3GPP TSG-RAN5 Meeting #103	R5-243273

# Fukuoka City, Fukuoka, Japan, 20th May 2024 - 24th May 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.579-1 | CR | 0360 | rev | - | Current version: | 16.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to clause 5.5.3.4.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NIST |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, MCenhUEConTest |  |  |  |  |  | Date: |  |  | 2024-05-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | There was a wrong Derivation Path in one of the tables |  |  |  |  |  |  |  |  |
| Summary of change: |  | Corrected the Derivation Path in Table 5.5.3.4.2-3 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A tester would not know where to go if there were questions about the elements defined in Table 5.5.3.4.2-3 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.5.3.4.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<START OF CHANGES>

##### 5.5.3.4.2	Location-info (Configuration sent by the SS)

##### …

##### -	MCData

Table 5.5.3.4.2-3: Location-info (Configuration sent by the SS) for MCData

| Derivation Path: TS 24.282 [87] clause D.4 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| location-info |  |  |  |  |
| Configuration |  |  |  |  |
| ConfigScope | "Full" | The MCData Client shall replace any previous configuration. |  |  |
| NonEmergencyLocationInformation |  |  |  |  |
| ServingEcgi | present | An optional element specifying that the serving E-UTRAN Cell Global Identity (ECGI) needs to be reported |  |  |
| NeighbouringEcgi | present | An optional element that can occur multiple times, specifying that neighbouring ECGIs need to be reported |  |  |
| MbmsSaId | present | An optional element specifying that the serving MBMS Service Area Id needs to be reported; |  |  |
| MbsfnArea | present | An optional element specifying that the MBSFN area Id needs to be reported; |  |  |
| GeographicalCoordinate | present | An optional element specifying that the geographical coordinate specified in clause 6.1 in 3GPP TS 23.032 [65] needs to be reported |  |  |
| minimumIntervalLength | "10" | A mandatory element specifying the minimum time the MCData client needs to wait between sending location reports. The value is given in seconds |  |  |
| EmergencyLocationInformation" |  |  |  |  |
| ServingEcgi | present | An optional element specifying that the serving E-UTRAN Cell Global Identity (ECGI) needs to be reported |  |  |
| NeighbouringEcgi | present | An optional element that can occur multiple times, specifying that neighbouring ECGIs need to be reported |  |  |
| MbmsSaId | present | An optional element specifying that the serving MBMS Service Area Id needs to be reported; |  |  |
| MbsfnArea | present | An optional element specifying that the MBSFN area Id needs to be reported; |  |  |
| GeographicalCoordinate | present | An optional element specifying that the geographical coordinate specified in clause 6.1 in 3GPP TS 23.032 [65] needs to be reported |  |  |
| minimumIntervalLength | "5" | A mandatory element specifying the minimum time the MCData client needs to wait between sending location reports. The value is given in seconds |  |  |
| TriggeringCriteria |  |  |  |  |
| CellChange | not present |  |  |  |
| TrackingAreaChange | not present |  |  |  |
| PlmnChange | not present |  |  |  |
| MbmsSaChange | not present |  |  |  |
| MbsfnAreaChange | not present |  |  |  |
| PeriodicReport | not present |  |  |  |
| TravelledDistance | not present |  |  |  |
| McdataSignallingEvent | not present |  |  |  |
| GeographicalAreaChange |  |  |  |  |
| AnyAreaChange | not present |  |  |  |
| EnterSpecificAreaType | not present |  |  |  |
| ExitSpecificAreaType | not present |  |  |  |

<END OF CHANGES>