3GPP TSG-RAN5 Meeting #103	R5-243348

# Fukuoka City, Fukuoka, Japan, 20th May 2024 - 24th May 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 4456 | rev | - | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrrection to RRC test case 8.2.3.6.2x |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2024-05-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The LTE OTA link is uncalibrated in the signalling test setup, and only serving cell power level is defined which is also a suggestive value. It is not possible to define multiple cell power levels for EUTRA cell for the purpose of performning measurements. Therefore these test casea can not be run in OTA test environment. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Table 8.2.3.6.2.3.2-2 is voided, also removed FR2 specific changes from the tables 8.2.3.6.2.3.3-11, 8.2.3.6.2a.3.3-2 and 8.2.3.6.2b.3.3-2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test specification remains unaligned with agreed test environment |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.3.6.2, 8.2.3.6.2a, 8.2.3.6.2b |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

##### 8.2.3.6.2	Measurement configuration control and reporting / Event A3 / Measurement of Neighbour E-UTRA and NR cells / Intra-frequency measurements / NE-DC

8.2.3.6.2.1	Test Purpose (TP)

(1)

with { UE in RRC_CONNECTED state in NE-DC, and, MCG(s) (NR PDCP) and SCG and intra-frequency measurements configured for event A3 }

ensure that {

when { Entry condition for event A3 is not met for neighbour E-UTRA cell }

then { UE does not send MeasurementReport }

}

(2)

with { UE in RRC_CONNECTED state in NE-DC, and, MCG(s) (NR PDCP) and SCG and intra-frequency measurements configured for event A3 }

ensure that {

when { Neighbour E-UTRA cell becomes offset better than serving E-UTRA PSCell }

then { UE sends MeasurementReport with correct measId for event A3 }

}

(3)

with { UE in RRC_CONNECTED state in NE-DC, and, MCG(s) (NR PDCP) and SCG and intra-frequency measurements configured for event A3 }

ensure that {

when { Neighbour NR cell becomes offset better than serving NR PCell }

then { UE sends MeasurementReport with correct measId for event A3 and includes measurement result for the PSCell }

}

8.2.3.6.2.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: 38.331:5.3.5.3, 5.5.5.1, 5.7.2a.3; TS 36.331:5.3.5.3, 5.5.2.3, 5.5.4.1, 5.5.4.4, 5.5.5.1. Unless otherwise stated these are Rel-15 requirements.

[TS 38.331, clause 5.3.5.3]

The UE shall perform the following actions upon reception of the RRCReconfiguration, or upon execution of the conditional reconfiguration (CHO or CPC):

…

1>	if the RRCReconfiguration includes the mrdc-SecondaryCellGroupConfig:

2>	if the mrdc-SecondaryCellGroupConfig is set to setup:

3>	if the mrdc-SecondaryCellGroupConfig includes mrdc-ReleaseAndAdd:

4>	perform MR-DC release as specified in clause 5.3.5.10;

3>	if the received mrdc-SecondaryCellGroup is set to nr-SCG:

4>	perform the RRC reconfiguration according to 5.3.5.3 for the RRCReconfiguration message included in nr-SCG;

3>	if the received mrdc-SecondaryCellGroup is set to eutra-SCG:

4>	perform the RRC connection reconfiguration as specified in TS 36.331 [10], clause 5.3.5.3 for the RRCConnectionReconfiguration message included in eutra-SCG;

2>	else (mrdc-SecondaryCellGroupConfig is set to release):

3>	perform MR-DC release as specified in clause 5.3.5.10;

…

1>	set the content of the RRCReconfigurationComplete message as follows:

…

2>	if the RRCReconfiguration message includes the mrdc-SecondaryCellGroupConfig with mrdc-SecondaryCellGroup set to eutra-SCG:

3>	include in the eutra-SCG-Response the E-UTRA RRCConnectionReconfigurationComplete message in accordance with TS 36.331 [10] clause 5.3.5.3;

…

[TS 38.331, clause 5.5.5.1]

Figure 5.5.5.1-1: Measurement reporting

For the measId for which the measurement reporting procedure was triggered, the UE shall set the measResults within the MeasurementReport message as follows:

1>	set the measId to the measurement identity that triggered the measurement reporting;

…

1>	if the reportConfig associated with the measId that triggered the measurement reporting is set to eventTriggered and eventID is set to eventA3, or eventA4, or eventA5, or eventB1, or eventB2:

2>	if the UE is in NE-DC and the measurement configuration that triggered this measurement report is associated with the MCG:

3>	set the measResultServFreqListEUTRA-SCG to include an entry for each E-UTRA SCG serving frequency with the following:

4>	include carrierFreq of the E-UTRA serving frequency;

4>	set the measResultServingCell to include the available measurement quantities that the UE is configured to measure by the measurement configuration associated with the SCG;

4>	if reportConfig associated with the measId that triggered the measurement reporting includes reportAddNeighMeas:

5>	set the measResultServFreqListEUTRA-SCG to include within measResultBestNeighCell the quantities of the best non-serving cell, based on RSRP, on the concerned serving frequency;

…

1>	if there is at least one applicable neighbouring cell to report:

2>	if the reportType is set to eventTriggered or periodical:

3>	set the measResultNeighCells to include the best neighbouring cells up to maxReportCells in accordance with the following:

4>	if the reportType is set to eventTriggered:

5>	include the cells included in the cellsTriggeredList as defined within the VarMeasReportList for this measId;

4>	else:

5>	include the applicable cells for which the new measurement results became available since the last periodical reporting or since the measurement was initiated or reset;

4>	for each cell that is included in the measResultNeighCells, include the physCellId;

4>	if the reportType is set to eventTriggered or periodical:

5>	for each included cell, include the layer 3 filtered measured results in accordance with the reportConfig for this measId, ordered as follows:

6>	if the measObject associated with this measId concerns NR:

7>	if rsType in the associated reportConfig is set to ssb:

8>	set resultsSSB-Cell within the measResult to include the SS/PBCH block based quantity(ies) indicated in the reportQuantityCell within the concerned reportConfig, in decreasing order of the sorting quantity, determined as specified in 5.5.5.3, i.e. the best cell is included first;

8>	if reportQuantityRS-Indexes and maxNrofRS-IndexesToReport are configured, include beam measurement information as described in 5.5.5.2;

7>	else if rsType in the associated reportConfig is set to csi-rs:

8>	set resultsCSI-RS-Cell within the measResult to include the CSI-RS based quantity(ies) indicated in the reportQuantityCell within the concerned reportConfig, in decreasing order of the sorting quantity, determined as specified in 5.5.5.3, i.e. the best cell is included first;

8>	if reportQuantityRS-Indexes and maxNrofRS-IndexesToReport are configured, include beam measurement information as described in 5.5.5.2;

6>	if the measObject associated with this measId concerns E-UTRA:

7>	set the measResult to include the quantity(ies) indicated in the reportQuantity within the concerned reportConfigInterRAT in decreasing order of the sorting quantity, determined as specified in 5.5.5.3, i.e. the best cell is included first;

6>	if the measObject associated with this measId concerns UTRA-FDD and if ReportConfigInterRAT includes the reportQuantityUTRA-FDD:

7>	set the measResult to include the quantity(ies) indicated in the reportQuantityUTRA-FDD within the concerned reportConfigInterRAT in decreasing order of the sorting quantity, determined as specified in 5.5.5.3, i.e. the best cell is included first;

2>	else:

…

[TS 38.331, clause 5.7.2a.3]

The UE shall set the contents of the ULInformationTransferMRDC message as follows:

1>	if there is a need to transfer MR-DC dedicated information related to NR:

2>	set the ul-DCCH-MessageNR to include the NR MR-DC dedicated information to be transferred (e.g., NR RRC MeasurementReport, UEAssistanceInformation, FailureInformation, RRCReconfigurationComplete or MCGFailureInformation message);

1>	else if there is a need to transfer MR-DC dedicated information related to E-UTRA:

2>	set the - to include the E-UTRA MR-DC dedicated information to be transferred (e.g., E-UTRA RRC MeasurementReport, or MCGFailureInformation message);

1>	submit the ULInformationTransferMRDC message to lower layers for transmission, upon which the procedure ends.

[TS 36.331, clause 5.3.5.3]

If the RRCConnectionReconfiguration message does not include the mobilityControlInfo and the UE is able to comply with the configuration included in this message, the UE shall:

…

1>	if the received RRCConnectionReconfiguration includes the sCellGroupToAddModList:

2>	perform SCell group addition or modification as specified in 5.3.10.3e;

1>	if the received RRCConnectionReconfiguration includes the scg-Configuration; or

1>	if the current UE configuration includes one or more split DRBs configured with pdcp-Config and the received RRCConnectionReconfiguration includes radioResourceConfigDedicated including drb-ToAddModList:

2>	perform SCG reconfiguration as specified in 5.3.10.10;

…

1>	if the RRCConnectionReconfiguration message includes the measConfig:

2>	perform the measurement configuration procedure as specified in 5.5.2;

…

1>	if the UE is configured with NE-DC:

2>	if the received RRCConnectionReconfiguration message was included in an NR RRCResume message:

3>	transfer the RRCConnectionReconfigurationComplete message via SRB1 embedded in NR RRC message RRCResumeComplete as specified in TS 38.331 [82], clause 5.3.13.4;

2>	else:

3>	transfer the RRCConnectionReconfigurationComplete message via SRB1 embedded in NR RRC message RRCReconfigurationComplete as specified in TS 38.331 [82], clause 5.3.5.3;

1>	else:

2>	submit the RRCConnectionReconfigurationComplete message to lower layers for transmission using the new configuration, upon which the procedure ends;

[TS 36.331, clause 5.5.2.3]

E-UTRAN applies the procedure as follows:

-	configure a measId only if the corresponding measurement object, the corresponding reporting configuration and the corresponding quantity configuration, are configured;

The UE shall:

1>	for each measId included in the received measIdToAddModList:

2>	if an entry with the matching measId exists in the measIdList within the VarMeasConfig:

3>	replace the entry with the value received for this measId;

2>	else:

3>	add a new entry for this measId within the VarMeasConfig;

2>	remove the measurement reporting entry for this measId from the VarMeasReportList, if included;

2>	stop the periodical reporting timer or timer T321, whichever one is running, and reset the associated information (e.g. timeToTrigger) for this measId;

…

[TS 36.331, clause 5.5.4.1]

If security has been activated successfully, the UE shall:

1>	for each measId included in the measIdList within VarMeasConfig:

…

2>	else:

3>	if the corresponding measObject concerns E-UTRA:

4>	if the ue-RxTxTimeDiffPeriodical is configured in the corresponding reportConfig:

5>	consider only the PCell to be applicable;

4>	else if the reportSSTD-Meas is set to true in the corresponding reportConfig:

5>	consider the PSCell to be applicable;

4>	else if the eventA1 or eventA2 is configured in the corresponding reportConfig:

5>	consider only the serving cell to be applicable;

4>	else if eventC1 or eventC2 is configured in the corresponding reportConfig; or if reportStrongestCSI-RSs is set to true in the corresponding reportConfig:

5>	consider a CSI-RS resource on the associated frequency to be applicable when the concerned CSI-RS resource is included in the measCSI-RS-ToAddModList defined within the VarMeasConfig for this measId;

4>	else if measRSSI-ReportConfig is configured in the corresponding reportConfig:

5>	consider the resource indicated by the rmtc-Config on the associated frequency to be applicable;

4>	else:

5>	if useAllowedCellList is set to TRUE:

6>	consider any neighbouring cell detected on the associated frequency to be applicable when the concerned cell is included in the allowedCellsToAddModList defined within the VarMeasConfig for this measId;

5>	else:

6>	consider any neighbouring cell detected on the associated frequency to be applicable when the concerned cell is not included in the excludedCellsToAddModList defined within the VarMeasConfig for this measId;

5>	for events involving a serving cell on one frequency and neighbours on another frequency, consider the serving cell on the other frequency as a neighbouring cell;

4>	if the corresponding reportConfig includes alternativeTimeToTrigger and if the UE supports alternativeTimeToTrigger:

5>	use the value of alternativeTimeToTrigger as the time to trigger instead of the value of timeToTrigger in the corresponding reportConfig for cells included in the altTTT-CellsToAddModList of the corresponding measObject;

…

[TS 36.331, clause 5.5.4.4]

The UE shall:

1>	consider the entering condition for this event to be satisfied when condition A3-1, as specified below, is fulfilled;

1>	consider the leaving condition for this event to be satisfied when condition A3-2, as specified below, is fulfilled;

1>	if usePSCell of the corresponding reportConfig is set to true:

2>	use the PSCell for Mp, Ofp and Ocp;

1>	else:

2>	use the PCell for Mp, Ofp and Ocp;

NOTE	The cell(s) that triggers the event is on the frequency indicated in the associated measObject which may be different from the frequency used by the PCell/ PSCell.

Inequality A3-1 (Entering condition)

Inequality A3-2 (Leaving condition)

The variables in the formula are defined as follows:

Mn is the measurement result of the neighbouring cell, not taking into account any offsets.

Ofn is the frequency specific offset of the frequency of the neighbour cell (i.e. offsetFreq as defined within measObjectEUTRA corresponding to the frequency of the neighbour cell).

Ocn is the cell specific offset of the neighbour cell (i.e. cellIndividualOffset as defined within measObjectEUTRA corresponding to the frequency of the neighbour cell), and set to zero if not configured for the neighbour cell.

Mp is the measurement result of the PCell/ PSCell, not taking into account any offsets.

Ofp is the frequency specific offset of the frequency of the PCell/ PSCell (i.e. offsetFreq as defined within measObjectEUTRA corresponding to the frequency of the PCell/ PSCell).

Ocp is the cell specific offset of the PCell/ PSCell (i.e. cellIndividualOffset as defined within measObjectEUTRA corresponding to the frequency of the PCell/ PSCell), and is set to zero if not configured for the PCell/ PSCell.

Hys is the hysteresis parameter for this event (i.e. hysteresis as defined within reportConfigEUTRA for this event).

Off is the offset parameter for this event (i.e. a3-Offset as defined within reportConfigEUTRA for this event).

Mn, Mp are expressed in dBm in case of RSRP, or in dB in case of RSRQ and RS-SINR.

Ofn, Ocn, Ofp, Ocp, Hys, Off are expressed in dB.

[TS 36.331, clause 5.5.5.1]

For the measId for which the measurement reporting procedure was triggered, the UE shall set the measResults within the MeasurementReport message as follows:

…

1>	if the measurement configuration that triggered the measurement reporting procedure was configured by an sl-ConfigDedicatedEUTRA that was received within an NR RRCReconfiguration message:

2>	submit the MeasurementReport message via SRB1 embedded in NR RRC message ULInformationTransferIRAT as specified in TS 38.331 [82].

1>	else if the UE is configured with NE-DC:

2>	submit the MeasurementReport message via SRB1 embedded in NR RRC message ULInformationTransferMRDC as specified in TS 38.331 [82].

1>	else:

2>	submit the MeasurementReport message to lower layers for transmission, upon which the procedure ends;

8.2.3.6.2.3	Test description

8.2.3.6.2.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1 is the PCell and EUTRA Cell 1 is the PS Cell.

-	NR Cell 2 is the intra-frequency neighbour NR cell.

-	EUTRA Cell 2 is the intra-frequency neighbour EUTRA cell.

-	System information combination NR-6 as defined in TS 38.508-1 [4] clause 4.4.3.1.2 is used in NR cells.

UE:

-	None

Preamble:

-	The UE is in state NE-DC RRC_CONNECTED using generic procedure parameter Connectivity (NE-DC) according to TS 38.508-1 [4], Table 4.5.4.2-6.

8.2.3.6.2.3.2	Test procedure sequence

Table 8.2.3.6.2.3.2-1 illustrates the downlink power levels to be applied for NR Cells and EUTRA Cells at various time instants of the test execution. Row marked "T0" denotes the conditions after the preamble, while row marked "T1" are to be applied subsequently. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 8.2.3.6.2.3.2-1: Time instances of cell power level and parameter changes for FR1

|  | Parameter | Unit | E-UTRA Cell 1 | E-UTRA Cell 2 | NR Cell 1 | NR Cell 2 | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- |
| T0 | Cell-specific RS EPRE | dBm/15kHz | -85 | -91 | - | - | Power levels are such that entry condition for event A3 is not satisfied for any of the neighbour E-UTRA cells: Mn + Ofn + Ocn + Hys < Mp + Ofp + Ocp + Off |
|  | SS/PBCH SSS EPRE | dBm/SCS | - | - | -85 | -91 |  |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -85 | -79 | - | - | Power levels are such that entry condition for event A3 is satisfied for intra-frequency neighbour E-UTRA cell (measId 1): Mn + Ofn + Ocn – Hys > Mp + Ofp + Ocp + Off |
|  | SS/PBCH SSS EPRE | dBm/SCS | - |  | -85 | -91 |  |
| T2 | Cell-specific RS EPRE | dBm/15kHz | -85 | -91 | - | - | Power levels are such that entry condition for event A3 is satisfied for intra-frequency neighbour NR cell (measId 2): Mn + Ofn + Ocn – Hys > Mp + Ofp + Ocp + Off |
|  | SS/PBCH SSS EPRE | dBm/SCS | - | - | -85 | -79 |  |

Table 8.2.3.6.2.3.2-2: Void

Table 8.2.3.6.2.3.2-3: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits an RRCReconfiguration message containing RRCConnectionReconfiguration message to setup measurements for neighbour E-UTRA Cells and reporting for event A3 | <-- | NR RRC: RRCReconfiguration (RRCConnectionReconfiguration) | - | - |
| 2 | The UE transmit an RRCReconfigurationComplete message. | --> | NR RRC: RRCReconfigurationComplete (RRCConnectionReconfigurationComplete) | - | - |
| 3 | Check: Does the UE transmit an ULInformationTransferMRDC message containing MeasurementReport message within the next 10s to report event A3? | --> | NR RRC: ULInformationTransferMRDC (MeasurementReport) | 1 | F |
| 4 | The SS re-adjusts the cell-specific reference signal level according to row "T1". | - | - | - | - |
| 5 | Check: Does the UE transmit an ULInformationTransferMRDC message containing MeasurementReport message to report event A3 (measId 1) with the measured value for E-UTRA Cell 2? | --> | NR RRC: ULInformationTransferMRDC (MeasurementReport) | 2 | P |
| 6 | SS re-adjusts the cell-specific reference signal level according to row "T0". | - | - | - | - |
| 7 | The SS transmits an RRCReconfiguration message including MeasConfig to setup NR measurement and reporting for intra-frequency event A3 (measId 2) and reportAddNeighMeas. | <-- | NR RRC: RRCReconfiguration | - | - |
| 8 | The UE transmit an RRCReconfigurationComplete message. | --> | NR RRC: RRCReconfigurationComplete | - | - |
| 9 | SS re-adjusts the cell-specific reference signal level according to row "T2". | - | - | - | - |
| 10 | Check: Does the UE transmit a MeasurementReport message to report event A3 (measId 2) with the measured RSRP value for NR Cell 2, E-UTRA PSCell and E-UTRA neighbour Cell 2. | --> | NR RRC: MeasurementReport | 3 | P |

8.2.3.6.2.3.3	Specific message contents

Table 8.2.3.6.2.3.3-1: RRCReconfiguration (step 1, Table 8.2.3.6.2.3.2-3)

Derivation Path: TS 38.508-1 [4], Table 4.6.1-13 with condition NE-DC

Table 8.2.3.6.2.3.3-2: SCG-Configuration-r12-NE-DC (step 1, Table 8.2.3.6.2.3.2-3)

| Derivation Path: 36.508, Clause 4.6.3, Table 4.6.3-19G |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| scg-Configuration-r12 CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| scg-ConfigPartSCG-r12 SEQUENCE { |  |  |  |
| measConfigSN-r15 | MeasConfig-A3-E-UTRA |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.3.6.2.3.3-3: MeasConfig-A3-E-UTRA (Table 8.2.3.6.2.3.3-2)

| Derivation path: 36.508 clause 4.6.6 table 4.6.6-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| measConfig ::= SEQUENCE { |  |  |  |
| measObjectToAddModList SEQUENCE (SIZE (1..maxObjectId)) OF SEQUENCE { | 1 entry |  |  |
| measObjectId[1] | 1 |  |  |
| measObject[1] | MeasObjectEUTRA-GENERIC(f1) |  |  |
| measObject[1] | MeasObjectEUTRA-GENERIC(maxEARFCN) |  | Band > 64 |
| } |  |  |  |
| reportConfigToAddModList SEQUENCE (SIZE (1..maxReportConfigId)) OF SEQUENCE { | 1 entry |  |  |
| reportConfigId[1] | 1 |  |  |
| reportConfig[1] | ReportConfig-A3-H |  |  |
| } |  |  |  |
| measIdToAddModList SEQUENCE (SIZE (1..maxMeasId)) OF SEQUENCE { | 1 entry |  |  |
| measId[1] | 1 |  |  |
| measObjectId[1] | 1 |  |  |
| reportConfigId[1] | 1 |  |  |
| } |  |  |  |
| measObjectToAddModList-v9e0 ::= SEQUENCE (SIZE (1..maxObjectId)) OF SEQUENCE { | 1 entry |  | Band > 64 |
| measObjectEUTRA-v9e0[1] SEQUENCE { |  |  |  |
| carrierFreq-v9e0 | Same downlink EARFCN as used for f1 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.3.6.2.3.3-4: ReportConfig-A3-H (step 1, Table 8.2.3.6.2.3.2-3)

| Derivation path: 36.508 clause 4.6.6 table 4.6.6-6 ReportConfigEUTRA-A3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportConfigEUTRA-A3 ::= SEQUENCE { |  |  |  |
| triggerType CHOICE { |  |  |  |
| event SEQUENCE { |  |  |  |
| timeToTrigger | ms0 |  |  |
| } |  |  |  |
| } |  |  |  |
| reportQuantity | sameAsTriggerQuantity |  |  |
| } |  |  |  |

Table 8.2.3.6.2.3.3-5: RRCReconfigurationComplete (step 2, step 8, Table 8.2.3.6.2.3.2-3)

Derivation Path: TS 38.508-1 [4], Table 4.6.1-14 Condition with NE-DC

Table 8.2.3.6.2.3.3-6: ULInformationTransferMRDC (step 5, Table 8.2.3.6.2.3.2-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-34 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ULInformationTransferMRDC ::= SEQUENCE { |  |  |  |
| ul-DCCH-MessageEUTRA | OCTET STRING including the MeasurementReport message according to Table 8.2.3.6.2.3.3-7 |  |  |
| } |  |  |  |

Table 8.2.3.6.2.3.3-7: MeasurementReport (Table 8.2.3.6.2.3.3-6)

| Derivation path: 36.508 4.6.1 table 4.6.1-5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| MeasurementReport ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| measurementReport-r8 SEQUENCE { |  |  |  |
| measResults ::= SEQUENCE { |  |  |  |
| measId | 1 |  |  |
| measResultPCell::= SEQUENCE { |  | Report E-UTRA Cell 1 |  |
| rsrpResult | (0..97) |  |  |
| rsrqResult | (0..34) |  |  |
| } |  |  |  |
| measResultNeighCells CHOICE { |  |  |  |
| measResultListEUTRA SEQUENCE (SIZE (1..maxCellReport)) OF SEQUENCE { | 1 entry | Report E-UTRA Cell 2 |  |
| physCellId | physCellId of the E-UTRA Cell 2. |  |  |
| measResult SEQUENCE{ |  |  |  |
| rsrpResult | (0..97) |  |  |
| rsrqResult | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| measResultListUTRA | Not present |  |  |
| measResultListGERAN | Not present |  |  |
| measResultsCDMA2000 | Not present |  |  |
| measResultNeighCellListNR-r15 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.3.6.2.3.3-8: RRCReconfiguration (step 7, Table 8.2.3.6.2.3.2-3)

Derivation Path: TS 38.508-1 [4] Table 4.6.1-13 with condition NR_MEAS

Table 8.2.3.6.2.3.3-9: MeasConfig-A3 (Table 8.2.3.6.2.3.3-8)

| Derivation Path: TS 38.508-1 [4] Table 4.6.3-69 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasConfig ::= SEQUENCE { |  |  |  |
| measObjectToAddModList SEQUENCE (SIZE (1..maxNrofMeasId)) OF MeasObjectToAddMod { | 1 entry |  |  |
| MeasObjectToAddMod[1] SEQUENCE { |  | entry 1 |  |
| measObjectId | 1 |  |  |
| measObject CHOICE { |  |  |  |
| measObjectNR | MeasObjectNR |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| reportConfigToAddModList SEQUENCE(SIZE (1..maxReportConfigId)) OF ReportConfigToAddMod { | 1 entry |  |  |
| ReportConfigToAddMod[1] SEQUENCE { |  | entry 1 |  |
| reportConfigId | 1 |  |  |
| reportConfig CHOICE { |  |  |  |
| reportConfigNR | ReportConfigNR-EventA3 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| measIdToAddModList SEQUENCE (SIZE (1..maxNrofMeasId)) OF MeasIdToAddMod { | 1 entry |  |  |
| MeasIdToAddMod[1] SEQUENCE { |  | entry 1 |  |
| measId | 2 |  |  |
| measObjectId | 1 |  |  |
| reportConfigId | 1 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.3.6.2.3.3-10: MeasObjectNR (Table 8.2.3.6.2.3.3-9)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-76 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasObjectNR::= SEQUENCE { |  |  |  |
| ssbFrequency | ARFCN-ValueNR for SSB of NR Cell 2 |  |  |
| absThreshSS-BlocksConsolidation | Not present |  |  |
| nrofSS-BlocksToAverage | Not present |  |  |
| } |  |  |  |

Table 8.2.3.6.2.3.3-11: ReportConfigNR-EventA3 (Table 8.2.3.6.2.3.3-9)

| Derivation Path: TS 38.508-1 [4] Table 4.6.3-142 with condition EVENT_A3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportConfigNR ::= SEQUENCE { |  |  |  |
| reportType CHOICE { |  |  |  |
| eventTriggered SEQUENCE { |  |  |  |
| eventId CHOICE { |  |  |  |
| eventA3 SEQUENCE { |  |  |  |
| a3-Offset CHOICE { |  |  |  |
| rsrp | 2 | 1 dB (2*0.5 dB) |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| reportAmount | 1 |  |  |
| reportQuantityCell SEQUENCE { |  |  |  |
| rsrp | true |  |  |
| rsrq | false |  |  |
| sinr | false |  |  |
| } |  |  |  |
| reportAddNeighMeas | setup |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.3.6.2.3.3-12: MeasurementReport (step 10, Table 8.2.3.6.2.3.2-3)

| Derivation Path: TS 38.508-1 [4] Table 4.6.1-5A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasurementReport ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| measurementReport SEQUENCE { |  |  |  |
| measResults SEQUENCE { |  |  |  |
| measId | 2 |  |  |
| measResultServingMOList SEQUENCE (SIZE (1..maxNrofServingCells)) OF MeasResultServMO { | 1 entry | Measurement report for NR Cell 1 |  |
| MeasResultServMO[1] SEQUENCE { |  | entry 1 |  |
| servCellId | ServCellIndex of NR Cell 1 |  |  |
| measResultServingCell SEQUENCE { |  |  |  |
| physCellId | PCI of NR Cell 1 |  |  |
| measResult SEQUENCE { |  |  |  |
| cellResults SEQUENCE { |  |  |  |
| resultsSSB-Cell SEQUENCE { |  |  |  |
| rsrp | (0..127) |  |  |
| rsrq | (0..127) |  |  |
| sinr | Not present |  |  |
|  | Not checked |  | pc_ss_SINR_Meas |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| measResultNeighCells CHOICE { |  |  |  |
| measResultListNR SEQUENCE (SIZE (1.. maxCellReport)) OF MeasResultNR { | 1 entry | Measurement report for NR Cell 2 |  |
| MeasResultNR[1] SEQUENCE { |  | entry 1 |  |
| physCellId | Physical layer cell identity of NR Cell 2 |  |  |
| measResult SEQUENCE { |  |  |  |
| cellResults SEQUENCE { |  |  |  |
| resultsSSB-Cell SEQUENCE { |  |  |  |
| rsrp | (0..127) |  |  |
| rsrq | Not present |  |  |
| sinr | Not present |  |  |
| } |  |  |  |
| resultsCSI-RS-Cell | Not present |  |  |
| } |  |  |  |
| rsIndexResults | Not present |  |  |
| } |  |  |  |
| cgi-Info | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| measResultServFreqListEUTRA-SCG SEQUENCE (SIZE (1..maxNrofServingCellsEUTRA)) OF MeasResult2EUTRA { | 1 entry |  |  |
| MeasResult2EUTRA[1] SEQUENCE { |  | entry 1 |  |
| carrierFreq | ARFCN-Value for E-UTRA Cell 1 |  |  |
| measResultServingCell SEQUENCE { |  |  |  |
| eutra-PhysCellId | PhysCellId of E-UTRA Cell 1 |  |  |
| measResult SEQUENCE { |  |  |  |
| rsrp | (0..97) |  |  |
| rsrq | Not present |  |  |
| sinr | Not present |  |  |
| } |  |  |  |
| cgi-Info | Not present |  |  |
| } |  |  |  |
| measResultBestNeighCell SEQUENCE { |  |  |  |
| eutra-PhysCellId | PhysCellId of E-UTRA Cell 2 |  |  |
| measResult SEQUENCE { |  |  |  |
| rsrp | (0..97) |  |  |
| rsrq | Not present |  |  |
| sinr | Not present |  |  |
| } |  |  |  |
| cgi-Info | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

##### 8.2.3.6.2a	Measurement configuration control and reporting / Event A3 / Measurement of Neighbor E-UTRA and NR cell / Inter-frequency measurements / NE-DC

8.2.3.6.2a.1	Test Purpose (TP)

(1)

with { UE in RRC_CONNECTED state in NE-DC, and, MCG(s) (NR PDCP) and SCG and inter-frequency measurements configured for event A3 }

ensure that {

when { Entry condition for event A3 is not met for neighbour E-UTRA cell }

then { UE does not send MeasurementReport }

}

(2)

with { UE in RRC_CONNECTED state in NE-DC, and, MCG(s) (NR PDCP) and SCG and inter-frequency measurements configured for event A3 }

ensure that {

when { Neighbour E-UTRA cell becomes offset better than serving E-UTRA PSCell }

then { UE sends MeasurementReport with correct measId for event A3 }

}

(3)

with { UE in RRC_CONNECTED state in NE-DC, and, MCG(s) (NR PDCP) and SCG and inter-frequency measurements configured for event A3 }

ensure that {

when { Neighbour NR cell becomes offset better than serving NR PCell }

then { UE sends MeasurementReport with correct measId for event A3 and includes measurement result for PScell }

}

8.2.3.6.2a.2	Conformance requirements

Same as test case 8.2.3.6.2.2 with the following difference:

[TS 38.331, clause 5.5.2.9]

The UE shall:

1>	if gapFR1 is set to setup:

2>	if an FR1 measurement gap configuration is already setup, release the FR1 measurement gap configuration;

2>	setup the FR1 measurement gap configuration indicated by the measGapConfig in accordance with the received gapOffset, i.e., the first subframe of each gap occurs at an SFN and subframe meeting the following condition:

SFN mod T = FLOOR(gapOffset/10);

subframe = gapOffset mod 10;

with T = MGRP/10 as defined in TS 38.133 [14];

2>	apply the specified timing advance mgta to the gap occurrences calculated above (i.e. the UE starts the measurement mgta ms before the gap subframe occurrences);

1>	else if gapFR1 is set to release:

2>	release the FR1 measurement gap configuration;

1>	if gapFR2 is set to setup:

2>	if an FR2 measurement gap configuration is already setup, release the FR2 measurement gap configuration;

2>	setup the FR2 measurement gap configuration indicated by the measGapConfig in accordance with the received gapOffset, i.e., the first subframe of each gap occurs at an SFN and subframe meeting the following condition:

SFN mod T = FLOOR(gapOffset/10);

subframe = gapOffset mod 10;

with T = MGRP/10 as defined in TS 38.133 [14];

2>	apply the specified timing advance mgta to the gap occurrences calculated above (i.e. the UE starts the measurement mgta ms before the gap subframe occurrences);

1>	else if gapFR2 is set to release:

2>	release the FR2 measurement gap configuration;

1>	if gapUE is set to setup:

2>	if a per UE measurement gap configuration is already setup, release the per UE measurement gap configuration;

2>	setup the per UE measurement gap configuration indicated by the measGapConfig in accordance with the received gapOffset, i.e., the first subframe of each gap occurs at an SFN and subframe meeting the following condition:

SFN mod T = FLOOR(gapOffset/10);

subframe = gapOffset mod 10;

with T = MGRP/10 as defined in TS 38.133 [14];

2>	apply the specified timing advance mgta to the gap occurrences calculated above (i.e. the UE starts the measurement mgta ms before the gap subframe occurrences);

1>	else if gapUE is set to release:

2>	release the per UE measurement gap configuration.

NOTE 1:	For gapFR2 configuration with synchronous CA, for the UE in NE-DC or NR-DC, the SFN and subframe of the serving cell indicated by the refServCellIndicator in gapFR2 is used in the gap calculation. Otherwise, the SFN and subframe of a serving cell on FR2 frequency is used in the gap calculation

NOTE 2:	For gapFR1 or gapUE configuration, for the UE in NE-DC or NR-DC, the SFN and subframe of the serving cell indicated by the refServCellIndicator in corresponding gapFR1 or gapUE is used in the gap calculation. Otherwise, the SFN and subframe of the PCell is used in the gap calculation.

NOTE 3:	For gapFR2 configuration with asynchronous CA, for the UE in NE-DC or NR-DC, the SFN and subframe of the serving cell indicated by the refServCellIndicator and refFR2ServCellAsyncCA in gapFR2 is used in the gap calculation. Otherwise, the SFN and subframe of a serving cell on FR2 frequency indicated by the refFR2ServCellAsyncCA in gapFR2 is used in the gap calculation

8.2.3.6.2a.3	Test description

8.2.3.6.2a.3.1	Pre-test conditions

Same as test case 8.2.3.6.2 with the following differences:

-	Cells configuration: NR Cell 3 replaces NR Cell 2; EUTRA Cell 3 replaces EUTRA Cell 2.

8.2.3.6.2a.3.2	Test procedure sequence

Same as test case 8.2.3.6.2 with the following differences:

-	Cells configuration: NR Cell 3 replaces NR Cell 2; EUTRA Cell 3 replaces EUTRA Cell 2.

8.2.3.6.2a.3.3	Specific message contents

Same as test case 8.2.3.6.2 with the following differences:

-	Cells configuration: Cells configuration: NR Cell 3 replaces NR Cell 2; EUTRA Cell 3 replaces EUTRA Cell 2.

Table 8.2.3.6.2a.3.3-1: MeasConfig-A3-E-UTRA (Table 8.2.3.6.2.3.3-3)

| Derivation path: 36.508 clause 4.6.6 table 4.6.6-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| measConfig ::= SEQUENCE { |  |  |  |
| measObjectToAddModList SEQUENCE (SIZE (1..maxObjectId)) OF SEQUENCE { | 2 entries |  |  |
| measObjectId[1] | 1 |  |  |
| measObject[1] | MeasObjectEUTRA-GENERIC(f1) |  |  |
| measObject[1] | MeasObjectEUTRA-GENERIC(maxEARFCN) |  | Band > 64 |
| measObjectId[2] | 2 |  |  |
| measObject[2] | MeasObjectEUTRA-GENERIC(f2) |  |  |
| measObject[2] | MeasObjectEUTRA-GENERIC(maxEARFCN) |  | Band > 64 |
| } |  |  |  |
| reportConfigToAddModList SEQUENCE (SIZE (1..maxReportConfigId)) OF SEQUENCE { | 1 entry |  |  |
| reportConfigId[1] | 1 |  |  |
| reportConfig[1] | ReportConfig-A3-H |  |  |
| } |  |  |  |
| measIdToAddModList SEQUENCE (SIZE (1..maxMeasId)) OF SEQUENCE { | 1 entry |  |  |
| measId[1] | 1 |  |  |
| measObjectId[1] | 2 |  |  |
| reportConfigId[1] | 1 |  |  |
| } |  |  |  |
| measGapConfig | MeasGapConfig-GP1 |  | INTER-FREQ |
|  | MeasGapConfig-CE |  | intraFreq-CE-NeedForGaps |
| measObjectToAddModList-v9e0 ::= SEQUENCE (SIZE (1..maxObjectId)) OF SEQUENCE { | 2 entries |  | Band > 64 |
| measObjectEUTRA-v9e0[1] SEQUENCE { |  |  |  |
| carrierFreq-v9e0 | Same downlink EARFCN as used for f1 |  |  |
| } |  |  |  |
| measObjectEUTRA-v9e0[2] SEQUENCE { |  |  |  |
| carrierFreq-v9e0 | Same downlink EARFCN as used for f2 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.3.6.2a.3.3-2: MeasConfig-A3 (Table 8.2.3.6.2.3.3-9)

| Derivation Path: TS 38.508-1 [4] Table 4.6.3-69 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasConfig ::= SEQUENCE { |  |  |  |
| measObjectToAddModList SEQUENCE (SIZE (1..maxNrofMeasId)) OF MeasObjectToAddMod { | 2 entries |  |  |
| MeasObjectToAddMod[1] SEQUENCE { |  | entry 1 |  |
| measObjectId | 1 |  |  |
| measObject CHOICE { |  |  |  |
| measObjectNR | MeasObjectNR-f1 |  |  |
| } |  |  |  |
| } |  |  |  |
| MeasObjectToAddMod[2] SEQUENCE { |  | entry 2 |  |
| measObjectId | 2 |  |  |
| measObject CHOICE { |  |  |  |
| measObjectNR | MeasObjectNR-f2 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| reportConfigToAddModList SEQUENCE(SIZE (1..maxReportConfigId)) OF ReportConfigToAddMod { | 1 entry |  |  |
| ReportConfigToAddMod[1] SEQUENCE { |  | entry 1 |  |
| reportConfigId | 1 |  |  |
| reportConfig CHOICE { |  |  |  |
| reportConfigNR | ReportConfigNR(2) Same as TS 38.508-1 Table 4.6.3-142 except for reportAmount set to ‘r1’ and reportAddNeighMeas is set to ‘setup’ | a3-Offset Thresh value set to 1 dB (2*0.5 dB) |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| measIdToAddModList SEQUENCE (SIZE (1..maxNrofMeasId)) OF MeasIdToAddMod { | 1 entry |  |  |
| MeasIdToAddMod[1] SEQUENCE { |  | entry 1 |  |
| measId | 2 |  |  |
| measObjectId | 2 |  |  |
| reportConfigId | 1 |  |  |
| } |  |  |  |
| } |  |  |  |
| measGapConfig | MeasGapConfig |  |  |
| } |  |  |  |

Table 8.2.3.6.2a.3.3-3: MeasObjectNR-f1 (Table 8.2.3.6.2a.3.3-2)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-76 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasObjectNR::= SEQUENCE { |  |  |  |
| ssbFrequency | ARFCN-ValueNR for SSB of NR Cell 1 |  |  |
| absThreshSS-BlocksConsolidation | Not present |  |  |
| nrofSS-BlocksToAverage | Not present |  |  |
| } |  |  |  |

Table 8.2.3.6.2a.3.3-4: MeasObjectNR-f2 (Table 8.2.3.6.2a.3.3-2)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-76 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasObjectNR::= SEQUENCE { |  |  |  |
| ssbFrequency | ARFCN-ValueNR for SSB of NR Cell 3 |  |  |
| absThreshSS-BlocksConsolidation | Not present |  |  |
| nrofSS-BlocksToAverage | Not present |  |  |
| } |  |  |  |

##### 8.2.3.6.2b	Measurement configuration control and reporting / Event A3 / Measurement of Neighbor E-UTRA and NR cell / Inter-band measurements / NE-DC

8.2.3.6.2b.1	Test Purpose (TP)

(1)

with { UE in RRC_CONNECTED state in NE-DC, and, MCG(s) (NR PDCP) and SCG and inter-band measurements configured for event A3 }

ensure that {

when { Entry condition for event A3 is not met for neighbour E-UTRA cell }

then { UE does not send MeasurementReport }

}

(2)

with { UE in RRC_CONNECTED state in NE-DC, and, MCG(s) (NR PDCP) and SCG and inter-band measurements configured for event A3 }

ensure that {

when { Neighbour E-UTRA cell becomes offset better than serving E-UTRA PSCell }

then { UE sends MeasurementReport with correct measId for event A3 }

}

(3)

with { UE in RRC_CONNECTED state in NE-DC, and, MCG(s) (NR PDCP) and SCG and inter-band measurements configured for event A3 }

ensure that {

when { Neighbour NR cell becomes offset better than serving NR PCell }

then { UE sends MeasurementReport with correct measId for event A3 and includes measurement result for PScell }

}

8.2.3.6.2b.2	Conformance requirements

Same as test case 8.2.3.6.2.2 with the following difference:

[TS 38.331, clause 5.5.2.9]

The UE shall:

1>	if gapFR1 is set to setup:

2>	if an FR1 measurement gap configuration is already setup, release the FR1 measurement gap configuration;

2>	setup the FR1 measurement gap configuration indicated by the measGapConfig in accordance with the received gapOffset, i.e., the first subframe of each gap occurs at an SFN and subframe meeting the following condition:

SFN mod T = FLOOR(gapOffset/10);

subframe = gapOffset mod 10;

with T = MGRP/10 as defined in TS 38.133 [14];

2>	apply the specified timing advance mgta to the gap occurrences calculated above (i.e. the UE starts the measurement mgta ms before the gap subframe occurrences);

1>	else if gapFR1 is set to release:

2>	release the FR1 measurement gap configuration;

1>	if gapFR2 is set to setup:

2>	if an FR2 measurement gap configuration is already setup, release the FR2 measurement gap configuration;

2>	setup the FR2 measurement gap configuration indicated by the measGapConfig in accordance with the received gapOffset, i.e., the first subframe of each gap occurs at an SFN and subframe meeting the following condition:

SFN mod T = FLOOR(gapOffset/10);

subframe = gapOffset mod 10;

with T = MGRP/10 as defined in TS 38.133 [14];

2>	apply the specified timing advance mgta to the gap occurrences calculated above (i.e. the UE starts the measurement mgta ms before the gap subframe occurrences);

1>	else if gapFR2 is set to release:

2>	release the FR2 measurement gap configuration;

1>	if gapUE is set to setup:

2>	if a per UE measurement gap configuration is already setup, release the per UE measurement gap configuration;

2>	setup the per UE measurement gap configuration indicated by the measGapConfig in accordance with the received gapOffset, i.e., the first subframe of each gap occurs at an SFN and subframe meeting the following condition:

SFN mod T = FLOOR(gapOffset/10);

subframe = gapOffset mod 10;

with T = MGRP/10 as defined in TS 38.133 [14];

2>	apply the specified timing advance mgta to the gap occurrences calculated above (i.e. the UE starts the measurement mgta ms before the gap subframe occurrences);

1>	else if gapUE is set to release:

2>	release the per UE measurement gap configuration.

NOTE 1:	For gapFR2 configuration with synchronous CA, for the UE in NE-DC or NR-DC, the SFN and subframe of the serving cell indicated by the refServCellIndicator in gapFR2 is used in the gap calculation. Otherwise, the SFN and subframe of a serving cell on FR2 frequency is used in the gap calculation

NOTE 2:	For gapFR1 or gapUE configuration, for the UE in NE-DC or NR-DC, the SFN and subframe of the serving cell indicated by the refServCellIndicator in corresponding gapFR1 or gapUE is used in the gap calculation. Otherwise, the SFN and subframe of the PCell is used in the gap calculation.

NOTE 3:	For gapFR2 configuration with asynchronous CA, for the UE in NE-DC or NR-DC, the SFN and subframe of the serving cell indicated by the refServCellIndicator and refFR2ServCellAsyncCA in gapFR2 is used in the gap calculation. Otherwise, the SFN and subframe of a serving cell on FR2 frequency indicated by the refFR2ServCellAsyncCA in gapFR2 is used in the gap calculation

8.2.3.6.2b.3	Test description

8.2.3.6.2b.3.1	Pre-test conditions

Same as test case 8.2.3.6.2 with the following differences:

-	Cells configuration: NR Cell 10 replaces NR Cell 2; EUTRA Cell 10 replaces EUTRA Cell 2.

8.2.3.6.2b.3.2	Test procedure sequence

Same as test case 8.2.3.6.2 with the following differences:

-	Cells configuration: NR Cell 10 replaces NR Cell 2; EUTRA Cell 10 replaces EUTRA Cell 2.

8.2.3.6.2b.3.3	Specific message contents

Same as test case 8.2.3.6.2 with the following differences:

-	Cells configuration: Cells configuration: NR Cell 10 replaces NR Cell 2; EUTRA Cell 10 replaces EUTRA Cell 2.

Table 8.2.3.6.2b.3.3-1: MeasConfig-A3-E-UTRA (Table 8.2.3.6.2.3.3-3)

| Derivation path: 36.508 clause 4.6.6 table 4.6.6-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| measConfig ::= SEQUENCE { |  |  |  |
| measObjectToAddModList SEQUENCE (SIZE (1..maxObjectId)) OF SEQUENCE { | 2 entries |  |  |
| measObjectId[1] | 1 |  |  |
| measObject[1] | MeasObjectEUTRA-GENERIC(f1) |  |  |
| measObject[1] | MeasObjectEUTRA-GENERIC(maxEARFCN) |  | Band > 64 |
| measObjectId[2] | 2 |  |  |
| measObject[2] | MeasObjectEUTRA-GENERIC(f5) |  |  |
| measObject[2] | MeasObjectEUTRA-GENERIC(maxEARFCN) |  | Band > 64 |
| } |  |  |  |
| reportConfigToAddModList SEQUENCE (SIZE (1..maxReportConfigId)) OF SEQUENCE { | 1 entry |  |  |
| reportConfigId[1] | 1 |  |  |
| reportConfig[1] | ReportConfig-A3-H |  |  |
| } |  |  |  |
| measIdToAddModList SEQUENCE (SIZE (1..maxMeasId)) OF SEQUENCE { | 1 entry |  |  |
| measId[1] | 1 |  |  |
| measObjectId[1] | 2 |  |  |
| reportConfigId[1] | 1 |  |  |
| } |  |  |  |
| measGapConfig | MeasGapConfig-GP1 |  | INTER-FREQ |
|  | MeasGapConfig-CE |  | intraFreq-CE-NeedForGaps |
| measObjectToAddModList-v9e0 ::= SEQUENCE (SIZE (1..maxObjectId)) OF SEQUENCE { | 2 entries |  | Band > 64 |
| measObjectEUTRA-v9e0[1] SEQUENCE { |  |  |  |
| carrierFreq-v9e0 | Same downlink EARFCN as used for f1 |  |  |
| } |  |  |  |
| measObjectEUTRA-v9e0[2] SEQUENCE { |  |  |  |
| carrierFreq-v9e0 | Same downlink EARFCN as used for f5 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.3.6.2b.3.3-2: MeasConfig-A3 (Table 8.2.3.6.2.3.3-9)

| Derivation Path: TS 38.508-1 [4] Table 4.6.3-69 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasConfig ::= SEQUENCE { |  |  |  |
| measObjectToAddModList SEQUENCE (SIZE (1..maxNrofMeasId)) OF MeasObjectToAddMod { | 2 entries |  |  |
| MeasObjectToAddMod[1] SEQUENCE { |  | entry 1 |  |
| measObjectId | 1 |  |  |
| measObject CHOICE { |  |  |  |
| measObjectNR | MeasObjectNR-f1 |  |  |
| } |  |  |  |
| } |  |  |  |
| MeasObjectToAddMod[2] SEQUENCE { |  | entry 2 |  |
| measObjectId | 2 |  |  |
| measObject CHOICE { |  |  |  |
| measObjectNR | MeasObjectNR-f2 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| reportConfigToAddModList SEQUENCE(SIZE (1..maxReportConfigId)) OF ReportConfigToAddMod { | 1 entry |  |  |
| ReportConfigToAddMod[1] SEQUENCE { |  | entry 1 |  |
| reportConfigId | 1 |  |  |
| reportConfig CHOICE { |  |  |  |
| reportConfigNR | ReportConfigNR(2) Same as TS 38.508-1 Table 4.6.3-142 except for reportAmount set to ‘r1’ and reportAddNeighMeas is set to ‘setup’ | a3-Offset Thresh value set to 1 dB (2*0.5 dB) |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| measIdToAddModList SEQUENCE (SIZE (1..maxNrofMeasId)) OF MeasIdToAddMod { | 1 entry |  |  |
| MeasIdToAddMod[1] SEQUENCE { |  | entry 1 |  |
| measId | 2 |  |  |
| measObjectId | 2 |  |  |
| reportConfigId | 1 |  |  |
| } |  |  |  |
| } |  |  |  |
| measGapConfig | MeasGapConfig |  |  |
| } |  |  |  |

Table 8.2.3.6.2b.3.3-3: MeasObjectNR-f1 (Table 8.2.3.6.2a.3.3-2)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-76 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasObjectNR::= SEQUENCE { |  |  |  |
| ssbFrequency | ARFCN-ValueNR for SSB of NR Cell 1 |  |  |
| absThreshSS-BlocksConsolidation | Not present |  |  |
| nrofSS-BlocksToAverage | Not present |  |  |
| } |  |  |  |

Table 8.2.3.6.2b.3.3-4: MeasObjectNR-f2 (Table 8.2.3.6.2a.3.3-2)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-76 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasObjectNR::= SEQUENCE { |  |  |  |
| ssbFrequency | ARFCN-ValueNR for SSB of NR Cell 10 |  |  |
| absThreshSS-BlocksConsolidation | Not present |  |  |
| nrofSS-BlocksToAverage | Not present |  |  |
| } |  |  |  |