3GPP TSG-RAN5 Meeting #103	R5-243511

# Fukuoka, Japan, 20th May 2024 – 24th May 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 4415 | rev | 1 | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NEDC test case 8.2.2.8.3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, MCC TF160, MediaTek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2024-04-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - In step 16 SS modify MN terminated MCG DRB to SN terminated Split DRB, and primary path is on SCG(E-UTRAN), so SS need to test data exchange in E-UTRA path instead of NR path in step18. - In step 6, SS test the data exchange in NR path, therefore SS has to configure primary path on MCG(NR) in step4. - In step 19 and Step25, SS release MCG RLC, PCellChange is not mandatory in this scenario. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. Change NR path to E-UTRA path in step18 2. Correct the primary path to NR in step6. 3.Delete the condition PCell_Change in step 19 and 25. 4.Clarify the reference table in the message content and correct typo. 5.The table references and specific message contents were updated. 6.Editorial corrections. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Conformant UE may fail this test |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.2.8.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revised from: R5-242816r1 |  |  |  |  |  |  |  |  |

<Start of modified section 1>

8.2.2.8.3	Bearer Modification / Handling for bearer type change with security key change / NE-DC

8.2.2.8.3.1	Test Purpose (TP)

(1)

with { UE in RRC_CONNECTED state with NE-DC and SCG }

ensure that {

when { UE receives an RRCReconfiguration message to modify the SN terminated SCG DRB to MN terminated SCG DRB with security key change to KgNB }

then { UE reconfigures the DRB and sends an RRCReconfigurationComplete message }

}

(2)

with { UE in RRC_CONNECTED state with NE-DC, and and MN terminated SCG DRB established with security key KgNB }

ensure that {

when { UE receives an RRCReconfiguration message to modify the MN terminated SCG DRB to SN terminated Split DRB with security key change to s-KeNB }

then { UE reconfigures the DRB and sends an RRCReconfigurationComplete message }

}

(3)

with { UE in NR RRC_CONNECTED state with NE-DC, and SN terminated Split DRB established with security key s-KeNB }

ensure that {

when { UE receives an RRCReconfiguration message to modify the SN terminated Split DRB to MN terminated Split DRB with security key change to KgNB }

then { UE reconfigures the DRB and sends an RRCReconfigurationComplete message }

}

(4)

with { UE in NR RRC_CONNECTED state with NE-DC, and, and MN terminated Split DRB established with security key KgNB }

ensure that {

when { UE receives an RRCConnectionReconfiguration message to modify the MN terminated Split DRB to SN terminated MCG DRB (NR PDCP) with security key change to s-KeNB }

then { UE reconfigures the DRB and sends an RRCConnectionReconfigurationComplete message }

}

(5)

with { UE in NR RRC_CONNECTED state with NE-DC, and, and SN terminated MCG DRB established with security key s-KeNB }

ensure that {

when { UE receives an RRCConnectionReconfiguration message to modify the SN terminated MCG DRB (NR PDCP) to MN terminated MCG DRB (NR PDCP) with security key change to KgNB }

then { UE reconfigures the DRB and sends an RRCConnectionReconfigurationComplete message }

}

(6)

with { UE in NR RRC_CONNECTED state with NE-DC, and, and MN terminated MCG DRB established with security key KgNB }

ensure that {

when { UE receives an RRCConnectionReconfiguration message to modify the MN terminated MCG DRB (NR PDCP) to SN terminated Split DRB with security key change to s-KeNB }

then { UE reconfigures the DRB and sends an RRCConnectionReconfigurationComplete message }

}

(7)

with { UE in NR RRC_CONNECTED state with NE-DC, and, and SN terminated Split DRB established with security key s-KeNB }

ensure that {

when { UE receives an RRCConnectionReconfiguration message to modify the SN terminated Split DRB to MN terminated SCG DRB with security key change to KgNB }

then { UE reconfigures the DRB and sends an RRCConnectionReconfigurationComplete message }

}

(8)

with { UE in NR RRC_CONNECTED state with NE-DC, and, and MN terminated SCG DRB established with security key KgNB }

ensure that {

when { UE receives an RRCConnectionReconfiguration message to modify the MN terminated SCG DRB to SN terminated MCG DRB (NR PDCP) with security key change to s-KeNB }

then { UE reconfigures the DRB and sends an RRCConnectionReconfigurationComplete message }

}

(9)

with { UE in NR RRC_CONNECTED state with NE-DC, and, and SN terminated MCG DRB established with security key s-KeNB }

ensure that {

when { UE receives an RRCConnectionReconfiguration message to modify the SN terminated MCG DRB (NR PDCP) to MN terminated SCG DRB with security key change to KgNB }

then { UE reconfigures the DRB and sends an RRCConnectionReconfigurationComplete message }

}

8.2.2.8.3.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 38.331, clause 5.3.5.3 and TS 36.331 clauses 5.3.5.3 and 5.3.10.3a1, TS 37.340, clause Annex A. Unless otherwise stated these are Rel-15 requirements.

[TS 38.331, clause 5.3.5.3]

The UE shall perform the following actions upon reception of the RRCReconfiguration, or upon execution of the conditional reconfiguration (CHO, CPA or CPC):

…

1>	if the RRCReconfiguration includes the mrdc-SecondaryCellGroupConfig:

2>	if the mrdc-SecondaryCellGroupConfig is set to setup:

3>	if the mrdc-SecondaryCellGroupConfig includes mrdc-ReleaseAndAdd:

4>	perform MR-DC release as specified in clause 5.3.5.10;

3>	if the received mrdc-SecondaryCellGroup is set to nr-SCG:

4>	perform the RRC reconfiguration according to 5.3.5.3 for the RRCReconfiguration message included in nr-SCG;

3>	if the received mrdc-SecondaryCellGroup is set to eutra-SCG:

4>	perform the RRC connection reconfiguration as specified in TS 36.331 [10], clause 5.3.5.3 for the RRCConnectionReconfiguration message included in eutra-SCG;

2>	else (mrdc-SecondaryCellGroupConfig is set to release):

3>	perform MR-DC release as specified in clause 5.3.5.10;

…

1>	set the content of the RRCReconfigurationComplete message as follows:

…

2>	if the RRCReconfiguration message includes the mrdc-SecondaryCellGroupConfig with mrdc-SecondaryCellGroup set to eutra-SCG:

3>	include in the eutra-SCG-Response the E-UTRA RRCConnectionReconfigurationComplete message in accordance with TS 36.331 [10] clause 5.3.5.3;

[TS 36.331, clause 5.3.5.3]

If the RRCConnectionReconfiguration message does not include the mobilityControlInfo and the UE is able to comply with the configuration included in this message, the UE shall:

…

1>	if the UE is configured with NE-DC:

2>	if the received RRCConnectionReconfiguration message was included in an NR RRCResume message:

3>	transfer the RRCConnectionReconfigurationComplete message via SRB1 embedded in NR RRC message RRCResumeComplete as specified in TS 38.331 [82], clause 5.3.13.4;

2>	else:

3>	transfer the RRCConnectionReconfigurationComplete message via SRB1 embedded in NR RRC message RRCReconfigurationComplete as specified in TS 38.331 [82], clause 5.3.5.3;

1>	else:

2>	submit the RRCConnectionReconfigurationComplete message to lower layers for transmission using the new configuration, upon which the procedure ends;

[TS 36.331, clause 5.3.10.3a1]

For the drb-Identity value for which this procedure is initiated, the UE shall:

1>	if drb-ToAddModListSCG is received and includes the drb-Identity value; and drb-Identity value is not part of the current UE configuration (i.e. DC specific DRB establishment):

2>	if drb-ToAddModList is received and includes the drb-Identity value (i.e. add split DRB):

3>	establish a PDCP entity and configure it with the current MCG security configuration and in accordance with the pdcp-Config included in drb-ToAddModList;

3>	establish an MCG RLC entity and an MCG DTCH logical channel in accordance with the rlc-Config, logicalChannelIdentity and logicalChannelConfig included in drb-ToAddModList;

3>	establish an SCG RLC entity and an SCG DTCH logical channel in accordance with the rlc-ConfigSCG, logicalChannelIdentitySCG and logicalChannelConfigSCG included in drb-ToAddModListSCG;

2>	else (i.e. add SCG DRB):

3>	establish a PDCP entity and configure it with the current SCG security configuration and in accordance with the pdcp-Config included in drb-ToAddModListSCG;

3>	establish a primary SCG RLC entity or entities and a primary SCG DTCH logical channel in accordance with the rlc-ConfigSCG, logicalChannelIdentitySCG and logicalChannelConfigSCG included in drb-ToAddModListSCG;

3>	if rlc-BearerConfigSecondary is included with value setup;

4>	establish a secondary SCG RLC entity or entities and an associated DTCH logical channel in accordance with the received rlc-BearerConfigSecondary and associate these with the E-UTRA PDCP entity with the same value of srb-Identity within the current UE configuration;

2>	indicate the establishment of the DRB(s) and the eps-BearerIdentity of the established DRB(s) to upper layers;

1>	else (i.e. DC specific DRB modification; drb-ToAddModList and/ or drb-ToAddModListSCG received):

2>	if the DRB indicated by drb-Identity is a split DRB:

3>	if drb-ToAddModList is received and includes the drb-Identity value, while for this entry drb-TypeChange is included and set to toMCG (i.e. split to MCG):

4>	release the SCG RLC entity or entities and the SCG DTCH logical channel(s);

4>	reconfigure the PDCP entity in accordance with the pdcp-Config, if included in drb-ToAddModList;

4>	reconfigure the primary MCG RLC entity and/ or the primary MCG DTCH logical channel in accordance with the rlc-Config and logicalChannelConfig, if included in drb-ToAddModList;

4>	if rlc-BearerConfigSecondary is included with value setup;

5>	establish a secondary MCG RLC entity or entities and an associated DTCH logical channel in accordance with the received rlc-BearerConfigSecondary and associate these with the E-UTRA PDCP entity with the same value of srb-Identity within the current UE configuration;

3>	else (i.e. reconfigure split):

4>	reconfigure the PDCP entity in accordance with the pdcp-Config, if included in drb-ToAddModList;

4>	reconfigure the MCG RLC entity and/ or the MCG DTCH logical channel in accordance with the rlc-Config and logicalChannelConfig, if included in drb-ToAddModList;

4>	reconfigure the SCG RLC entity and/ or the SCG DTCH logical channel in accordance with the rlc-ConfigSCG and logicalChannelConfigSCG, if included in drb-ToAddModListSCG;

2>	if the DRB indicated by drb-Identity is an SCG DRB:

3>	if drb-ToAddModList is received and includes the drb-Identity value, while for this entry drb-TypeChange is included and set to toMCG (i.e. SCG to MCG):

4>	reconfigure the PDCP entity with the current MCG security configuration and in accordance with the pdcp-Config, if included in drb-ToAddModList;

4>	reconfigure the SCG RLC entity or entities (both primary and secondary, if configured) and the SCG DTCH logical channel (both primary and secondary, if configured) to be an MCG RLC entity or entities and an MCG DTCH logical channel;

4>	reconfigure the primary MCG RLC entity or entities and/ or the primary MCG DTCH logical channel in accordance with the rlc-Config, logicalChannelIdentity and logicalChannelConfig, if included in drb-ToAddModList;

4>	if rlc-BearerConfigSecondary is included with value release:

5>	release the secondary MCG RLC entity or entities as well as the associated DTCH logical channel;

4>	if rlc-BearerConfigSecondary is included with value setup;

5>	if the current DRB configuration does not include a secondary RLC bearer:

6>	establish a secondary MCG RLC entity or entities and an associated DTCH logical channel in accordance with the received rlc-BearerConfigSecondary and associate these with the E-UTRA PDCP entity with the same value of srb-Identity within the current UE configuration;

5>	else:

6>	reconfigure the secondary MCG RLC entity or entities and the associated DTCH logical channel in accordance with the received rlc-BearerConfigSecondary;

3>	else (i.e. drb-ToAddModListSCG is received and includes the drb-Identity value i.e. reconfigure SCG):

4>	reconfigure the PDCP entity in accordance with the pdcp-Config, if included in drb-ToAddModListSCG;

4>	reconfigure the primary SCG RLC entity or entities and/ or the primary SCG DTCH logical channel in accordance with the rlc-ConfigSCG and logicalChannelConfigSCG, if included in drb-ToAddModListSCG;

4>	if rlc-BearerConfigSecondary is included with value release:

5>	release the secondary SCG RLC entity or entities as well as the associated DTCH logical channel;

4>	if rlc-BearerConfigSecondary is included with value setup;

5>	if the current DRB configuration does not include a secondary RLC bearer:

6>	establish a secondary SCG RLC entity or entities and an associated DTCH logical channel in accordance with the received rlc-BearerConfigSecondary and associate these with the E-UTRA PDCP entity with the same value of srb-Identity within the current UE configuration;

5>	else:

6>	reconfigure the secondary SCG RLC entity or entities and the associated DTCH logical channel in accordance with the received rlc-BearerConfigSecondary;

2>	if the DRB indicated by drb-Identity is an MCG DRB:

3>	if drb-ToAddModListSCG is received and includes the drb-Identity value, while for this entry drb-Type is included and set to split (i.e. MCG to split):

4>	reconfigure the PDCP entity in accordance with the pdcp-Config, if included in drb-ToAddModList;

4>	reconfigure the primary MCG RLC entity and/ or the primary MCG DTCH logical channel in accordance with the rlc-Config and logicalChannelConfig, if included in drb-ToAddModList;

4>	if rlc-BearerConfigSecondary is included with value release:

5>	release the secondary MCG RLC entity or entities as well as the associated DTCH logical channel;

4>	establish an SCG RLC entity and an SCG DTCH logical channel in accordance with the rlc-ConfigSCG, logicalChannelIdentitySCG and logicalChannelConfigSCG, included in drb-ToAddModListSCG;

3>	else (i.e. drb-Type is included and set to scg i.e. MCG to SCG):

4>	reconfigure the PDCP entity with the current SCG security configuration and in accordance with the pdcp-Config, if included in drb-ToAddModListSCG;

4>	reconfigure the MCG RLC entity or entities (both primary and secondary, if configured) and the MCG DTCH logical channel (both primary and secondary, if configured) to be an SCG RLC entity or entities and an SCG DTCH logical channel;

4>	reconfigure the primary SCG RLC entity or entities and/ or the primary SCG DTCH logical channel in accordance with the rlc-ConfigSCG, logicalChannelIdentitySCG and logicalChannelConfigSCG, if included in drb-ToAddModListSCG;

4>	if rlc-BearerConfigSecondary is included with value release:

5>	release the secondary SCG RLC entity or entities as well as the associated DTCH logical channel;

4>	if rlc-BearerConfigSecondary is included with value setup;

5>	if the current DRB configuration does not include a secondary RLC bearer:

6>	establish a secondary SCG RLC entity or entities and an associated DTCH logical channel in accordance with the received rlc-BearerConfigSecondary and associate these with the E-UTRA PDCP entity with the same value of srb-Identity within the current UE configuration;

5>	else:

6>	reconfigure the secondary SCG RLC entity or entities and the associated DTCH logical channel in accordance with the received rlc-BearerConfigSecondary;

[TS 37.340, Annex A]

This clause provides for information an overview on L2 handling for bearer type change in MR-DC, with and without a security key change due to a change of the termination point.

Table A-1: L2 handling for bearer type change with and without a security key change due to a change of the termination point.

| Bearer type change from row to col | MCG |  | Split |  | SCG |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | no change of termination point (no key change) | change of termination point (key change) | no change of termination point (no key change) | change of termination point (key change) | no change of termination point (no key change) | change of termination point (key change) |
| MCG | N/A | PDCP: Re-establish MCG RLC: See Note 1 MCG MAC: See Note 1 SCG RLC: No action SCG MAC: No action | PDCP: Reconfigure MCG RLC: No action MCG MAC: No action SCG RLC: Establish SCG MAC: Reconfigure | PDCP: Re-establish MCG RLC: See Note 1 MCG MAC: See Note 1 SCG RLC: Establish SCG MAC: Reconfigure | PDCP: Recovery MCG RLC: See Note 3 MCG MAC: Reconfigure SCG RLC: Establish SCG MAC: Reconfigure | PDCP: Re-establish MCG RLC: See Note 3 MCG MAC: Reconfigure SCG RLC: Establish SCG MAC: Reconfigure |
| Split | PDCP: Recovery MCG RLC: No action MCG MAC: No action SCG RLC: See Note 4 SCG MAC: Reconfigure | PDCP: Re-establish MCG RLC: See Note 1 MCG MAC: See Note 1 SCG RLC: See Note 4 SCG MAC: Reconfigure | N/A | PDCP: Re-establish MCG RLC: See Note 1 MCG MAC: See Note 1 SCG RLC: See Note 1 SCG MAC: See Note 1 | PDCP: Recovery MCG RLC: See Note 3 MCG MAC: Reconfigure SCG RLC: No action SCG MAC: No action | PDCP: Re-establish MCG RLC: See Note 3 MCG MAC: Reconfigure SCG RLC: See Note 1 SCG MAC: See Note 1 |
| SCG | PDCP: Recovery MCG RLC: Establish MCG MAC: Reconfigure SCG RLC: See Note 4 SCG MAC: Reconfigure | PDCP: Re-establish MCG RLC: Establish MCG MAC: Reconfigure SCG RLC: See Note 4 SCG MAC: Reconfigure | PDCP: Reconfigure MCG RLC: Establish MCG MAC: Reconfigure SCG RLC: No action SCG MAC: No action | PDCP: Re-establish MCG RLC: Establish MCG MAC: Reconfigure SCG RLC: See Note 1 SCG MAC: See Note 1 | N/A | PDCP: Re-establish MCG RLC: No action MCG MAC: No action SCG RLC: See Note 1 SCG MAC: See Note 1 |

NOTE 1:	For EN-DC and NGEN-DC MCG, NE-DC SCG: the MAC/RLC behaviour depends on the solution selected by the network. It can be PCell handover (for EN-DC and NGEN-DC) or PSCell change (for NE-DC), which triggers MAC reset and RLC re-establishment. Alternatively, the logical channel identity can be changed, either via RLC bearer release and add for the same DRB (including RLC re-establishment), or via reconfiguration of the RLC bearer with RLC-re-establishment.

For EN-DC and NGEN-DC SCG, NE-DC MCG, NR-DC MCG and SCG: the MAC/RLC behaviour depends on the solution selected by the network. It can be reconfiguration with sync, with MAC reset and RLC re-establishment. Alternatively, the logical channel identity can be changed via RLC bearer release and add.

NOTE 2:	Void

NOTE 3:	For EN-DC and NGEN-DC: Re-establishment and release. For NE-DC and NR-DC: Release.

NOTE 4:	For NE-DC: Re-establishment and release. For EN-DC, NGEN-DC and NR-DC: Release.

8.2.2.8.3.3	Test description

8.2.2.8.3.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1 is the PCell and E-UTRA Cell 1 is the PSCell.

UE:

-	None.

Preamble:

-	If pc_IP_Ping is set to TRUE then, the UE is in state RRC_CONNECTED using generic procedure parameter Connectivity (NE-DC), Bearers (MCG(s) and SCG) established according to TS 38.508-1 [4], clause 4.5.4.

-	Else, the UE is in state RRC_CONNECTED using generic procedure parameter Connectivity (NE-DC), Bearers (MCG(s) and SCG) established and Test Loop Function (On) with UE test loop mode B according to TS 38.508-1 [4], clause 4.5.4.

8.2.2.8.3.3.2	Test procedure sequence

Table 8.2.2.8.3.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits an RRCReconfiguration message containing E-UTRA RRCConnectionReconfiguration message to modify SN terminated SCG DRB with security key s-KeNB to MN terminated SCG DRB with security key change to KgNB. | <-- | RRCReconfiguration (RRCConnectionReconfiguration) | - | - |
| 2 | Check: Does the UE transmit an RRCReconfigurationComplete message containing E-UTRA RRCConnectionReconfigurationCompletemessage? | --> | RRCReconfigurationComplete (RRCConnectionReconfigurationComplete) | 1 | P |
| 3 | Check: Does the test result of generic test procedure in TS 38.508-1 subclause 4.9.1 indicate that the UE is capable of exchanging IP data on SCG DRB#2? | - | - | 1 | - |
| 4 | The SS transmits an RRCReconfiguration message containing E-UTRA RRCConnectionReconfiguration message to modify MN terminated SCG DRB with security key KgNB to SN terminated Split DRB with security key change to s-KeNB. | <-- | RRCReconfiguration (RRCConnectionReconfiguration) | - | - |
| 5 | Check: Does the UE transmit an RRCReconfigurationComplete message containing E-UTRA RRCConnectionReconfigurationCompletemessage? | --> | RRCReconfigurationComplete (RRCConnectionReconfigurationComplete) | 2 | P |
| 6 | Check: Does the test result of generic test procedure in TS 38.508-1 subclause 4.9.1 indicate that the UE is capable of exchanging IP data on Split DRB#2 using NR radio path? | - | - | 2 | - |
| 7 | The SS transmits an RRCReconfiguration message containing E-UTRA RRCConnectionReconfiguration message to modify SN terminated Split DRB with security key s-KeNB to MN terminated Split DRB with security key change to KgNB. | <-- | RRCReconfiguration (RRCConnectionReconfiguration) | - | - |
| 8 | Check: Does the UE transmit an RRCReconfigurationComplete message containing E-UTRA RRCConnectionReconfigurationComplete message? | --> | RRCReconfigurationComplete (RRCConnectionReconfigurationComplete) | 3 | P |
| 9 | Check: Does the test result of generic test procedure in TS 38.508-1 subclause 4.9.1 indicate that the UE is capable of exchanging IP data on Split DRB#2 using NR radio path? | - | - | 3 | - |
| 10 | The SS transmits an RRCReconfiguration message containing E-UTRA RadioBearerConfig to release to modify MN terminated Split DRB with security key KgNB to SN terminated MCG DRB with security key change to s-KeNB. | <-- | RRCReconfiguration | - | - |
| 11 | Check: Does the UE transmit an RRCReconfigurationComplete message? | --> | RRCReconfigurationComplete | 4 | P |
| 12 | Check: Does the test result of generic test procedure in TS 38.508-1 subclause 4.9.1 indicate that the UE is capable of exchanging IP data on MCG DRB#2? | - | - | 4 | - |
| 13 | The SS transmits an RRCReconfiguration message containing NR RadioBearerConfig to modify SN terminated MCG DRB with security key s-KeNB to MN terminated MCG DRB with security key change to KgNB. | <-- | RRCReconfiguration | - | - |
| 14 | Check: Does the UE transmit an RRCReconfigurationComplete message? | --> | RRCReconfigurationComplete | 5 | P |
| 15 | Check: Does the test result of generic test procedure in TS 38.508-1 subclause 4.9.1 indicate that the UE is capable of exchanging IP data on MCG DRB#2? | - | - | 5 | - |
| 16 | The SS transmits an RRCReconfiguration message containing E-UTRA RRCConnectionReconfiguration message to modify MN terminated MCG DRB with security key KgNB to SN terminated Split DRB with security key change to s-KeNB. | <-- | RRCReconfiguration (RRCConnectionReconfiguration) | - | - |
| 17 | Check: Does the UE transmit an RRCReconfigurationComplete message containing E-UTRA RRCConnectionReconfigurationCompletemessage? | --> | RRCReconfigurationComplete (RRCConnectionReconfigurationComplete) | 6 | P |
| 18 | Check: Does the test result of generic test procedure in TS 38.508-1 subclause 4.9.1 indicate that the UE is capable of exchanging IP data on Split DRB#2 using E-UTRA radio path? | - | - | 6 | - |
| 19 | The SS transmits an RRCReconfiguration message containing E-UTRA RRCConnectionReconfiguration message to modify SN terminated Split DRB with security key s-KeNB to MN terminated SCG DRB with security key change to KgNB. | <-- | RRCReconfiguration (RRCConnectionReconfiguration) | - | - |
| 20 | Check: Does the UE transmit an RRCReconfigurationComplete message containing E-UTRA RRCConnectionReconfigurationComplete message? | --> | RRCReconfigurationComplete (RRCConnectionReconfigurationComplete) | 7 | P |
| 21 | Check: Does the test result of generic test procedure in TS 38.508-1 subclause 4.9.1 indicate that the UE is capable of exchanging IP data on SCG DRB#2? | - | - | 7 | - |
| 22 | The SS transmits an RRCReconfiguration message containing E-UTRA RRCConnectionReconfiguration message to modify MN terminated SCG DRB with security key KgNB to SN terminated MCG DRB with security key change to s-KeNB. | <-- | RRCReconfiguration (RRCConnectionReconfiguration) | - | - |
| 23 | Check: Does the UE transmit an RRCReconfigurationComplete message containing E-UTRA RRCConnectionReconfigurationComplete message? | --> | RRCReconfigurationComplete (RRCConnectionReconfigurationComplete) | 8 | P |
| 24 | Check: Does the test result of generic test procedure in TS 38.508-1 subclause 4.9.1 indicate that the UE is capable of exchanging IP data on MCG DRB#2? | - | - | 8 | - |
| 25 | The SS transmits an RRCReconfiguration message containing E-UTRA RRCConnectionReconfiguration message to modify SN terminated MCG DRB with security key s-KeNB to MN terminated SCG DRB with security key change to KgNB. | <-- | RRCReconfiguration (RRCConnectionReconfiguration) | - | - |
| 26 | Check: Does the UE transmit an RRCReconfigurationComplete message containing E-UTRA RRCConnectionReconfigurationComplete message? | --> | RRCReconfigurationComplete (RRCConnectionReconfigurationComplete) | 9 | P |
| 27 | Check: Does the test result of generic test procedure in TS 38.508-1 subclause 4.9.1 indicate that the UE is capable of exchanging IP data on SCG DRB#2? | - | - | 9 | - |

8.2.2.8.3.3.3	Specific message contents

Table 8.2.2.8.3.3.3-1: RRCReconfiguration (step 1, Table 8.2.2.8.3.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| mrdc-SecondaryCellGroupConfig CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| mrdc-SecondaryCellGroup CHOICE { |  |  |  |
| eutra-SCG | OCTET STRING (CONTAINING RRCConnectionReconfiguration) | See Table 8.2.2.8.3.3.3-2 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| radioBearerConfig2 | RadioBearerConfig-SCG | See Table 8.2.2.8.3.3.3-4 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.2.8.3.3.3-2: RRCConnectionReconfiguration (Table 8.2.2.8.3.3.3-1)

| Derivation Path: TS 36.508 [7], Table 4.6.1-8 with condition NE-DC |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| rrcConnectionReconfiguration-r8 SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| scg-Configuration-r12 | SCG-Configuration-r12-NE-DC | See Table 8.2.2.8.3.3.3-3 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.2.8.3.3.3-3: SCG-Configuration-r12-NE-DC (Table 8.2.2.8.3.3.3-2)

Derivation Path: TS 36.508 [7], Table 4.6.3-19G

Table 8.2.2.8.3.3.3-4: RadioBearerConfig-SCG (Table 8.2.2.8.3.3.3-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-132 with condition DRB1 and Re-establish_PDCP |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioBearerConfig ::= SEQUENCE { |  |  |  |
| drb-ToAddModList SEQUENCE (SIZE (1..maxDRB)) OF DRB-ToAddMod { | 1 entry |  |  |
| DRB-ToAddMod[1] SEQUENCE { |  | entry 1 |  |
| drb-Identity | DRB-Identity using condition DRBn | DRBn is allocated for SCG according to internal TTCN mapping |  |
| } |  |  |  |
| } |  |  |  |
| securityConfig SEQUENCE { |  |  |  |
| keyToUse | master |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.2.8.3.3.3-5: RRCReconfiguration (step 4, Table 8.2.2.8.3.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| masterCellGroup | CellGroupConfig-MCG | OCTET STRING (CONTAINING CellGroupConfig) See Table 8.2.2.8.3.3.3-6 |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| mrdc-SecondaryCellGroupConfig CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| mrdc-SecondaryCellGroup CHOICE { |  |  |  |
| eutra-SCG | OCTET STRING (CONTAINING RRCConnectionReconfiguration) | See Table 8.2.2.8.3.3.3-2 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| radioBearerConfig2 | RadioBearerConfig-SCG | See Table 8.2.2.8.3.3.3-7 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.2.8.3.3.3-6: CellGroupConfig-MCG (Table 8.2.2.8.3.3.3-5)

| Derivation Path: TS 38.508-1, table 4.6.3-19 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |
| rlc-BearerToAddModList SEQUENCE (SIZE(1..maxLC-ID)) OF RLC-BearerConfig { | 1 entry |  |  |
| RLC-BearerConfig[1] | RLC-BearerConfig as per TS 38.508-1[4] Table 4.6.3-148: RLC-BearerConfig with conditions DRBn | DRBn is allocated for SCG according to internal TTCN mapping |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.2.8.3.3.3-7: RadioBearerConfig-SCG (Table 8.2.2.8.3.3.3-5, Table 8.2.2.8.3.3.3-17)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-132 with condition DRB1 and Re-establish_PDCP |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioBearerConfig ::= SEQUENCE { |  |  |  |
| drb-ToAddModList SEQUENCE (SIZE (1..maxDRB)) OF DRB-ToAddMod { | 1 entry |  |  |
| DRB-ToAddMod[1] SEQUENCE { |  | entry 1 |  |
| drb-Identity | DRB-Identity using condition DRBn | DRBn is allocated for SCG according to internal TTCN mapping |  |
| pdcp-Config | PDCP-Config-Split | Table 8.2.2.8.3.3.3-8 |  |
| } |  |  |  |
| } |  |  |  |
| securityConfig SEQUENCE { |  |  |  |
| keyToUse | secondary |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.2.8.3.3.3-8: PDCP-Config-Split (Table 8.2.2.8.3.3.3-7)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-99 with condition Split |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDCP-Config ::= SEQUENCE { |  |  |  |
| moreThanOneRLC SEQUENCE { |  |  |  |
| primaryPath SEQUENCE { |  |  |  |
| cellGroup | 0 |  |  |
| logicalChannel | LogicalChannelIdentity |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.2.8.3.3.3-9: RRCReconfiguration (step 7, Table 8.2.2.8.3.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |
| radioBearerConfig | RadioBearerConfig according to TS 38.508-1[4] Table 4.6.3-132: RadioBearerConfig with conditions SRB1, SRB2, DRBm, and Re-establish_PDCP | DRBm is DRB(s) on MCG are allocated according to internal TTCN mapping |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| masterCellGroup | CellGroupConfig-MCG | OCTET STRING (CONTAINING CellGroupConfig) See Table 8.2.2.8.3.3.3-12 |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| mrdc-SecondaryCellGroupConfig CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| mrdc-SecondaryCellGroup CHOICE { |  |  |  |
| eutra-SCG | OCTET STRING (CONTAINING RRCConnectionReconfiguration) | See Table 8.2.2.8.3.3.3-2 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| radioBearerConfig2 | RadioBearerConfig-SCG | See Table 8.2.2.8.3.3.3-10 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.2.8.3.3.3-10: RadioBearerConfig-SCG (Table 8.2.2.8.3.3.3-9)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-132 with condition DRB1 and Re-establish_PDCP |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioBearerConfig ::= SEQUENCE { |  |  |  |
| drb-ToAddModList SEQUENCE (SIZE (1..maxDRB)) OF DRB-ToAddMod { | 1 entry |  |  |
| DRB-ToAddMod[1] SEQUENCE { |  | entry 1 |  |
| drb-Identity | DRB-Identity using condition DRBn | DRBn is allocated for SCG according to internal TTCN mapping |  |
| pdcp-Config | PDCP-Config-Split | Table 8.2.2.8.3.3.3-11 |  |
| } |  |  |  |
| } |  |  |  |
| securityConfig SEQUENCE { |  |  |  |
| keyToUse | master |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.2.8.3.3.3-11: PDCP-Config-Split (Table 8.2.2.8.3.3.3-10)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-99 with condition Split |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDCP-Config ::= SEQUENCE { |  |  |  |
| moreThanOneRLC SEQUENCE { |  |  |  |
| primaryPath SEQUENCE { |  |  |  |
| cellGroup | 0 |  |  |
| logicalChannel | LogicalChannelIdentity |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.2.8.3.3.3-12: CellGroupConfig-MCG (Table 8.2.2.8.3.3.3-9)

| Derivation Path: TS 38.508-1 [4], table 4.6.3-19 with condition PCell_Change |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |
| rlc-BearerToAddModList SEQUENCE (SIZE(1..maxLC-ID)) OF RLC-BearerConfig { | 4 entries |  |  |
| RLC-BearerConfig[1] | RLC-BearerConfig as per TS 38.508-1[4] Table 4.6.3-148: RLC-BearerConfig with conditions SRB1and Re-establish_RLC |  |  |
| RLC-BearerConfig[2] | RLC-BearerConfig as per TS 38.508-1[4] Table 4.6.3-148: RLC-BearerConfig with conditions SRB2 and Re-establish_RLC |  |  |
| RLC-BearerConfig[3] | RLC-BearerConfig as per TS 38.508-1[4] Table 4.6.3-148: RLC-BearerConfig with conditions DRBm and Re-establish_RLC | DRBm is DRB(s) on MCG |  |
| RLC-BearerConfig[4] | RLC-BearerConfig as per TS 38.508-1[4] Table 4.6.3-148: RLC-BearerConfig with conditions DRBn and Re-establish_RLC | DRBn is DRB(s) on MCG |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.2.8.3.3.3-13: RRCReconfiguration (step 10, Table 8.2.2.8.3.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |
| radioBearerConfig | RadioBearerConfig according to TS 38.508-1[4] Table 4.6.3-132: RadioBearerConfig with conditions SRB1, SRB2, DRBm, and Re-establish_PDCP | DRBm is DRB(s) on MCG are allocated according to internal TTCN mapping |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| masterCellGroup | CellGroupConfig | OCTET STRING (CONTAINING CellGroupConfig) See Table 8.2.2.8.3.3.3-14 |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| mrdc-SecondaryCellGroupConfig CHOICE { |  |  |  |
| release |  |  |  |
| } |  |  |  |
| radioBearerConfig2 | RadioBearerConfig | See Table 8.2.2.8.3.3.3-15 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.2.8.3.3.3-14: CellGroupConfig (Table 8.2.2.8.3.3.3-13)

| Derivation Path: TS 38.508-1 [4], table 4.6.3-19 with condition PCell_Change |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |
| rlc-BearerToAddModList SEQUENCE (SIZE(1..maxLC-ID)) OF RLC-BearerConfig { | 4 entries |  |  |
| RLC-BearerConfig[1] | RLC-BearerConfig as per TS 38.508-1[4] Table 4.6.3-148: RLC-BearerConfig with conditions SRB1 and Re-establish_RLC |  |  |
| RLC-BearerConfig[2] | RLC-BearerConfig as per TS 38.508-1[4] Table 4.6.3-148: RLC-BearerConfig with conditions SRB2 and Re-establish_RLC |  |  |
| RLC-BearerConfig[3] | RLC-BearerConfig as per TS 38.508-1[4] Table 4.6.3-148: RLC-BearerConfig with conditions DRBm and Re-establish_RLC | DRBm is DRB(s) on MCG |  |
| RLC-BearerConfig[4] | RLC-BearerConfig as per TS 38.508-1[4] Table 4.6.3-148: RLC-BearerConfig with conditions DRBn and Re-establish_RLC | DRBn is DRB(s) on MCG |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.2.8.3.3.3-15: RadioBearerConfig (Table 8.2.2.8.3.3.3-13)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioBearerConfig ::= SEQUENCE { |  |  |  |
| drb-ToAddModList SEQUENCE (SIZE (1..maxRB)) OF DRB-ToAddMod { | 1 entry |  |  |
| DRB-ToAddMod[1] SEQUENCE { |  |  |  |
| cnAssociation CHOICE { |  |  |  |
| sdap-Config | SDAP-Config |  |  |
| } |  |  |  |
| drb-Identity | DRB-Identity using condition DRBn |  |  |
| reestablishPDCP | True |  |  |
| recoverPDCP | Not present |  |  |
| pdcp-Config | PDCP-Config |  |  |
| } |  |  |  |
| } |  |  |  |
| securityConfig SEQUENCE { |  |  |  |
| keyToUse | Secondary |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.2.8.3.3.3-16: RRCReconfiguration (step 13, Table 8.2.2.8.3.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |
| radioBearerConfig | RadioBearerConfig according to TS 38.508-1[4] Table 4.6.3-132: RadioBearerConfig with conditions SRB1, SRB2, DRBn, DRBm, Re-establish_PDCP | DRBn and DRBm is allocated according to internal TTCN mapping |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| masterCellGroup | CellGroupConfig | OCTET STRING (CONTAINING CellGroupConfig) See Table 8.2.2.8.3.3.3-17 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.2.8.3.3.3-17: CellGroupConfig (Table 8.2.2.8.3.3.3-16)

| Derivation Path: TS 38.508-1 [4], table 4.6.3-19 with condition PCell_Change |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |
| rlc-BearerToAddModList SEQUENCE (SIZE(1..maxLC-ID)) OF RLC-BearerConfig { | 4 entries |  |  |
| RLC-BearerConfig[1] | RLC-BearerConfig as per TS 38.508-1[4] Table 4.6.3-148: RLC-BearerConfig with conditions SRB1 and Re-establish_RLC |  |  |
| RLC-BearerConfig[2] | RLC-BearerConfig as per TS 38.508-1[4] Table 4.6.3-148: RLC-BearerConfig with conditions SRB2 and Re-establish_RLC |  |  |
| RLC-BearerConfig[3] | RLC-BearerConfig as per TS 38.508-1[4] Table 4.6.3-148: RLC-BearerConfig with conditions DRBm and Re-establish_RLC | DRBm is DRB(s) on MCG |  |
| RLC-BearerConfig[4] | RLC-BearerConfig as per TS 38.508-1[4] Table 4.6.3-148: RLC-BearerConfig with conditions DRBn and Re-establish_RLC | DRBn is DRB(s) on MCG |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.2.8.3.3.3-18: RRCReconfiguration (step 16, Table 8.2.2.8.3.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |
| radioBearerConfig | RadioBearerConfig according to TS 38.508-1[4] Table 4.6.3-132: RadioBearerConfig with conditions SRB1, SRB2, DRBm, Re-establish_PDCP | DRBm on MCG is allocated according to internal TTCN mapping |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| masterCellGroup | CellGroupConfig-MCG | OCTET STRING (CONTAINING CellGroupConfig) See Table 8.2.2.8.3.3.3-21 |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| mrdc-SecondaryCellGroupConfig CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| mrdc-SecondaryCellGroup CHOICE { |  |  |  |
| eutra-SCG | OCTET STRING (CONTAINING RRCConnectionReconfiguration) | See Table 8.2.2.8.3.3.3-2 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| radioBearerConfig2 | RadioBearerConfig-SCG | See Table 8.2.2.8.3.3.3-19 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.2.8.3.3.3-19: RadioBearerConfig-SCG (Table 8.2.2.8.3.3.3-18)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-132 with condition DRB1 and Re-establish_PDCP |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioBearerConfig ::= SEQUENCE { |  |  |  |
| drb-ToAddModList SEQUENCE (SIZE (1..maxDRB)) OF DRB-ToAddMod { | 1 entry |  |  |
| DRB-ToAddMod[1] SEQUENCE { |  | entry 1 |  |
| drb-Identity | DRB-Identity using condition DRBn | DRBn is allocated for SCG according to internal TTCN mapping |  |
| pdcp-Config | PDCP-Config-Split | Table 8.2.2.8.3.3.3-20 |  |
| } |  |  |  |
| } |  |  |  |
| securityConfig SEQUENCE { |  |  |  |
| keyToUse | secondary |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.2.8.3.3.3-20: PDCP-Config-Split (Table 8.2.2.8.3.3.3-19)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-99 with condition Split |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDCP-Config ::= SEQUENCE { |  |  |  |
| moreThanOneRLC SEQUENCE { |  |  |  |
| primaryPath SEQUENCE { |  |  |  |
| cellGroup | 1 |  |  |
| logicalChannel | LogicalChannelIdentity |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.2.8.3.3.3-21: CellGroupConfig-MCG (Table 8.2.2.8.3.3.3-18)

| Derivation Path: TS 38.508-1 [4], table 4.6.3-19 with condition PCell_Change |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |
| rlc-BearerToAddModList SEQUENCE (SIZE(1..maxLC-ID)) OF RLC-BearerConfig { | 4 entries |  |  |
| RLC-BearerConfig[1] | RLC-BearerConfig as per TS 38.508-1[4] Table 4.6.3-148: RLC-BearerConfig with conditions SRB1 and Re-establish_RLC |  |  |
| RLC-BearerConfig[2] | RLC-BearerConfig as per TS 38.508-1[4] Table 4.6.3-148: RLC-BearerConfig with conditions SRB2 and Re-establish_RLC |  |  |
| RLC-BearerConfig[3] | RLC-BearerConfig as per TS 38.508-1[4] Table 4.6.3-148: RLC-BearerConfig with conditions DRBm and Re-establish_RLC | DRBm is DRB(s) on MCG |  |
| RLC-BearerConfig[4] | RLC-BearerConfig as per TS 38.508-1[4] Table 4.6.3-148: RLC-BearerConfig with conditions DRBn and Re-establish_RLC | DRBn is DRB(s) on MCG |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.2.8.3.3.3-22: RRCReconfiguration (step 19, Table 8.2.2.8.3.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| masterCellGroup | CellGroupConfig-MCG | OCTET STRING (CONTAINING CellGroupConfig) See Table 8.2.2.8.3.3.3-24 |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| mrdc-SecondaryCellGroupConfig CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| mrdc-SecondaryCellGroup CHOICE { |  |  |  |
| eutra-SCG | OCTET STRING (CONTAINING RRCConnectionReconfiguration) | See Table 8.2.2.8.3.3.3-2 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| radioBearerConfig2 | RadioBearerConfig-SCG | See Table 8.2.2.8.3.3.3-23 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.2.8.3.3.3-23: RadioBearerConfig-SCG ( Table 8.2.2.8.3.3.3-22)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-132 with condition DRB1 and Re-establish_PDCP |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioBearerConfig ::= SEQUENCE { |  |  |  |
| drb-ToAddModList SEQUENCE (SIZE (1..maxDRB)) OF DRB-ToAddMod { | 1 entry |  |  |
| DRB-ToAddMod[1] SEQUENCE { |  | entry 1 |  |
| drb-Identity | DRB-Identity using condition DRBn | DRBn is allocated for SCG according to internal TTCN mapping |  |
| pdcp-Config | PDCP-Config | TS 38.508-1[4] Table 4.6.3-99 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.2.8.3.3.3-24: CellGroupConfig-MCG (Table 8.2.2.8.3.3.3-22)

| Derivation Path: TS 38.508-1[4], table 4.6.3-19 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |
| rlc-BearerToReleaseList SEQUENCE (SIZE(1..maxLC-ID)) OF LogicalChannelIdentity { |  |  |  |
| LogicalChannelIdentity[1] | Logical ChannelID of DRBn |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.2.8.3.3.3-25: RRCReconfiguration (step 22, Table 8.2.2.8.3.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| masterCellGroup | CellGroupConfig | OCTET STRING (CONTAINING CellGroupConfig) See Table 8.2.2.8.3.3.3-26 |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| mrdc-SecondaryCellGroupConfig CHOICE { |  |  |  |
| Release |  |  |  |
| } |  |  |  |
| radioBearerConfig2 | RadioBearerConfig | See Table 8.2.2.8.3.3.3-27 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.2.8.3.3.3-26: CellGroupConfig (Table 8.2.2.8.3.3.3-25)

| Derivation Path: TS 38.508-1[4], table 4.6.3-19 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |
| rlc-BearerToAddModList SEQUENCE (SIZE(1..maxLC-ID)) OF RLC-BearerConfig { | 1 entry |  |  |
| RLC-BearerConfig[1] | RLC-BearerConfig as per TS 38.508-1[4] Table 4.6.3-148: RLC-BearerConfig with conditions DRBn | DRBn of SCG are allocated according to internal TTCN mapping |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.2.8.3.3.3-27: RadioBearerConfig (Table 8.2.2.8.3.3.3-25)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioBearerConfig ::= SEQUENCE { |  |  |  |
| drb-ToAddModList SEQUENCE (SIZE (1..maxRB)) OF DRB-ToAddMod { | 1 entry |  |  |
| DRB-ToAddMod[1] SEQUENCE { |  |  |  |
| cnAssociation CHOICE { |  |  |  |
| sdap-Config | SDAP-Config |  |  |
| } |  |  |  |
| drb-Identity | DRB-Identity using condition DRBn | DRBn of SCG are allocated according to internal TTCN mapping |  |
| reestablishPDCP | True |  |  |
| recoverPDCP | Not present |  |  |
| pdcp-Config | PDCP-Config |  |  |
| } |  |  |  |
| } |  |  |  |
| securityConfig SEQUENCE { |  |  |  |
| keyToUse | Secondary |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.2.8.3.3.3-28: RRCReconfiguration (step 25, Table 8.2.2.8.3.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| masterCellGroup | CellGroupConfig-MCG | OCTET STRING (CONTAINING CellGroupConfig) See Table 8.2.2.8.3.3.3-29 |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| mrdc-SecondaryCellGroupConfig CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| mrdc-SecondaryCellGroup CHOICE { |  |  |  |
| eutra-SCG | OCTET STRING (CONTAINING RRCConnectionReconfiguration) | See Table 8.2.2.8.3.3.3-2 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| radioBearerConfig2 | RadioBearerConfig-SCG | See Table 8.2.2.8.3.3.3-30 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.2.8.3.3.3-29: CellGroupConfig-MCG (Table 8.2.2.8.3.3.3-28)

| Derivation Path: TS 38.508-1 [4], table 4.6.3-19 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |
| rlc-BearerToReleaseList SEQUENCE (SIZE(1..maxLC-ID)) OF LogicalChannelIdentity { |  |  |  |
| LogicalChannelIdentity[1] | Logical ChannelID of DRBn |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.2.8.3.3.3-30: RadioBearerConfig-SCG (Table 8.2.2.8.3.3.3-28)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-132 with condition DRB1 and Re-establish_PDCP |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioBearerConfig ::= SEQUENCE { |  |  |  |
| drb-ToAddModList SEQUENCE (SIZE (1..maxDRB)) OF DRB-ToAddMod { | 1 entry |  |  |
| DRB-ToAddMod[1] SEQUENCE { |  | entry 1 |  |
| drb-Identity | DRB-Identity using condition DRBn | DRBn is allocated for SCG according to internal TTCN mapping |  |
| pdcp-Config | PDCP-Config |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

<End of modified section 1>