3GPP TSG-RAN5 Meeting #103	R5-243470

# Fukuoka City, Fukuoka, Japan, 20th May 2024 - 24th May 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-1 | CR | 5313 | rev | 1 | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to LTE RACS Test case 9.2.5.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu EMEA Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16_Test, RACS-UEConTest |  |  |  |  |  | Date: |  |  | 2024-05-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Depending on the UE configuration, either normal or combined attach, the Tracking Area Update (TAU) request can be sent with either a combined or a normal EPS update. TAU should be permissible for both types of updates. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updated Table 9.2.5.1.3.3-6 and Table 9.2.5.1.3.3-8 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Conformant UE may fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.2.5.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Associated with TTCN CR: R5s240351 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This is the final verson of R5-242758 and one revision r1: Corrected WIC |  |  |  |  |  |  |  |  |

<Start of Change>

#### 9.2.5.1	RACS / Network assigned UE radio capability ID

9.2.5.1.1	Test Purpose (TP)

(1)

with { UE is powered on or switched on and has an applicable UE radio capability ID for the current UE radio configuration for the selected PLMN }

ensure that {

when { UE initiates an Attach procedure }

then { UE indicates the UE radio capability ID availability in ATTACH REQUEST message }

}

(2)

with { UE indicating UE radio capability ID availability in ATTACH REQUEST message }

ensure that {

when { the UE receives a SECURITY MODE COMMAND message requesting UE radio capability ID }

then { UE includes UE radio capability ID in SECURITY MODE COMPLETE message }

}

(3)

with { UE in state EMM-REGISTERED and EMM-IDLE mode and having assigned a new UE Radio Capability ID in the ATTACH ACCEPT message }

ensure that {

when { the UE detects entering a tracking area that is not in the list of tracking areas that the UE previously registered in the MME }

then { UE sends TRACKING AREA UPDATE REQUEST message including the indication of UE radio capability ID availability }

}

(4)

with { UE indicating UE radio capability ID availability in TRACKING AREA UPDATE REQUEST message }

ensure that {

when { the UE receives a SECURITY MODE COMMAND message requesting UE radio capability ID }

then { UE includes UE radio capability ID in SECURITY MODE COMPLETE message }

}

(5)

with { UE having assigned a UE radio capability ID }

ensure that {

when { the UE enters a cell belonging to another PLMN }

then { either UE indicates that UE radio capability ID is not available in UE radio capability ID availability indication IE in TRACKING AREA UPDATE REQUEST or UE indicates that UE radio capability ID is available in UE radio capability ID availability indication IE in TRACKING AREA UPDATE REQUEST and includes the manufacturer-assigned UE Radio Capability ID in SECURITY MODE COMPLETE message}

}

9.2.5.1.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 24.301, clause 5.3.20.

[TS 24.301, clause 5.3.20]

UE radio capability signalling optimisation (RACS) is a feature that is optional at both the UE and the network and which aims to optimise the transmission of UE radio capability over the radio interface (see 3GPP TS 23.401 [10]). RACS works by assigning an identifier to represent a set of UE radio capabilities. This identifier is called the UE radio capability ID. A UE radio capability ID can be either manufacturer-assigned or network-assigned. The UE radio capability ID is an alternative to the signalling of the radio capabilities container over the radio interface.

In this release of the specification, RACS is not applicable to NB-S1 mode.

If the UE supports RACS:

-	the UE shall indicate support for RACS by setting the RACS bit to "RACS supported" in the UE network capability IE of the ATTACH REQUEST and TRACKING AREA UPDATE REQUEST messages;

-	if the UE performs an attach procedure and the UE has an applicable UE radio capability ID for the current UE radio configuration in the selected network, the UE shall include the UE radio capability ID availability IE in the ATTACH REQUEST message and set the IE to "UE radio capability ID available";

-	if the UE performs a tracking area updating procedure and the UE has an applicable UE radio capability ID for the current UE radio configuration in the selected network, the UE shall include the UE radio capability ID availability IE in the TRACKING AREA UPDATE REQUEST message and set the IE to "UE radio capability ID available";

-	If the UE is requested to provide the UE radio capability ID by the network during a security mode control procedure, the UE shall include the UE radio capability ID in the UE radio capability ID IE of the SECURITY MODE COMPLETE message according to the rules in clause 5.4.3.3.;

…

-	upon receiving a network-assigned UE radio capability ID in the ATTACH ACCEPT message in the TRACKING AREA UPDATE ACCEPT message or in the GUTI REALLOCATION COMMAND message, the UE shall store the network-assigned UE radio capability ID and the PLMN ID of the serving network along with a mapping to the current UE radio configuration in its non-volatile memory as specified in annex C.

…

If the network supports RACS:

-	if the UE has included the UE radio capability ID availability IE in the ATTACH REQUEST message and set the IE to "UE radio capability ID available", the network shall initiate a security mode control procedure to retrieve the UE radio capability ID from the UE;

-	if the UE has included the UE radio capability ID availability IE in the TRACKING AREA UPDATE REQUEST message and set the IE to "UE radio capability ID available", the network may initiate a security mode control procedure to retrieve the UE radio capability ID from the UE;

-	if the UE has included the UE radio capability ID availability IE in the TRACKING AREA UPDATE REQUEST message, set the URCIDA bit to "UE radio capability ID available" in the UE radio capability ID availability IE and no UE radio capability ID is available in the UE context in the MME, the network shall initiate a security mode control procedure to retrieve the UE radio capability ID from the UE;

-	the network may assign a network-assigned UE radio capability ID to a UE which supports RACS by including a UE radio capability ID IE in the ATTACH ACCEPT message, in the TRACKING AREA UPDATE ACCEPT message or in the GUTI REALLOCATION COMMAND message; and

9.2.5.1.3	Test description

9.2.5.1.3.1	Pre-test conditions

System Simulator:

-	cell A (belongs to TAI-1, home PLMN) is set to ''Serving cell'';

-	cell B (belongs to TAI-2, home PLMN) is set to ''Non- Suitable cell'';

-	cell G (belongs to TAI-7, visited PLMN) is set to ''Non- Suitable cell''.

UE:

- 	the UE is previously registered on E-UTRAN, and when on E-UTRAN, the UE is last authenticated and registered on cell A using default message contents according to TS 36.508 [18];

Preamble:

-	the UE is in state Switched OFF (state 1) according to TS 36.508 [18].

9.2.5.1.3.2	Test procedure sequence

Table 9.2.5.1.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The UE is switched on. | - | - | - | - |
| 2 | Check: Does the UE include UE radio capability availability indication in ATTACH REQUEST message on Cell A? | --> | ATTACH REQUEST | 1 | P |
| 3-4 | Steps 5-6 of the generic procedure for UE registration specified in TS 36.508 subclause 4.5.2.3 are performed. | - | - | - | - |
| 5 | The SS transmits a NAS SECURITY MODE COMMAND message requesting UE radio capability ID. | <-- | SECURITY MODE COMMAND | - | - |
| 6 | Check: Does the UE transmit a NAS SECURITY MODE COMPLETE message including UE radio capability ID 1? | --> | SECURITY MODE COMPLETE | 2 | P |
| 7-11 | Steps 9-13 of the generic procedure for UE registration specified in TS 36.508 subclause 4.5.2.3 are performed. | - | - | - | - |
| 12 | SS transmits a ATTACH ACCEPT message with a new UE radio capability ID 2. | <-- | ATTACH ACCEPT | - | - |
| 13-16 | Steps 15-18 of the generic procedure for UE registration specified in TS 36.508 subclause 4.5.2.3 are performed. | - | - | - | - |
| - | The following messages are sent and shall be received on cell B. | - | - | - | - |
| 17 | The SS configures: - Cell A as the "Non-Suitable cell". - Cell B as a "Serving cell". | - | - | - | - |
| 18 | Void |  |  |  |  |
| - | EXCEPTION: Steps 25Aa1-25Ab1 describe behaviour that depends on UE configuration; the "lower case letter" identifies a step sequence that takes place depending on whether the UE has a manufacturer-assigned UE Radio Capability ID | - | - | - | - |
| 25Aa1 | IF the UE has a manufacturer-assigned UE Radio Capability ID THEN Check: Does the UE indicate UE radio Capability ID available in UE radio capability availability indication IE in TRACKING AREA UPDATE REQUEST message? | --> | TRACKING AREA UPDATE REQUEST | 5 | P |
| 25Aa2 | The SS transmits a NAS SECURITY MODE COMMAND message requesting UE radio capability ID. | <-- | SECURITY MODE COMMAND | - | - |
| 25Aa3 | Check: Does the UE transmit a NAS SECURITY MODE COMPLETE message including the manufacturer-assigned UE Radio Capability ID? | --> | SECURITY MODE COMPLETE | 5 | P |
| 25Ab1 | ELSE Check: Does the UE not indicate a UE radio Capability ID is available in TRACKING AREA UPDATE REQUEST message? | --> | TRACKING AREA UPDATE REQUEST | 5 | P |
| 19 | The SS transmits a NAS SECURITY MODE COMMAND message requesting UE radio capability ID. | <-- | SECURITY MODE COMMAND | - | - |
| 20 | Check: Does the UE transmit a NAS SECURITY MODE COMPLETE message including UE radio capability ID 2? | --> | SECURITY MODE COMPLETE | 4 | P |
| 21 | The SS transmits a TRACKING AREA UPDATE ACCEPT message. | <-- | TRACKING AREA UPDATE ACCEPT | - | - |
| 22 | The UE transmits a TRACKING AREA UPDATE COMPLETE message. | --> | TRACKING AREA UPDATE COMPLETE | - | - |
| 23 | The SS releases the RRC connection. | - | - | - | - |
| - | The following messages are sent and shall be received on cell G. | - | - | - | - |
| 24 | The SS configures: - Cell B as the "Non-Suitable cell". - Cell G as a "Serving cell". | - | - | - | - |
| 25 | Check: Does the UE include UE radio capability availability indication in TRACKING AREA UPDATE REQUEST message? | --> | TRACKING AREA UPDATE REQUEST | 5 | F |
| 26 | The SS transmits a TRACKING AREA UPDATE ACCEPT message. | <-- | TRACKING AREA UPDATE ACCEPT | - | - |
| 27 | The UE transmits a TRACKING AREA UPDATE COMPLETE message. | --> | TRACKING AREA UPDATE COMPLETE | - | - |
| 28 | The SS releases the RRC connection. | - | - | - | - |

9.2.5.1.3.3	Specific message contents

Table 9.2.5.1.3.3-1: Message ATTACH REQUEST (preamble)

| Derivation path: 36.508 table 4.7.2-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| UE network capability |  |  |  |
| Radio capability signalling optimisation (RACS) capability (octet 10, bit 1) | '1'B | RACS supported |  |

Table 9.2.5.1.3.3-2: Message ATTACH ACCEPT (preamble)

| Derivation path: 36.508 table 4.7.2-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| UE radio capability ID | ‘01000000000010’O | Type Field (TF): 1 Version ID: 00 Radio Configuration Identifier (RCI): 000000000001 |  |

Table 9.2.5.1.3.3-3: Message ATTACH REQUEST (step 2)

| Derivation path: 36.508 table 4.7.2-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| UE radio capability ID availability | '001'B | UE radio capability ID available |  |

Table 9.2.5.1.3.3-4: Message SECURITY MODE COMPLETE (step 6)

| Derivation path: 36.508 table 4.7.2-20 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| UE radio capability ID | ‘01000000000010’ O | Type Field (TF): 1 Version ID: 00 Radio Configuration Identifier (RCI): 000000000001 |  |

Table 9.2.5.1.3.3-5: Message ATTACH ACCEPT (step 12)

| Derivation path: 36.508 table 4.7.2-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| UE radio capability ID | ‘01000000000020’ O | Type Field (TF): 1 Version ID: 00 Radio Configuration Identifier (RCI): 000000000002 |  |

Table 9.2.5.1.3.3-6: Message TRACKING AREA UPDATE REQUEST (step 18)

| Derivation path: 36.508 table 4.7.2-27 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| UE radio capability ID availability | '001'B | UE radio capability ID available |  |

Table 9.2.5.1.3.3-7: Message SECURITY MODE COMPLETE (step 20)

| Derivation path: 36.508 table 4.7.2-20 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| UE radio capability ID | ‘01000000000020’ O | Type Field (TF): 1 Version ID: 00 Radio Configuration Identifier (RCI): 000000000002 |  |

Table 9.2.5.1.3.3-8: Message TRACKING AREA UPDATE REQUEST (step 25Aa1 and step 25Ab1)

| Derivation path: 36.508 table 4.7.2-27 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| UE radio capability ID availability | '000'B or Not Present | 000: UE radio capability ID not available | 25Ab1 |
|  | ‘001’B | 001: UE radio capability ID available | 25Aa1 |

Table 9.2.5.1.3.3-9: Message SECURITY MODE COMPLETE (step 25Aa3)

| Derivation path: 36.508 table 4.7.2-20 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| UE radio capability ID | ‘00XXXXXXXXXXXX’ O | Type Field (TF): 0 Version ID: XX Radio Configuration Identifier (RCI): XXXXXXXXXXXX |  |

<End of Change>