3GPP TSG-RAN5 Meeting #103	R5-243604

# Fukuoka, Japan, 20th - 24th May 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 3199 | rev | 1 | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of missing TRS configurations |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Rohde & Schwarz |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16_Test, NR_perf_enh-UEConTest |  |  |  |  |  | Date: |  |  | 2024-05-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | For CA testing many new bandwidths need to be tested in demod or CSI test cases. For these bandwidths the TRS configuration is missing in TS 38.508-1. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add missing TRS configurations to chapter 5.4.2.0. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Required configurations remain missing |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.2.0 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### < Unchanged sections omitted >

CSI-FrequencyOccupation

Table 5.4.2.0-11: CSI-FrequencyOccupation for TRS

| Derivation Path: Table 4.6.3-33 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-FrequencyOccupation ::= SEQUENCE { |  |  |  |
| nrofRBs | 28 | BW 5 MHz SCS 15kHz | TRS |
|  | 52 | BW 10 MHz SCS 15kHz | TRS |
|  | 24 | BW 10 MHz SCS 30kHz | TRS |
|  | 80 | BW 15 MHz SCS 15kHz | TRS |
|  | 40 | BW 15 MHz SCS 30kHz | TRS |
|  | 108 | BW 20 MHz SCS 15kHz | TRS |
|  | 52 | BW 20 MHz SCS 30kHz | TRS |
|  | 136 | BW 25 MHz SCS 15kHz | TRS |
|  | 68 | BW 25 MHz SCS 30kHz | TRS |
|  | 160 | BW 30 MHz SCS 15kHz | TRS |
|  | 80 | BW 30 MHz SCS 30kHz | TRS |
|  | 216 | BW 40 MHz SCS 15kHz | TRS |
|  | 108 | BW 40 MHz SCS 30kHz | TRS |
|  | 272 | BW 50 MHz SCS 15kHz | TRS |
|  | 136 | BW 50 MHz SCS 30kHz | TRS |
|  | 164 | BW 60 MHz SCS 30kHz | TRS |
|  | 220 | BW 80 MHz SCS 30kHz | TRS |
|  | 248 | BW 90 MHz SCS 30kHz | TRS |
|  | 276 | BW 100 MHz SCS 30kHz | TRS |
|  | 68 | BW 100 MHz SCS 120kHz | TRS |
|  | 132 | BW 200 MHz SCS 120kHz | TRS |
| } |  |  |  |

#### < End of changes >