3GPP TSG-RAN5 Meeting #103	R5-242334

# Fukuoka City, Fukuoka, Japan, 20th May 2024 - 24th May 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4821 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | NTN-IoT: Addition of NGSO NB-IoT NTN Test Model |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17_Test, LTE_NBIOT_eMTC_NTN_plus_EPS-UEConTest |  |  |  |  |  | Date: |  |  | 2024-05-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - At RAN5#103 the SIG testing environment for the NGSO scenario was finalized. The NB-IoT NTN test model needs to be updated to add support for the NGSO scenario. - The SIG testing environment for the NGSO scenario requires that the satellite movement prediction is disabled at the UE. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Updated NTN clauses with the NGSO relevant information and calculation results. - Introduced an MMI command to turn off the satellite movement prediction functionality at the UE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | NB-IoT NTN test model for NGSO scenario will remain unspecified and the UE will be continuously predicting the satellite movement thus loosing the connection with the simulated cell. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2, 7A.14.1, 7A14.2.2, 7A.14.3(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# 5	Upper Tester Interface

This clause describes the handling of Upper Tester Commands at the system interface. The internal handling of those commands in TTCN is out of scope.

In the TTCN, the Upper Tester is located at the MTC; therefore there is one interface to the system adaptor common for all RATs.

## 5.2	Upper Tester ASPs

There is one primitive defined carrying either an MMI or an AT command to be sent to the system adaptor and one common confirmation primitive to be sent by the system adaptor.

| TTCN-3 ASP Definition |  |  |  |
| --- | --- | --- | --- |
| Type Name | UT_SYSTEM_REQ |  |  |
| TTCN-3 Type | Record |  |  |
| Cmd |  | TTCN-3 Type | union |
| AT |  | charstring carrying the AT command as defined in TS 27.007 [32], TS 27.005 [31] and TS 27.060 [33] |  |
| MMI |  | - Cmd (charstring) - List of parameters: - Name (charstring) - Value (charstring) |  |
| CnfRequired |  | TTCN-3 Type | Ut_CnfReq_Type |
|  |  | CNF_REQUIRED: SS shall reply with one confirmation (NOTE) NO_CNF_REQUIRED: SS shall swallow any confirmation generated LOCAL_CNF_REQUIRED: SS shall immediately send one confirmation when the command is submitted to the UE i.e. in case of operator interaction when the operator has confirmed the command, but SS shall not wait for the UE responding(NOTE) |  |
| NOTE: In the TTCN, a confirmation shall only be requested in cases when there is no signalling from the UE being triggered by the MMI/AT command |  |  |  |

| TTCN-3 ASP Definition |  |  |  |
| --- | --- | --- | --- |
| Type Name | UT_COMMON_CNF |  |  |
| TTCN-3 Type | Record |  |  |
| Result |  | TTCN-3 Type | boolean |
|  |  | true: success false: failure NOTE: The return value false will result in the test immediately finishing with a fatal error. In case of AT commands, the SS shall return false if and only if the UE comes back with "ERROR" as result code according to ITU-T Rec V.250 clause 5.7 [60]. In case of e.g. "+CME ERROR" the SS shall return true and put the UE's response into the ResultString |  |
| ResultString |  | TTCN-3 Type | charstring |
|  |  | response by the UE for commands which request the UE to return a result, optional In case of AT commands, ResultString shall contain the complete line according to the specification of the AT command. Any carriage return and linefeed characters, defined in TS 27.007 [32] clause 3.1, contained in the response shall be included as the textual representations "<CR>" and "<LF>" respectively. |  |

The following MMI commands are defined.

Table 5.1: MMI commands

| Command | Parameters |  |
| --- | --- | --- |
|  | Name | Value |
| "SWITCH_ON" | (none) |  |
| "SWITCH_OFF" | (none) |  |
| "POWER_ON" | (none) |  |
| "POWER_OFF" | (none) |  |
| "INSERT USIM" | "USIM" | <USIM> |
| "REMOVE_USIM" | "USIM" (optional) | <USIM> |
| "CHECK_PLMN" | "PLMN" | <PLMN ID> |
| "CHECK_ETWS_INDICATION" | “WARNING1” | <WARNING1> |
|  | “WARNING2” | <WARNING2> |
| “CHECK_ETWS_ALERT” | (none) |  |
| “CHECK_ETWS_NO_ALERT” | (none) |  |
| "CHECK_CMAS_INDICATION" | “WARNING1” | <WARNING1> |
|  | “WARNING2” | <WARNING2> |
| “CHECK_CMAS_ALERT” | (none) |  |
| “CHECK_CMAS_NO_ALERT” | (none) |  |
| "HRPD_PDN_CONNECTION” | (none) |  |
| “CHECK_SMS_LENGTH_CONTENTS" | "Length" | <Length> |
|  | "Msg" | <Msg> |
| "CONFIGURE_SMS_ONLY" | (none) |  |
| "DISABLE EPS CAPABILITY" | (none) |  |
| "DETACH_NON_EPS" | (none) |  |
| "CLEAR_STORED_ASSISTANCE_DATA" | (none) |  |
| "CHECK_DTCH_THROUGHCONNECTED" | (none) |  |
| "GERAN_UPLINK_DATA" | (none) |  |
| "SELECT_CSG" | "PLMN" | <PLMN ID> |
|  | "CSG" | <CSG ID> |
| “TRIGGER_USER_RESELECTION” | (none) |  |
| "REQUEST_NON_CALL_RELATED_SS" | (none) |  |
| "MBMS_SERVICE_INTEREST" | "PLMN" | <PLMN ID> |
|  | "Service" | <MBMS Service ID> |
|  | "Interest" | "ON" / "OFF" |
|  | "SAI" | <MBMS SAI> |
| “MBMS_SERVICE_ACTIVE” | "Service" | <MBMS Service ID> |
|  | "SAI" | <MBMS SAI> |
|  | “Active” | "ON" / "OFF" |
| “MBMS_PRIORITY_OVER_UNICAST” | "Priority Over Unicast" | "FALSE" / "TRUE" |
| "CHECK_ETWS_NO_INDICATION" | “WARNING1” | <WARNING1> |
|  | “WARNING2” | <WARNING2> |
| "CHECK_CMAS_NO_INDICATION" | “WARNING1” | <WARNING1> |
|  | “WARNING2” | <WARNING2> |
| "CHECK_CALL_DISPLAY" | "ToDisplay" | <NAME>/<NUMBER> |
| "CHECK_CSG" | "CSG" | < CSG ID > |
|  | "Included" | "FALSE" / "TRUE" |
| "CHECK_NITZ_DST" | "DaylightSavingTime:" | DST |
| "CHECK_RESPONSE_DISPLAY" | "ToDisplay" | <STRING> |
| "CHECK_SS_USER_IND" | "Supplementary Service" | <SUPPLEMENTARY SERVICE> |
|  | "ToDisplay" | <STRING> |
| “CHECK_SS_USER_IND_FAILURE” | (none) |  |
| "CONFIGURE_OPERATIONMODE" | "OPERATION MODE" | <OPERATION MODE> |
| “CONFIGURE_ANDSF_RULE” | “ANDSF Rule” | <ANDSF RULE> |
| “WLAN_ANDSF_CONFIGURED” | (none) |  |
| “WLAN_ANDSF_NOT_CONFIGURED” | (none) |  |
| "CMTLR_DUMMY" (see Note) | ”Notification Type” | <NOTIFICATION TYPE> |
| "CCFC_DUMMY" (see Note) | ”Status” | <STATUS> |
| "CNAP_DUMMY" (see Note) | ”Testname” | <TESTNAME> |
| "CTZE_DUMMY" (see Note) | "Year | <YEAR> |
|  | "Month" | <MONTH> |
|  | "Day" | <DAY> |
|  | "Hour" | <HOUR> |
|  | "Minutes" | <MINUTES> |
|  | "Seconds" | <SECONDS> |
|  | “Daylight SavingTime” | <DAYLIGHT SAVING TIME> |
|  | "TimeZone" | <TIMEZONE> |
| "CMWN_DUMMY" (see Note) | (none) |  |
| “D2D_DISCOVERY_CLEAR” | (none) |  |
| “D2D_DISCOVERY_CONFIGURATION” | “ProSeApplicationID” | <charstring> |
|  | "PLMN_List" | <PLMN ID_List> |
|  | T4005 | <T4005> |
| “D2D_DISCOVERY” | “ProSeApplicationID” | <charstring> |
|  | “Announce” | "ON" / "OFF" |
|  | “Monitor” | "ON" / "OFF" |
| “D2D_COMMUNICATION” | “ProSe Layer-2 Group ID” | <charstring> |
|  | “Transmit” | "ON" / "OFF" |
|  | “Receive” | "ON" / "OFF" |
|  | “Max size of UE transmitted Direct Communication data” | <bytes> |
| “D2D_COMMUNICATION_CONFIGURATION” | “ProSe Layer-2 Group ID” | <charstring> |
|  | "PLMN_List" | <PLMN ID_List> |
|  | T4005_Minutes | <T4005> |
|  | “Security Life time timer in minutes” (Default: 0) | <SecurityTimer> |
| "V2X_SIDELINK" | "Data Type" | "IP" / "NON-IP" |
| “INSERT_USIM_OR_PRECONFIGURE” | "USIM" | <USIM> |
| “DELETE_NTN_PLMN_NA” | (none) |  |
| “CONFIGURE_NTN_UE_POSITION” | "SpecReference" | <charstring> |
| “SATELLITE_TRAJECTORY_PREDICTION” | “Prediction” | “ACTIVATE” / ”DEACTIVATE” |
| Note: These MMI commands are used to indicate to the SS that a UT response is expected by the TTCN in the case when the AT command is not supported. They can be ignored if the UE supports the AT command. The prefix of these commands matches the start of the AT response expected by the TTCN. |  |  |

<<SKIPPED SECTIONS>>

## 7A.14	Non-Terrestrial Network (NTN)

### 7A.14.1	NTN cell configuration

An NB-IoT cell to be used in a Non-Terrestrial Network (NTN) test environment is an NB-IoT cell that is broadcasting a system information block combination, defined in TS 36.508 [3] clause 8.1.4.3.1.1, that contains SystemInformationBlockType31-NB (SIB31-NB). It is called an NB-IoT NTN cell.

In addition to that, an NB-IoT NTN cell shall be configured in SIB1-NB as "barred" in cellAccessRelatedInfo-r13 IE and as "notBarred" in cellAccessRelatedInfo-NTN-r17 IE.

Typically an NTN test case can be executed under one of the following two sub-NTN test environments:

-	the Geosynchronous Orbit (GSO) scenario, or

-	the Non-Geosynchronous Orbit (NGSO) scenario.

### 7A.14.2	NTN timing considerations

#### 7A.14.2.1	General

DL and UL are frame aligned at the UL time synchronization reference point (RP). To accommodate the long propagation delays in NTN, the timing relationships are defined using a Common Timing Advance (Common TA) and two offsets: Koffset and Kmac, with Koffset ≥ TTA.

The various NTN timing relationships are illustrated in Figure 7A.14.2.1-1.

Figure 7A.14.2.1-1: NTN timing relationship parameters

#### 7A.14.2.2	NTN timing advance

The Timing Advance (TA) formula, specified in TS 36.211 [35] clause 8.1, can be summarized as follows:

TTA = TTA,legacy + TTA,NTN

TTA,NTN = TTA,common + TTA,UE-specific

with:

-	TTA,legacy = (NTA + NTA,offset) * Ts: the (legacy) part of the TA formula that applies also to Terrestrial Networks,

-	TTA,common = NTA,adjcommon * Ts: the Common TA between the RP and the NTN payload,

-	TTA,UE-specific = NTA,adjUE * Ts: the TA corresponding to the service link delay.

The NTN test environment in TS 36.508 [3] specifies that both the serving satellite (NTN payload) and the UE under test have a static position during the lifetime of a test case execution. Therefore, the corresponding TTA is also static for a given NTN cell. TTCN provides to the SS the following inputs to enable the SS to calculate TTA:

Table 7A.14.2.2-1: NTN timing advance - input parameters

| TTA element | Associated parameter(s) |  | Relationship |
| --- | --- | --- | --- |
| (in sec) | Protocol parameter | ASP field | (TS 36.213 [30] clause 16.1.2) |
| TTA,legacy |  | InitialValue integer (0..2047) | NTA = InitialValue * 16 |
| TTA,common | NTA,adjcommon | NtnCommonTA integer (0..8316827) | See below. |
| TTA,UE-specific | NTA,adjUE | NtnUeSpecificTA integer (0..8316827) | NTA,adjUE= NtnUeSpecificTA See below. |
| NOTE: The time unit TS is defined in TS 36.211 [35] clause 4. |  |  |  |

From the nta-CommonParameters-17 specified in TS 36.331 [19], only the value of nta-Common-r17 is defined in TS 36.508 [3] SIB31-NB for the GSO and NGSO scenarios. TTCN will forward this value to the SS as NtnCommonTA for the calculation by the SS of NTA,adjcommon according to the formula given in TS 36.213 [30] clause 16.1.2.

NOTE:	It is assumed that the SS uses the calculation: NTA,adjcommon = NtnCommonTA.

NTA,adjUE is calculated using the NTN serving satellite static position specified in TS 36.508 [3] SIB31-NB's ephemeris data and the UE static position specified in TS 36.508 [3] clause 4.13. The end result is provided to the SS by TTCN. The calculation is provided hereafter:

The two-way propagation delay in sec between the NTN serving satellite and the non-moving UE is assumed to be constant at any time and is calculated according to the formula:

t=2*d/c

where c is the speed of light (299792458 m/sec) and d is the distance in meters between the serving satellite and the UE.

For two points in Euclidian geometry d in meters is derived by:

d=∆x+∆y+∆z

with

∆x=(xsat-xUE)2

∆y= (ysat-yUE)2

∆z=(zsat-zUE)2

and

xsat, ysat, zsat being serving satellite coordinates, in m

xUE, yUE, zUE being UE coordinates, in m

To use above formulas the positioning data provided in TS 36.508 [3] is first converted to the coordinates accordingly.

Example of the calculation of ∆x:

given UE latitude 25.08439333 and positionX-r17 of -16976014:

xsat= -16976014* 1.3 m = -22068818.2 m

xUE= -3025296.94 m

∆x=(-22068818.2 m-(-3025296.94 m))2=(-19043521.26 m)2= 362655701980071.94 m2

Following the above calculations, the final value t for a GSO scenario results in 0.24338972709427653 sec that yields NTA,adjUE to have a value of 7476932.416336175, that is rounded to 7476932.

For a NGSO scenario, the calculated final value of t results in 0.004062662259880647 sec and a corresponding NTA,adjUE has a value of 124804.98462353348. The last one is rounded to 124805.

#### 7A.14.2.3	NTN scheduling offsets

The NTN timing parameters listed in Table 7A.14.2.3-1 come into play when scheduling data in an NTN cell.

Table 7A.14.2.3-1: NTN timing parameters in an NTN cell

| NTN timing parameter | Definition | Reference |
| --- | --- | --- |
| DLoffset | Koffset + Kmac | TS 36.321 [16] clause 7.7 |
| UE-eNB RTT | TTA + Kmac | TS 36.321 [16] clause 3.1 |
| Kmac | k-Mac or 0 | TS 36.213 [30] clause 16.6 |
| Koffset | k-Offset - Differential Koffset | TS 36.213 [30] clause 16 |
| RTToffset | UE-eNB RTT | TS 36.321 [16] clause 7.7 |

TTCN configures the NTN offsets Koffset and Kmac in the SS as part of the NTN cell configuration. The SS shall use those parameters to adjust the timing & scheduling of DL & UL transmissions specified in clauses 7A.4, 7A.5 and 7A.6 as follows:

-	Table 7A.4.1-1: Timing of random access procedure

| tdelay1 | 3ms + UE-eNB RTT | according to TS 36.321 [16] clause 5.1.4. |
| --- | --- | --- |
| tdelay3 | 12ms + Koffset | according to TS 36.213 [30] clause 16.5.1. |
| tdelay4 | 3ms + UE-eNB RTT | according to TS 36.321 [16] clause 5.1.5. |
| tdelay5 | DL: 13ms UL: 13ms + Koffset | depending on whether UL or DL transmission follows, see below. |

-	Table 7A.4.2-1: Timing of uplink transmissions

| tdelay1 | 8ms + Koffset | according to TS 36.213 [30] clause 16.5.1. |
| --- | --- | --- |
| tdelay2 | 3ms + RTToffset | according to TS 36.321 [16] clause 7.7. |

-	Table 7A.4.3-1: Timing of downlink transmissions

| tdelay2 | 12ms + Koffset | according to TS 36.213 [30] clause 16.4.2. |
| --- | --- | --- |

-	Table 7A.4.4-1: Uplink transmissions

-	The end of the period during which NPDCCH is not monitored by the UE is n1+Kmac+3: according to TS 36.213 [30] clause 16.6.

In addition to the NTN timing parameters referenced in Table 7A.14.2.3-1, the following NTN timing parameters are defined in the present specification for the purpose of conformance testing:

-	TNTN,offset = DLoffset

-	NPDCCH period bundle:

-	in a Terrestrial Networks (TN) test environment:

-	NPDCCH period bundle = NPDCCH period

-	in a Non Terrestrial Networks (NTN) test environment:

-	NPDCCH period bundle = CEIL(TNTN,offset / NPDCCH period) * NPDCCH period + NPDCCH period

#### 7A.14.2.4	RRC connection release sequence

The sequence specified in clause 7A.7 applies, with the following differences:

TTCN schedules the RRC connection release sequence at time T being at least TNTN,offset in advance:

2.	At T + 1 NPDCCH period bundle:
Scheduling of UL grants for search space cycle starting at (T + 1 NPDCCH period bundle) and four subsequent cycles

3.	At T + 1 NPDCCH period bundle + 16ms

4.	At T + 1 NPDCCH period bundle + 32ms

### 7A.14.3	NTN frequency considerations

For the NGSO scenario, according to TS 36.508 [3] clause 6.3.5.2, for each configured NB-IoT NTN cell, a constant frequency Doppler shift is applied.

### Once TTCN has configured in the SS an NB-IoT NTN cell with a constant frequency Doppler shift, the SS shall alter the operating carrier frequency of this cell (EARFCN) by the value of this shift.