3GPP TSG WG RAN5 Meeting #103	R5-242601

# Fukuoka, Japan, 20th - 24th May 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 3151 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of CBW 70MHz 90MHz and 100MHz for n40 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CAICT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17_Test, NR_lic_bands_BW_R17-UEConTest |  |  |  |  |  | Date: |  |  | 2024-05-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | CBW 70MHz, 90MHz and 100MHz for n40 are still missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | CBW 70MHz, 90MHz and 100MHz for n40 were introduced. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | CBW 70MHz, 90MHz and 100MHz for n40 will remain missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<< Unchanged sections omitted >>

4.3.1	Test frequencies

Editor's Note: n7 (High: 50MHz), n8 (High: 35MHz), n65 (High: 50MHz), n66 (High: 45MHz), n75(High: 50MHz), and n80 (High: 40MHz) are currently not yet included in the test channel bandwidth tables due to test frequency have not been defined yet. Further analysis is needed for the test coverage of these test channel bandwidths and bands.

4.3.1.0	General

<< Unchanged sections omitted >>

4.3.1.0A	Mid test channel bandwidth

The Mid test channel bandwidth definition for RF is given in Table 4.3.1.0A-1, Table 4.3.1.0A-2 and Table 4.3.1.0A-3 for FR1, FR2 and NTN respectively.

Table 4.3.1.0A-1: Mid Test Channel bandwidths for each NR band, FR1

| NR Band | UE Mid Test Channel bandwidth [MHz]1, 1a, 1b | RedCap UE Mid Test Channel bandwidth [MHz] 1, 1a, 1b |
| --- | --- | --- |
| n1 | 25 | 15 |
| n2 | 20 | 15 |
| n3 | 25 | 15 |
| n5 | 15 | 15 |
| n7 | 25 | 15 |
| n8 | 15 | 15 |
| n12 | 10 | 10 |
| n13 | 10 | 10 |
| n14 | 10 | 10 |
| n20 | 15 | 15 |
| n24 | 10 | 10 |
| n25 | 2511 | 15 |
| n26 | 15 | 15 |
| n28 | 15 | 15 |
| n29 | 102 | N/A |
| n30 | 10 | 10 |
| n34 | 10 | 10 |
| n38 | 20 | 15 |
| n39 | 20 | 15 |
| n40 | 50 | 15 |
| n41 | 50 | 15 |
| n46 | 604, 405 | N/A |
| n48 | 2011 | 15 |
| n50 | 404, 205,11 | 15 |
| n51 | 5 | 5 |
| n53 | 10 | 10 |
| n65 | 15 | 15 |
| n66 | 20 | 15 |
| n70 | 152, 103 | 152, 103 |
| n71 | 153, 202 | 15 |
| n74 | 15 | 15 |
| n75 | 152 | N/A |
| n76 | 52 | N/A |
| n77 | 50 | 15 |
| n78 | 50 | 15 |
| n79 | 50 | 20 |
| n80 | 203 | [15] |
| n81 | 153 | [15] |
| n82 | 153 | [15] |
| n83 | 153 | [15] |
| n84 | 253 | [15] |
| n86 | 203 | [15] |
| n91 | 52, 103 | [52,103] |
| n92 | 15 | [15] |
| n93 | 52, 103 | [52,103] |
| n94 | 15 | [15] |
| n95 | 103 | [10] |
| n96 | 60 | N/A |
| n97 | 503 | [15] |
| n99 | 103 | [10] |
| n100 | 5 | 5 |
| n101 | 10 | 10 |
| Note 1: Mid channel BW is the closest channel BW to the arithmetic average channel BW of all the possible channel BWs per band in Table 5.3.5-1 of TS 38.521-1 [14] among all SCSs. If there are two channel bandwidths that have same distance to the average value, the higher one is selected. In case such bandwidth is not applicable for a given subcarrier spacing, the closest bandwidth to the value in this table applicable for such subcarrier spacing shall be tested. In case such bandwidth is not defined in the UE release specification according to 38.101-1 [7] Table 5.3.5-1: - If the bandwidth listed above is equal or higher than maximum bandwidth supported in the UE release according to 38.101-1 [7] Table 5.3.5-1, the closest channel BW to the arithmetic average among the values supported in the UE release specification shall be tested. - Otherwise, the closest bandwidth to the value in this table defined for that band in the UE release specification shall be tested. If there are two channel bandwidths that have same distance to the arithmetic average, the higher one is selected Note 1a: Values listed in this table assume that the non-optional channel bandwidths specified in Table 5.3.5-1 of TS 38.101-1 (most updated version of latest release available) [7] lower than the maximum are supported. However, these channel bandwidths are mandatory with capability parameter as defined in [55] TS 38.306 clause 4.2.1 for channelBWs-DL/channelBWs-UL parameters. Hence the UE might indicate them as not supported. In such case, select the closest channel bandwidth in both DL and UL. Note 1b: For CA, DC, SDL and SUL, the mid-test channel bandwidth per component carrier is chosen to test the closest aggregated bandwidth to the mathematical centre between minimum and maximum aggregated bandwidth defined for and within a given bandwidth combination set. In case no set of channel bandwidths per component carrier supported by the UE can achieve such aggregated bandwidth, select one combination of bandwidths per component carrier within the bandwidth combination set that minimizes the difference to the target aggregated bandwidth. Note 2: This UE channel bandwidth is applicable only to downlink. Note 3: This UE channel bandwidth is applicable only to uplink. Note 4: Applicable when for use as single carrier, PCell in CA or PCell in DC configuration. Note 5: Applicable for use as SCell in CA or SCell in DC configuration. Note 6: Void Note 7: Void Note 8: Void Note 9: Void Note 10: This Mid test channel bandwidth is chosen since it is more commonly used. Note 11: This is an exception to Note 1 in this table to avoid not allowed asymmetric channel bandwidths. |  |  |

<< Unchanged sections omitted >>

4.3.1.0C	High test channel bandwidth

The high test channel bandwidth definition for RF is given in Table 4.3.1.0C-1, Table 4.3.1.0C-2 and Table 4.3.1.0C-3 for FR1, FR2 and NTN respectively.

Table 4.3.1.0C-1: High Test Channel bandwidths for each NR band, FR1

| NR Band | UE High Test Channel bandwidth [MHz]10, 11, 12 | RedCap UE High Test Channel bandwidth [MHz] 10, 11, 12 |
| --- | --- | --- |
| n1 | 50 | 20 |
| n2 | 40 | 20 |
| n3 | 50 | 20 |
| n5 | 201,14, 252 | 20 |
| n7 | 50 | 20 |
| n8 | 201,14, 352 | 20 |
| n12 | 15 | 15 |
| n13 | 10 | 10 |
| n14 | 10 | 10 |
| n20 | 20 | 20 |
| n24 | 10 | 10 |
| n25 | 401, 452 | 20 |
| n26 | 20 | 20 |
| n28 | 30 | 20 |
| n29 | 102 | N/A |
| n30 | 10 | 10 |
| n34 | 15 | 15 |
| n38 | 25, 4013 | 15 |
| n39 | 40 | 20 |
| n40 | 100 | 20 |
| n41 | 100 | 20 |
| n46 | 80 | N/A |
| n48 | 403, 1004 | 20 |
| n50 | 601,14, 802 | 20 |
| n51 | 5 | 5 |
| n53 | 10 | 10 |
| n65 | 20 | 20 |
| n66 | 40 | 20 |
| n70 | 151,14, 252 | 151, 202 |
| n71 | 201, 352 | 20 |
| n74 | 20 | 20 |
| n75 | 202 | N/A |
| n76 | 52 | N/A |
| n77 | 100 | 20 |
| n78 | 100 | 20 |
| n79 | 100 | 20 |
| n80 | 301 | [20] |
| n81 | 201 | [20] |
| n82 | 201 | [20] |
| n83 | 301 | [20] |
| n84 | 501 | [20] |
| n86 | 401 | [20] |
| n91 | 101, 52,14 | [10] |
| n92 | 20 | [20] |
| n93 | 101, 52,14 | [10] |
| n94 | 20 | [20] |
| n95 | 151 | [15] |
| n96 | 80 | N/A |
| n97 | 1001 | [20] |
| n99 | 101 | [10] |
| n100 | 5 | 5 |
| n101 | 10 | 10 |
| Note 1: This UE channel bandwidth is applicable only to uplink. Note 2: This UE channel bandwidth is applicable only to downlink. Note 3: Applicable for use as single carrier, PCell in CA or PCell in DC configuration. Note 4: Applicable for use as DL SCell in CA or DL SCell in DC configuration. Note 5: Void Note 6: Void Note 7: Void Note 8: Void Note 9: Void Note 10: Maximum channel BW value among all the possible channel BW-SCS combinations per band in Table 5.3.5-1 of TS 38.521-1 [14] are listed. In case such bandwidth is not applicable for a given subcarrier spacing, the maximum bandwidth applicable for such subcarrier spacing shall be tested. In case such bandwidth is not defined in the UE release specification according to 38.101-1 [7] Table 5.3.5-1, the maximum bandwidth defined for that band in the UE release specification shall be tested. In case such bandwidth is optional in the UE release specification according to 38.101-1 [7] Table 5.3.5-1 and not supported by the UE, the maximum non-optional bandwidth for the UE release specification shall be tested. Note 11: Values listed in this table assume that the maximum (non-optional) channel bandwidth specified in Table 5.3.5-1 of TS 38.101-1 [7]is mandatory without capability parameter (i.e., purely mandatory) as defined in [55] TS 38.306 clause 4,2,1 for supportedBandwidthDL/ supportedBandwidthUL parameters in a band combination with a single band entry and a single CC entry (i.e., non-CA band combination). Note 12: For CA, DC, SDL and SUL, the High-test channel bandwidth per component carrier is chosen to allow maximum aggregated bandwidth defined for a given bandwidth combination set. In case no set of channel bandwidths per component carrier supported by the UE can achieve maximum aggregated bandwidths in CA, DC, SDL or SUL, some flexibility could be provided to the ecosystem for Rel-15 and Rel-16 so one combination of bandwidth per component carrier within the bandwidth combination set that maximizes the aggregated bandwidth is tested instead values described in Note 12 in Table 4.3.1.0C-1. Note 13: These UE channel bandwidths are applicable to sidelink operation. Note 14: This UE channel bandwidth is applicable to both uplink and downlink when both uplink and downlink are allocated and testing asymmetric BW is not explicitly intended. |  |  |

NOTE 1 (Informative): In case values listed in table above are higher than those signalled by the UE in supportedBandwidthDL/supportedBandwidthUL, some flexibility could be provided to the ecosystem for Rel-15 and Rel-16 so the value signalled by the UE in supportedBandwidthDL/supportedBandwidthUL is used in single carrier operation instead values described in Table 4.3.1.0C-1.

<< Unchanged sections omitted >>

4.3.1.1.1.40	Reference test frequencies for NR operating band n40

Table 4.3.1.1.1.40-1: Test frequencies for NR operating band n40 and SCS 15 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | 25 | Downlink | Low | 2302.5 | 460500 | 2300.25 | 460050 | 0 | (Note 3) | - | 460770 | - | - | - | - |
|  |  | & | Mid | 2350 | 470000 | 2329.39 | 465878 | 102 |  | - | 470270 | - | - | - | - |
|  |  | Uplink | High | 2397.5 | 479500 | 2304.53 | 460906 | 504 |  | - | 479770 | - | - | - | - |
| 10 | 52 | Downlink | Low | 2305 | 461000 | 2300.32 | 460064 | 0 | 30 | 5764 | 461090 | 6 | 2 | 1 (6) | 8 |
|  |  | & | Mid | 2350 | 470000 | 2326.96 | 465392 | 102 |  | 5875 | 469970 | 2 | 3 | 0 (2) | 107 |
|  |  | Uplink | High | 2395 | 479000 | 2299.6 | 459920 | 504 |  | 5989 | 479090 | 6 | 2 | 1 (6) | 512 |
| 15 | 79 | Downlink | Low | 2307.5 | 461500 | 2300.39 | 460078 | 0 | 30 | 5762 | 461050 | 0 | 1 | 1 (6) | 7 |
|  |  | & | Mid | 2350 | 470000 | 2324.53 | 464906 | 102 |  | 5869 | 469490 | 4 | 3 | 0 (2) | 107 |
|  |  | Uplink | High | 2392.5 | 478500 | 2294.67 | 458934 | 504 |  | 5973 | 477870 | 0 | 0 | 0 (2) | 506 |
| 20 | 106 | Downlink | Low | 2310 | 462000 | 2300.46 | 460092 | 0 | 30 | 5763 | 461070 | 2 | 1 | 1 (6) | 7 |
|  |  | & | Mid | 2350 | 470000 | 2322.1 | 464420 | 102 |  | 5863 | 469010 | 6 | 3 | 0 (2) | 107 |
|  |  | Uplink | High | 2390 | 478000 | 2289.74 | 457948 | 504 |  | 5960 | 476890 | 2 | 0 | 0 (2) | 506 |
| 25 | 133 | Downlink | Low | 2312.5 | 462500 | 2300.53 | 460106 | 0 | 30 | 5764 | 461090 | 4 | 1 | 1 (6) | 7 |
|  |  | & | Mid | 2350 | 470000 | 2319.67 | 463934 | 102 |  | 5857 | 468530 | 8 | 3 | 0 (2) | 107 |
|  |  | Uplink | High | 2387.5 | 477500 | 2284.81 | 456962 | 504 |  | 5950 | 475970 | 0 | 2 | 0 (2) | 508 |
| 30 | 160 | Downlink | Low | 2315 | 463000 | 2300.6 | 460120 | 0 | 30 | 5762 | 461050 | 10 | 3 | 0 (2) | 5 |
|  |  | & | Mid | 2350 | 470000 | 2317.24 | 463448 | 102 |  | 5851 | 468050 | 10 | 3 | 0 (2) | 107 |
|  |  | Uplink | High | 2385 | 477000 | 2279.88 | 455976 | 504 |  | 5937 | 474990 | 2 | 2 | 0 (2) | 508 |
| 40 | 216 | Downlink | Low | 2320 | 464000 | 2300.56 | 460112 | 0 | 30 | 5764 | 461090 | 2 | 1 | 1 (6) | 7 |
|  |  | & | Mid | 2350 | 470000 | 2312.2 | 462440 | 102 |  | 5839 | 467090 | 2 | 1 | 1 (6) | 109 |
|  |  | Uplink | High | 2380 | 476000 | 2269.84 | 453968 | 504 |  | 5914 | 473090 | 2 | 1 | 1 (6) | 511 |
| 50 | 270 | Downlink | Low | 2325 | 465000 | 2300.7 | 460140 | 0 | 30 | 5763 | 461070 | 10 | 3 | 0 (2) | 5 |
|  |  | & | Mid | 2350 | 470000 | 2307.34 | 461468 | 102 |  | 5827 | 466130 | 6 | 1 | 1 (6) | 109 |
|  |  | Uplink | High | 2375 | 475000 | 2259.98 | 451996 | 504 |  | 5888 | 471130 | 6 | 1 | 1 (6) | 511 |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-1 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. Note 3: No SS/PBCH block fits within the channel bandwidth. The channel bandwidth can only be used as SCell |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.40-2: Test frequencies for NR operating band n40 and SCS 30 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 24 | Downlink | Low | 2305 | 461000 | 2300.68 | 460136 | 0 | 30 | 5764 | 461090 | 6 | 0 | 3 (3) | 6 |
|  |  | & | Mid | 2350 | 470000 | 2308.96 | 461792 | 102 |  | 5875 | 469970 | 14 | 0 | 1 (1) | 206 |
|  |  | Uplink | High | 2395 | 479000 | 2209.24 | 441848 | 504 |  | 5989 | 479090 | 6 | 0 | 3 (3) | 1014 |
| 15 | 38 | Downlink | Low | 2307.5 | 461500 | 2300.66 | 460132 | 0 | 30 | 5762 | 461050 | 18 | 0 | 2 (2) | 4 |
|  |  | & | Mid | 2350 | 470000 | 2306.44 | 461288 | 102 |  | 5869 | 469490 | 22 | 0 | 1 (1) | 206 |
|  |  | Uplink | High | 2392.5 | 478500 | 2204.22 | 440844 | 504 |  | 5973 | 477870 | 6 | 0 | 0 (0) | 1008 |
| 20 | 51 | Downlink | Low | 2310 | 462000 | 2300.82 | 460164 | 0 | 30 | 5763 | 461070 | 14 | 0 | 2 (2) | 4 |
|  |  | & | Mid | 2350 | 470000 | 2304.1 | 460820 | 102 |  | 5863 | 469010 | 18 | 0 | 1 (1) | 206 |
|  |  | Uplink | High | 2390 | 478000 | 2199.38 | 439876 | 504 |  | 5960 | 476890 | 2 | 0 | 0 (0) | 1008 |
| 25 | 65 | Downlink | Low | 2312.5 | 462500 | 2300.8 | 460160 | 0 | 30 | 5764 | 461090 | 22 | 0 | 2 (2) | 4 |
|  |  | & | Mid | 2350 | 470000 | 2301.58 | 460316 | 102 |  | 5857 | 468530 | 2 | 0 | 2 (2) | 208 |
|  |  | Uplink | High | 2387.5 | 477500 | 2194.36 | 438872 | 504 |  | 5950 | 475970 | 6 | 0 | 1 (1) | 1010 |
| 30 | 78 | Downlink | Low | 2315 | 463000 | 2300.96 | 460192 | 0 | 30 | 5762 | 461050 | 22 | 0 | 1 (1) | 2 |
|  |  | & | Mid | 2350 | 470000 | 2299.24 | 459848 | 102 |  | 5851 | 468050 | 22 | 0 | 1 (1) | 206 |
|  |  | Uplink | High | 2385 | 477000 | 2189.52 | 437904 | 504 |  | 5937 | 474990 | 2 | 0 | 1 (1) | 1010 |
| 40 | 106 | Downlink | Low | 2320 | 464000 | 2300.92 | 460184 | 0 | 30 | 5764 | 461090 | 14 | 0 | 2 (2) | 4 |
|  |  | & | Mid | 2350 | 470000 | 2294.2 | 458840 | 102 |  | 5839 | 467090 | 14 | 0 | 2 (2) | 208 |
|  |  | Uplink | High | 2380 | 476000 | 2179.48 | 435896 | 504 |  | 5914 | 473090 | 14 | 0 | 2 (2) | 1012 |
| 50 | 133 | Downlink | Low | 2325 | 465000 | 2301.06 | 460212 | 0 | 30 | 5763 | 461070 | 22 | 0 | 1 (1) | 2 |
|  |  | & | Mid | 2350 | 470000 | 2289.34 | 457868 | 102 |  | 5827 | 466130 | 18 | 0 | 2 (2) | 208 |
|  |  | Uplink | High | 2375 | 475000 | 2169.62 | 433924 | 504 |  | 5888 | 471130 | 18 | 0 | 2 (2) | 1012 |
| 60 | 162 | Downlink | Low | 2330 | 466000 | 2300.84 | 460168 | 0 | 30 | 5762 | 461050 | 6 | 0 | 2 (2) | 4 |
|  |  | & | Mid | 2350 | 470000 | 2284.12 | 456824 | 102 |  | 5812 | 464930 | 14 | 0 | 0 (0) | 204 |
|  |  | Uplink | High | 2370 | 474000 | 2159.4 | 431880 | 504 |  | 5862 | 468990 | 10 | 0 | 1 (1) | 1010 |
| 70 | 189 | Downlink | Low | 2335 | 467000 | 2300.98 | 460196 | 0 | 30 | 5764 | 461090 | 10 | 0 | 2 (2) | 4 |
|  |  | & | Mid | 2350 | 470000 | 2279.26 | 455852 | 102 |  | 5800 | 463970 | 18 | 0 | 0 (0) | 204 |
|  |  | Uplink | High | 2365 | 473000 | 2149.54 | 429908 | 504 |  | 5839 | 467090 | 10 | 0 | 2 (2) | 1012 |
| 80 | 217 | Downlink | Low | 2340 | 468000 | 2300.94 | 460188 | 0 | 30 | 5763 | 461070 | 6 | 0 | 2 (2) | 4 |
|  |  | & | Mid | 2350 | 470000 | 2274.22 | 454844 | 102 |  | 5788 | 463010 | 10 | 0 | 1 (1) | 206 |
|  |  | Uplink | High | 2360 | 472000 | 2139.5 | 427900 | 504 |  | 5813 | 465130 | 2 | 0 | 3 (3) | 1014 |
| 90 | 245 | Downlink | Low | 2345 | 469000 | 2300.9 | 460180 | 0 | 30 | 5762 | 461050 | 2 | 0 | 2 (2) | 4 |
|  |  | & | Mid | 2350 | 470000 | 2269.18 | 453836 | 102 |  | 5776 | 462050 | 2 | 0 | 2 (2) | 208 |
|  |  | Uplink | High | 2355 | 471000 | 2129.46 | 425892 | 504 |  | 5787 | 462990 | 6 | 0 | 1 (1) | 1010 |
| 100 | 273 | Downlink | Low | 2350 | 470000 | 2300.86 | 460172 | 0 | 30 | 5764 | 461090 | 18 | 0 | 2 (2) | 4 |
|  |  | & | Mid | 2350 | 470000 | 2264.14 | 452828 | 102 |  | 5764 | 461090 | 18 | 0 | 2 (2) | 208 |
|  |  | Uplink | High | 2350 | 470000 | 2119.42 | 423884 | 504 |  | 5764 | 461090 | 18 | 0 | 2 (2) | 1012 |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-2 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.40-3: Test frequencies for NR operating band n40 and SCS 60 kHz without CORESET#0

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 11 | Downlink | Low | 2305 | 461000 | 2301.04 | 460208 | 0 | 30 | - | 460928 |
|  |  | & | Mid | 2350 | 470000 | 2272.6 | 454520 | 102 |  | - | 469928 |
|  |  | Uplink | High | 2395 | 479000 | 2028.16 | 405632 | 504 |  | - | 478928 |
| 15 | 18 | Downlink | Low | 2307.5 | 461500 | 2301.02 | 460204 | 0 | 30 | - | 460924 |
|  |  | & | Mid | 2350 | 470000 | 2270.08 | 454016 | 102 |  | - | 469424 |
|  |  | Uplink | High | 2392.5 | 478500 | 2023.14 | 404628 | 504 |  | - | 477924 |
| 20 | 24 | Downlink | Low | 2310 | 462000 | 2301.36 | 460272 | 0 | 30 | - | 460992 |
|  |  | & | Mid | 2350 | 470000 | 2267.92 | 453584 | 102 |  | - | 468992 |
|  |  | Uplink | High | 2390 | 478000 | 2018.48 | 403696 | 504 |  | - | 476992 |
| 25 | 31 | Downlink | Low | 2312.5 | 462500 | 2301.34 | 460268 | 0 | 30 | - | 460988 |
|  |  | & | Mid | 2350 | 470000 | 2265.4 | 453080 | 102 |  | - | 468488 |
|  |  | Uplink | High | 2387.5 | 477500 | 2013.46 | 402692 | 504 |  | - | 475988 |
| 30 | 38 | Downlink | Low | 2315 | 463000 | 2301.32 | 460264 | 0 | 30 | - | 460984 |
|  |  | & | Mid | 2350 | 470000 | 2262.88 | 452576 | 102 |  | - | 467984 |
|  |  | Uplink | High | 2385 | 477000 | 2008.44 | 401688 | 504 |  | - | 474984 |
| 40 | 51 | Downlink | Low | 2320 | 464000 | 2301.64 | 460328 | 0 | 30 | - | 461048 |
|  |  | & | Mid | 2350 | 470000 | 2258.2 | 451640 | 102 |  | - | 467048 |
|  |  | Uplink | High | 2380 | 476000 | 1998.76 | 399752 | 504 |  | - | 473048 |
| 50 | 65 | Downlink | Low | 2325 | 465000 | 2301.6 | 460320 | 0 | 30 | - | 461040 |
|  |  | & | Mid | 2350 | 470000 | 2253.16 | 450632 | 102 |  | - | 466040 |
|  |  | Uplink | High | 2375 | 475000 | 1988.72 | 397744 | 504 |  | - | 471040 |
| 60 | 79 | Downlink | Low | 2330 | 466000 | 2301.56 | 460312 | 0 | 30 | - | 461032 |
|  |  | & | Mid | 2350 | 470000 | 2248.12 | 449624 | 102 |  | - | 465032 |
|  |  | Uplink | High | 2370 | 474000 | 1978.68 | 395736 | 504 |  | - | 469032 |
| 70 | 93 | Downlink | Low | 2335 | 467000 | 2301.52 | 460304 | 0 | 30 | - | 461024 |
|  |  | & | Mid | 2350 | 470000 | 2243.08 | 448616 | 102 |  | - | 464024 |
|  |  | Uplink | High | 2365 | 473000 | 1968.64 | 393728 | 504 |  | - | 467024 |
| 80 | 107 | Downlink | Low | 2340 | 468000 | 2301.48 | 460296 | 0 | 30 | - | 461016 |
|  |  | & | Mid | 2350 | 470000 | 2238.04 | 447608 | 102 |  | - | 463016 |
|  |  | Uplink | High | 2360 | 472000 | 1958.6 | 391720 | 504 |  | - | 465016 |
| 90 | 121 | Downlink | Low | 2345 | 469000 | 2301.44 | 460288 | 0 | 30 | - | 461008 |
|  |  | & | Mid | 2350 | 470000 | 2233 | 446600 | 102 |  | - | 462008 |
|  |  | Uplink | High | 2355 | 471000 | 1948.56 | 389712 | 504 |  | - | 463008 |
| 100 | 135 | Downlink | Low | 2350 | 470000 | 2301.4 | 460280 | 0 | 30 | - | 461000 |
|  |  | & | Mid | 2350 | 470000 | 2227.96 | 445592 | 102 |  | - | 461000 |
|  |  | Uplink | High | 2350 | 470000 | 1938.52 | 387704 | 504 |  | - | 461000 |
| Note: FR1 carrier without CORSESET#0 is indicated in the MIB by setting =31, controlResourceSetZero=0 and searchSpaceZero = 0 (TS 38.213 [22], clause 13). |  |  |  |  |  |  |  |  |  |  |  |

<< End of changes >>