3GPP TSG-RAN5 Meeting #103	R5-242883

# Fukuoka, Japan, May 20th 2024 - May 24th 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.533 | CR | 3143 | rev | - | Current version: | 18.2.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to RedCap RRM test case 17.3.1.1 with TT |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17_Test, NR_redcap_plus_ARCH-UEConTest |  |  |  |  |  | Date: |  |  | 2024-05-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - To align with latest version of 38.133; - To update test parameters taking test tolerance into consideration. - To update Editor’s note |  |  |  |  |  |  |  |  |
| Summary of change: |  | Issues mentioned above are solved. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case is incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 17.3.1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.533 CR 3165 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | RRM TT |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

17.3.1.1	NR SA FR2 Intra-frequency handover from FR2 to FR2; unknown target cell for 2 Rx

Editor’s Note: This test case has been completed for the following configurations:

-	Test frequency f ≤ 40.8 GHz.

-	UE PC3.

-	The test is incomplete for UE power classes other than PC3

-	The test is incomplete for test frequencies > 40.8 GHz

17.3.1.1.1	Test purpose

To verify the requirement for the NR FR2 intra frequency handover requirements specified in TS 38.133 [6] clause 6.1D.1.3.

17.3.1.1.2	Test applicability

This test applies to all types of NR RedCap UE with 2Rx from Release 17 onwards and supporting FR2.

17.3.1.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 17.3.1.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.17.3.1.1.

17.3.1.1.4	Test description

17.3.1.1.4.1	Initial conditions

Same as the initial conditions given in clause 7.3.1.2.4.1 with following exceptions:

-	Table 7.3.1.2.4.1-1 is replaced by Table 17.3.1.1.4.1-1;

-	Table 7.3.1.2.4.1-2 is replaced by Table 17.3.1.1.4.1-2;

-	Table 7.3.1.2.4.1-3 is replaced by Table 17.3.1.1.4.1-3;

Table 17.3.1.1.4.1-1: Supported test configurations

| Config | Description |
| --- | --- |
| 17.3.1.1-1 | Source cell: NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode Target cell: NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table 17.3.1.1.4.1-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, table E.15-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 17.3.1.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in clause C.2.2. |
| Connection Diagram | TE Part | A.3.3.3.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.4.1.1 |  |
| Exceptions to connection diagram | N/A |  |  |

Table 17.3.1.1.4.1-3: General test parameters

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Initial conditions | Active cell |  | Cell 1 |  |
|  | Neighbouring cell |  | Cell 2 |  |
| Final condition | Active cell |  | Cell 2 |  |
| Access Barring Information |  | - | Not Sent | No additional delays in random access procedure. |
| Time offset between cells |  |  | 3 s | Synchronous cells |
| T1 |  | s | 5 |  |
| T2 |  | s | 10 |  |

17.3.1.1.4.2	Test procedure

Same as the test procedure given in clause 7.3.1.2.4.2.

17.3.1.1.4.3	Message contents

Same as the message contents given in 7.3.1.2.4.3.

17.3.1.1.5	Test requirements

Same as the test requirements given in clause 7.3.1.2.5 with following exceptions:

-	Table 7.3.1.2.5-1 is replaced by Table 17.3.1.1.5-1.

Table 17.3.1.1.5-1: Cell-specific test parameters (NR Cell 1)

| Parameter |  | Unit | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| Assumption for UE beamsNote 6 |  |  | Rough |  | Rough |  |
| AoA setup |  |  | Setup 1 as defined in A.9 |  |  |  |
| NR RF Channel Number |  |  | 1 |  |  |  |
| Duplex mode |  |  | TDD |  |  |  |
| TDD configuration |  |  | TDDConf.3.1 |  |  |  |
| BWchannel |  | MHz | 100: NRB,c = 66 |  |  |  |
| BWP BW |  | MHz | 100: NRB,c = 66 |  |  |  |
| Data RBs allocated |  |  | 66 |  |  |  |
| DRX Cycle |  | ms | Not Applicable |  |  |  |
| PDSCH Reference measurement channel |  |  | SR.3.1 TDD |  |  |  |
| RMSI CORESET Reference Channel |  |  | CR.3.1 TDD |  |  |  |
| Control Channel RMC |  |  | CCR.3.1 TDD |  |  |  |
| OCNG Patterns |  |  | OP.1 |  |  |  |
| SMTC Configuration |  |  | SMTC.1 |  |  |  |
| SSB Configuration |  |  | SSB.3 FR2 |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  | kHz | 120 |  |  |  |
| PUCCH/PUSCH subcarrier spacing |  | kHz | 120 |  |  |  |
| PRACH configuration |  |  | PRACH.1 FR2 |  |  |  |
| TRS configuration |  |  | TRS.2.1 TDD |  |  |  |
| PDSCH/PDCCH TCI state |  |  | TCI.State.2 |  |  |  |
| BWP configuration | Initial DL BWP |  | DLBWP.0.1 |  |  |  |
|  | Dedicated DL BWP |  | DLBWP.1.1 |  |  |  |
|  | Initial UL BWP |  | ULBWP.0.1 |  |  |  |
|  | Dedicated UL BWP |  | ULBWP.1.1 |  |  |  |
| EPRE ratio of PSS to SSS |  | dB | 0 |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |  |
| Note2 |  | dBm/15kHz | -104.7 |  |  |  |
| Note2 |  | dBm/SCS | -95.7 |  |  |  |
|  |  | dB | 6 | -1.8 | -Infinity | 0 |
|  |  | dB | 6 | 6 | -Infinity | 7 |
| IoNote3 |  | dBm/ BW | -59.7 | -56.7 | -59.7 | -56.7 |
| Propagation condition |  | - | AWGN |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Equivalent power received by an antenna with 0 dBi gain at the centre of the quiet zone Note 5: As observed with 0 dBi gain antenna at the centre of the quiet zone Note 6: Information about types of UE beam is given in 38.133 [6] clause B.2.1.3, and does not limit UE implementation or test system implementation |  |  |  |  |  |  |