3GPP TSG RAN Meeting #104								          RP-241039

Shanghai, China, June 17-20, 2024

Status Report to TSG

Agenda item:			9.3.1.4

| WI / SI Name | Low-power wake-up signal and receiver for NR (LP-WUS/WUR) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: Yes |  | Testing part: No |
| Acronym | NR_LPWUS |  |  |  |  |
| Unique ID | 1020094 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-240801 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: N/A | Core part: 09/2025 | Performance part: 03/2026 | Testing part: N/A |  |
| Overall Completion level | Study Item: n/a | Core part: 20% | Performance Part: 0% | Testing part: N/A |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN 1 |
| --- | --- | --- |
| Rapporteur (primary) | Name | Xueming Pan |
|  | Company | vivo |
|  | Email | panxueming@vivo.com |
| Rapporteur | Name | Shinya Kumagai |
|  | Company | NTT DOCOMO |
|  | Email | shinya.kumagai.yw@nttdocomo.com |

1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | Yes |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

The WI rapporteurs suggest to increase the core part RAN4 RD (RRM&Demod) TU by 0.25 TU per meeting, starting from 2024Q3 till the completion of the core part, i.e. 2025Q3. The reason for change is due to TU under-estimation given the amount of RAN4 RRM impact according to the existing work item objectives.

2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

2.1	RAN1

2.1.1	Agreements

LP-WUS and LP-SS design

- RAN1#116bis

Agreement

For OOK-4 with M >1, support M=2 & M=4 (working assumption) for LP-WUS.

- FFS whether value of M depends on SCS

- FFS M=1 for OOK-4

Agreement

For evaluation purpose on LP-WUS, companies report the overlaid OFDM sequence(s), including:

- Sequence(s) generation and how sequence(s) map in time or frequency domain (including any details with multiplexing and IFFT).

- Number of candidate overlaid OFDM sequences used for information conveying

  - Including details on whether the number of candidate overlaid sequences is per OFDM symbol or per OOK symbol

- How the proposed sequence design is processed by OFDM-based LP-WUR, e.g., in time domain or in frequency domain or in both time and frequency domain.

Agreement

Support to specify multiple binary LP-SS sequences for the ‘ON-OFF’ pattern:

- The LP-SS sequence used in a cell is

  - Option 1: a sequence is configured

  - Option 2: a sequence is determined by predefined rule

  - FFS: Whether both options will be supported or only one will be supported

- FFS: the number of LP-SS sequences

- Note: Multiple sequences are used to differentiate LP-SS from different cells

Agreement

From RAN1 perspective, support X PRBs for LP-WUS and LP-SS with SCS 30kHz (blanked guard RBs are not included) for a channel bandwidth equal or larger than 5MHz

- X to be down-selected between 11 and 12 PRBs

- FFS the number of PRBs for 15kHz

- FFS if other number of PRBs needed, for LP-SS and LP-WUS with a channel bandwidth equal or less than 5MHz

FFS: Whether the above is applicable to FR2

Agreement

For timing error evaluation purpose, the following two options for residual frequency error are considered:

- Option 1: The maximum frequency error (Fe) of RTC/oscillator is assumed, companies report Fe value and the applied LP-WUR type.

- Option 2: The residual frequency error (Fr) after frequency error correction/clock calibration by LR or after assistance from MR is assumed, companies report Fr value, how to achieve it and the applied LP-WUR type.

Agreement

For frequency error evaluation purpose, the following two options for residual frequency error are considered:

- Option 1: The maximum frequency error (Fe) of oscillator is assumed, companies report Fe value and the applied LP-WUR type.

- Option 2: The residual frequency error (Fr) after frequency error correction by LR or after assistance from MR is assumed, companies report Fr value, how to achieve it and the applied LP-WUR type.

Working Assumption

Support the following options for LP-SS

- Option 1: OOK-1

- Option 2: OOK-4 with M=2,4, FFS:1,8,16

  - FFS whether value of M depends on SCS

- The SCS of a CP-OFDM symbol used for LP-SS generation is the same as that used for LP-WUS generation

- FFS how OOK-1 and OOK-4 are specified

Agreement

Regarding the LP-WUS information for idle/inactive UEs, at least consider the following：

- Option 1: A bitmap with each bit corresponding to [one or more] subgroups

- Option 2: A codepoint value corresponding to one or more subgroup(s)

- Option 3: Multiple codepoint values with each corresponding to one or more subgroup(s)

- Combination of above options are not precluded

- FFS how to carry LP-WUS information, e.g., by encoded bits (with/without CRC) and/or by OOK sequence selection for ‘ON-OFF’ pattern for OOK symbols of LP-WUS.

- FFS how to carry LP-WUS information by overlaid OFDM sequences.

  - It doesn’t preclude considering the configuration where a single candidate overlaid OFDM sequence is used

- Other options are not precluded

Agreement

Regarding the LP-WUS information to trigger PDCCH monitoring of RRC connected UEs, at least consider the following：

- Option 1: A bitmap with each bit corresponding to [one or more] UEs

- Option 2: A codepoint value corresponding to one or part of UE identity, e.g., C-RNTI

- Option 3: A codepoint value corresponding to [one or more] UEs

- Option 4: Multiple codepoint values with each corresponding to [one or more] UE(s)

- Option 5: Multiple bit blocks with each corresponding to [one or more] UE(s)

- Combination of above options are not precluded.

- FFS how to carry LP-WUS information, e.g, by encoded bits (with/without CRC) and/or by OOK sequence selection for ‘ON-OFF’ pattern for OOK symbols of LP-WUS.

- FFS how to carry LP-WUS information by overlaid OFDM sequences.

  - It doesn’t preclude considering the configuration where a single candidate overlaid OFDM sequence is used

- FFS details of LP-WUS information to trigger PDCCH monitoring (e.g. whether above is applicable to one or more serving cells)

Conclusion:

For calibration purposes, companies are encouraged to report the SNR to achieve the coverage of PUSCH for message3, at least with the following assumptions:

- Carrier frequency: 2.6 GHz

- The number of Tx chains: 1

- MIL of MSG 3: use the average one in R17 coverage, i.e.,153.51 dB for non-redcap UE

- Transmit antenna gain correction factors for WUS: up to company report

- Noise Figure: All three values +2dB, +5dB, +8dB on top of NF of MR (7dB) are to be reported, SNR for different assumptions on NF are determined separately

Agreement

For the purpose of further study and evaluation in RAN1, the following candidate sequences for the overlaid OFDM sequence are considered:

- Gold sequence

- M-sequence

- ZC sequence

- Chirp sequence

- Walsh sequence

- Golay sequence

- Kasami sequence

- Low density sequence

- DFT/FFT sequence

- QAM symbol-based sequence

- Combinations and optimizations of above are not precluded

Companies are encouraged to provide an assessment on performance, required complexity, and power consumption to support their preferred sequence. Companies are encouraged to provide details on their preferred sequence (e.g. references).

Agreement

Regarding the overlaid OFDM sequence(s) of LP-WUS, consider the following options:

- Option 1: Single overlaid sequence is on each OOK ‘ON’ symbol or OFDM symbol duration. OFDM-based LP-WUR can obtain the whole information bits by the presence of the overlaid sequence.

- Option 1-2: The overlaid OFDM sequence is pre-determined from multiple sequences. This sequence carry NO information bits of LP-WUS. OFDM-based LP-WUR can obtain the whole information bits by the OOK ON/OFF pattern.

- Option 2: One sequence is selected from multiple candidates overlaid OFDM sequences on each OOK ‘ON’ symbol or OFDM symbol duration, and OFDM-based LP-WUR obtain LP-WUS information at least by overlaid OFDM sequence(s). Consider the following two sub-options.

- Option 2-1: The overlaid OFDM sequence(s) carry part of information bits of LP-WUS. OFDM-based LP-WUR can obtain the whole information bits by OFDM sequence(s) and location of the OFDM sequence(s)/OOK symbols.

- Option 2-2: The overlaid OFDM sequence(s) carry all information bits of LP-WUS. OFDM-based LP-WUR can obtain the whole information bits by the overlaid OFDM sequence(s)

- Option 3: One sequence is selected from multiple candidates overlaid OFDM sequences on one or more OOK ‘ON’ symbols, and OFDM-based LP-WUR obtain LP-WUS information at least by overlaid OFDM sequence(s).

- Option 4: Use of modulated overlay sequence with constellation point: overlay sequence acting as a spreading sequence and constellation point carrying information for OFDM-based LP-WUR.

- Other options are not precluded.

- RAN1#117

Agreement

Support overlaid OFDM sequence based on existing NR sequence type for LP-WUS

- Down select among gold sequence, m sequence and ZC sequence

  - FFS the overlaid OFDM sequence is time or frequency domain sequence.

  - FFS how to reuse the existing sequences

  - Note: Strive to minimize the impact on OOK detection performance

- If overlaid OFDM sequence is supported for LP-SS, the same sequence type is used for both LP-SS and LP-WUS

Agreement

For the LP-SS sequence used in a cell,

- Option 1: the information necessary for determining the sequence is explicitly configured

- FFS: Additional support of determining the sequence by predefined rule without configuration

Agreement

The LP-WUS and LP-SS design assumes the residual frequency error after frequency error correction without considering impact of drift, is up to X ppm for OOK-based LP-WUR.

- FFS X which is no larger than 20ppm

- Initial frequency error assumption: up to company report

For the overlaid OFDM sequence design of LP-WUS, it is assumed that the residual frequency error for OFDM-based LP-WUR after frequency error correction without considering impact of drift is not larger than Y.

- FFS Y which is no larger than 20ppm and lower than X

- Initial frequency error assumption: up to company report

Agreement

For overlaid OFDM sequence(s) for LP-WUS in time or frequency domain, down-selection from the following:

- Option 1-1: overlaid sequence(s) are the sequence(s) of an OOK on symbol before DFT/LS processing

  - The length of overlaid sequence(s) depends on the number of REs used for LP-WUS and the value of M

- Option 1-2: overlaid sequence(s) are the sequence(s) of an OFDM symbol before DFT/LS processing

  - The length of overlaid sequence(s) depends on the number of REs used for LP-WUS

- Option 2: overlaid sequence(s) are the sequence(s) of an OFDM symbol before IFFT processing

  - The length of overlaid sequence(s) depends on the number of REs used for LP-WUS

- Option 3: overlaid sequence(s) are the sequence(s) of an OOK on symbol in time domain after IFFT processing

  - The length of overlaid sequence(s) depends on IFFT size and the value of M

FFS: same or different options are applied for OOK-1 and OOK-4 M>1.

Agreement

Update agreement in last meeting as below:

From RAN1 perspective, support X PRBs for LP-WUS and LP-SS with SCS 30kHz (blanked guard RBs are not included) for a channel bandwidth equal or larger than 5MHz

- X to be down-selected between = 11 and 12 PRBs

- FFS the number of PRBs for 15kHz

- FFS if other number of PRBs needed, for LP-SS and LP-WUS with a channel bandwidth equal or less than 5MHz

FFS: Whether the above is applicable to FR2

Agreement

Further down-select the number of binary LP-SS sequences for the ‘ON-OFF’ pattern:

- 3

- 4

- 8

- 16

Agreement

For the binary LP-SS sequence type for the ‘ON-OFF’ pattern in a LP-SS, further down-selection from the following:

- Gold sequence

- M sequence

- Computer searched sequence

- FFS: the length of LP-SS sequence

Agreement

The following SNR values for LP-WUR are reported by companies to achieve coverage of PUSCH for message3 for difference noise figures for calibration purposes:

- NF of LR = NF of MR+ 8dB: the reported SNR value range is [-9.05,2.94] dB

- NF of LR = NF of MR+ 5dB:  the reported SNR value range is [-6.5, 5.58] dB

- NF of LR = NF of MR+ 2dB: the reported SNR value range is [-4.04,7.95] dB

- Note 1: The above is observed based on the following assumptions used for calibration:

-	Carrier frequency: 2.6 GHz

-	The number of Tx chains: 1

-	MIL of MSG 3: use the average one in R17 coverage, i.e.,153.51 dB for non-redcap UE

-	Transmit antenna gain correction factors for WUS: up to company report

-	Noise Figure: All three values +2dB, +5dB, +8dB on top of NF of MR (7dB) are to be reported, SNR for different assumptions on NF are determined separately

Agreement

For evaluation of LP-WUS and LP-SS design to achieve coverage of PUSCH for message3 from RAN1 perspective, at least the following SNR values should be considered:

- SNR=-3dB for NF of LR = NF of MR+ 8dB

- SNR= -0.5dB for NF of LR = NF of MR+ 5dB

- SNR=2dB for NF of LR = NF of MR+ 2dB

- Note 1: The NF of MR is assumed as 7dB

Agreement

For RRC idle/inactive state, down-select among the following options for at least indicating subgroup information using LP-WUS:

- Option 1: A LP-WUS indicates a bitmap with each bit corresponding to one subgroup of N subgroups for part of, one or more PO(s), e.g., N is 8~16, 24

  - Number of information bits for a LP-WUS is at least N, single LP-WUS to wake up one or more subgroups

- Option 2: A LP-WUS indicates a codepoint value corresponding to one or more subgroup(s) from N subgroups for part of, one or more POs, e.g., N is 8~256

  - Number of information bits for a LP-WUS is at least ceil (log2(X1)), where X1 is the number of codepoints indicating one or more subgroups. X1 is reported by companies, X1 could be smaller, equal to or larger than N.

- Option 3: A LP-WUS indicates multiple codepoint values with each corresponding to one or more subgroup(s) from N subgroups for part of, one or more POs, e.g., N is 8~256

  - Number of information bits for a LP-WUS is at least K*ceil (log2(X2)), where X2 is the number of codepoints indicating one or more subgroups. X2 is reported by companies, X2 could be smaller, equal to or larger than N.

  - K is the number of multiple codepoint values in a LP-WUS where K is larger than 1

- How to satisfy FAR is reported by companies, e.g., FEC/CRC

- Note: multiple TDMed LP-WUSs can be used to support more subgroups for each option.

- Note: Y% effective paging rate per PO is reported by companies

- The followings are considered when down-select among options:

  - The number of supported UE subgroups per PO: M

  - Average network overhead to indicate the number of UE subgroups M per PO

  - False wake up rate due to subgroup-based indication, which will impact the power saving gain

  - Paging latency

  - Note: Coverage target shall be met under 1%BLER, 1% FAR (for false alarm from noise)

LP-WUS operation in IDLE/INACTIVE modes

- RAN1#116bis

Agreement

For multi-beam operation of LP-WUS, UE assumes the same LP-WUS information payload is repeated in all transmitted beams corresponding to LP-WUS

- the selection of the beam(s) for the reception of the LP-WUS is up to UE implementation

Agreement

Each LO consists of N * K LP-WUS MOs, where N is the number of beams corresponding to LP-WUS, and K is the number of LP-WUS MOs for each beam.

- Option 1: K = 1

- Option 2: K can be larger than or equal to 1

  - FFS if more than 1 LP-WUS is transmitted from the same beam, whether the information in these multiple LP-WUS is always the same or can be different

Agreement

From RAN1 perspective, at least the following metrics can be supported for RRM serving cell measurement performed by OOK-based receiver based on LP-SS:

- LP-RSRP

  - LP-RSRP is the linear average of received power of LP-SS in OOK ON symbols.

    - FFS: How to determine the received power of LP-SS in OOK ON symbols

- LP-RSRQ

  - LP-RSRQ = LP-RSRP/LP-RSSI

  - For the definition of LP-RSSI for determination of LP-RSRQ, further consider the following options:

    - Option 1: LP-RSSI is the linear average of total received power in all LP-SS OOK symbols.

    - Option 2: LP-RSSI is the linear average of total received power in LP-SS OOK OFF symbols.

    - Option 3: LP-RSSI is the linear average of total received power in LP-SS OOK ON symbols.

- FFS: LP-SINR, Power ratio of OOK-ON symbol and OOK-OFF symbol

Note: RAN1 will send an LS to RAN2 and RAN4 on the measurement metrics that can be supported from RAN1 perspective, to facilitate RAN2/RAN4 discussions. The exact metrics for OOK-based receiver to be used and defined in the specifications depend on the outcome of [RAN1]/RAN2/RAN4 discussions.

Working Assumption

From RAN1 perspective, for the entry/exit conditions for LP-WUS monitoring in IDLE/inactive mode,

- The UE may start LP-WUS monitoring if

  - the serving cell measurement performed by the MR is above entry threshold(s), if configured by the gNB, and/or

  - FFS other conditions, and if any, whether all or one or some of the conditions need to be satisfied

- If UE starts LP-WUS monitoring, it may stop the legacy PO monitoring before UE receives LP-WUS indicating wake-up

- The UE monitors the legacy PO (and may monitor PEI) and may stop LP-WUS monitoring if

  - the serving cell measurement performed by the LR is below exit threshold(s), if configured by the gNB, and/or

  - FFS other conditions, and if any, whether all or one or some of the conditions need to be satisfied

- FFS the serving cell measurement metrics

- The entry/exit thresholds can be configured separately for different types of LR

- It is left to RAN2 discussion whether the threshold(s) are always configured by the gNB.

- Note: This may be revisited based on the RAN2/RAN4 discussion.

Conclusion

LP-SINR is not considered further as a metric for RRM serving cell measurement for OOK-based receiver.

- RAN1#117

Conclusion

Regarding the “FFS: How to determine the received power of LP-SS in OOK ON symbols” for LP-RSRP, no additional work in RAN1.

Agreement

It is supported that UEs monitoring the same PO are divided into multiple subgroups, where LP-WUS can provide wake-up indication for each subgroup. Consider the following options:

- Option 1: UEs monitoring the same PO monitor the same LO

- Option 2: UEs corresponding to different POs monitor the same LO

- Option 3: UEs monitoring the same PO are divided into multiple sets of subgroups, with UEs within each set of subgroups monitoring the same LO.

- Combinations of the above options can be considered.

Agreement

For LP-SS based LP-RSRQ, LP-RSRP and LP-RSSI are measured within the same bandwidth.

Agreement

For LP-RSSI definition for LP-RSRQ, the following is agreed

- Option 1: LP-RSSI is the linear average of total received power in ON and OFF LP-SS OOK symbols.

Note: Above does not constrain LP-SS sequence design for OOK

Working Assumption

From RAN1 perspective, for the RRM measurement metrics based on SSS for OFDM-based LP-WUR, use the same definition of SS-RSRP and SS-RSRQ for LP-SSS-RSRP and LP-SSS-RSRQ, respectively.

- Above is applicable for both time-domain processing or frequency-domain processing

- Above does not imply that RAN1 will introduce LP-SSS-RSRP and LP-SSS-RSRQ in the specifications

Agreement

For idle/inactive mode, the maximum number of information bits (excluding CRC) in a LP-WUS is Z, where Z <= [8 or 16].

- FFS the exact value of Z

Agreement

For idle/inactive mode, the maximum number of subgroups per PO is X, where 8 <= X <= 256.

- FFS the exact value of X.

LP-WUS operation in CONNECTED modes

- RAN1#116bis

Agreement

Update the following agreement in RAN1#116 in red:

Agreement

- For RRC CONNECTED mode, from RAN1 perspective, further study following LP-WUS procedures to trigger PDCCH monitoring:

  - Case 1: PDCCH monitoring is triggered by LP-WUS with C-DRX configuration

    - Option 1-1: LP-WUS monitoring according to the LP-WUS monitoring configuration before drx-onDurationTimer to trigger the starting of the drx-onDurationTimer.

      - This option may replace DCP functionality

    - Option 1-2: LP-WUS monitoring outside at least legacy C-DRX active time according to the LP-WUS monitoring configuration to trigger PDCCH monitoring.

      - PDCCH monitoring possibly irrespective of drx-onDurationTimer

        - Option 1-2-1: PDCCH monitoring may be additionally triggered based on legacy C-DRX cycle and drx-onDurationTimer when monitoring LP-WUS

          - If this is adopted, it should be configured together with Option 1-1 to achieve power saving gain compared to legacy C-DRX

        - Option 1-2-2: PDCCH monitoring is not triggered by legacy C-DRX cycle and drx-onDurationTimer when monitoring LP-WUS

    - Option 1-3: LP-WUS monitoring inside at least legacy C-DRX active time according to the LP-WUS monitoring configuration to trigger PDCCH monitoring.

  - Case 2: PDCCH monitoring is triggered by LP-WUS without C-DRX configuration. LP-WUS can be monitored at any time according to the LP-WUS monitoring configuration

    - FFS duty-cycled and/or continuous LP-WUS monitoring

- Combination of options in Case 1 and combination of options in Case 1 and Case 2 are not precluded should be considered.

- RAN1 does not discuss C-DRX related timers other than drx-onDurationTimer, this topic is up to RAN2

- Note: Above does not preclude to support fallback mechanism to trigger PDCCH monitoring, if any

Agreement

For RRC CONNECTED mode, support UE capability report for determination of minimum time gap between LP-WUS reception and MR to start PDCCH monitoring.

- FFS: exact value(s) of the minimum time gap

- FFS: support of multiple minimum time gaps

- FFS whether the reported value includes the duration for time/frequency synchronization of MR

Agreement

For LP-WUS monitoring in RRC CONNECTED mode, a LP-WUS is QCLed with existing NR signal/channel/CORESET for the TCI state

- FFS which existing NR signal/channel/CORESET is the QCL source of LP-WUS

- FFS exact definition of QCL relationship between LP-WUS and existing NR signal/channel/CORESET

Agreement

LP-WUS monitoring occasions (MOs) are configured by RRC, where UE can monitor for LP-WUS transmission in RRC CONNECTED mode.

- FFS whether to define a time window for Mos

- It is at least supported that a UE can monitor MOs with a periodicity.

  - FFS details of the periodicity, e.g. derived from DRX cycle, separately configured

Agreement

Study whether/how LP-WUS works when UE is configured with CA in RRC CONNECTED mode

- FFS: The cell(s) where PDCCH monitoring triggered by a LP-WUS is applicable

  - Option 1: one or more serving cells based on gNB indication/configuration

  - Option 2: all activated serving cells

  - Note: other options are not precluded

Agreement

For RRC CONNECTED mode, LP-WUS can be configured without following existing features.

- Rel-16 DCP

- Rel-17 PDCCH skipping

- Rel-17 SSSG switching

- Rel-18 cell DTX

Further study whether/how LP-WUS works with following existing features (PDCCH skipping, SSSG switching, cell DTX)

2.1.2	Remaining Open issues

- Remaining details of an LP-WUS design commonly applicable to both IDLE/INACTIVE and CONNECTED modes

  - Remaining details of OOK (OOK-1 and/or OOK-4) based LP-WUS with overlaid OFDM sequence(s) over OOK symbol

- For IDLE/INACTIVE modes

  - Remaining details of procedure and configuration of LP-WUS indicating paging monitoring triggered by LP-WUS, including at least configuration, sub-grouping and entry/exit condition for LP-WUS monitoring

  - Remaining details of LP-SS with periodicity with Yms for LP-WUR, for synchronization and/or RRM for serving cell.

    - Remaining details of LP-SS based on OOK-1 and/or OOK-4 waveform with or without overlaid OFDM sequences.

- For CONNECTED mode, remaining details of procedures to allow UE MR PDCCH monitoring triggered by LP-WUS including activation and deactivation procedure of LP-WUS monitoring

2.2	RAN2

2.2.1	Agreements

Procedure and configuration of LP-WUS in RRC_IDLE INACTIVE

RAN2#125bis

The LP-WUS related configuration for IDLE/INACTIVE state is provided via system information. FFS if dedicated configuration is needed.

Working assumption: the LP-WUS configuration in SIB at least includes the following information:

- -	LP-SS configuration

- -	LP-WUS configuration

- -	FFS on Entry/exit condition for LP-WUS monitoring

The PEI subgrouping method is taken as baseline for LP-WUS subgrouping, i.e. CN assigned and UE_ID based subgrouping. FFS the maximum number of subgroups.

RAN2#126

RAN2 will further discuss the details about LP-WUS monitoring entry/exit conditions based on RAN1’s existing working assumptions.

The LP-WUS related configuration in SIB at least include the following information for IDLE/INACTIVE:

- -	LP-SS configuration

- -	LP-WUS configuration

- -	Entry/exit condition for LP-WUS monitoring (FFS if it always configured)

Baseline for entry condition definition: If the serving cell quality, e.g. RSRP, RSRQ from MR, is above threshold(s) (if configured), UE may start to monitor LP-WUS, if UE monitors LP-WUS, it may stop monitoring the legacy PO. FFS if any measurement from LR is needed.

Baseline for exit condition definition: If the serving cell measurement result based on LR is below a threshold (if configured), UE monitors PO as in legacy and it may stop monitoring the LP-WUS.

RAN2 understand that if UE is configured with CN-based LP-WUS subgrouping, it is up to CN to assign the LP-WUS subgroup ID to the UE.

RAN2 assume the maximum number of subgroups that can be configured for LP-WUS subgrouping is no less than 8.

From RAN2 perspective, no new procedure is introduced for SI reception/updates.

RRM measurement relaxation and offloading in RRC_IDLE INACTIVE

RAN2#126

For serving cell measurement offloading (i.e., serving cell measurement fully offloaded to LR  and no serving cell measurement via MR is required), RAN2 should focus on specifying the offloading criterion for serving cell for UEs supporting LP-WUS, and assume that RAN4 will define the measurement offloading requirements for serving cell.

RAN2 understand that the RRM measurement of the neighboring cell can only be performed by MR. Can discuss again if RAN1 inform us otherwise.

RAN2 will further discuss the neighbor cell measurement relaxation criteria (if the UE is using LR to measure the serving cell), e.g., considering reuse Rel-16 criteria for ‘not at cell edge’ and ‘low mobility’.

Procedures for LP-WUS in RRC_CONNECTED

RAN2#126

In RRC_CONNECTED mode, RAN2 to further discuss the impacts of LP-WUS operation methods identified in RAN1.

For Option 1-1 (as described in RAN1 agreement), the LP-WUS monitoring occasion locates at a configured time offset before the start of drx-onDurationTimer. The range of time offset can be determined by RAN1.

For Option 1-1, RAN2 assumes the solutions/ operations introduced for DCP mechanism is taken as baseline.

RAN2 assume that legacy DCP and Option 1-1 is not configured simultaneously for a UE.

The LP-WUS related configuration for RRC CONNECTED state UE is provided via dedicated RRC message.

2.2.2	Remaining Open issues

- For IDLE/INACTIVE modes

  - Remaining details of procedure and configuration of LP-WUS indicating paging monitoring triggered by LP-WUS, including at least configuration, sub-grouping and entry/exit condition for LP-WUS monitoring

  - Remaining details of RRM relaxation of UE MR for both serving and neighbor cell measurements, and UE serving cell RRM measurement offloaded from MR to LP-WUR, including the necessary conditions

- For CONNECTED mode, remaining details of procedures to allow UE MR PDCCH monitoring triggered by LP-WUS including activation and deactivation procedure of LP-WUS monitoring

2.3	RAN3

2.3.1	Agreements

2.3.2	Remaining Open issues

2.4	RAN4

2.4.1	Agreements

LP-WUS RRM

- RAN4#110bis

Issue 2-1-1: Core requirements to be specified for LP-WUR measurement

Agreement:

  - At Rel-19 LP-WUR WI, for LP-WUR measurement, RAN4 specifies measurement requirements for the following:

    - Measurement requirements for LP-WUR serving cell measurement based on LP-SS at Idle/Inactive state

    - Measurement requirements for LP-WUR serving cell measurement based on existing PSS/SSS at Idle/Inactive state

  - Other related requirements are FFS

Issue 2-2-1: Methodology on specifying LP-WUR RRM requirements at Idle/Inactive mode

Agreement:

  - The outcome of RAN1’s study in Rel-19 WI on SNR/SINR target is used as the starting point for RAN4 LP-WUR requirement study.

  - Aspects not considered in RAN1 (if any) can be further discussed in RAN4.

Issue 2-2-3: Accuracy

Agreement:

RAN4 requirement for LP-WUR RRM measurement in Idle/inactive states:

- RAN4 to discuss the simulation assumptions considering the target SNR, accuracy and number of samples.

- RAN4 will define the delay requirement in core part, and further discuss whether/how to define the accuracy requirement.

- RAN4#111

Issue 1-1-2: Cases/states to be considered for RRM relaxation

Agreement: Discuss the RAN4 requirements first for the following case #1, and FFS for case #2 to #5.

| RRM measurement case index | MR serving cell measurement | MR neighboring cell measurement | LR measurement |
| --- | --- | --- | --- |
| #1 Fully offloading case | Off | Off: FFS the condition and the details | ON |

Issue 1-4-2: LP-SS periodicity for evaluation

Agreement:

- As starting point for RAN4 simulation purpose, uses 320 ms as the LP-SS periodicity in IDLE/Inactive mode, and other values based on RAN1/2 discussion are not precluded.

  - Not consider the MR DRX configuration in the simulation.

Issue 1-4-5: Simulation assumptions

Agreement:

First start the simulation for FR1, and FFS for FR2 pending on the RF progress.

LP-WUS UE RF

- RAN4#110bis

Sub-topic 2-1 General for UE RF

Issue 2-1-1: Operation bands for LP-WUR

Agreement:

- Focus on FR1 licensed bands

  - FR2 is not precluded

- FFS on which licensed bands will be chosen as example bands for band specific requirements

Issue 2-1-3: one or multi-Sets of requirements

Way forward:

  - Discussions could be separated for OOK-based receiver and OFDM-based receiver. And latter stage to discuss whether a generic requirement could be defined, or not.

    - Further discuss the possibility of a dedicated set of requirements for OOK LP-WUR

    - FFS whether there is any other factors should be considered for LP-WUR requirements discussion

  - SNR and NF may be different for these two types.

Issue 2-1-4: Performance metric for Rx requirements

Way forward:

  - Agreements have been reached in issue 2-2-1.

Issue 2-1-5: Rx antenna assumption for LP-WUR

Way forward:

  - 1Rx is supported in FR1. FFS RX diversity

Issue 2-1-6: CBW and RB number for LP-WUR

Way forward:

  - This can be discussed in simulation assumption for ACS/ASCS.

Issue 2-1-7: SCS for LP-WUR

Way forward:

  - The SCS of LP-WUS is considered same as in-band NR signals

Issue 2-1-8: sync raster for LP-WUR

Way forward:

  - No sync raster is needed for LP-WUR based on RAN1 design

Sub-topic 2-2 REFSENS requirements

Issue 2-2-1: Performance metric for REFSENS

Agreement:

- Use X% missed detection rate as the starting point for performance metric for LP-WUS RF requirements

  - FFS on X values

  - FFS on whether to have false alarm rate

Issue 2-2-2: How to specify REFSENS requirements

Agreement:

- Reuse legacy approach to derive REFSENS, further discuss SNR, NF, IM

  - FFS whether to design REFSENS requirements or other requirements to ensure LP-WUR meet the coverage target

  - Side condition for REFSENS test: DL test signal will only have LP-WUS signal.

Issue 2-2-3: How to decide SNR value (not requirement) for REFSENS

Way forward:

  - After concluding WUS design in RAN1, the SNR to specify REFSENS requirements should be decided in RAN4

Issue 2-2-4: How to decide NF value (not requirement) for REFSENS

Way forward:

  - Encourage companies input on NF analysis for different LP-WUR types

Issue 2-2-5: RF/Antenna Architecture considerations for LP-WUS receiver

Way forward:

  - Detailed antenna/RF architecture can be considered in issue 2-2-4 analysis

  - FFS other RAN4 impacts than REFSENS

Sub-topic 2-3 ASCS requirements

Issue 2-3-1: Simulation work for ASCS

Agreement:

- LLS simulation for ASCS is sufficient

  - The same level PSD for LP-WUS and NR signals is assumed

Issue 2-3-2: Detailed Methodology for simulation to evaluate ASCS value and guard RB

Way forward:

  - Use SI assumption as a starting point, further confirm and align the simulation parameters in the group next meeting

    - Metric for link-level simulation should be aligned

    - Alignment required on RF impairments before starting the work.

Issue 2-3-3: ASCS requirements value

Way forward:

  - FFS ASCS requirements value

Issue 2-3-4: Required number of guard RB

Way forward:

  - FFS required number of guard RB for ASCS

Issue 2-3-5: RF impairment considerations for ASCS

Way forward:

  - RF impairments can be considered and aligned for ASCS simulation

Issue 2-3-6: Side conditions for ASCS test

Way forward:

  - LP-WUS along with required guard RBs is packed with NR legacy DL signal on both sides.

Sub-topic 2-4 ACS requirements

Issue 2-4-1: coexistence System-level simulation to evaluate ACS

Agreement:

- The same interference level as for main radio is assumed for LP-WUR

  - Guard RB number needs be evaluated by link level simulation for ACS requirements

Issue 2-4-2: Link-level simulation to evaluate ACS

Way forward:

  - Use SI assumption as a starting point, further confirm and align the simulation parameters next meeting

    - Necessary update and alignment on parameters, e.g., number of WUS RB, RF impairment, ADC bit, and performance metric is required before starting the work.

Issue 2-4-3: ACS requirements value

Way forward:

  - FFS ACS requirements value

Issue 2-4-4: How to evaluate required guard RB for ACS case

Way forward:

  - FFS required number of guard RB for ACS

Issue 2-4-5: Detailed coexistence System-level simulation assumptions

Way forward:

  - System-level simulation is not needed for ACS evaluation

Sub-topic 2-5 other UE RF requirements

Issue 2-5-1: Any other Rx requirements should be specified

Agreement:

- Apart from REFSENS, ACS, Rx requirements of IBB, OBB, intermodulation as well as spurious emissions should be specified for LP-WUR.

- Other legacy receiver requirements are not precluded

Sub-topic 2-6 Testability for UE RF requirements

Issue 2-6-1: Test metric for LP-WUR receiver

Way forward:

  - Test metric should be aligned with performance metric for Rx requirements

- RAN4#111

Sub-topic 1-1 General and system parameters

Issue 1-1-1: Operation bands for LP-WUS feature

Agreements:

RAN4 confirm LP-WUS is a general feature not limited to specific example band(s)

Issue 1-1-2: FR1 example bands for requirements as phase 1

Agreements:

No need to list specific example band(s)

Issue 1-1-5: Rx diversity gain assumption for LP-WUR

Agreements:

No diversity gain as baseline for FR1. More Rx could be implementation choice

Issue 1-1-7: Channel raster for LP-WUR

Agreements:

RAN4 further check whether no channel raster is needed for LP-WUR.

Issue 1-1-9: Side condition for LP-WUR requirements

Agreements:

  - FFS RAN4 should consider both idle and connected mode conditions in the side conditions for the LPWUR requirements.

Issue 1-1-11: Metric for LP_WUR requirements

Agreements:

  - MDR is used as the metric for LP_WUR Rx RF requirements

    - FFS MDR values.

  - FAR will be defined as demodulation requirements

    - FFS FAR values.

Sub-topic 2-1 Alignment of LLS parameters to specify ACS/ASCS requirements

Issue 2-1-1: Center frequencies for LLS simulation

Agreement:

- For ACS and ASCS simulation, select 900MHz, 2.6GHz and 3.5GHz as example frequencies for FR1.

- FFS on FR2 example band(s)

Issue 2-1-2: Performance metric (MDR/BLER value) for LLS simulation (apply to ACS/ASCS and REFSENS)

Agreement:

- For LLS simulation (apply to ACS/ASCS and REFSENS), the metric includes

  - 1% MDR/BLER as baseline and 5% MDR/BLER as optional

  - The following false alarm rate can be considered

    - 1%

    - 5%

    - Providing the information whether the false alarm rate is considered or not

- Further down-select the performance metric for the requirements and testing

Issue 2-1-3: Waveform for LLS simulation

Agreement:

- Use both OOK1 and OOK4 based on RAN1 agreements for link level simulations

Issue 2-1-4: number of LP-WUS RBs for LLS simulation

Agreement:

- Assume 11PRB for LP-WUS signal with 30kHz SCS for simulations

- FFS on RB number(s) for 15KHz SCS depending on RAN1 decision

Issue 2-1-5: number of ADC assumption for LLS simulation

Agreement:

- Number of ADC assumption for LLS simulation

  - Option 1: 8 bit

  - Option 2: 4 bit

  - Encourage companies to provide simulation results with both options for comparison

Issue 2-1-6: Frequency error assumption for LLS simulation

Agreement

- Frequency error assumption for LLS simulation

  - Up to 20ppm

Issue 2-1-7: Phase noise model for LLS simulation

Agreement:

- Not needed in LLS. Consider as RF impairment of implementation.

Issue 2-1-8: For ASCS, the BWinterferer for ASCS evaluation

Agreement:

- For ASCS evaluation, only 10MHz and 20MHz NR CBWs are assumed and the BWinterferer is

  - All RBs between WUS edge to channel edge

Issue 2-1-10: The overall simulation parameters for LLS simulation

Agreement:

- Wait for RAN1 conclusion on FR2 to decide the parameters for evaluations

- Adopt the following parameters in table below for FR1 simulation evaluations

| Attributes | Assumptions |  |  |
| --- | --- | --- | --- |
| Case name (waveform) | OOK-1 waveform |  | OOK-4 waveform |
| Center frequency | 900MHz, 2.6GHz and 3.5GHz |  |  |
| Channel structure | Total 8/16 bits |  |  |
| Chip rate | M=1 | M=1/2/4 |  |
| WUS duration | FFS |  |  |
| Coding | 1/2 rate Manchester coding |  |  |
| Time error | 0 |  |  |
| Residual Frequency error | 0/10/20 ppm |  |  |
| SCS | 30kHz |  |  |
| UE Channel BW | 20MHz (51 RB)-case 1 10MHz (24 RB)-case 2 5MHz (11 RB)-case 3 |  |  |
| WUS RB | - Fixed 11RB ~ 3.96MHz for 10MHz and 20MHz cases |  |  |
| Position within channel | - For 10/20MHz CBW, Center for ASCS, edge for ACS [assume no ASCS impact] - For 5MHz CBW, fixed center of channel |  |  |
| Guardband of NR channel, both wanted cell and interfer cell (ACS) | - For wanted signal: 505kHz for 5MHz, 665kHz for 10MH, 805kHz for 20Mhz - For interference cell2 5MHz: fixed 505kHz |  |  |
| Guard RB | - For ASCS: 0 or 1RB on each side of LP-WUS bandwidth - For ACS: 1/2/3/4 RB |  |  |
| Filter | - 3th/5th Order lowpass Butterworth matching fixed 3.96MHz RF bandwidth for 10MHz/20MHz case - Other order lowpass filter is not precluded - The filter bandwidth is adapted with actual WUS RBs, for 5MHz case |  |  |
| ASCS | PDSCH mapped on RBs not used for LP-WUS and Guard RB; EPRE of PDSCH /EPRE of LP-WUS = 0 dB Same PSD with WUS signal |  |  |
| ACS | PDSCH mapped on interference RBs (11RB for 5MHz CBW), one side; EPRE of PDSCH /EPRE of in-band LP-WUS = [20~33] dB NOTE: decide the interference level depending on SNR |  |  |
| Wanted signal level | For ACS, REFSENS + 14 dB for LP-WUS |  |  |
| Sampling rate | 7.68MHz |  |  |
| ADC bit width | 4/8 bits ADC for ASCS/ACS Encourage companies to provide simulation results with both options for comparison |  |  |
| Phase noise | Not modelled |  |  |
| Non-linearities | Not modelled |  |  |
| Power boosting | EPRE ratio: 0dB/3dB for OOK-1/OOK-4 NOTE: 3dB is optional for simulation |  |  |
| Channel Model | Option 1: TDL-C 300 Option 2: AWGN Note: encourage companies to provide simulation results with both options |  |  |
| Performance metric | - 1% MDR/BLER as baseline and 5% MDR/BLER as optional - The following false alarm rate can be considered - 1% - 5% Providing the information whether the false alarm rate is considered or not |  |  |

Sub-topic 2-2 REFSENS requirements

Issue 2-2-3: SNR value (not requirement) for REFSENS

Agreement:

  - Stick to last meeting agreements: Wait for RAN1 SNR progress

Issue 2-2-4: NF Gap between LR and MR for REFSENS (assume MR as 9dB)

Agreement:

  - RAN4 further discuss potential NF for OOK-based and OFDM-based receiver.

  - Linked to resolution of Issue 2-2-8

Issue 2-2-6: IM value for REFSENS

Agreement:

  - RAN4 further discuss potential IM value for OOK-based and OFDM-based receiver.

Issue 2-2-8: Whether a baseline architecture is needed for LP-WUS receiver

Agreement:

  - RAN4 will further discuss and decide baseline architecture(s) for OOK-based receiver and OFDM-based receiver.

Sub-topic 2-3 ASCS requirements

Issue 2-3-1: Align the definition of ASCS requirements

Agreement:

  - RAN4 can use the following ASCS definition as starting point.

    - Adjacent SubCarrier Selectivity (ASCS) is a measure of a receiver's ability to receive an LP-WUS signal at its assigned channel frequency in the presence of adjacent subcarrier NR signal(s) at a given frequency offset (guard RB) between LP-WUS and NR. ASCS is the ratio of the receive filter attenuation on the assigned LP-WUR channel frequency to the receive filter attenuation on the adjacent NR subcarrier

Issue 2-3-2: ASCS requirements value

Agreement:

  - RAN4 further discuss whether ASCS requirements value should be specified, or just the test parameter are specified, i.e., LP_WUS and NR signal bandwidths and power levels, and guard RBs.

Issue 2-3-3: Required number of guard RB for ASCS

Agreement:

  - RAN4 further discuss required number of guard RB for ASCS based on LLS simulation outcome next meeting.

Issue 2-3-4: Test case for ASCS

Agreement:

  - RAN4 further discuss detailed test parameters for ASCS.

Sub-topic 2-4 ACS requirements

Issue 2-4-1: Whether to update the ACS definition for LP-WUS

Agreement:

  - RAN4 consider the following clarified ACS requirements definition for LP-WUS as starting point.

    - Adjacent channel selectivity (ACS) is a measure of a receiver's ability to receive an NR LP-WUS signal at its assigned channel frequency in the presence of an adjacent channel signal at a given frequency offset from the center frequency of the assigned channel. ACS is the ratio of the receive filter attenuation on the assigned channel frequency to the receive filter attenuation on the adjacent channel(s).

Issue 2-4-2: ACS requirements value

Agreement:

  - RAN4 consider the how to specify ACS requirements. FFS same as MR.

    - Decision will be made based on LLS outcome

Issue 2-4-3: Required guard RB for ACS case

Agreement:

  - RAN4 further discuss required number of guard RB for ACS based on LLS simulation outcome next meeting.

Sub-topic 3-1 General for UE RF

Issue 3-1-1: IBB and OBB assumption

Agreement:

  - RAN4 further check and discuss whether same interference level of IBB and OBB as MR is assumed for LP-WUR.

    - With assumption that Rx dynamic range for LR may not be as good as MR

Issue 3-1-2: IBB and OBB requirements

Agreement:

  - RAN4 further discuss detailed test condition for IBB and OBB, e.g., location of LP-WUS in-band, MR channel bandwidth, whether guard RBs are allocated, power level.

    - FFS IBB and OBB requirements

Issue 3-1-4: Intermodulation requirements

Agreement:

  - RAN4 further discuss detailed test condition for Intermodulation requirements, e.g., location of LP-WUS in-band, MR channel bandwidth, whether guard RBs are allocated, power level.

    - FFS Intermodulation requirements requirement

Sub-topic 3-2 spurious emissions and response requirements

Issue 3-2-1: Spurious emissions requirements

Agreement:

  - RAN4 conclude Spurious emissions requirements can be reused for LP-WUR.

Issue 3-2-2: spurious response requirements

Agreement:

  - RAN4 further discuss whether spurious response requirements is needed for LP-WUR.

Sub-topic 3-3 Other Rx requirements

Issue 3-3-1: Maximum input level requirements

Agreement:

  - RAN4 further discuss whether maximum input level requirements can be reused for LP-WUR.

    - FFS side condition

Issue 3-3-2: Narrow band blocking requirements

Agreement:

  - FFS RAN4 no need to define narrow band blocking requirements for LP-WUR.

LP-WUS BS RF

- RAN4#110bis

Whether RAN4 evaluation of power boosting needs to wait for RAN1 conclusion

Way forward:

- RAN4 can work in parallel and rely on liaison-based approach to communicate with RAN1 when necessary.

On the definition of dynamic range for LP-WUS (DL power boosting)

Way forward:

- Concept of LP-WUS dynamic range/power boosting is considered as starting point

  - The LP-WUS RB power dynamic range (or LP-WUS power boosting) is the difference between the average power of LP-WUS REs (which occupy certain REs within a NR transmission bandwidth configuration and the average power over all REs (from both LP-WUS and the NR carrier containing the LP-WUS REs).

- With following assumption to guide future discussion, RAN4 will revisit the above concept after BS core requirements are settled.

  - Transmission power is shared between NR OFDM signal and NR LP-WUS for the same carrier.

  - The rated carrier output power and rated total output power are not changed with LP-WUS power boosting.

On what can be considered for the future discussion on dynamic range for LP-WUS (DL power boosting)

FFS:

- The applicable BS types. e.g. BS type 1-C, BS type 1-H and BS type 1-O

- Power degradation of RBs other than LP-WUS signal within the carrier

- Whether BS power boosting is declaration based or not

- Others

- RAN4#111

1.1 Manufacture declaration on LP-WUS

WF

FFS on the following options for next meeting:

- Option 1: Set the LP-WUS power boosting a complete manufacture declaration feature, including whether supporting LP-WUS power boosting and the supported boosting level.

- Option 2: Minimum power boosting level in core specification together with manufacturer declaration in the conformance test specification.

1.2. Concept of LP-WUS dynamic range/power boosting

WF

FFS on the following options for next meeting:

- Option 1: Stick with the one in RAN4#110bis approved WF R4-2406140, which is:

  - The LP-WUS RB power dynamic range (or LP-WUS power boosting) is the difference between the average power of LP-WUS REs (which occupy certain REs within a NR transmission bandwidth configuration and the average power over all REs (from both LP-WUS and the NR carrier containing the LP-WUS REs).

- Option 2: Define EPRE ratio between LP-WUS and NR signals instead of power dynamic range.

1.3. Whether to preclude small CBW for consideration of LP-WUS power boosting

WF

FFS on the following options for next meeting:

- Option 1: Consider a power degradation limit, e.g., 2dB, for validating a configuration for LP-WUS representing by (EPRE ratio, channel bandwidth).

- Option 2: Only consider LP-WUS power boosting for CBWs larger than 20MHz.

- Option 3: Focus on CBWs >= 10MHz and different power boosting values can be considered for different CBW.

- Option 4: Not preclude small CBW for LP-WUS power boosting.

1.4. On applicable BS type for LP-WUS

WF

FFS on the following proposals for next meeting:

- Proposal 1: Not to set restriction on applicable BS types to support LP-WUS.

- Proposal 2: To decide on the applicable BS types after the applicable frequency range and bands for LP-WUS have been decided.

- Proposal 3: Consider BS type 1-C as applicable type to further discuss of LP-WUS power boosting. FFS other BS types.

- Proposal 4: Depend on power boosting level for certain BS type.

  - If power boosting is limited to 3 dB, all BS type 1-C, 1-H and 1-O can be considered.

1.5. On minimum value for LP-WUS power boosting

WF

FFS on the following options for next meeting:

- Option 1: Use [3dB] as starting point

  - It should be considered in conjunction with the supported CBWs.

- Option 2: To consider the power degradation of RBs other than LP-WUS signal within the carrier after the number of LP-WUS RBs have been decided in RAN1.

1.6. Whether a cap for LP-WUS power boosting should be considered

WF

FFS on the following options for next meeting:

- Option 1: Limit to 3dB for BS type 1-C, 1-H and 1-O.

- Option 2: FFS on the value in conjunction with the supported CBWs.

- Option 3: To consider the power degradation of RBs other than LP-WUS signal within the carrier after the number of LP-WUS RBs have been decided in RAN1.

- Option 4: Not consider to cap the LP-WUS power boosting.

1.7. Requirements other than power boosting that should be considered

WF

FFS on the following proposals for next meeting:

- Proposal 1: Unwanted emissions requirements of SEM and spurious emissions should be considered for transmitted signal with LP-WUS and NR in the same carrier.

- Proposal 2: FFS whether transmitted signal quality requirements should be defined for LP-WUS, at least for the EVM requirement.

- Proposal 3: Multi-band requirements at gNB side for LP-WUS.

1.8. Whether UE needs to know BS power boosting information for RRM measurement

WF

This issue will be discussed in RRM session. Not precluded in RF session if identified as necessary.

1.9. Whether to consider FR2 for LP-WUS

WF

- RAN4 focus on FR1 licensed bands for BS RF requirements.

  - FR2 is not precluded for further study.

2.4.2	Remaining Open issues

- RRM：For IDLE/INACTIVE modes

  - Remaining details of procedure and configuration of LP-WUS indicating paging monitoring triggered by LP-WUS, including at least configuration, sub-grouping and entry/exit condition for LP-WUS monitoring.

  - Remaining details of LP-SS with periodicity with Yms for LP-WUR, for synchronization and/or RRM for serving cell.

  - Remaining details of requirements for further RRM relaxation of UE MR for both serving and neighbor cell measurements, and requirements for UE serving cell RRM measurement offloaded from MR to LP-WUR.

- RF：

  - UE RF requirements related issues

  - BS RF requirements related issues

- Demod：

  - Not started

2.5	RAN5

2.5.1	Agreements

2.5.2	Remaining Open issues

2.5.3	Remaining Open issues with cross-WG dependencies

2.6	RAN6

2.6.1	Agreements

2.6.2	Remaining Open issues

3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

3.1	SAx/CTs

3.1.1	Agreements with cross-TSG impacts

3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

4.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

### RAN1#116bis

- R1-2401982	Discussion on LP-WUS and LP-SS Design 	TCL

- R1-2401999	Signal Design of LP-WUS and LP-SS	Huawei, HiSilicon

- R1-2402046	Discussion on LP-WUS and LP-SS Design	FUTUREWEI

- R1-2402114	Discussion on LP-WUS and LP-SS design	Spreadtrum Communications

- R1-2402193	Discussion on LP-WUS design	ZTE, Sanechips

- R1-2402251	Discussion on LP-WUS and LP-SS design	vivo

- R1-2402337	Signal design for LP-WUS and LP-SS	OPPO

- R1-2402392	Design of LP-WUS and LP-SS	CATT

- R1-2402475	Discussion on LP-WUS and LP-SS design	Samsung

- R1-2402519	Discussion on LP-WUS/LPSS design	China Telecom

- R1-2402539	Discussion on LP-WUS and LP-SS Design	EURECOM

- R1-2402544	Discussion on the LP-WUS and LP-SS design	Panasonic

- R1-2402574	Discussion on LP-WUS and LP-SS design	CMCC

- R1-2402610	Discussion on LP-WUS and LP-SS design framework for Low power WUS	InterDigital, Inc.

- R1-2402617	Discussion on LP-WUS and LP-SS design	Everactive

- R1-2402675	Discussion on LP-WUS and LP-SS design	Xiaomi

- R1-2402728	Discussion on LP-WUS and LP-SS design	Honor

- R1-2402740	Discussion on LP-WUS and LP-SS design	Sharp

- R1-2402760	Discussion on LP-WUS and LP-SS design	NEC

- R1-2402890	LP-WUS and LP-SS design	Apple

- R1-2402906	LP-WUS and LP-SS design	Nokia

- R1-2402952	On LP-WUS and LP-SS designs	MediaTek

- R1-2402976	LP-WUS and LP-SS design	Sony

- R1-2403105	Discussion on LP-WUS and LP-SS design	Lenovo

- R1-2403126	Discussion on LP-WUS and LP-SS design	LG Electronics

- R1-2403203	LP-WUS and LP-SS design	Qualcomm Incorporated

- R1-2403253	Discussion on LP-WUS and LP-SS design	NTT DOCOMO, INC.

- R1-2403276	LP-WUS and LP-SS design	Ericsson

- R1-2403376	On LP-WUS and LP-SS design	Nordic Semiconductor ASA

- R1-2403554	Summary #1 of discussions on LP-WUS and LP-SS design	Moderator (vivo)

- R1-2403616	Summary #2 of discussions on LP-WUS and LP-SS design	Moderator (vivo)

- R1-2403751	Summary #3 of discussions on LP-WUS and LP-SS design	Moderator (vivo)

- R1-2401983	Discussion on LP-WUS Operation in IDLE/INACTIVE modes	TCL

- R1-2402000	Procedures and functionalities of LP-WUS in IDLE/INACTIVE mode	Huawei, HiSilicon

- R1-2402047	Discussion on LP-WUS Operation in IDLE/INACTIVE Modes	FUTUREWEI

- R1-2402115	Discussion on LP-WUS operation in IDLE/INACTIVE modes	Spreadtrum Communications

- R1-2402194	Discussion on LP-WUS operation in IDLE/INACTIVE mode	ZTE, Sanechips

- R1-2402252	Discussion on LP-WUS operation in IDLE/INACTIVE modes	vivo

- R1-2402338	Discussion on LP-WUS operation in RRC_IDLE/INACTIVE modes	OPPO

- R1-2402393	System design and procedure of LP-WUS operation for UE in IDLE/Inactive Modes	CATT

- R1-2402476	Discussion on LP-WUS operation in IDLE/INACTIVE modes	Samsung

- R1-2402520	Discussion on LP-WUS operation in IDLE/INACTIVE modes	China Telecom

- R1-2402545	Discussion on LP-WUS operation in IDLE/INACTIVE modes	Panasonic

- R1-2402575	Discussion on LP-WUS operation in IDLE/INACTIVE modes	CMCC

- R1-2402611	Discussion on LP-WUS operation in IDLE/INACTIVE modes	InterDigital, Inc.

- R1-2402676	Discussion on LP-WUS operation in Idle/Inactive modes	Xiaomi

- R1-2402741	Discussion on LP-WUS operation in IDLE/INACTIVE modes	Sharp

- R1-2402761	Discussion on LP-WUS operation in RRC IDLE/INACTIVE mode	NEC

- R1-2402835	Discussion on LP-WUS operation in IDLE/INACTIVE mode	Fujitsu

- R1-2402891	LP-WUS operation in IDLE/INACTIVE modes	Apple

- R1-2402893	Summary #1 on LP-WUS operation in IDLE/INACTIVE mode	Moderator (Apple)

- R1-2402894	Summary #2 on LP-WUS operation in IDLE/INACTIVE mode	Moderator (Apple)

- R1-2402895	Summary #3 on LP-WUS operation in IDLE/INACTIVE mode	Moderator (Apple)

- R1-2402907	LP-WUS operation in ILDE/Inactive mode	Nokia

- R1-2402953	On LP-WUS operation in IDLE/INACTIVE modes	MediaTek

- R1-2402977	LP-WUS operation in IDLE / INACTIVE mode	Sony

- R1-2403027	Discussion on LP-WUS operation in IDLE/INACTIVE modes	ETRI

- R1-2403106	Discussion on LP-WUS operation in Idle/Inactive modes	Lenovo

- R1-2403127	Discussion on LP-WUS operation in IDLE/INACTIVE modes	LG Electronics

- R1-2403138	Views on LP-WUS operation in IDLE/INACTIVE modes	KT Corp.

- R1-2403204	LP-WUR operation in idle and inactive modes	Qualcomm Incorporated

- R1-2403254	Discussion on LP-WUS operation in IDLE/INACTIVE modes	NTT DOCOMO, INC.

- R1-2403277	LP-WUS operation in IDLE and INACTIVE modes	Ericsson

- R1-2403375	On LP-WUS operation in IDLE/Inactive	Nordic Semiconductor ASA

- R1-2402001	Procedures and functionalities of LP-WUS in CONNECTED mode	Huawei, HiSilicon

- R1-2402116	Discussion on LP-WUS operation in CONNECTED modes	Spreadtrum Communications

- R1-2402195	Discussion on LP-WUS operation in CONNECTED mode	ZTE, Sanechips

- R1-2402253	Discussion on LP-WUS operation in CONNECTED modes	vivo

- R1-2402339	Consideration on wake-up procedure in connected mode	OPPO

- R1-2402394	System design and procedure of LP-WUS operation for UE in CONNECTED Modes	CATT

- R1-2402477	Discussion on LP-WUS operation in CONNECTED modes	Samsung

- R1-2402521	Discussion on LP-WUS operation in CONNECTED modes	China Telecom

- R1-2402546	Discussion on LP-WUS operation in CONNECTED mode	Panasonic

- R1-2402576	Discussion on LP-WUS operation in CONNECTED mode	CMCC

- R1-2402588	Discussion on LP-WUS operation in CONNECTED modes	Lenovo

- R1-2402612	Discussion on RRC CONNECTED mode LP-WUS monitoring	InterDigital, Inc.

- R1-2402677	Discussion on LP-WUS operation in Connected mode	Xiaomi

- R1-2402742	Discussion on LP-WUS operation in CONNECTED modes	Sharp

- R1-2402762	Discussion on LP-WUS operation in RRC CONNECTED mode	NEC

- R1-2402892	LP-WUS operation in CONNECTED modes	Apple

- R1-2402908	LP-WUS operation in CONNECTED mode	Nokia

- R1-2402954	On LP-WUS operation in CONNECTED modes	MediaTek

- R1-2402978	LP-WUS operation in CONNECTED mode	Sony

- R1-2403028	Discussion on LP-WUS operation in CONNECTED modes	ETRI

- R1-2403037	Discussion on LP-WUS procedures in Connected mode	TCL

- R1-2403128	Discussion on LP-WUS operation in CONNECTED modes	LG Electronics

- R1-2403205	LP-WUR operation in connected mode	Qualcomm Incorporated

- R1-2403255	Discussion on LP-WUS operation in CONNECTED mode	NTT DOCOMO, INC.

- R1-2403278	LP-WUS operation in CONNECTED mode	Ericsson

- R1-2403517	FL summary #1 on LP-WUS operation in CONNECTED mode	Moderator (NTT DOCOMO)

- R1-2403713	FL summary #2 on LP-WUS operation in CONNECTED mode	Moderator (NTT DOCOMO)

- R1-2402893	Summary #1 on LP-WUS operation in IDLE/INACTIVE mode	Moderator (Apple)

### RAN1#117

- R1-2403864	Discussion on LP-WUS and LP-SS Design	FUTUREWEI

- R1-2403879	Discussion on LP-WUS and LP-SS Design	EURECOM

- R1-2403948	Signal Design of LP-WUS and LP-SS	Huawei, HiSilicon

- R1-2404035	Discussion on LP-WUS and LP-SS design	Spreadtrum Communications

- R1-2404059	Discussion on LP-WUS and LP-SS Design 	TCL

- R1-2404124	Discussion on LP-WUS and LP-SS design	Samsung

- R1-2404186	Discussion on LP-WUS and LP-SS design	vivo

- R1-2404296	LP-WUS and LP-SS design	Apple

- R1-2404312	Discussion on LP-WUS and LP-SS design framework for Low power WUS	InterDigital, Inc.

- R1-2404320	Discussion on LP-WUS and LP-SS design	Everactive

- R1-2404410	Design of LP-WUS and LP-SS	CATT

- R1-2404465	Discussion on LP-WUS and LP-SS design	CMCC

- R1-2404509	LP-WUS and LP-SS design	Sony

- R1-2404563	Discussion on LP-WUS design	ZTE, Sanechips

- R1-2404579	Discussion on LP-WUS and LP-SS design	HONOR

- R1-2404627	Discussion on LP-WUS and LP-SS design	Xiaomi

- R1-2404664	Discussion on LP-WUS and LP-SS design	NEC

- R1-2404705	LP-WUS and LP-SS design	Nokia

- R1-2404760	Discussion on the LP-WUS and LP-SS design	Panasonic

- R1-2404852	Signal design for LP-WUS and LP-SS	OPPO

- R1-2404897	Discussion on LP-WUS and LP-SS design	LG Electronics

- R1-2404942	Discussion on LP-WUS and LP-SS design	Lenovo

- R1-2404966	Discussion on LP-WUS and LP-SS design	Sharp

- R1-2405051	Discussion on LP-WUS and LP-SS design	NTT DOCOMO, INC.

- R1-2405073	LP-WUS and LP-SS design	MediaTek Inc.

- R1-2405108	LP-WUS and LP-SS design	Ericsson

- R1-2405164	LP-WUS and LP-SS design	Qualcomm Incorporated

- R1-2405254	On LP-WUS and LP-SS design	Nordic Semiconductor ASA

- R1-2405499	Summary #1 of discussions on LP-WUS and LP-SS design	Moderator (vivo)

- R1-2405519	Summary #2 of discussions on LP-WUS and LP-SS design	Moderator (vivo)

- R1-2405645	Summary #4 of discussions on LP-WUS and LP-SS design	Moderator (vivo)

- R1-2405708	Summary #5 of discussions on LP-WUS and LP-SS design	Moderator (vivo)

- R1-2403865	Discussion on LP-WUS Operation in IDLE/INACTIVE Modes	FUTUREWEI

- R1-2403949	Procedures and functionalities of LP-WUS in IDLE/INACTIVE mode	Huawei, HiSilicon

- R1-2404036	Discussion on LP-WUS operation in IDLE/INACTIVE modes	Spreadtrum Communications

- R1-2404125	Discussion on LP-WUS operation in IDLE/INACTIVE modes	Samsung

- R1-2404136	Discussion on LP-WUS Operation in IDLE/INACTIVE state 	TCL

- R1-2404187	Discussion on LP-WUS operation in IDLE/INACTIVE modes	vivo

- R1-2404297	LP-WUS operation in IDLE/INACTIVE modes	Apple

- R1-2404299	Summary #1 on LP-WUS operation in IDLE/INACTIVE mode	Moderator (Apple)

- R1-2404300	Summary #2 on LP-WUS operation in IDLE/INACTIVE mode	Moderator (Apple)

- R1-2404301	Summary #3 on LP-WUS operation in IDLE/INACTIVE mode	Moderator (Apple)

- R1-2404313	Discussion on LP-WUS operation in IDLE/INACTIVE modes	InterDigital, Inc.

- R1-2404411	System design and procedure of LP-WUS operation for UE in IDLE/Inactive Modes	CATT

- R1-2404466	Discussion on LP-WUS operation in IDLE/INACTIVE modes	CMCC

- R1-2404510	LP-WUS operation in IDLE / INACTIVE modes	Sony

- R1-2404564	Discussion on LP-WUS operation in IDLE/INACTIVE mode	ZTE, Sanechips

- R1-2404628	Discussion on LP-WUS operation in Idle/Inactive modes	Xiaomi

- R1-2404665	Discussion on LP-WUS operation in RRC IDLE/INACTIVE mode	NEC

- R1-2404706	LP-WUS operation in IDLE/Inactive mode	Nokia

- R1-2404761	Discussion on LP-WUS operation in IDLE/INACTIVE modes	Panasonic

- R1-2404782	Discussion on LP-WUS operation in IDLE/INACTIVE modes	ETRI

- R1-2404805	Discussion on LP-WUS operation in IDLE/INACTIVE mode	Fujitsu

- R1-2404853	Discussion on LP-WUS operation in RRC_IDLE/INACTIVE modes	OPPO

- R1-2404898	Discussion on LP-WUS operation in IDLE/INACTIVE modes	LG Electronics

- R1-2404943	Discussion on LP-WUS operation in Idle/Inactive modes	Lenovo

- R1-2404967	Discussion on LP-WUS operation in IDLE/INACTIVE modes	Sharp

- R1-2405052	Discussion on LP-WUS operation in IDLE/INACTIVE modes	NTT DOCOMO, INC.

- R1-2405074	LP-WUS operation in IDLE/INACTIVE modes	MediaTek Inc.

- R1-2405109	LP-WUS operation in IDLE and INACTIVE modes	Ericsson

- R1-2405165	LP-WUR operation in idle and inactive modes	Qualcomm Incorporated

- R1-2405180	Discussion on LP-WUS operation in IDLE/INACTIVE modes	KT Corp.

- R1-2405253	On LP-WUS operation in IDLE/Inactive	Nordic Semiconductor ASA

- R1-2405681	Summary #4 on LP-WUS operation in IDLE/INACTIVE mode	Moderator (Apple)

- R1-2403950	Procedures and functionalities of LP-WUS in CONNECTED mode	Huawei, HiSilicon

- R1-2404001	Discussion on LP-WUS procedures in Connected mode	TCL

- R1-2404037	Discussion on LP-WUS operation in CONNECTED modes	Spreadtrum Communications

- R1-2404126	Discussion on LP-WUS operation in CONNECTED modes	Samsung

- R1-2404188	Discussion on LP-WUS operation in CONNECTED modes	vivo

- R1-2404298	LP-WUS operation in CONNECTED modes	Apple

- R1-2404314	Discussion on RRC CONNECTED mode LP-WUS monitoring	InterDigital, Inc.

- R1-2404412	System design and procedure of LP-WUS operation for UE in CONNECTED Modes	CATT

- R1-2404440	Discussion on LP-WUS operation in CONNECTED modes	Lenovo

- R1-2404467	Discussion on LP-WUS operation in CONNECTED mode	CMCC

- R1-2404511	LP-WUS operation in CONNECTED mode	Sony

- R1-2404565	Discussion on LP-WUS operation in CONNECTED mode	ZTE, Sanechips

- R1-2404629	Discussion on LP-WUS operation in Connected mode	Xiaomi

- R1-2404666	Discussion on LP-WUS operation in RRC CONNECTED mode	NEC

- R1-2404707	LP-WUS operation in CONNECTED mode	Nokia

- R1-2404762	Discussion on LP-WUS operation in CONNECTED mode	Panasonic

- R1-2404783	Discussion on LP-WUS operation in CONNECTED modes	ETRI

- R1-2404854	Consideration on wake-up procedure in connected mode	OPPO

- R1-2404899	Discussion on LP-WUS operation in CONNECTED modes	LG Electronics

- R1-2404968	Discussion on LP-WUS operation in CONNECTED modes	Sharp

- R1-2405053	Discussion on LP-WUS operation in CONNECTED mode	NTT DOCOMO, INC.

- R1-2405075	LP-WUS operation in CONNECTED modes	MediaTek Inc.

- R1-2405110	LP-WUS operation in CONNECTED mode	Ericsson

- R1-2405166	LP-WUR operation in connected mode	Qualcomm Incorporated

- R1-2405471	FL summary #1 on LP-WUS operation in CONNECTED mode	Moderator (NTT DOCOMO)

- R1-2405592	FL summary #2 on LP-WUS operation in CONNECTED mode	Moderator (NTT DOCOMO)

- R1-2405659	FL summary #3 on LP-WUS operation in CONNECTED mode	Moderator (NTT DOCOMO)

### RAN2#125bis

- R2-2402635	Work plan for Rel-19 WI on LP-WUSWUR	vivo (Rapporteur)	discussion	Rel-19	NR_LPWUS-Core

- R2-2402159	Views on procedure and configuration of LP-WUS in RRC_IDLE/INACTIVE	China Telecom	discussion	Rel-19	NR_LPWUS-Core

- R2-2402194	Discussion on procedure and configuration of LP-WUS in RRC_IDLE/INACTIVE	OPPO	discussion	Rel-19	NR_LPWUS-Core

- R2-2402320	Discussion on LP-WUS/WUR for RRC IDLE/INACTIVE state	KT Corp.	discussion	Rel-19	NR_LPWUS

- R2-2402347	Discussion on LP-WUS operation in IDLE/INACTIVE modes	Spreadtrum Communications	discussion	Rel-19

- R2-2402446	General considerations on the procedure for RRC_IDLE_INACTIVE	Xiaomi Communications	discussion

- R2-2402539	Analysis on Procedure and Configuration of LP-WUS for IDLE/INACTIVE Modes	CATT	discussion	Rel-19	NR_LPWUS-Core

- R2-2402592	Discussion on procedure of LP-WUS in RRC_IDLE INACITVE 	NEC	discussion	Rel-19	NR_LPWUS-Core

- R2-2402597	Discussion on entry exit conditions for LP-WUS monitoring	Sharp	discussion

- R2-2402624	Discussion on LP-WUS WUR in RRC_IDLE INACTIVE	vivo	discussion	Rel-19	NR_LPWUS-Core withdrawn

- R2-2402690	Overall procedure of LP-WUS in RRC_IDLE/INACTIVE	HONOR	discussion	Rel-19	NR_LPWUS-Core

- R2-2402754	Procedure and configuration of LP-WUS for IDLE and INACTIVE modes	ZTE Corporation, Sanechips	discussion

- R2-2402875	Procedure and configuration of LP-WUS in RRC_IDLE/INACTIVE	Apple	discussion	Rel-19	NR_LPWUS-Core

- R2-2403698	Procedure and configuration of LP-WUS in RRC_IDLE and RRC_INACTIVE	Apple	discussion	Rel-19	NR_LPWUS-Core

- R2-2402933	Procedure and Configuration of LP-WUS in RRC Idle Inactive Mode	Samsung	discussion	Rel-19

- R2-2402963	Discussion on procedure and configuration of LP-WUS in RRC_IDLE/INACTIVE	Huawei, HiSilicon	discussion

- R2-2402972	Procedure and configuration of LP-WUS	LG Electronics Inc.	discussion	Rel-19	NR_LPWUS-Core

- R2-2403028	LP-WUS operation in IDLE INACTIVE modes	CMCC	discussion	Rel-19	NR_LPWUS-Core

- R2-2403037	Low Power Receiver: First points to address	Vodafone	discussion

- R2-2403043	Procedure and Configuration of LP-WUS in RRC IDLE/INACTIVE	Lenovo	discussion	NR_LPWUS-Core

- R2-2403057	RAN2 aspects on LP-WUS/WUR in RRC Idle/Inactive mode	Sony	discussion	Rel-19	NR_LPWUS-Core

- R2-2403135	LP-WUS based paging	Qualcomm Incorporated	discussion	NR_LPWUS-Core

- R2-2403272	LP-WUS in IDLE and INACTIVE	Nokia	discussion	Rel-19	NR_LPWUS-Core

- R2-2403332	Discussion on Procedure and configuration in RRC_IDLE/INACTIVE	NTT DOCOMO INC..	discussion	Rel-19	NR_LPWUS-Core

- R2-2403333	Discussion on LP-WUS operation in RRC_IDLE/INACTIVE modes	InterDigital, Inc.	discussion	Rel-19	NR_LPWUS-Core

- R2-2403550	WUR in Idle and Inactive	Ericsson	discussion	Rel-19	NR_LPWUS-Core

- R2-2403551	WUR and RRM measurements	Ericsson	discussion	Rel-19	NR_LPWUS-Core

- R2-2402201	Discussion on RRM measurement in RRC IDLE and INACTIVE	OPPO	discussion	Rel-19	NR_LPWUS-Core

- R2-2402348	Discussion on RRM measurement relaxation and offloading in IDLE/INACTIVE mode	Spreadtrum Communications	discussion	Rel-19

- R2-2402447	RRM measurement relaxation for RRC_IDLE_INACTIVE	Xiaomi Communications	discussion

- R2-2402477	Discussion on RRM measurement relaxation and offloading in RRC_IDLE/INACTIVE	Huawei, HiSilicon	discussion	Rel-19	NR_LPWUS-Core

- R2-2402540	RRM Relaxation and Offloading in RRC_IDLE/INACTIVE	CATT	discussion	Rel-19	NR_LPWUS-Core

- R2-2402593	Discussion on RRM measurement for LP-WUS in RRC_IDLE INACTIVE 	NEC	discussion	Rel-19	NR_LPWUS-Core

- R2-2402598	Discussion on serving cell RRM measurement offloading	Sharp	discussion

- R2-2402625	Discussion on RRM measurement relaxation and offloading in RRC_IDLE/INACTIVE	vivo	discussion	Rel-19	NR_LPWUS-Core

- R2-2402728	RRM measurement relaxation and offloading in RRC_IDLE/INACTIVE	Lenovo	discussion	Rel-19

- R2-2402755	RRM measurement relaxation for IDLEINACTIVE modes	ZTE Corporation, Sanechips	discussion

- R2-2402876	RRM measurement relaxation and offloading in RRC_IDLE/INACTIVE	Apple	discussion	Rel-19	NR_LPWUS-Core

- R2-2403699	RRM measurement relaxation and offloading in RRC_IDLE and RRC_INACTIVE	Apple	discussion	Rel-19	NR_LPWUS-Core

- R2-2402934	RRM measurement relaxation and offloading in RRC Idle Inactive Mode	Samsung	discussion	Rel-19

- R2-2402973	RRM Measurement offloading to LR	LG Electronics Inc.	discussion	Rel-19	NR_LPWUS-Core

- R2-2403029	Discussion on LP-WUR measurement offloading in IDLE INACTIVE modes	CMCC	discussion	Rel-19	NR_LPWUS-Core

- R2-2403058	Discussion on RRMRAN2 aspects foron LP-WUS/WUR	Sony	discussion	Rel-19	NR_LPWUS-Core

- R2-2403116	Discussion on RRM measurement relaxation in RRC_IDLE/INACTIVE	China Telecom	discussion

- R2-2403136	LP-WUS RRM measurement relaxation and offloading	Qualcomm Incorporated	discussion	NR_LPWUS-Core

- R2-2403273	RRM measurement relaxation in RRC_IDLE/INACTIVE	Nokia	discussion	Rel-19	NR_LPWUS-Core

- R2-2403059	Considerations on LP-WUS/WUR in RRC Connected mode	Sony	discussion	Rel-19	NR_LPWUS-Core

- R2-2403334	Discussion on LP-WUS operation in RRC_CONNECTED mode	InterDigital, Inc.	discussion	Rel-19	NR_LPWUS-Core

- R2-2403696	Discussion on LP-WUS WUR in RRC_CONNECTED	vivo	discussion	Rel-19	NR_LPWUS-Core	Late

### RAN2#126

- R2-2404295	General considerations on the procedure for RRC_IDLE_INACTIVE	Xiaomi Communications	discussion

- R2-2404314	LP-WUS procedure in RRC_IDLE INACTIVE 	NEC	discussion	Rel-19	NR_LPWUS-Core

- R2-2404376	LP-WUS Operation in RRC_IDLE/INACTIVE	CATT	discussion	Rel-19	NR_LPWUS-Core

- R2-2404418	Discussion on LP-WUS WUR in RRC_IDLE INACTIVE	vivo	discussion	Rel-19	NR_LPWUS-Core

- R2-2404459	Discussion on procedure and configuration of LP-WUS in RRC_IDLE/INACTIVE	Huawei, HiSilicon	discussion

- R2-2404460	Entering/Exit conditions, relaxed serving cell measurements on the main receiver and offload of  measurements to LP-WUR	Vodafone 	discussion	Rel-19

- R2-2404469	LP-WUS in IDLE and INACTIVE	Nokia	discussion	Rel-19	NR_LPWUS-Core

- R2-2404562	Discussion on LP-WUS in RRC_IDLE/INACTIVE	HONOR	discussion	Rel-19	NR_LPWUS-Core

- R2-2404588	Discussion on procedure and configuration of LP-WUS in RRC_IDLE/INACTIVE	OPPO	discussion	Rel-19	NR_LPWUS-Core

- R2-2404674	Procedure and configuration of LP-WUS in RRC_IDLE/INACTIVE	Apple	discussion	Rel-19	NR_LPWUS-Core

- R2-2404860	Procedure and configuration of LP-WUS for IDLE and INACTIVE mode	ZTE Corporation, Sanechips	discussion	Rel-19	NR_LPWUS-Core

- R2-2404906	RAN2 aspects on LP-WUS/WUR in RRC Idle/Inactive mode	Sony	discussion	Rel-19	NR_LPWUS-Core

- R2-2404927	Discussion on LP-WUS operation in IDLEI/NACTIVE mode	Spreadtrum Communications	discussion	Rel-19

- R2-2404996	WUR in Idle and Inactive	Ericsson	discussion	Rel-19	NR_LPWUS-Core

- R2-2405223	LP-WUS in RRC_IDLE and RRC_INACTIVE	LG Electronics Inc.	discussion	Rel-19	NR_LPWUS-Core

- R2-2405308	Discussion on procedure and configuration of LP-WUS in RRC_IDLE/INACTIVE	China Telecom	discussion	Rel-19	NR_LPWUS-Core

- R2-2405325	Discussion on LP-WUS operation in RRC_IDLE/INACTIVE modes	InterDigital, Inc.	discussion	Rel-19	NR_LPWUS-Core

- R2-2405354	Discussion on entry exit conditions for LP-WUS monitoring	Sharp	discussion

- R2-2405409	Procedure and Configuration of LP-WUS in RRC Idle Inactive Mode	Samsung	discussion	Rel-19

- R2-2405497	LP-WUS operation in IDLE/INACTIVE modes	CMCC	discussion	Rel-19	NR_LPWUS-Core

- R2-2405577	LP-WUS operation in IDLE/Inactive state	Qualcomm Incorporated	discussion	NR_LPWUS-Core

- R2-2405637	Procedure and Configuration of LP-WUS in RRC IDLE/INACTIVE	Lenovo	discussion	Rel-19	NR_LPWUS-Core

- R2-2405638	Discussion on Procedure and configuration in RRC_IDLE-INACTIVE	NTT DOCOMO INC..	discussion	Rel-19	NR_LPWUS-Core

- R2-2405695	Discussion on LP-WUS_WUR entry and exit conditions for RRC Idle_Inactive mode	KT Corp.	discussion	Rel-19	NR_LPWUS-Core	Late

- R2-2404301	Discussion on RRM measurement relaxation for RRC_IDLE_INACTIVE	Xiaomi Communications	discussion

- R2-2404315	LP-WUS RRM measurement relaxation 	NEC	discussion	Rel-19	NR_LPWUS-Core

- R2-2404323	Discussion on RRM measurement relaxation and offloading in RRC_IDLE/INACTIVE	Huawei, HiSilicon	discussion	Rel-19	NR_LPWUS-Core

- R2-2404377	RRM Relaxation and Offloading in RRC_IDLE/INACTIVE	CATT	discussion	Rel-19	NR_LPWUS-Core

- R2-2404399	Discussion on RRM measurement relaxation and offloading in RRC_IDLE/INACTIVE	China Telecom 	discussion

- R2-2404419	Discussion on RRM measurement relaxation and offloading in RRC_IDLE/INACTIVE	vivo	discussion	Rel-19	NR_LPWUS-Core

- R2-2404470	RRM measurement relaxation in RRC_IDLE/INACTIVE	Nokia	discussion	Rel-19	NR_LPWUS-Core

- R2-2404583	Discussion on RRM measurement in RRC IDLE and INACTIVE	OPPO	discussion	Rel-19	NR_LPWUS-Core

- R2-2404675	RRM measurement relaxation and offloading in RRC_IDLE/INACTIVE	Apple	discussion	Rel-19	NR_LPWUS-Core

- R2-2404808	RRM measurement relaxation and offloading in RRC_IDLE/INACTIVE	Lenovo	discussion	Rel-19

- R2-2404861	RRM measurement relaxation for IDLE and INACTIVE mode	ZTE Corporation, Sanechips	discussion	Rel-19	NR_LPWUS-Core

- R2-2404907	Discussion on RRMRAN2 aspects foron LP-WUS/WUR	Sony	discussion	Rel-19	NR_LPWUS-Core

- R2-2404928	Discussion on RRM measurement relaxation and offloading in IDLE/INACTIVE mode	Spreadtrum Communications	discussion	Rel-19

- R2-2404997	WUR and RRM measurements	Ericsson	discussion	Rel-19	NR_LPWUS-Core

- R2-2405013	Discussion on RRM measurement relaxation/offloading in IDLE/INACTIVE modes	CMCC	discussion	Rel-19	NR_LPWUS-Core

- R2-2405224	RRM relaxation and RRM offloading	LG Electronics Inc.	discussion	Rel-19	NR_LPWUS-Core

- R2-2405328	Discussion on RRM measurement relaxation and offloading	InterDigital, Inc.	discussion	Rel-19	NR_LPWUS-Core

- R2-2405355	Discussion on serving cell RRM measurement offloading	Sharp	discussion

- R2-2405410	RRM measurement relaxation and offloading in RRC Idle Inactive Mode	Samsung	discussion	Rel-19

- R2-2405579	LP-WUS RRM measurement relaxation and offloading	Qualcomm Incorporated	discussion	NR_LPWUS-Core

- R2-2404244	Discussion on LP-WUS for RRC_CONNECTED mode	Huawei, HiSilicon	discussion	Rel-19	NR_LPWUS-Core

- R2-2404302	Discussing on LP-WUS monitoring for RRC_Connected	Xiaomi Communications	discussion

- R2-2404316	LP-WUS procedure in RRC_CONNECTED 	NEC	discussion	Rel-19	NR_LPWUS-Core

- R2-2404378	Analysis on LP-WUS in RRC_CONNECTED	CATT	discussion	Rel-19	NR_LPWUS-Core

- R2-2404380	LP-WUS in RRC_CONNECTED	Nokia	discussion	NR_LPWUS-Core

- R2-2404420	Discussion on LP-WUS WUR in RRC_Connected	vivo	discussion	Rel-19	NR_LPWUS-Core  Revised in R2-2405935

- R2-2405935	Discussion on LP-WUS/WUR in RRC_CONNECTED	vivo	discussion	Rel-19	NR_LPWUS-Core

- R2-2404584	Discussion on LP-WUS in RRC_CONNECTED	OPPO	discussion	Rel-19	NR_LPWUS-Core

- R2-2404676	Procedures for LP-WUS in RRC_CONNECTED	Apple	discussion	Rel-19	NR_LPWUS-Core

- R2-2404862	Procedure for LP-WUS in RRC_CONNECTED	ZTE Corporation, Sanechips	discussion	Rel-19	NR_LPWUS-Core

- R2-2404908	Considerations on LP-WUS/WUR in RRC Connected mode	Sony	discussion	Rel-19	NR_LPWUS-Core

- R2-2404998	WUR in Connected	Ericsson	discussion	Rel-19	NR_LPWUS-Core

- R2-2405033	Discussion on LP-WUS operation in CONNECTED mode	CMCC	discussion	Rel-19	NR_LPWUS-Core

- R2-2405309	Discussion on procedures for LP-WUS in RRC_CONNECTED	China Telecom	discussion	Rel-19	NR_LPWUS-Core

- R2-2405332	Discussion on LP-WUS operation in RRC_CONNECTED mode	InterDigital, Inc.	discussion	Rel-19	NR_LPWUS-Core

- R2-2405411	Procedures for LP-WUS in RRC Connected Mode	Samsung	discussion	Rel-19

- R2-2405468	Discussion on LP-WUS in RRC_CONNECTED mode	LG Electronics Inc.	discussion	Rel-19

- R2-2405578	LP-WUS operation in CONNECTED state	Qualcomm Incorporated	discussion	NR_LPWUS-Core

- R2-2405639	LP-WUS in RRC Connected Mode	Lenovo	discussion	Rel-19	NR_LPWUS-Core

- R2-2405687	Discussion on LP-WUS in RRC_CONNECTED	NTT DOCOMO INC..	discussion	Rel-19	NR_LPWUS-Core

### RAN4#110bis

- R4-2404195	Initial views on LPWUS for NR	Apple

- R4-2404252	On receiver sensitivity and ACS/ASCS of the low power wake up radio	Sony

- R4-2404265	UE RF requirements for LP-WUS	Nokia

- R4-2404289	Required changes for BS RF requirements for low-power wake-up signal for NR	Nokia

- R4-2404367	On RRM core requirements for LP-WUR	Apple

- R4-2404421	Discussion on RRM core requirements for R19 LP-WUS/WUR	CATT

- R4-2404446	On UE RF requirements for LP-WUS/LP-WUR	CATT

- R4-2404447	On BS RF requirements for LP-WUS/LP-WUR	CATT

- R4-2404498	Work plan on RRM for LP-WUR WI	vivo

- R4-2404499	Initial consideration on RRM for LP-WUR	vivo

- R4-2404524	Discussion on BS RF requirements for LP-WUS/LP-SS.	NTT DOCOMO, INC.

- R4-2404536	Discussion on RRM requirements for LP-WUS,WUR	Samsung

- R4-2404550	Discussion on RF requirements for LP-WUS	Xiaomi

- R4-2404565	Initial discussion on RRM core requirements for LP-WUS/WUR	Xiaomi

- R4-2404589	Discussion on UE RF requirements and testability for low-power wake-up receiver	Samsung

- R4-2404590	Discussion on BS RF requirements for low-power wake-up signal	Samsung

- R4-2404655	Workplan for Rel-19 LP-WUS WI	vivo

- R4-2404656	Views on LP-WUS UE RF requirements	vivo

- R4-2404657	Views on LP-WUS BS RF requirements	vivo

- R4-2404691	(NR_LPWUS-Core) Discussion on RRM impact of LP-WUR	CMCC

- R4-2404732	Discussion on RRM requirements for LP-WUS/WUR	LG Electronics Inc.

- R4-2404842	Topic summary for [110bis][231] NR_LPWUS	Moderator (vivo)

- R4-2404872	General overview on WUR RF requirement testability	Ericsson

- R4-2404873	WUR RF requirement overview	Ericsson

- R4-2404874	BS RF requirement overview for LP-WUS	Ericsson

- R4-2404927	Discussion on RRM core requirements for LP-WUS/WUR	OPPO

- R4-2404980	Discussion on LP-WUS RRM requirement	Ericsson

- R4-2405119	Discussion on UE RF requirements for LP-WUS/WUR	ZTE Corporation

- R4-2405120	Discussion on BS RF requirements for LP-WUS/WUR	ZTE Corporation

- R4-2405290	Topic summary for [110bis][137] NR_LPWUS	Moderator(vivo)

- R4-2405307	Discussion on LP-WUS BS RF requirement	CMCC

- R4-2405308	Discussion on LP-WUS UE RF requirement	CMCC

- R4-2405381	Initial discussion on UE RF requirement for LP-WUS	OPPO

- R4-2405487	Consideration on UE RF aspects for Rel-19 LP-WUS	Huawei, HiSilicon

- R4-2405488	Consideration on BS RF aspects for Rel-19 LP-WUS	Huawei, HiSilicon

- R4-2405540	Discussion on RRM core requirements for LP-WUS/WUR	Nokia

- R4-2405604	Initial views on RRM requirements for LP-WUR	Huawei, HiSilicon

- R4-2405834	Topic summary for [110bis][314] NR_LPWUS	Moderator (Huawei)

- R4-2405937	Scope of RRM requirements for LP-WUR	Qualcomm Incorporated

- R4-2405958	Discussion on LPWUS for NR	ZTE Corporation

- R4-2405970	Discussion on the RRM requirements for LP-WUS	MediaTek inc.

- R4-2406108	Way Forward for [110bis][314] NR_LPWUS	Huawei

- R4-2406139	Way Forward for [110bis][314] NR_LPWUS	Huawei

- R4-2406140	Way Forward for [110bis][314] NR_LPWUS	Huawei

- R4-2406366	Work plan on RRM for LP-WUR WI	vivo

- R4-2406367	WF for RRM requirements for LP-WUS/WUR	Vivo

- R4-2406619	WF on NR LP-WUS UE requirements	Vivo

### RAN4#111

- R4-2407069	On Low-power Wake-up Signal and Receiver for NR	Apple

- R4-2407311	On RRM core requirements for LP-WUR	Apple

- R4-2407330	Discussion on RRM core requirements for LP-WUS/WUR	Samsung

- R4-2407377	Discussions on RRM core requirements for LP-WUS/WUR	NTT DOCOMO, INC.

- R4-2407412	On receiver sensitivity of the low-power wake-up receiver	Sony

- R4-2407440	BS RF requirements for low-power wake-up signal for NR	Nokia

- R4-2407487	Discussion on RRM core requirements for R19 LP-WUS/WUR	CATT

- R4-2407546	On system parameters for LP-WUS/LP-WUR	CATT

- R4-2407547	On BS RF requirements for LP-WUS/LP-WUR	CATT

- R4-2407649	On general aspects for Rel-19 LP-WUS	Huawei, HiSilicon

- R4-2407650	On UE RF REFSENS, ACS, ASCS requirements for Rel-19 LP-WUS	Huawei, HiSilicon

- R4-2407651	On UE RF other Rx requirements for Rel-19 LP-WUS	Huawei, HiSilicon

- R4-2407652	On UE RF testability issue for Rel-19 LP-WUS	Huawei, HiSilicon

- R4-2407653	On BS RF for Rel-19 LP-WUS	Huawei, HiSilicon

- R4-2407794	On Rx requirements of REFSENS, ASCS and ACS	CATT

- R4-2407825	Discussion on Rx requirements of REFSENS, ASCS and ACS for LP-WUS	Xiaomi

- R4-2407826	Discussion on Rx requirements of IBB, OBB, intermodulation, spurious emissions and others for LP-WUS	Xiaomi

- R4-2407844	Discussion on RRM core requirements for LP-WUS/WUR	Xiaomi

- R4-2407886	Discussion on core requirements for LP-WUS WUR	OPPO

- R4-2407894	Discussion on REFSENS for LP-WUR	Samsung

- R4-2407895	Discussion on power boosting for LP-WUS	Samsung

- R4-2407939	(NR_LPWUS-Core) Discussion on RRM impact of LP-WUR	CMCC

- R4-2407953	(NR_LPWUS-Core) Discussion on LP-WUS UE system parameters requirements	CMCC

- R4-2407954	(NR_LPWUS-Core) Discussion on LP-WUS UE RF Rx requirements of REFSENS, ASCS and ACS	CMCC

- R4-2407955	(NR_LPWUS-Core) Discussion on LP-WUS UE RF Rx requirements of IBB, OBB, intermodulation, spurious emissions and others	CMCC

- R4-2407956	(NR_LPWUS-Core) Discussion on LP-WUS BS RF requirements	CMCC

- R4-2407966	Discussion on RRM requirements for LP-WUS/WUR	LG Electronics Inc.

- R4-2408026	Topic summary for [111][229] NR_LPWUS	Moderator (vivo)

- R4-2408032	On LPWUR system considerations	Qualcomm Incorporated

- R4-2408040	RRM requirements for LP-WUR	Qualcomm Incorporated

- R4-2408046	Fundamental RX requirements for LP-WUR	Nokia Poland

- R4-2408047	Other RX requirements for LP-WUR	Nokia Poland

- R4-2408049	System parameters for LP-WUR	Nokia Poland

- R4-2408050	Testability aspects of LP-WUR	Nokia Poland

- R4-2408108	Discussions on LP-WUS system parameters	vivo

- R4-2408109	Discussions on LP-WUS REFSENS, ASCS and ACS	vivo

- R4-2408110	Discussions on LP-WUS IBB, OBB, intermodulation, spurious emissions	vivo

- R4-2408111	Discussions on LP-WUS Testability	vivo

- R4-2408112	Discussions on LP-WUS BS RF requirements	vivo

- R4-2408137	Discussion on RX requirements of REFSENS for LP-WUR	Spreadtrum Communications

- R4-2408316	Discussion on RRM core requirements for LP-WUS/WUR	China Telecom

- R4-2408329	Discussion on LP-WUS RRM requirement	Ericsson

- R4-2408362	Discussion on system parameters for LP-WUS/WUR	ZTE Corporation, Sanechips

- R4-2408363	Discussion on REFSENS, ASCS, ACS for LP-WUR	ZTE Corporation, Sanechips

- R4-2408364	Discussion on testability for LP-WUS UE RF requirements	ZTE Corporation, Sanechips

- R4-2408365	Discussion on BS RF requirements for LP-WUS/WUR	ZTE Corporation, Sanechips

- R4-2408624	Consideration on RRM for LP-WUR	vivo

- R4-2408673	Discussion on RRM core requirements for LP-WUS and LR	Nokia

- R4-2408824	on LPWUS other RX requirements	OPPO

- R4-2408825	on LPWUS REFSENS requirement	OPPO

- R4-2408826	on LPWUS testability issue	OPPO

- R4-2408947	Topic summary for [111][136] NR_LPWUS_UERF	Moderator(vivo)

- R4-2409099	BS RF requirement overview	Ericsson

- R4-2409100	On general issues for WUR	Ericsson

- R4-2409101	On system paramter for WUR	Ericsson

- R4-2409102	On WUR RF requirement testability	Ericsson

- R4-2409103	WUR RF requirement other than REFSENS	Ericsson

- R4-2409104	WUR RF requirement REFSESN ASC ASCS	Ericsson

- R4-2409295	Discussion on RRM requirements for LP-WUR	Huawei, HiSilicon

- R4-2409687	Discussion on NLP-WUS for core part	ZTECorporation,Sanechips

- R4-2409729	Discussion on the RRM requirements for LP-WUS	MediaTek inc.

- R4-2410109	Topic summary for [111][314] NR_LPWUS	Moderator (Huawei)

- R4-2410296	WF for RRM core requirements for LP-WUS/WUR	Vivo

- R4-2410569	WF on NR LP-WUS UE RF	Vivo

17.05.2021		minor adaptations for RAN #92e

28.01.2021		minor adaptations for RAN #91e

09.11.2020		minor adaptations for RAN #90e

31.08.2020		minor adaptations for RAN #89e

20.04.2020		minor adaptations for RAN #88e

18.02.2020		minor adaptations for RAN #87e

14.11.2019		minor adaptations for RAN #86

18.08.2019		minor adaptations for RAN #85

12.05.2019		minor adaptations for RAN #84

27.02.2019		minor adaptations for RAN #83

21.11.2018		completion levels with colours added (for RAN #82)

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects (for RAN #81)

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template