3GPP TSG RAN Meeting #104									RP-241043

Shanghai, China, June 17-20, 2024

Status Report to TSG

Agenda item:		9.3.4.8

| WI / SI Name | WI on TRP (Total Radiated Power), TRS (Total Radiated Sensitivity) and MIMO OTA (Over the Air) testing enhancement Phase 3 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: Yes |  | Testing part: No |
| Acronym | TRP_TRS_MIMO_OTA_Ph3 |  |  |  |  |
| Unique ID | 1030085 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-240841 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: | Core part: Sep/2025 | Performance part: Mar/2026 | Testing part: |  |
| Overall Completion level | Study Item: | Core part: 10% | Performance Part: 0% | Testing part: |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN4 |
| --- | --- | --- |
| Rapporteur | Name | Wang, Ruixin |
|  | Company | vivo |
|  | Email | ruixin.wang@vivo.com |
| Rapporteur | Name | Zhu, Siting |
|  | Company | CAICT |
|  | Email | zhusiting@caict.ac.cn |

1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

2.4	RAN4

2.4.1	Agreements

RAN4 #110bis

- General

- WI workplan is approved in [29]

- Topic summary for FR1 TRP TRS WI in [28]

- LS to CTIA on Head Phantoms for XR devices OTA testing was approved in [30]

- WF for Rel-19 TRP_TRS_MIMO_OTA was approved in [31]

Agreements:

Sub-topic 2-1 XR Head phantom related topics

Issue 2-1-1: Head phantom for XR devices

Way forward:

- Send a LS to CTIA on collaboration for XR head phantom development this meeting.

Issue 2-1-2: LS to CTIA on XR head phantom

Way forward:

- Approve the LS to CTIA on Head Phantoms for XR devices OTA testing in R4-2406085.

Sub-topic 2-2 XR test scnarios and configurations

Issue 2-2-1: XR device type

Way forward:

  - RAN4 further discuss the XR device type for test method discussion

Issue 2-2-2: Phantom-based and Free space XR test scenarios

Way forward:

  - Consider Head phantom scenario (1st priority) and FFS Free space for head-worn XR devices

    - For Free Space, the positioning guideline of XR device should be studied in RAN4. The corresponding XR UE mechanical mode description should also be defined

Issue 2-2-4: XR test methodology

Way forward:

  - Test method, test configurations and measurement grid for 1Tx/2Tx handheld UE with SA mode should be reused as much as possible for 1Tx/2Tx XR devices OTA testing.

Sub-topic 2-3 Testing time reduction for XR OTA

Issue 2-3-1: Testing time reduction solutions

Way forward:

  - Further discuss whether and how to address the potential limited battery volume, e.g. testing with maximum output power or lower power configuration, testing with charging cable, fast sensitivity searching approach, etc.

    - FFS testing with existing coarse measurement grid

Sub-topic 3-1 UE type and usage scenarios for NTN (NR-NTN and IoT-NTN)

Issue 3-1-1: UE type for NR-NTN

Way forward:

  - RAN4 further check whether the test method/system can be generic for different UE type. If not, further discuss potential prioritization for different UE types (e.g., Handheld and FR1 VSAT-like device).

Issue 3-1-2: UE type for IoT-NTN

Way forward:

  - RAN4 could further discuss the target UE form factors and corresponding size, antenna design, etc., for IoT NTN OTA

Issue 3-1-3: Usage scenarios for NR-NTN and IoT-NTN handheld UE

Way forward:

  - RAN4 further discuss the usage scenarios for NR-NTN and IoT-NTN. Input from satellite operators is encouraged

    - usage scenario can be considered with performance metric together .

Sub-topic 3-2 UE performance metric

Issue 3-2-1: whether RAN4 should consider different performance metric for different UE types or satellite orbits

Way forward:

  - RAN4 further discuss potential performance metric for different FR1 UE types.

    - FFS whether different performance metric for different UE type. FFS Handheld UE is prioritized.

    - FFS whether separate metrics for NR/IoT NTN UEs based on their support for either GSO or NGSO or both

    - FFS whether different performance metrics should be defined for high earth orbit (such as GEO) and low earth orbit (i.e. LEO) due to they have different requirements on the radiating characteristics

Issue 3-2-2: Proper performance metric for NR-NTN

Way forward:

Consider the following initial input as a starting point for further discussions:

  - Option 1: consider the following aspects

    - Option 1: integrated power/sensitivity within declared half sphere

    - Option 2: peak EIRP/EIS only

    - Option 3: peak EIRP/EIS + X%-tile spherical coverage within declared half sphere

    - Option 4: peak EIRP/EIS + Y%-tile spherical coverage from whole sphere

  - Option 2: consider the following aspects

    - Consider a general framework for each UE type, to specify performance metric for different use cases / power classes, e.g., a set of metric for Handheld, and other set for FR1 VSAT-like UE.

    - Consider the assumption: UE elevation angles for NTN are typically >20°, in majority of cases.

    - Consider TRP, TRS, EIRP, and EIS as starting point. Further discuss other performance metric based on NTN usage scenarios, e.g., directivity requirements, Antenna Gain.

  - Option 3: consider the following aspects

    - consider WRP and WRS defined in R4-2404278 as performance metrics for NTN devices

    - consider EIRP and EIS CDF percentile thresholds as performance metrics for NTN devices

  - Option 4: Conical TRP/EIRP for Tx and conical TRS/EIS for Rx could be considered as the starting point for the metric of NTN device OTA testing. FFS on the range of q and f.

  - Option 5: Consider the following aspects for NTN performance metric

- Adopting a directional antenna pattern as the starting point for the antenna characteristics of an FR1 NTN device.

- Take UIRP and UHIS as the as the starting point for the performance metric of devices using directional antenna.

- Further discuss whether a smaller angle of test scan can be used or whether a EIRP/EIS CDF-like performance metric and be considered.

  - Option 6: other aspects are not precluded

Sub-topic 3-3 NTN OTA test methodologies

Issue 3-3-1: Test method for NR-NTN and IoT-NTN

Way forward:

  - RAN4 further discuss potential test methods based on consideration in topics 3-1 and 3-2.

  - Based on FR1 frequency range, above 10 GHz bands are not included in the NTN OTA discussion.

Issue 3-3-2: Test parameters for NR-NTN and IoT-NTN

Way forward:

  - RAN4 should define detailed OTA test parameters for NTN bands, e.g., CBW, SCS, Modulation, RB allocation, for both NR-NTN and IoT-NTN. Alignment with conducted test parameters can be considered. Input from satellite operators is encouraged.

  - RAN4 further discuss and check whether NR/IoT NTN radiated requirements shall be verified when Doppler conditions are set to zero and delay conditions are set to constant for all types of satellites.

Issue 3-3-3: Settings of UE

Way forward:

  - RAN4 further discuss whether specific setting for NTN UE is needed, e.g., consider Soft-assistant function, allowance of TAS.

Issue 3-3-4: test campaign of NTN UE

Way forward:

  - Given no requirements work for NTN devices in Rel-19, no test campaign is planned.

Sub-topic 4-1 Dynamic channel model generation and validation methodology

Issue 4-1-1: Framework for dynamic Channel model generation

Way forward:

- Consider the following proposals for further study of framework for dynamic channel model generation, whether any of the aspect is agreeable depends on further discussions and analysis

  - Proposal 1: dynamic channel model should consist of number of segments with fixed channel models available in 3GPP.

    - the number of segments in a dynamic channel should be in the range of [8] to [12].

    - parameters would need to be interpolated between segments to ensure continuity of dynamic channels

    - Pathloss variation in a dynamic model should account for achievable chamber dynamic range

    - A dynamic channel should balance between high and low throughput segments by using lengths of corresponding segments

  - Proposal 2: RAN4 should not reopen the basic channel modeling discussions in TR 38.901 to general new CDL models. Using CDL models defined in TR 38.827 as starting point to generate the dynamic channel model.

  - Proposal 3: RAN4 to consider the following aspects regarding the dynamic OTA modelling.

    - How to create a drive route, DoT (Direction of Travel), velocity, AoA, etc, parameters

    - How to define the midway points on the drive route

    - How to interpolate channel parameters for continuous channel modelling

    - Whether/how to change UE orientation, such as via switching the probes and/or UE positioner

    - Other aspects are not precluded

  - Proposal 4: RAN4 should define dynamic channel models which emulate typical real-world scenarios with time-varying multi-path propagation conditions, based on CDL channel models defined in TR 38.901 and TR 38.827.  The following general principles should be considered:

    - The performance of “good” and “bad” UEs can be effectively differentiated under the dynamic channel models.

    - The test time should be kept within an acceptable level.

Issue 4-1-2: Alignment of dynamic channel model and validation procedure in different SDOs

Way forward:

  - RAN4 should consider the progress on dynamic channel model in other SDOs. FFS whether RAN4 will develop full-aligned or partial-aligned channel model and validation procedure with other SDO.

Issue 4-1-3: Whether dynamic link adaption is adopted

Way forward:

  - RAN4 consider introducing dynamic link adaptation schemes as a starting point for further study, e.g., variable MCS/rank/MIMO precoding by the BS emulator/gNB.

    - RAN4 should discuss how to ensure the consistency of different test equipment/test system on Scheduling Algorithm.

Issue 4-1-4: Environmental condition

Way forward:

  - Follow WID, RAN4 use noise-limited environmental condition as baseline. Further comparison of different environmental condition can be contribution driven.

Sub-topic 4-2 Test system for FR1 dynamic MIMO OTA

Issue 4-2-1: Test system for FR1 dynamic MIMO OTA

Way forward:

  - Follow WID, 3GPP to re-use the MPAC system with uniformly spaced probes from 38.827, 38.151 for FR1 dynamic MIMO OTA.

Issue 4-2-2: Test zone of test system for dynamic channel model

Way forward:

  - RAN4 should discuss proper test zone size (e.g., 30cm test zone size as target) for the newly defined dynamic FR1 MIMO OTA channel model. Acceptable criteria (e.g., Spatial Correlation deviation) for dynamic channel model generation in the chamber should also be discussed.

Sub-topic 4-3 UE Performance metric

Issue 4-3-1: UE performance metric for dynamic MIMO OTA testing

Way forward:

Consider the following input as starting point for further discussion, whether any metric is selected depends on further discussions and analysis

  - Proposal 1: 3GPP to adopt the same/similar performance metrics of TMT and CTMT.

  - Propsoal 2: Throughput CDF can be a good candidate for performance metric with either a fixed percentile or a combination of a number of percentiles.

  - Proposal 3: For dynamic MIMO OTA, the measured throughput under pre-defined power profile is as performance metric.

  - Proposal 4: The CDF of UE throughput could be considered as the starting point for performance metric of FR1 dynamic MIMO OTA testing.

RAN4 #111

- General

- The TR skeleton for FR1 dynamic MIMO OTA is approved in [38]

- WF for Rel-19 TRP_TRS_MIMO_OTA was approved in [59]

- Topic summary for [111][338] TRP_TRS_MIMO_OTA is presented in [60]

Agreements:

Sub-topic 2-1 XR test scnarios and configurations

Issue 2-1-1: XR device type

Agreements

FFS The defined test methodologies of TRP/TRS for FR1 non-Redcap XR devices shall be applied to all possible 5G XR device types except the XR device form-factors based on smartphone UE i.e. XR5G-P1.

RAN4 further discuss prioritization of XR devices type, e.g.,

- Whether 1Tx XR device should be prioritized in Rel-19

- whether head-worn XR devices with external 5G (e.g. XR5G-V1/V2/V3, XR5G-A1/A2/A5) should be considered.

Issue 2-1-3: How to identify 2Rx-XR device?

Agreements

  - RAN4 further check the following configuration for 2Rx XR:

    - The parameter cellBarred2RxXR-r18 (ENUMERATED) should be set to {false} during OTA tests for XR devices with 2Rx.

    - Use parameter supportOf2RxXR-r18 to identify XR devices with 2Rx.

Sub-topic 2-2 Testing time reduction for XR OTA

Issue 2-2-1: alternative test configurations for Testing time reduction solutions

Agreements

  - RAN4 can consider low transmission power configuration (e.g., 13dBm) for XR device TRS testing if a Single Point Offset Test (SPOT) with maximum transmit power is performed and the end of the measurement to scale the TRS result.

Issue 2-2-2: Coarse measurement grids

Agreements

  - Legacy measurement grid can be baseline for XR devices TRP TRS measurement and measurement campaign used to derive performance requirements, RAN4 can further check and confirm coarse measurement grid based on measurement results and simulations.

  - MU analysis for coarse measurement grid is also needed.

Sub-topic 3-1 UE type and usage scenarios for NTN (NR-NTN and IoT-NTN)

Issue 3-1-1: UE type for NR-NTN

Agreements

  - Prioritize Handheld UE and further study VSAT-like UE.

Issue 3-1-2: Alignment on understanding of VSAT-like UE for NR-NTN

Agreements

  - Encourage OEMs/Satellite Operators to provide more device information for VSAT-like device types, e.g., detailed definition, device size, weight, typical antenna patterns.

Issue 3-1-3: Power Class for NR-NTN devices

Agreements

  - Prioritize PC3.

Issue 3-1-4: UE type for IoT-NTN

Agreements

  - FFS detailed UE type for IoT-NTN.

Issue 3-1-5: Alignment on understanding of IoT-NTN devices

Agreements

  - Encourage OEMs/Satellite Operators to provide more device information for IoT-NTN device types, e.g., detailed definition, device size, weight, typical antenna patterns.

Issue 3-1-6: Usage scenarios for NR-NTN and IoT-NTN handheld UE

Agreements

  - RAN4 further discuss prioritized usage scenarios. Free space only is not considered.

    - Option 1: Prioritize the browsing mode (with hand phantom)

    - Option 2: Prioritize the browsing mode (with hand phantom) and Free Space

    - Option 3: Prioritize browsing mode (with hand phantom) and talk mode (Head+Hand)

    - Option 4: Prioritize Head+Hand talk mode, hand only browsing mode and talk mode (new positioning guideline), and Free Space

    - Option 5: Prioritize Free Space mode only

Sub-topic 3-2 UE performance metric

Issue 3-2-1: Categorized performance metric for NR-NTN handheld

Agreements

- RAN4 should not adopt direction 1, other directions are FFS

  - Direction 1: Only single point performance metric, i.e., Peak EIRP/EIS

  - Direction 2: Single point + measured full sphere (integrated or CDF), e.g.,

    - D2a, Peak EIRP/EIS+TRP/TRS,

    - D2b, Peak EIRP/EIS+ full Spherical coverage CDF

  - Direction 3: Single point + measured partial sphere (integrated or CDF), e.g.,

    - D3a, Peak EIRP/EIS+ partial TRP/TRS (e.g., within selected 30, 60, 90, 180 degree range of angles),

    - D3b, Peak EIRP/EIS+ partial Spherical coverage CDF (e.g., within selected 30, 60, 90, 180 degree range of angles)

  - Direction 4: Only Full sphere (integrated TRP/TRS or Spherical coverage CDF) with/without weighting,

    - D4a, Spherical Coverage with EIRP-CDF and EIS-CCDF at [>50%]-ile

    - D4b, TRP/TRS or Weighted metric, e.g., weighted TRP/TRS (with larger weight for declared hemisphere)

  - Direction 5: Only Partial sphere (integrated or CDF) (e.g., within selected 30, 60, 90, 180 degree range of angles),

Issue 3-2-3: Whether performance metric should be different for GEO and NGEO (including LEO and MEO)

Agreements

  - FFS whether define different metrics for GSO and NGSO

Sub-topic 3-3 NTN OTA test methodologies

Issue 3-3-2: CBW for NR-NTN bands

Agreements

  - Due to limited spectrum resource for NTN, RAN4 further consider whether min test channel BW defined in RAN5 38.508-1 should be adopted for FR1 NTN OTA testing.

Issue 3-3-4: Doppler parameter of NR-NTN UE

Agreements

  - NR/IoT NTN radiated requirements shall be verified when Doppler conditions are set to zero and delay conditions are set to constant for all types of satellites

Issue 3-3-5: New positioning guideline of hand only talk mode

Agreements

  - Consider a new position for hand only, i.e., DUT’s main display is aligned with vertical axis for further discussion.

    - The need to define new positioning guidelines for testing depends on the metric selected for testing as per Issue 3-2-1. E.g. full sphere does not require new positioning guidelines.

Sub-topic 4-1 Dynamic channel model generation and validation

Issue 4-1-1: Alignment of dynamic channel model parameters in different SDOs

Agreements

  - RAN4 adopt the UMa and the UMi dynamic channel models defined in CTIA. FFS whether minor modification is needed.

  - RAN4 develop few new channel models is not precluded.

Issue 4-1-3: Channel model validation methodology

Agreements

  - RAN4 adopt the similar channel model validation procedures as other SDOs

Sub-topic 4-2 Test system for FR1 dynamic MIMO OTA

Issue 4-2-1: Test zone of test system for dynamic channel model

Agreements

  - Define 30cm QZ for dynamic FR1 MIMO OTA testing. Dynamic channel model Test zone of UMi and UMa needs analysis and confirmation.

Issue 4-2-2: Link adaption configuration for Dynamic FR1 MIMO OTA

Agreements

  - RAN4 adopt dynamic link adaptation. FFS details, e.g.,

    - mapping tables between CQI and MCS for available rank values

Issue 4-2-3: Test system Dynamic-link-adaption verification

Agreements

  - RAN4 should study a procedure to verify the consistency of different test equipment/test system on Scheduling Algorithm in dynamic channel model system. This can be considered as an additional aspect of system validation.

Issue 4-2-4: Environmental condition for RAN4 FR1 dynamic MIMO OTA

Agreements

  - RAN4 adopt the full path loss model from TR 38.901 for the UE Noise-limited environment condition.

  - With understanding of noise-limited is baseline for dynamic MIMO OTA in RAN4, the study of Interference-limited environment condition for RAN4 FR1 Dynamic MIMO OTA is not precluded.

Issue 4-2-5: Noise floor of noise-limited system for FR1 dynamic MIMO OTA

Agreements

  - Encourage system integrators to confirm the system noise floor levels.

Sub-topic 4-3 UE Performance metric

Issue 4-3-1: UE measurement campaign to decide final performance metric

Agreements

  - To study/verify the UE performance metrics, RAN4 could use the UE throughput data collected during the measurement campaign

Issue 4-3-3: UE orientations for dynamic MIMO OTA testing

Agreements

  - RAN4 further study and check whether the four device orientations DMP, DML (both left and right tilt), and DMSU and four rotations (0°, 90°, 180°, 270°) per orientation can be used.

Sub-topic 4-4 Draft TR skeleton

Issue 4-4-1: Skeleton for FR1 dynamic MIMO OTA TR 38.8xy

Agreements

  - Agree the draft skeleton in R4-2407667.

2.4.2	Remaining Open issues

- Core part

    - Further discuss XR OTA test method

    - Further discuss NTN OTA test method

    - Further discuss FR1 dynamic MIMO OTA test method

- Performance part

    - Not started

2.5	RAN5

2.5.1	Agreements

RAN5 #103

- No discussion this meeting

2.4.2	Remaining Open issues

- XR OTA test methods related MU assessment

- FR1 NTN OTA (NR-NTN and IoT-NTN) test methods related MU assessment

- FR1 dynamic MIMO OTA related MU assessment

3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

N/A

4.	References

RAN4 #110bis

- R4-2404142	On FR1 dynamic MIMO OTA methodology	Keysight Technologies UK Ltd

- R4-2404167	On headworn XR TRP TRS methodology	Apple

- R4-2404168	On NTN TRP TRS methodology	Apple

- R4-2404169	On work scope for the TRP TRS performance phase	Apple

- R4-2404198	On FR1 MIMO OTA with Dynamic Channel Model	Apple

- R4-2404277	on test methodology for FR1 non-RedCap headworn XR devices	Huawei, HiSilicon

- R4-2404278	on test methodology and radiated performance metric for FR1 NTN devices	Huawei, HiSilicon

- R4-2404279	on FR1 dynamic MIMO OTA test methodology	Huawei, HiSilicon

- R4-2404540	Initial view on XR OTA evaluation Methodology of glass type Device	Meta Ireland

- R4-2404554	Intial discussion on OTA test for FR1 NTN devices	Xiaomi

- R4-2404555	Intial discussion on Dynamic MIMO OTA test	Xiaomi

- R4-2404591	Initial discussion on Rel-19 FR1 TRP TRS test method for XR UE	Samsung

- R4-2404592	Initial discussion on Rel-19 FR1 TRP TRS test method for NTN UE	Samsung

- R4-2404593	Initial discussion on Rel-19 FR1 dynamic MIMO OTA test method	Samsung

- R4-2404650	Workplan for Rel-19 OTA WI	vivo,CAICT

- R4-2404651	General views on XR OTA test method	vivo

- R4-2404652	General views on NTN OTA test method	vivo

- R4-2404653	General views on FR1 MIMO OTA dynamic channel model	vivo

- R4-2404654	LS to CTIA on Head Phantoms for XR devices OTA testing	vivo, Keysight

- R4-2405177	Views on FR1 XR OTA methodology	OPPO

- R4-2405178	Views on performance metric of FR1 NTN	OPPO

- R4-2405398	Views on OTA test method for XR devices	Qualcomm Incorporated

- R4-2405399	Views on OTA test method for FR1 NTN devices	Qualcomm Incorporated

- R4-2405400	Views on FR1 dynamic MIMO OTA test method	Qualcomm Incorporated

- R4-2405457	Initial views on test methodology for non-RedCap headworn XR	CAICT, SAICT

- R4-2405458	Initial views on test methodology for FR1 NTN devices	CAICT, SAICT

- R4-2405470	On FR1 dynamic MIMO OTA test methodology	CAICT, SAICT

- R4-2405857	Topic summary for [110bis][338] TRP_TRS_MIMO_OTA	Moderator (vivo)

- R4-2406084	Workplan for Rel-19 OTA WI	vivo,CAICT

- R4-2406085	LS on Head Phantoms for XR devices OTA testing	vivo, Keysight

- R4-2406086	Way Forward for [110bis][338] TRP_TRS_MIMO_OTA	 Vivo

RAN4 #111

- R4-2407059	On headworn XR TRP TRS methodology	Apple

- R4-2407060	On NTN TRP TRS methodology	Apple

- R4-2407103	on FR1 dynamic MIMO OTA test methodology	Huawei, HiSilicon

- R4-2407104	on test methodology for FR1 non-RedCap headworn XR devices	Huawei, HiSilicon

- R4-2407105	on test methodology and radiated performance metric for FR1 NTN devices	Huawei, HiSilicon

- R4-2407666	On FR1 dynamic MIMO OTA	CAICT

- R4-2407667	TR 38.xyz skeleton for Dynamic MIMO OTA	CAICT

- R4-2407809	Discussion on OTA test for FR1 NTN devices	Xiaomi

- R4-2407896	Discussion on XR UE OTA test	Samsung

- R4-2407897	Discussion on NTN UE OTA test	Samsung

- R4-2407898	Discussion on figure of merit for dynamic MIMO OTA	Samsung

- R4-2407997	XR OTA evaluation Methodology of non-RedCap XR Device	Facebook Japan G.K.

- R4-2408105	Discussions on XR OTA test method	vivo

- R4-2408106	Discussions on NTN OTA test method	vivo

- R4-2408107	Discussions on FR1 dynamic MIMO OTA	vivo

- R4-2408610	On Test methodology for FR1 NTN devices	CAICT.

- R4-2408691	Discussion for XR device types	Nokia

- R4-2408692	Discussion for NR NTN OTA test	Nokia

- R4-2408693	Discussion for for dynamic MIMO OTA	Nokia

- R4-2408904	Test methodology of XR OTA	OPPO

- R4-2408905	On NTN OTA	OPPO

- R4-2408906	On dynamic MIMO OTA	OPPO

- R4-2409187	Discusion on test methodology for non-RedCap headworn XR	CAICT, SAICT

- R4-2409430	Discussion on FR1 NTN devices OTA	Qualcomm Incorporated

- R4-2409431	Discussion on FR1 dynamic MIMO OTA test method	Qualcomm Incorporated

- R4-2409769	On FR1 dynamic MIMO OTA methodology	Keysight Technologies UK Ltd

- R4-2409771	NTN OTA Testability Aspects	Keysight Technologies UK Ltd

- R4-2409931	Way Forward for [111][338] TRP_TRS_MIMO_OTA	Vivo

- R4-2410128	Topic summary for [111][338] TRP_TRS_MIMO_OTA	Moderator (vivo)

RAN5 #103

N/A

09.11.2020		minor adaptations for RAN #90e

31.08.2020		minor adaptations for RAN #89e

20.04.2020		minor adaptations for RAN #88e

18.02.2020		minor adaptations for RAN #87e

14.11.2019		minor adaptations for RAN #86

18.08.2019		minor adaptations for RAN #85

12.05.2019		minor adaptations for RAN #84

27.02.2019		minor adaptations for RAN #83

21.11.2018		completion levels with colours added (for RAN #82)

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects (for RAN #81)

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template