3GPP TSG RAN Meeting #104									RP-241046

Shanghai, China, June 17-20, 2024

Status Report to TSG

Agenda item:		9.4.4.10

| WI / SI Name | Enhancement of UE TRP (Total Radiated Power) and TRS (Total Radiated Sensitivity) requirements and test methodologies for FR1 (NR SA and EN-DC) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: Yes |  | Testing part: No |
| Acronym | NR_FR1_TRP_TRS_enh |  |  |  |  |
| Unique ID | 970082 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-234064 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: | Core part: Dec/2023 | Performance part: Jun/2024 -> Sep/2024 | Testing part: |  |
| Overall Completion level | Study Item: | Core part: 100% | Performance Part: 98% | Testing part: |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN4 |
| --- | --- | --- |
| Rapporteur | Name | Wang, Ruixin |
|  | Company | vivo |
|  | Email | ruixin.wang@vivo.com |

1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | Yes |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

RAN4 agree to have one more meeting cycle (RAN4#112) to finalize the TRP TRS requirements for bands n1/n28/n41/n78.

2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

2.4	RAN4

2.4.1	Agreements

RAN4 #110bis

- General

- Topic summary for FR1 TRP TRS WI in [30]

- Ad-hoc minutes for NR_FR1_TRP_TRS_enh was captured in [35]

- draft CR to TR 38.870 on Rel-18 AC lab alignment activity outcome was endorsed in [40]

- Analysis of Rel-18 TRP TRS Performance Test Campaign and Proposals for requirements is presented in [19]

- Analysis of 3GPP TRP TRS AC lab alignment and RC harmonization measurement results-phase 3 is presented in [17]

- WF for Rel-18 FR1 TRP TRS was approved in [41]

- Reply LS to RAN5 on TT work for Rel-18 NR FR1 TRP TRS was approved in [39]

Agreements:

Issue 1-1-1: Performance metric for Coherent UE support multiple TPMI index 2~5

Agreements:

- RAN4 consider both option 1 and option 2 as performance metrics for Coherent UL-MIMO. These two options will be captured in the TR 38.870.

Issue 1-1-2: Confirmation of relative phase variation for coherent UL-MIMO

Agreements:

- A reference detailed test procedure for coherent UL MIMO testing is helpful for understanding the impacts of relative phase issue during OTA testing. Further study if the chipset resets relative phase between the two transmitting ports every time a TPMI is received (i.e. with every DCI received over PDCCH) in case the index does not change from last value (i.e. same TPMI on consecutive DCI)

- The detailed test procedure might be updated based on above feedback.

- FFS whether if there is relative-phase-variation difference between conductive testing and OTA testing.

- FFS the side conditions for OTA testing should follow the condition described in TS 38.101-1 clause 6.4D.4.

- Get feedback from TE vendors about the confirmation of conductive UL-MIMO test case procedure.

Issue 2-1-1: Final Analysis and conclusion of RAN4 Rel-18 AC Lab alignment activity

Agreements:

- Based on analysis in R4-2404644 and agreed pass fail limits, 3GPP Rel-18 FR1 TRP TRS AC lab alignment activity for BHH can be successfully concluded within the 8 labs with anechoic chamber system are well aligned.

Issue 2-2-1: Updated RC Lab alignment outcome

Agreements:

  - Based on the analysis in R4-2404644, 3GPP Rel-18 FR1 TRP TRS RC lab alignment activity can be successfully concluded (phase 3):

- 6 labs with Reverb chamber system are well aligned at n78

- 5 labs with Reverb chamber system are well aligned at n28

Issue 2-2-2: Updated RC Lab alignment schedule

Agreements:

- Conclude final Rel-18 RC lab alignment activity in RAN4#111 meeting, with receiving the final results on RC from Samsung lab.

Issue 2-2-3: RC vs AC harmonization criteria

Agreements:

- Average of AC value per-band per-device in R4-2404644 should be reference value. Pass fail limits as 0.75*Total harmonization MU. The harmonization MU value will be decided next meeting.

Issue 2-3-1: How to scale the defined large CBW to narrow CBW requirements?

Agreement:

- Option 1: REFSENS RB scaled factor, or Option 2: 7dB, as the basic scaling factor for Band n41/77/78, additional allowance value on top of it is also considered, the exact value is FFS. Need input from companies.

Issue 2-3-2: How to reflect the requirements for additional CBW in spec?

Agreements:

- The core specification shall only list the requirement based on the channel bandwidth for which performance campaign has been carried out. A specific NOTE indicating derivation/scaling of the requirement for additional channel bandwidths can be listed.

Issue 2-3-3: Feasibility of single point offset method for different CBW measurements of same band

Agreements:

- Testing result preliminary confirmed the feasibility of single point offset method for different CBW measurements of same band.

Issue 2-3-4: detailed test parameters for additional CBW

Agreements:

- RAN4 to confirm the Option 1 (Case B) (i.e., aligned with test frequencies defined in TS 38.508-1) as the test parameter for additional CBW.

Issue 2-4-1: TT for Rel-18 TRP TRS

Agreements:

- Follow the same approach in Rel-17, reuse factor of 0.62. The recommended TT is as following:

  - TT value is recommended as a ratio of MU, where TT=0.62*MU; according to currently available analysis of MU of AC system in RAN4, these TT values should be:

    - For browsing mode: TRP TT is 1.1dB for above 3GHz bands and 1dB for below 3GHz bands, TRS TT is 1.3 dB for above 3GHz bands and 1.2 dB for below 3GHz bands

    - For talk mode: TRP TT is 1.2dB for above 3GHz bands and 1.1dB for below 3GHz bands, TRS TT is 1.5 dB for above 3GHz bands and 1.4 dB for below 3GHz bands

Issue 2-4-3: Measurements plan shared by test labs for performance measurement campaign

Agreement:

- Suggest the test labs to finalize the testing for both TRP and TRS case at same device, and also talk mode and browsing mode. Rapporteur to check with MCC this week about the latest UE information collection status and share with group.

Issue 2-4-6: How to define TRP PC3 requirements based on PC2

Agreements:

- For the TDD bands support PC2, and the measurement campaign is performed with PC2, RAN4 to define PC3 TRP for that band based on 3dB offset of PC2 requirements. For FDD bands, the offset value will not be specified in Rel-18.

RAN4 #111

- General

- Topic summary for FR1 TRP TRS WI in [80]

- Ad-hoc minutes for NR_FR1_TRP_TRS_enh was captured in [78]

- CR to TR 38.870 on Rel-18 AC lab alignment activity outcome was agreed in [61]

- Updated Rel-18_TRP TRS data collection template is approved in [64]

- CR to TR 38.870 to update RC TRS definition in 5.22 and 8.6.1.1 is approved in [65]

- Editorial CR to TR 38.870 is agreed in [72]

- CRs on preliminary MU alignment are agreed in [73][74]

- WF for Rel-18 FR1 TRP TRS was approved in [84]

- CR to TR 38.870 to add CA combinations to section 4.3.5 and editorial corrections was agreed in [87]

- Final Analysis of 3GPP TRP TRS AC lab alignment and RC harmonization measurement results was presented in [56]

- CR to TR38.870 on CBW scaling of the TRS requirement was agreed in [90]

- CR to TS38.161 on PC3 scaling of the TRP requirement was agreed in [91]

- CR to TR38.870 on UL MIMO radiated output power metric was agreed in [86]

- Final Analysis of Rel-18 TRP TRS Performance Test Campaign is presented in [58]

Agreements:

Issue 2-1-1: Total Harmonization MU

Conclusions:

Total harmonization MU=AC MU+0.2dB

Issue 2-1-2: Maintenance work for RC unavailable results

Conclusions:

Allow to update RC harmonization data in maintenance phase when the unfinished n28 testing from lab 9 is done.

Issue 2-1-3: Rel-18 RC harmonization conclusions

Conclusions:

Conclude RC test method harmonization for bands>3GHz. FFS below 3GHz

Sub-topic 2-2 Additional CBW for band n28/n41/n77/n78 requirements

Issue 2-2-1: Scaling factor to derive requirements on alternate narrow CBW for n28/n41/n77/n78?

Conclusions:

7dB offset value for n41/77/78 TRS, between 100MHz and 20MHz CBW.

3dB offset value for n28 TRS, between 20MHz and 10MHz CBW.

Issue 2-2-3: SPOT method for alternative CBW testing

Conclusions:

Capture SPOT method into TR as alternative for TRS measurements when using different channel bandwidth configurations on the same channel.

Sub-topic 2-3 Rel-18 TRP TRS requirements work

Issue 2-3-1: Thresholds of UE information for measured data pool

Conclusions:

- Adopt the following values for the thresholds related to the devices pool for Rel-18.

  - Minimum number of devices for each band, each device size, each power class: 40

  - Minimum number of device models: 30

  - Minimum number of devices' vendors: 5

  - Percentage of devices from second-half 2021 to 2024: 100%

  - Percentage of the devices that are certified by PTCRB/GCF/NAL-CTA/CE/FCC: 100%

Issue 2-3-2: Requirements discussion

Working procedure:

- RAN4 will go to band-by-band discussion for new bands.

Issue 2-3-3: Requirements decision

Conclude discussions of requirements for band n1 and n28:

| requirements | Talk mode TRP |  | Talk mode TRS |  | Browsing mode TRP |  | Browsing mode TRS |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | n1 PC3 | n28 PC3 | n1 TRS | n28 TRS | n1 PC3 | n28 PC3 | n1 TRS | n28 TRS |
| requirements | [10.5] | [7.0] | [-87.3] | [-80.2] | [11.5] | [9.5] | [-88.5] | [-83.5] |

Rel-18 measurement campaign is closed, there will be no data pool update.

Annex: UE information statistics by RAN4 MCC (for information)

| Total Devices | 83 |  |  |  |
| --- | --- | --- | --- | --- |
| Total Models | 81 |  |  |  |
| Total number of device vendors | 19 |  |  |  |
| Percentage of devices per vendor | 10 | 12.05% | Vendor A |  |
|  | 15 | 18.07% | Vendor B |  |
|  | 3 | 3.61% | Vendor C |  |
|  | 21 | 25.30% | Vendor D |  |
|  | 12 | 14.46% | Vendor E |  |
|  | 1 | 1.20% | Vendor F |  |
|  | 3 | 3.61% | Vendor G |  |
|  | 1 | 1.20% | Vendor H |  |
|  | 6 | 7.23% | Vendor I |  |
|  | 1 | 1.20% | Vendor J |  |
|  | 1 | 1.20% | Vendor k |  |
|  | 1 | 1.20% | Vendor N |  |
|  | 1 | 1.20% | Vendor O |  |
|  | 1 | 1.20% | Vendor P |  |
|  | 2 | 2.41% | Vendor Q |  |
|  | 1 | 1.20% | Vendor R |  |
|  | 1 | 1.20% | Vendor J |  |
| Percentage of devices per power class per band | 2 | n1 | PC2 | 2.41% |
|  | 78 |  | PC3 | 93.98% |
|  | 0 | n28 | PC2 | 0.00% |
|  | 77 |  | PC3 | 92.77% |
|  | 61 | n41 | PC2 | 73.49% |
|  | 14 |  | PC3 | 16.87% |
|  | 70 | n78 | PC2 | 84.34% |
|  | 6 |  | PC3 | 7.23% |
| Percentage of devices per supported band |  |  | n1 | 96.39% |
|  |  |  | n28 | 88.89% |
|  |  |  | n41 | 90.36% |
|  |  |  | n78 | 91.57% |
| Percentage of devices per year of production | 8 |  | 2021 | 9.64% |
|  | 30 |  | 2022 | 36.14% |
|  | 35 |  | 2023 | 42.17% |
|  | 10 |  | 2024 | 12.05% |
| Percentage of devices with at least one certification | 83 |  |  | 100.00% |
| Percentage of devices for each certification | 27 |  | GCF | 32.53% |
|  | 12 |  | PTCRB | 14.46% |
|  | 30 |  | NAL(CTA) | 36.14% |
|  | 30 |  | FCC | 36.14% |
|  | 35 |  | CE | 42.17% |
|  | 0 |  | Not Available |  |

2.4.2	Remaining Open issues

- Performance part

    - Finalize TRP TRS requirements and Cat B CR for bands n1/n28/n41/n78

2.5	RAN5

2.5.1	Agreements

RAN5 #103

- N/A

2.4.2	Remaining Open issues

- None

3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

N/A

4.	References

RAN4 #110bis

- R4-2404141	On Single-Layer UL MIMO for coherent UEs	Keysight Technologies UK Ltd, vivo

- R4-2404144	3GPP Rel-18 TRP TRS Measurement Campaign Results - Sporton	Sporton International Inc

- R4-2404190	Inputs to Rel-18 FR1 TRP TRS Performance Phase	Apple

- R4-2404196	Draft CR to TR38.870 on UL MIMO radiated output power metric	Apple

- R4-2404197	On performance metric for coherent UL MIMO UE	Apple

- R4-2404201	Huawei Measurement results for TRP TRS RC harmonization n28 AC talk and browsing mode	Huawei, HiSilicon

- R4-2404203	on AC and RC alignment criteria	Huawei, HiSilicon

- R4-2404204	on the TRP offset between PC2 and PC3	Huawei, HiSilicon

- R4-2404274	Element - 3GPP Rel-18 TRP TRS Performance Test Campaign	Element Materials Technology

- R4-2404595	Discussion on Rel-18 FR1 TRP TRS remaining issues in test method	Samsung

- R4-2404596	Measurement results for 3GPP Rel-18 TRP TRS RC harmonization activity_n78	Samsung

- R4-2404597	Discussion on Rel-18 FR1 TRP TRS RC harmonization and AC requirements	Samsung

- R4-2404600	3GPP Rel-18 TRP TRS AC and RC lab alignment result update from SRTC	SRTC

- R4-2404610	TRP TRS Measurement campaign result from SRTC	SRTC

- R4-2404613	On AC and RC Harmonization Requirements	Bluetest AB

- R4-2404618	Measurement results for Rel-18 TRP TRS Performance Test Campaign 	SGS Wireless

- R4-2404644	Analysis of 3GPP TRP TRS AC lab alignment and RC harmonization measurement results-phase 3	vivo

- R4-2404645	Measurement results for Rel-18 TRP TRS Performance Test Campaign	vivo

- R4-2404646	Analysis of Rel-18 TRP TRS Performance Test Campaign and Proposals for requirements	vivo

- R4-2404647	Proposals on concluding performance metric for Coherent UL MIMO	vivo, CAICT, CMCC, China Telecom, China Unicom, Huawei, HiSilcon, Xiaomi, OPPO, MediaTek, Google

- R4-2404648	Reply LS on TT work for Rel-18 NR FR1 TRP TRS	vivo

- R4-2404649	draft CR to TR 38.870 on Rel-18 AC lab alignment activity outcome	vivo

- R4-2405170	Consideration on UL MIMO metrics	OPPO

- R4-2405171	On RC harmonization with AC	OPPO, SRTC

- R4-2405172	3GPP Rel-18 TRP TRS Performance Test Campaign-OPPO	OPPO

- R4-2405396	Discussion on Rel-18 TRP and TRS requirements	Qualcomm Incorporated

- R4-2405455	Discussion on TRP TRS requirements issues	CAICT, SAICT

- R4-2405456	CAICT - Measurement results for Rel-18 TRP TRS Performance Test Campaign	CAICT

- R4-2405500	CR to TR 38.870 to add CA combinations to section 4.3.5 and editorial corrections	Huawei, HiSilicon

- R4-2405855	Topic summary for [110bis][336] NR_FR1_TRP_TRS_enh	Moderator (vivo)

- R4-2405872	Detailed timing for UL MIMO TRP procedure	ROHDE & SCHWARZ

- R4-2405897	On completion of the measurement campaign for FR1 OTA TRP-TRS minimum performance requirements definition	TELECOM ITALIA S.p.A., Vodafone, Orange, Deutsche Telekom

- R4-2406072	Ad-hoc meeting minutes for [110bis][336] NR_FR1_TRP_TRS_enh	Vivo

- R4-2406073	Way Forward for [110bis][336] NR_FR1_TRP_TRS_enh	Vivo

- R4-2406074	Ad-hoc meeting minutes for [110bis][336] NR_FR1_TRP_TRS_enh	Vivo

- R4-2406075	Way Forward for [110bis][336] NR_FR1_TRP_TRS_enh	Vivo

- R4-2406077	Huawei Measurement results for TRP TRS RC harmonization n28 AC talk and browsing mode	Huawei, HiSilicon

- R4-2406078	Element - 3GPP Rel-18 TRP TRS Performance Test Campaign	Element Materials Technology

- R4-2406079	Reply LS on TT work for Rel-18 NR FR1 TRP TRS	vivo

- R4-2406080	draft CR to TR 38.870 on Rel-18 AC lab alignment activity outcome 	vivo

- R4-2406081	Way Forward for [110bis][336] NR_FR1_TRP_TRS_enh	Vivo

RAN4 #111

- R4-2407054	CR to TR38.870 on UL MIMO radiated output power metric	Apple

- R4-2407055	On remaining issues for coherent UL-MIMO	Apple

- R4-2407056	On CBW scaling of the TRS requirement	Apple

- R4-2407057	CR to TR38.870 on CBW scaling of the TRS requirement	Apple

- R4-2407058	CR to TS38.161 on PC3 scaling of the TRP requirement	Apple

- R4-2407099	3GPP Rel-18 TRP TRS Measurement Campaign Results - Sporton Final	Sporton International Inc

- R4-2407101	Huawei TRP TRS results for measurement campaign	Huawei, HiSilicon

- R4-2407102	On AC and RC alignment MU value	Huawei, HiSilicon

- R4-2407120	CR to TR 38.870 to add CA combinations to section 4.3.5 and editorial corrections	Huawei, HiSilicon

- R4-2407167	Element - 3GPP Rel-18 TRP TRS Performance Test Campaign	Element Materials Technology

- R4-2407370	Measurement results for Rel-18 TRP TRS Performance Test Campaign	SGS Wireless

- R4-2407623	Rel-18 TRP TRS Measurement campaign result from SRTC	SRTC

- R4-2407714	A Qualitative Analysis of AC-RC Harmonization	Bluetest AB

- R4-2407900	Discussion on Rel-18 FR1 TRP TRS remaining issues in performance phase	Samsung

- R4-2408096	Final Analysis of 3GPP TRP TRS AC lab alignment and RC harmonization measurement results	vivo

- R4-2408097	Measurement results for Rel-18 TRP TRS Performance Test Campaign-vivo	vivo

- R4-2408098	Final Analysis of Rel-18 TRP TRS Performance Test Campaign	vivo

- R4-2408099	Proposals for concluding TRP TRS performance part WI	vivo

- R4-2408100	Reply LS on 3GPP NR TRP TRS OTA requirements	vivo

- R4-2408101	CR to TR 38.870 on Rel-18 AC lab alignment activity outcome	vivo

- R4-2408102	CR to TR 38.870 on Rel-18 RC lab alignment and harmonization outcome	vivo

- R4-2408103	Draft CR to TS 38.161 on FR1 TRP TRS requirements	vivo

- R4-2408104	Updated Rel-18_TRP TRS data collection template	vivo

- R4-2408274	CR to TR 38.870 to update RC TRS definition in 5.22 and 8.6.1.1	Bluetest AB

- R4-2408897	On RC harmonization	OPPO

- R4-2408898	On PC3 TRP requirement	OPPO

- R4-2408899	3GPP Rel-18 TRP TRS Performance Test Campaign-OPPO	OPPO

- R4-2409043	On FR1 OTA TRP-TRS minimum performance requirements definition	Telecom Italia S.p.A., Vodafone, BT plc, Orange, Deutsche Telekom

- R4-2409185	Discussion on TRP TRS requirements issues	CAICT, SAICT

- R4-2409186	Discussion on RC and AC harmonization	CAICT, SAICT

- R4-2409773	Editorial CR to TR 38.870	ROHDE & SCHWARZ

- R4-2409775	CR on preliminary MU alignment	ROHDE & SCHWARZ

- R4-2409776	CR on preliminary MU alignment	ROHDE & SCHWARZ

- R4-2409906	Reply LS on 3GPP NR TRP TRS OTA requirements	vivo

- R4-2409907	Draft CR to TS 38.161 on FR1 TRP TRS requirements	vivo

- R4-2409908	On FR1 OTA TRP-TRS minimum performance requirements definition	Telecom Italia S.p.A., Vodafone, BT plc, Orange, Deutsche Telekom

- R4-2409920	Ad-hoc meeting minutes for [111][336] NR_FR1_TRP_TRS_enh	Vivo

- R4-2409930	Way Forward for [111][336] NR_FR1_TRP_TRS_enh	Vivo

- R4-2410126	Topic summary for [111][336] NR_FR1_TRP_TRS_enh	Moderator (vivo)

- R4-2409930	Way Forward for [111][336] NR_FR1_TRP_TRS_enh	Vivo

- R4-2409998	Way Forward for [111][336] NR_FR1_TRP_TRS_enh	Vivo

- R4-2410025	Way Forward for [111][336] NR_FR1_TRP_TRS_enh	Vivo

- R4-2410765	Way Forward for [111][336] NR_FR1_TRP_TRS_enh	Vivo

- R4-2409902	CR to TR38.870 on UL MIMO radiated output power metric	Apple, Samsung, vivo

- R4-2409996	CR to TR38.870 on UL MIMO radiated output power metric	Apple, Samsung, vivo

- R4-2409993	CR to TR 38.870 to add CA combinations to section 4.3.5 and editorial corrections	Huawei

- R4-2409903	CR to TR 38.870 on Rel-18 RC lab alignment and harmonization outcome	vivo

- R4-2409997	CR to TR 38.870 on Rel-18 RC lab alignment and harmonization outcome	vivo

- R4-2409904	CR to TR38.870 on CBW scaling of the TRS requirement	Apple, Samsung, vivo

- R4-2409905	CR to TS38.161 on PC3 scaling of the TRP requirement	Apple, Samsung, vivo

RAN5 #103

N/A

09.11.2020		minor adaptations for RAN #90e

31.08.2020		minor adaptations for RAN #89e

20.04.2020		minor adaptations for RAN #88e

18.02.2020		minor adaptations for RAN #87e

14.11.2019		minor adaptations for RAN #86

18.08.2019		minor adaptations for RAN #85

12.05.2019		minor adaptations for RAN #84

27.02.2019		minor adaptations for RAN #83

21.11.2018		completion levels with colours added (for RAN #82)

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects (for RAN #81)

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template