3GPP TSG RAN Meeting #104										RP-241051

Shanghai, China, June 17-20, 2024

## Status Report to TSG

Agenda item:			9.3.1.1

| WI / SI Name | NR MIMO Phase 5 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: Yes |  | Testing part: No |
| Acronym | NR_MIMO_Ph5 |  |  |  |  |
| Unique ID | 1020089 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-234007 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: n/a | Core part: 09/2025 | Performance part: 03/2026 | Testing part: n/a |  |
| Overall Completion level | Study Item: n/a | Core part: 35% | Performance Part: 0% | Testing part: n/a |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN1 |
| --- | --- | --- |
| Rapporteur (primary) | Name | Eko Onggosanusi |
|  | Company | Samsung |
|  | Email | eko.o@samsung.com |
| Rapporteur | Name | Darcy Tsai |
|  | Company | MediaTek |
|  | Email | darcy.tsai@mediatek.com |
| Rapporteur | Name | Chai Li |
|  | Company | China Mobile |
|  | Email | chaili@chinamobile.com |

## 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

## 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

## 2.1	RAN1

#### 2.1.1	Agreements

In RAN1#116bis, the following agreements were made.

UE-initiated/event-driven beam management

Agreement

On beam report transmission procedure for UE-initiated/event-driven beam reporting, following modes are supported:

- Mode A (dynamically scheduling UCI by gNB):

  - Step 1: UE transmits a first PUCCH (one-bit/multi-bit) to request a resource for a second UL channel to carry beam report

    - FFS: Request format, e.g., SR or a new UCI type.

  - Step 2: UE detects the DCI format to indicate a resource for a second UL channel to carry beam report.

  - Step 3: Beam report is transmitted in second UL channel.

    - FFS: Details on the second UL channel, e.g., whether the second UL channel is PUCCH, PUSCH or both

  - This mode is basic UE capability (i.e. all UE supporting UE-initiated/event-driven beam reporting should support this feature).

  - No new DCI format is introduced.

- Mode B (UCI in pre-configured resource(s) for second UL channel):

  - Step 1: UE transmits a first PUCCH (one-bit/multi-bit) notifying a second UL channel to carry beam report

    - FFS: Notification format, e.g., SR or a new UCI type.

  - Step 2: UE transmits the beam report in the second UL channel.

    - FFS: Details on the second UL channel, e.g., whether the second UL channel is PUCCH, PUSCH or both

  - The notification in Step1 is in a separate reporting instance from the beam report in Step 2.

FFS: Whether UE receives acknowledge information with response to each step for all modes

For above procedures, cross-CC beam reporting is supported for both modes.

- FFS: Details.

Agreement

On UE-initiated/event-driven beam reporting, regarding trigger-event detection for beam reporting, at least support Event-2: Quality of at least one new beam, such as L1-RSRP, becomes a threshold value better than the current beam.

- At least L1-RSRP is supported as quality metrics used for Event-2

  - FFS: How the L1-RSRP is used to determine the triggering event (e.g. timer, counter, filter coefficient)

  - FFS: Whether the network controls how the L1-RSRP is used to determine the triggering event

- Regarding RS measurement for the new beam for Event-2, down-select one or more of the following:

  - Option-3a (explicit manner): The RS(s) for new beam(s) are explicitly configured by RRC (e.g., reusing legacy configuration of RS measurement or in TCI-State) or MAC-CE

  - Option-3b (implicit manner): The RS(s) for new beam(s) are implicitly derived from QCL RS(s) of activated TCI state(s).

  - Option-3c (implicit manner): The RS(s) for new beam(s) are implicitly derived from QCL RS(s) of configured TCI state(s).

- Note-1: ‘New/current beam’ is for discussion purpose.

- Note-2: Other trigger events/quality metrics (e.g., L1-SINR) are not precluded.

- Note-3: For above implicit manner(s), if there are two QCL RSs in a TCI state, the measurement RS is derived from RS w.r.t. QCL-TypeD, if applicable.

Agreement

On UE-initiated/event-driven beam reporting, regarding Event-2, the threshold value is RRC configured

Agreement

On UE-initiated/event-driven beam reporting, regarding Event-2, ‘current beam’ is a beam corresponding to the indicated TCI state.

- Regarding RS measurement for the current beam for Event-2, Option-2a is supported:

  - Option-2a (implicit manner): The RS for current beam is implicitly derived from a QCL RS of indicated TCI state.

    - FFS: The RS for current beam can be either the QCL RS in the indicated TCI state or the SSB which is QCLed with the QCL RS in the indicated TCI state.

  - FFS: Option-2c (explicit manner): The RS for current beam is explicitly configured by RRC or MAC-CE.

    - Note: SSB or CSI-RS can be configured

Agreement

On UE-initiated/event-driven beam reporting, further study the following trigger events:

- Event-1: Quality of the current beam is worse than a certain threshold.

- Event-3: Quality of a new beam is better than a certain threshold.

- Event-4: Quality of the current beam is worse than a threshold 1, and quality of at least one new beam is better than a threshold 2.

- Event-5: Absolute value of the difference between the quality of the current beam and the quality of at least one new beam is lower than a threshold.

- Event-6: When the current beam is not in the best K>1 beams (out of configured beams for measurement and reporting).

- Event-7a: Quality of at least one new beam, such as L1-RSRP, becomes a threshold value better than the RS derived from the activated TCI state with the worst quality.

- Event-7b: Quality of at least one new beam, such as L1-RSRP, becomes a threshold value better than the RS derived from the activated TCI state with the best quality.

- Event-8: Quality of M>1 new beams, such as L1-RSRP, become a threshold value better than the current beam.

- Event-9: Quality of at least one new beam, such as L1-RSRP, becomes a threshold value better than the configured reference RS (can be SSB or CSI-RS).

Agreement

On UE-initiated/event-driven beam reporting, regarding UL signaling content(s) of L1-RSRP report depending on Event-2, in a report instance, the following options are provided for down-selection (other options are not precluded) in RAN1#117

- Option-1 (variable size): N beam(s) are reported in the report instance, where N ∈ {1, 2, ..., Nmax}

  - The N beam(s) should satisfy the condition of Event-2

  - Nmax is configured by gNB

  - FFS: Whether the indication of payload size should be provided additionally.

- Option-1a (variable size): N beam(s) are reported in the report instance, where N ∈ {1, 2, ..., Nmax}

  - At least one of N reported beam(s) should satisfy the condition of Event-2

  - Nmax is configured by gNB

  - FFS: Whether the indication of payload size should be provided additionally.

  - FFS: Details on how value of N is determined by the UE

- Option-1b: N beam(s) are reported in the report instance, where N ∈ {1, 2, ..., Nmax}

  - The N beam(s) should satisfy the condition of Event-2

  - Nmax is configured by gNB

  - Payload size does not vary as a function of N

  - FFS: Zero-padding can be provided if N is less than Nmax.

- Option-2: Only N=1 beam is reported in the report instance

  - The reported beam should satisfy the condition of Event-2

- Option-3: N ≥ 1 beam(s) are reported in the report instance,

  - At least one of N reported beam(s) should satisfy the condition of Event-2

  - N is configured by gNB

- Other options are not precluded.

- FFS: Whether the measurement results for current beam is always reported or can be enabled by RRC.

- FFS: When current beam is reported, whether the current beam is counted in the N reported beams.

- The selected option shall satisfy Event-2.

CSI

Agreement

For the Rel-19 aperiodic standalone CJT calibration reporting, given the NTRP configured NZP CSI-RS resources/resource sets and the selected N resources/resource sets, support reporting, in one CSI reporting instance, {n, , n=0, 1, …, N – 1, n≠nref, =0,1,…,-1}, where n, denotes the measured phase offset between the n-th CSI-RS resource/resource set and the reference CSI-RS resource/resource set nref for the -th frequency unit

-  =1 is supported

  - FFS: whether >1 (sub-band reporting) is also supported. For this decision, companies are encouraged to evaluate performance loss without the support of >1 due to phase offset induced by TX-RX timing misalignment.

- The value n, indicates a uniformly quantized phase between –A and A, or 0 and A

  - FFS: supported quantization alphabet(s) (including A and resolution) for n,

- FFS: Detailed UCI design

Agreement

For the Rel-19 aperiodic standalone CJT calibration reporting, for a given CJT calibration report of one [or more] CJT calibration report type, the nref is selected by the UE and reported as a part of the CJT calibration report

- Note: CJT calibration report type refers to the Doffset/d report, FO report, and, TDD PO report

Agreement

For the Rel-19 aperiodic standalone CJT calibration reporting of {FOn , n=0, 1, …, NTRP – 1, n≠nref}, the value of FOn indicates a uniformly quantized frequency offset between 0 and AFO

Agreement

For the Rel-19 aperiodic standalone CJT calibration reporting of {(Dn,offset, dn), n=0, 1, …, NTRP – 1, n≠nref}, regarding the interval [δi,δi+1] which Dn,offset falls into, δ0,δ1,…,δMD-1 is uniformly spaced between 0 and AD, i.e. δi=i×ADMD-1,   i=0,1,…, MD-1, with  MD=2B and [δMD-1,∞] represent ‘out-of-range’

Agreement

For the Rel-19 aperiodic standalone CJT calibration reporting, the UE reports for all the configured NTRP NZP CSI-RS resources/resource sets

- FFS (by RAN1#116bis): Whether an ‘invalid’ or ‘out-of-range’ quantization state/hypothesis is supported for all the types of CJT calibration reporting. Note that ‘out-of-range’ is supported for the (Dn,offset, dn) reporting

Agreement

For the Rel-19 Type-I single-panel (SP) codebook refinement for 48, 64, and 128 CSI-RS ports, for RI=1-4, support the following:

- Scheme-A (based on Scheme1 in RAN1#116 agreement): Adding new (N1, N2) values for the Rel-15 Type-I single-panel codebook mode-1 (L=1) where 2N1N2 (>32) is the total number of CSI-RS ports across aggregated NZP CSI-RS resources, and for rank-3/4, follow legacy mechanisms for <16 ports

- Scheme-B (based on Scheme2 in RAN1#116 agreement): Adding new (N1, N2) values where 2N1N2 (>32) is the total number of CSI-RS ports across aggregated NZP CSI-RS resources, and

  - W1 structure:

    - For each layer, reuse legacy Rel-16 eType-II SD basis with L=1 to determine the DFT-based SD basis candidates

    - For 1<RI≤4, L=1 SD basis vector is independently selected for different layers

      - The SD basis selection indication includes layer-common (q1,q2) and log2N1N2 bits for each layer

      - Note: This implies that each of the SD basis vectors is selected from a group of N1N2 orthogonal basis vectors

  - W2 structure: Layer-specific inter-polarization co-phasing with the alphabet {+1, +j, -1, -j}

FFS (RAN1#116bis): For Rel-19 Type-I SP, whether to support Mode-C based on Scheme5 in RAN1#116 agreement with L=1 for RI=2-4

FFS (RAN1#116bis): For Rel-19 Type-I SP, whether inter-polarization amplitude for Mode-B can also be supported

FFS: Discuss further if Rel-19 Type-I MP extension based on scheme 4 is needed

Agreement

For the Rel-19 Type-I single-panel (SP) codebook refinement for 48, 64, and 128 CSI-RS ports, for RI=1-4, O1=O2 is 4

- FFS: Additional support for O1=O2 is 2 when RI=1-4 (including separate UE capability)

Agreement

For the Rel-19 Type-I and Type-II codebook refinement for 48, 64, and 128 CSI-RS ports, regarding the mapping from CSI-RS resource index/port index per resource and port index to CSI/PMI calculation, support NW to configure UE with one of the following mapping methods via higher-layer (RRC) signaling,

- Mapping method 1: Sequential ordering/indexing within (1st resource, 1st polarization), then (2nd resource, 1st polarization), …, then (Kth resource, 1st polarization), then (1st resource, 2nd polarization), then (2nd resource, 2nd polarization), …, then (Kth resource, 2nd polarization)

- Mapping method 2: Sequential ordering/indexing within (where K*n2 = N2):

  - for the 1st polarization, (1st n2 ports in 1st resource, 1st polarization), (1st n2 ports in 2nd resource, 1st polarization), …, (1st n2 ports in Kth resource, 1st polarization), then (2nd n2 ports in 1st resource, 1st polarization), (2nd n2 ports in 2nd resource, 1st polarization), …, (2nd n2 ports in Kth resource, 1st polarization), … then (N1th n2 ports in 1st resource, 1st polarization), (N1th n2 ports in 2nd resource, 1st polarization), …, (N1th n2 ports in Kth resource, 1st polarization) ,

  - and then for the 2nd polarization, (1st n2 ports in 1st resource, 2nd polarization), (1st n2 ports in 2nd resource, 2nd polarization), …, (1st n2 ports in Kth resource, 2nd polarization), then (2nd n2 ports in 1st resource, 2nd polarization), (2nd n2 ports in 2nd resource, 2nd polarization), …, (2nd n2 ports in Kth resource, 2nd polarization), … then (N1th n2 ports in 1st resource, 2nd polarization), (N1th n2 ports in 2nd resource, 2nd polarization), …, (N1th n2 ports in Kth resource, 2nd polarization)

FFS: Exact port indexing within each CSI-RS resource or across K CSI-RS resources

FFS: Whether the following is also supported:

- Mapping method 3 (for K=4): Sequential ordering/indexing within (where N1=2*n1, N2 = 2*n2):

  - for the 1st polarization, (1st n2 ports in 1st resource, 1st polarization), (1st n2 ports in 2nd resource, 1st polarization), then (2nd n2 ports in 1st resource, 1st polarization), (2nd n2 ports in 2nd resource, 1st polarization), …, then (n1th n2 ports in 1st resource, 1st polarization), (n1th n2 ports in 2nd resource, 1st polarization),

  - for the 1st polarization, (1st n2 ports in 3rd resource, 1st polarization), (1st n2 ports in 4th resource, 1st polarization), then (2nd n2 ports in 3rd resource, 1st polarization), (2nd n2 ports in 4th resource, 1st polarization), then (n1th n2 ports in 3rd resource, 1st polarization), (n1th n2 ports in 4th resource, 1st polarization),

  - and then for the 2nd polarization, (1st n2 ports in 1st resource, 2nd polarization), (1st n2 ports in 2nd resource, 2nd polarization), then (2nd n2 ports in 1st resource, 2nd polarization), (2nd n2 ports in 2nd resource, 2nd polarization), … then (n1th n2 ports in 1st resource, 2nd polarization), (n1th n2 ports in 2nd resource, 2nd polarization),

  - and then for the 2nd polarization, (1st n2 ports in 3rd resource, 2nd polarization), (1st n2 ports in 4th resource, 2nd polarization), then (2nd n2 ports in 3rd resource, 2nd polarization), (2nd n2 ports in 4th resource, 2nd polarization), then (n1th n2 ports in 3rd resource, 2nd polarization), (n1th n2 ports in 4th resource, 2nd polarization),

- Other methods are not precluded

Agreement

For the Rel-19 Type-I and Type-II codebook refinement for 48, 64, and 128 CSI-RS ports, regarding NZP CSI-RS resource aggregation to attain 32 < P (or PCSI-RS) ≤ 128, all K NZP CSI-RS resources shall be located within 1 slot or 2 consecutive slots (following legacy principle from Rel-18 Type-II CJT), and are associated with a same CSI-RS resource set:

- FFS(RAN1#116bis): Whether ‘within 1 slot’ should be basic feature and ‘within 2 consecutive slots’ should be UE capability

- FFS (RAN1#116bis): Verify if this can be achieved without refinement on CSI-RS resource set restrictions (i.e. same CDM type, same RE density, same starting RB for 0.5 RE/RB/port density, same number of RBs, for AP-CSI-RS same slot offset). If not, the supported refinement(s)

- FFS (RAN1#116bis): Whether additional restriction(s) beyond the restrictions on the CSI-RS resources associated with a same resource set are needed (e.g. same QCL, PCoffset, PCOffsetSS)

- FFS (RAN1#116bis): Extension for Rel-19 Type-II based on Rel-18 Type-II Doppler with aperiodic CMR

Agreement

For the Rel-19 Type-I and Type-II codebook refinement for 48, 64, and 128 CSI-RS ports, the legacy resource configuration for interference measurement is reused, i.e. only one NZP CSI-RS resource for interference measurement or only one CSI-IM resource can be configured where the one IM resource is associated with all the K CSI-RS resources in the CSI-RS resource set for channel measurement

Conclusion

For the Rel-19 Type-II codebook refinement for 48, 64, and 128 CSI-RS ports, on SD basis selection indication, there is no consensus on additional spec enhancement. Therefore, the same approach as legacy (using a layer-common log2P/2L-bit indicator) is reused

- Note: How to trade-off memory and calculation is left to implementation

Agreement

For the Rel-19 Type-II codebook refinement for 48, 64, and 128 CSI-RS ports, on CBSR, refine the legacy CBSR as follows:

- Only 1-bit hard restriction is supported (analogous to Rel-18 Type-II)

- Moving (N1, N2) configuration out from CBSR IE and the CBSR can be optional configured

  - Send LS to RAN2, and subject to RAN2 consent. Final LS in R1-2403650.

- Group-based CBSR granularity where each bit in the CBSR is associated with a set of X1X2 SD basis vectors, where the set includes X1 adjacent SD basis vectors along the N1 direction and/or X2 adjacent SD bases along the N2 direction

  - FFS: Value(s) of X1 and X2 and detailed design/spec impact

FFS: Whether/how to enable shared CBSR in RRC configuration for Type-I/-II codebooks with a same (N1,N2).

Agreement

For the Rel-19 Type-II codebook refinement for 48, 64, and 128 CSI-RS ports based on Rel-18 Type-II Doppler codebook,

- On UCI omission rules, fully reuse the legacy Rel-18 Type-II Doppler design

- On the definition and detailed design of UCI parameters, fully reuse the legacy Rel-18 Type-II Doppler design

Agreement

For the Rel-19 Type-I codebook refinement for 48, 64, and 128 CSI-RS ports, the (N1,N2) values for P=64 are supported as a part of the respective basic feature, while those for P=48 and P=128 are supported as two separate UE capabilities

Agreement

For the Rel-19 CRI-based CSI refinement for up to 128 CSI-RS ports,

- For Rel-15 Type-I Single Panel codebook, M is NW-configured via higher-layer (RRC) signaling with candidate value(s) of {1, …, min(4,KS)}

  - The maximum value of M is subject to UE capability

- For Rel-16 eType-II, M=1 is supported

  - The maximum value of KS is {1,2,3,4} and subject to UE capability

    - The support for Rel-16 eType-II is a separate UE capability at least from the support for Rel-19 Type-I and Type-II codebook refinements

  - FFS (RAN1#116bis): The support for M=2, and if so, the value of M={1, 2} is NW-configured via higher-layer (RRC) signaling, and if additional restriction(s) are needed

FFS: The determination of M reported beams

Note: Selection algorithm of CRI(s) from measurement of KS>1 NZP-CSI-RS resources is up to UE implementation.

Agreement

For the Rel-19 CRI-based CSI refinement for up to 128 CSI-RS ports, support the following time-domain behaviours:

- For Rel-15 Type-I SP codebook:

  - Aperiodic CSI (channel(s) and multiplexing rules follows legacy)

  - Periodic CSI (channel(s) and multiplexing follows legacy), hence wideband PMI only

  - Semi-persistent CSI (channel(s) and multiplexing rules follows legacy)

- For Rel-16 eType-II codebook:

  - Semi-persistent CSI on PUSCH (channel(s) and multiplexing rules follows legacy)

  - Aperiodic CSI (channel(s) and multiplexing rules follows legacy)

Agreement

For the Rel-19 CRI-based CSI refinement for up to 128 CSI-RS ports, the following report quantities are supported:

- ‘cri-RI-PMI-CQI ‘

- ‘cri-RI-LI-PMI-CQI’ (only for Type-I)

- FFS: ‘cri-RI-i1-CQI’ (only for Type-I)

- FFS: ‘cri-RI-i1’ (only for Type-I)

Agreement

For the Rel-19 CRI-based CSI refinement for up to 128 CSI-RS ports, on the configured KS>1 NZP CSI-RS resources, reuse the legacy CMR and IMR rules for the Rel-15 CRI-based reporting. This includes:

- All the KS NZP CSI-RS resources are associated with a same CSI-RS resource set

- KS CSI-IM resources can be configured (implying one-to-one correspondence between KS CMRs and KS CSI-IMs)

FFS: Whether all the KS NZP CSI-RS resources share a same Pcoffset and PcoffsetSS

FFS: Whether or not NZP CSI-RS resource for interference measurement can be configured. FFS further details.

Agreement

For the Rel-19 aperiodic standalone CJT calibration reporting, assuming the Rel-17/18 unified TCI framework, regarding TCI/QCL, the following is assumed:

- Based on the legacy support of up to 2 TCI states for PDSCH-CJT

Agreement

For the Rel-19 aperiodic standalone CJT calibration reporting, regarding the applicable type(s) of the configured NTRP NZP CSI-RS resources/resource sets when ReportQuantity is ‘cjtc-Dd’ (Doffset+d) or ‘cjtc-F’ (frequency offset), periodic TRS (‘CSI-RS for tracking’) resource set is used for each of the NTRP NZP CSI-RS resource sets

- Extend the maximum allowed number of TRS resource sets to 4 (note: legacy supports max. 3 from Rel-18 TDCP)

- FFS: Whether all the resources across the NTRP TRS resource sets are configured with the same bandwidth

- FFS: Whether aperiodic TRS resource set can also be used

- FFS: Whether CSI-RS for CSI can also be used

- FFS: Whether different RE locations (FDM) are supported for the RSs

- FFS: additional time separation between RSs

- FFS: The exact number of CSI-RS resource(s) within each TRS resource set

- FFS: applicable type(s) if joint reporting of both Doffset/d and FO is supported

Agreement

For the Rel-19 aperiodic standalone CJT calibration reporting, regarding the applicable type(s) of the configured NTRP NZP CSI-RS resources/resource sets when ReportQuantity is ‘cjtc-P’ (DL/UL phase offset), single-port CSI-RS(s) for CSI is used

- FFS: Whether multi-port CSI-RS for CSI can also be used

- FFS: Whether all the ‘CSI-RS for CSI’ resources within each resource set follow the legacy pre-Rel-19 rules of CSI-RS resources associated with a same resource set, and whether only 1 or NTRP >1 resource sets are used

- FFS: The exact number of CSI-RS resource(s) within each resource set

- FFS: Whether different RE locations (FDM) are supported for the RSs

- FFS: additional restrictions e.g. time separation between RSs, bandwidth

Agreement

For the Rel-19 aperiodic standalone CJT calibration reporting, regarding phase offset reporting, the value n, indicates a uniformly quantized phase between 0 and 2

Agreement

For the Rel-19 aperiodic standalone CJT calibration reporting, the dynamic range and resolution parameters for delay offset reporting Dn,offset, i.e. (AD, MD), are NW-configured via higher-layer (RRC) signalling from the following candidate values:

- AD ={0.5CP, 0.75CP, CP, 1.5CP, 2CP, 14∆f,  112∆f, 124∆f} where CP and ∆f denote the length of the cyclic prefix according to the current specifications (for normal CP) within a slot and the SCS, respectively

  - FFS: Further down-selection of the above candidate values for AD, including the use of a same unit for all supported values

- MD ={32, 64}

  - FFS: If TDD TX/RX timing misalignment report is supported, whether different set of candidate MD values is needed

In addition, the inside/outside range for the 1-bit indicator dn is equal to [0, CP].

FFS: Further implicit/explicit restriction(s) on candidate value(s) depending on the CSI-RS configuration

Agreement

For the Rel-19 aperiodic standalone CJT calibration reporting, an ‘invalid’ quantization state/hypothesis is supported for frequency offset and phase offset CJT calibration reporting

- Note: already supported as ‘out-of-range’ for the (Dn,offset, dn) reporting

- FFS (RAN1#117): The need for a condition/event for ‘invalid’ to be specified as a UE procedure e.g. RSRP-based

Agreement

For the Rel-19 aperiodic standalone CJT calibration reporting, at least for a CJT calibration report consisting only one type, support one-part UCI on PUSCH

Agreement

For the Rel-19 Type-I multi-panel (MP) codebook refinement for 48, 64, and 128 CSI-RS ports, for RI=1-4, decide, by RAN1#117, whether to support Type-I multi-panel (MP) codebook refinement in Rel-19.

If supported, decide from the following alternatives:

- Scheme1. Based on Rel-15 Type-I MP design directly extended with Ng=K (2, 3, and 4), and new (N1, N2) values

- Scheme2. Based on Scheme4/6 as described in the RAN1#116 agreement

  - W1 structure: Reuse legacy Rel-15 Type-I SP SD basis selection with L=1 independently for each of the K NZP CSI-RS resources

  - W2 structure:

    - Legacy Rel-15 Type-I inter-polarization co-phasing rules independently in each resource,

    - Layer-common inter-resource M-PSK co-phasing, where M is further down-selected from {2,4}

      - FFS: Whether inter-resource co-phasing is wideband or per subband.

If so, decide, by RAN1#117, whether port mapping scheme similar to, e.g. Rel-18 Type-II CJT, needs to be specified.

Note: This topic is lower priority compared to the Rel-19 Type-I SP codebook refinement

Conclusion

For the Rel-19 Type-I SP codebook refinement for 48, 64, and 128 CSI-RS ports with RI=1-4, there is no consensus on supporting the following additional enhancements: Mode-C, inter-polarization amplitude for Scheme-B, larger values of L (>1, including 2, …, 10)

Agreement

For the Rel-19 Type-I and Type-II codebook refinement for 48, 64, and 128 CSI-RS ports, regarding NZP CSI-RS resource aggregation to attain 32 < P (or PCSI-RS) ≤ 128, support the following refinement on the K>1 CSI-RS resources associated with a same CSI-RS resource set:

- Allow per-resource configuration of evenPRBs or oddPRBs for 0.5 RE/RB/port density

- For AP-CSI-RS, allow resource-specific slot offset when the K NZP CSI-RS resources are located in two consecutive slots

  - FFS: details on how to configure/determine the slot offsets

Agreement

For the Rel-19 Type-I and Type-II codebook refinement for 48, 64, and 128 CSI-RS ports, regarding NZP CSI-RS resource aggregation to attain 32 < P (or PCSI-RS) ≤ 128, all the K>1 NZP CSI-RS resources also share the same QCL, PCoffset, and PCoffsetSS. In addition:

- ‘within 1 slot’ should be basic feature and ‘within 2 consecutive slots’ should be UE capability

Agreement

For the Rel-19 Type-II codebook refinement for 48, 64, and 128 CSI-RS ports based on the Rel-18 Type-II Doppler codebook, support the following aperiodic CMR configuration:

- A UE can be configured with KDOPP = {4, 8, 12} CSI-RS resource groups for the purpose of aperiodic CMR as needed by Type-II Doppler CSI

  - The time separation between the first resources from two consecutive groups (=m) can be configured from {1, 2}

  - FFS: The need for additional restriction in time domain

- Each CSI-RS resource group comprises K NZP CSI-RS resources (K defined in previous agreements) for aggregation associated with a same CSI-RS resource set assuming the agreed resource set rules for Rel-19 Type-I/II codebooks

- All the KDOPP CSI-RS resource groups are associated with a same CSI-RS resource set configuration

FFS: the determination of CSI-RS resource group that a CSI-RS resource is associated with

Agreement

For the Rel-19 CRI-based CSI refinement for up to 128 CSI-RS ports, for M>1, the M CRIs (each with log2KS bits) are separated indicated

- FFS: whether to support NW configuring/requesting the UE to report CRI/RI/PMI/CQI associated with MR (<M) of KS CSI-RS resources, including whether further reduction in the number of hypotheses is supported, i.e. reporting (M – MR) CRIs (each with log2KS-MR bits)

Agreement

For the Rel-19 Type-I SP and Type-II codebook refinements for 48, 64, and 128 CSI-RS ports via aggregating K>1 CSI-RS resources, regarding timeline, introduce two UE capabilities:

- Capability 1: Reuse legacy Z/Z’ values

- Capability 2: Scale the legacy timeline Z/Z’ by ceil(P/32) where P is the total number of ports across all the K aggregated CSI-RS resources

FFS: CPU occupation and active resource counting

Note:

- The legacy timeline Z/Z’ for Type-I corresponds to Z1/Z1’ in Table 5.4-2 of TS38.214 for Type-I WB SP-CSI with at most 4 CSI-RS ports in a single resource without CRI, and Z2/Z2’ for other Type-I cases

- The legacy timeline Z/Z’ for Type-II corresponds to Z2/Z2’

Agreement

For the Rel-19 aperiodic standalone CJT calibration reporting, regarding frequency offset reporting, FOi=i×AFOMFO-1,   i=0,1,…,MFO-2 and FOMFO-1 represents an ‘invalid’ state

Agreement

For the Rel-19 aperiodic standalone CJT calibration reporting, the dynamic range and resolution parameters for frequency offset reporting FOn, i.e. (AFO, MFO), are NW-configured via higher-layer (RRC) signalling from the following candidate values:

- AFO = {0.01ppm, 0.1ppm, 0.2ppm, f, f/2, f/4,f/8, 1/(4t), 1/(8t), 1/(16t), 1/(32t), 1/(512t)} where f and t denote the SCS and duration of one OFDM symbol, respectively

  - FFS: Further down-selection of the above candidate values for AFO, including the use of a same unit for all supported values

- MFO = {16,32}

FFS: Whether additional restriction(s) based on CSI-RS configuration is supported, including implicit configuration of quantization range

Agreement

For the Rel-19 aperiodic standalone CJT calibration reporting, the resolution parameters for n, i.e. M, are NW-configured via higher-layer (RRC) signalling from the candidate values {16, 32}, where Φn,σ∈0,2πMΦ-1,4πMΦ-1,…,MΦ-22πMΦ-1,'invalid'.

Agreement

For the Rel-19 aperiodic standalone CJT calibration reporting, regarding QCL assumptions, each of the NTRP DL-RSs can be configured with a TCI state via RRC signaling

- ‘The DL-RS’ refers to the CSI-RS configured for CJT calibration measurement

- FFS: Whether additional constraints are needed for the TCI state of the ‘DL-RS’

Agreement

For a UE indicated with two TCI states, regarding QCL assumptions for PDSCH, at least the following are supported:

- Scheme C: The PDSCH DMRS port(s) are QCLed with the DL-RS associated with the first TCI state with respect to QCL-TypeA and QCLed with the DL-RS in the second TCI state with respect to QCL-TypeA except for {Doppler shift}

- Scheme D: The PDSCH DMRS port(s) are QCLed with the DL-RS associated with the first TCI state with respect to QCL-TypeA and QCLed with the DL-RS in the second TCI state with respect to QCL-TypeA except for {average delay}

Per Rel-18, the support of two TCI states is a UE capability

Agreement

For the Rel-19 Type-I SP codebook refinement for 48, 64, and 128 CSI-RS ports, the UCI parameters are captured in the tables below for Scheme-A and Scheme-B:

- Note: The second column includes the location of the parameters when reported with two-part UCI

- FFS (RAN1#117): Select between Alt1 and Alt2 for Scheme-B

Scheme-A

| Parameter | UCI | Details/description | Status |
| --- | --- | --- | --- |
| RI | Part 1 | Same as Rel-15 Type-I SP: RI=v | Complete |
| Wideband CQI for the first TB | Part 1 | Same as Rel-15 Type-I SP | Complete |
| Subband differential CQI for the first TB (*) | Part 1 | Same as Rel-15 Type-I SP | Complete |
| Wideband CQI of the second TB | Part 2 Wideband | Same as Rel-15 Type-I SP Only present when v >4 | Complete |
| Subband CQI of the second TB (*) | Part 2 Subband | Same as Rel-15 Type-I SP Only present when v >4 | Complete |
| First SD basic vector selection indicator | Part 2 Wideband | v=1-4: Same as Rel-15 Type-I SP with the scheme following < 16-port design of Rel-15 Type-I SP codebookMode=1 v=5-8: FFS | v=1-4: Complete v=5-8: Pending |
| Second SD basis vector selection indicator | Part 2 Wideband | v=1-4: Same as Rel-15 Type-I SP with the scheme following < 16-port design of R15 Type-I codebookMode=1 v=5-8: FFS | v=1-4: Complete v=5-8: Pending |
| Inter-pol co-phase selection indicator | Part 2 Wideband or Subband (**) | v=1-4: Same as Rel-15 Type-I SP with the scheme following < 16-port design of R16 Type-I codebookMode=1 v=5-8: FFS | v=1-4: Complete v=5-8: Pending |

Scheme-B

| Parameter | UCI | Details/description | Status |
| --- | --- | --- | --- |
| RI | Part 1 | Same as Rel-15 Type-I SP: RI=v | Complete |
| Wideband CQI for the first TB | Part 1 | Same as Rel-15 Type-I SP | Complete |
| Subband differential CQI for the first TB (*) | Part 1 | Same as Rel-15 Type-I SP | Complete |
| Wideband CQI of the second TB | Part 2 Wideband | Same as Rel-15 Type-I SP Only present when v>4 | Complete |
| Subband CQI of the second TB (*) | Part 2 Subband | Same as Rel-15 Type-I SP Only present when v >4 | Complete |
| SD basis oversampling (rotation) factor q1, q2 | Part 2 Wideband | v=1-4: Values of q1, q2 follow Rel-16 eType-II, log2O1O2 bit indicator v=5-8: FFS | v=1-4: Complete v=5-8: Pending |
| SD basis vector selection indicator for each layer | Alt1: Part 1 Alt2: Part 2 Wideband | v=1-4: - Alt1: log2N1N2 bit indicator per layer l=1, …, RIMAX - Alt2: log2N1N2 bit indicator per layer l=1, …, v v=5-8: FFS | Pending |
| Inter-pol co-phase selection indicator for each layer | Part 2 Wideband or Subband (**) | v=1-4: - Alt1: QPSK with orthogonality constraints across v layers - Alt2: QPSK: 2-bit indicator per layer l=1,…,v v=5-8: FFS | Pending |

(*): Not included when CQI reporting granularity is set to ‘wideband’

(**): Wideband when PMI reporting is set to ‘wideband’, Subband when PMI reporting granularity is set to ‘subband’

Agreement

For the Rel-19 Type-I SP codebook refinement for 48, 64, and 128 CSI-RS ports with RI=5-8, decide, by RAN1#117, from the following schemes:

- Scheme1: adding new (N1, N2) values for the Rel-15 Type-I RI=5-8

- Scheme2:

  - W1 structure: Independent selection of different ceil(v/2) SD basis vectors for RI = v, where each SD basis vector is applied to two respective layers except that, if v is odd, the last SD basis vector is applied to the orphan layer. Each of the SD basis vectors is freely selected from a group of N1N2 orthogonal SD DFT basis vectors via combinatorial indication

    - FFS: mapping between v layers and ceil(v/2) SD basis vectors

    - FFS: support of 4 selected SD basis vectors for RI=5-6

  - W2 structure:

    - For inter-polarization co-phasing, M (e.g., M = 4) codepoints for the orphan layer and M/2 codepoints for two layers sharing a same SD basis vector;

    - A fixed  rotation of inter-polarization co-phasing between two layers sharing a same SD basis vector to achieve layer orthogonality.

- Scheme3: the 1st beam is freely selected and subsequent 2 beams (RI=5-6) or 3 beams (RI=7-8) are freely selected such that they are orthogonal in at least one dimension (horizontal or vertical). Layers are mapped to the selected SD basis vectors following legacy Rel-15 for RI=5-8. One co-phasing across all layers ∈{1,j} following legacy Rel-15 Type-I RI=5-8

- Scheme4: concatenate two independently calculated RI=1-4 PMIs for RI=5-8 to reduce UE complexity where each PMI is calculated from the agreed RI=1-4 codebook (Scheme-A or Scheme-B) and the CQI for each of the two CWs is derived assuming it is received by one antenna group of 4 antenna ports (FFS: Whether additional mapping between the two PMIs and the two UE antenna groups is needed)

- Other schemes are not precluded

Agreement

For the Rel-19 Type-II codebook refinement for 48, 64, and 128 CSI-RS ports, except for Parameter Combination 8 from Rel-17 FeType-II PS, all legacy Parameter Combinations from Rel-16 eType-II (regular), Rel-18 Type-II Doppler (regular), and Rel-17 FeType-II PS are supported

Agreement

For the Rel-19 Type-I SP codebook refinement for 48, 64, and 128 CSI-RS ports, regarding CBSR design:

- 1-bit hard restriction is supported (analogous to Rel-15 Type-I)

- FFS: 3-bit scaling factor for soft restriction with the scaling factor taken into account in CQI/PMI calculation

- Moving (N1, N2) configuration out from CBSR IE and the CBSR can be optional configured

  - Send LS to RAN2, and subject to RAN2 consent

- N1⋅O1⋅N2⋅O2X1⋅X2-bit CBSR where each bit in the CBSR is associated with a set of X1X2 SD basis vectors, where the set includes X1 adjacent SD basis vectors along the N1 direction and/or X2 adjacent SD bases along the N2 direction

  - FFS: Value(s) of X1 and X2 and detailed design/spec impact

FFS: Whether/how to enable shared CBSR in RRC configuration for Type-I/II codebooks with a same (N1,N2).

Agreement

For the Rel-19 CRI-based CSI refinement for up to 128 CSI-RS ports, on the configured KS>1 NZP CSI-RS resources, Pcoffset and PcoffsetSS are CSI-RS-resource-specific (i.e. configured independently across resources)

Agreement

For the Rel-19 CRI-based CSI refinement for up to 128 CSI-RS ports, for M>1, SD basis selection is independently signalled per CRI (per CSI-RS resource)

Agreement

For the Rel-19 aperiodic standalone CJT calibration reporting, in addition to reporting one type of CJT calibration report in one report, at least support reporting {(Dn,offset, dn), n=0, 1, …, NTRP – 1, n≠nref1} and {FOn , n=0, 1, …, NTRP – 1, n≠nref2} in one report

- nref1 and nref2 are independently selected and indicated by the UE,

- One-part UCI is used

Agreement

For a UE indicated with two TCI states, regarding QCL assumptions for PDSCH, support the following QCL assumption for PDSCH:

- Scheme E: The PDSCH DMRS port(s) are QCLed with the DL-RS associated with the first TCI state with respect to QCL-TypeA and QCLed with the DL-RS in the second TCI state with respect to QCL-TypeA except for {Doppler shift, average delay}

Agreement

For the Rel-19 CRI-based CSI refinement for up to 128 CSI-RS ports,

- When M>1, the M PMIs are independently calculated and indicated

- with the Rel-16 eType-II codebook and KS={1,2,3,4}, support M=2 with a maximum of 16 ports per resource, R=1 only, and a maximum UCI payload of 1706 bits.

  - The value of M={1, 2} is NW-configured via higher-layer (RRC) signalling

  - The maximum value of M is subject to UE capability

- on the configured KS>1 NZP CSI-RS resources, reuse the legacy IMR rule for the Rel-15 CRI-based reporting for NZP CSI-RS resource for interference measurement, i.e. only 1 NZP CSI-RS resource for interference measurement can be configured

Conclusion

For the Rel-19 Type-I and Type-II codebook refinement for 48, 64, and 128 CSI-RS ports, for a given massive TXRU/antenna array, there is no consensus on the need for specification support for cell-specific precoder(s) for many-to-one mapping (virtualization) from multiple TXRUs to each CSI-RS port to facilitate full usage of all TXRUs for a pre-Rel-19 UE (i.e. cell-specific beamformed CSI-RS for pre-Rel-19 UEs)

Conclusion

For the Rel-19 Type-I and Type-II codebook refinement for 48, 64, and 128 CSI-RS ports, regarding the mapping from CSI-RS resource index/port index per resource and port index to CSI/PMI calculation, there is no consensus on supporting mapping method#3 (for K=4, 2x2 aggregation).

3Tx uplink

Agreement

To support codebook-based UL transmission by a 3TX UE, the agreed rank1 precoders in RAN1#116 can also be used when transform precoding is enabled (DFT-s-OFDM ).

Agreement

To indicate precoding information for codebook-based UL transmission by a 3TX UE,

- Reuse legacy TPMI indication framework where TPMI and TRI are jointly indicated

- TPMI field is 2 or 3bits for 3-antenna-port transmission

  - For maxRank equals to 1, TPMI field is 2 bits for DFT-s-OFDM and CP-OFDM

  - For maxRank equals to 2 or 3, TPMI field is 3 bits for CP-OFDM

Agreement

For SRS configuration supporting codebook-based UL transmission by a 3TX UE, support Alt1,

- Alt1: Support configuration of X 4-port SRS resources in a resource set where one the ports is muted

  - FFS muting mechanism

where X can be up to 2, subject to UE capability.

Agreement

- For codebook-based UL transmission by a 3TX UE, when 2 PTRS ports are configured by maxNrofPorts in PTRS-UplinkConfig, PTRS-DMRS association indication is as follows:

| Value of MSB | DMRS port | Value of LSB | DMRS port |
| --- | --- | --- | --- |
| 0 | 1st DMRS port which shares PTRS port 0 | 0 | 1st DMRS port which shares PTRS port 1 |
| 1 | 2nd DMRS port which shares PTRS port 0 | 1 | 2nd DMRS port which shares PTRS port 1 |

- Note: PUSCH antenna port 1000 and 1002 in indicated TPMI(s) share PT_RS port 0, and PUSCH antenna port 1001 is associated with PT_RS port 1

- Number of bits used for the indication

  - 1 bit

Agreement

For a 3TX UE, to support 3-port SRS transmission with reusing a 4-port SRS resource, support the following for muting one of the ports of the configured 4-port SRS resource,

- Option 3: Always a same port is muted, e.g., the 4th port

Agreement

For a 3TX UE, to support 3-port SRS transmission with reusing a 4-port SRS resource, UE splits a linear SRS power equally across the 3 unmuted antenna ports of the 4-port SRS resource.

Agreement

For 3-port codebook-based PUSCH transmission for a 3TX UE, scale factor s should be the ratio of the number of antenna ports with a non-zero PUSCH transmission power to 3 (except for full-power Mode 0).

- FFS: Whether specification needs to be updated to reflect the above

Agreement

For codebook-based UL transmission by a 3TX UE, when 1 PTRS port is configured by maxNrofPorts in PTRS-UplinkConfig, PTRS-DMRS association indication is as follows:

- Alt2: 2-bit indication

PTRS-DMRS association when 1 PT-RS port is configured

| Value | DMRS port |
| --- | --- |
| 0 | 1st scheduled DMRS port |
| 1 | 2nd scheduled DMRS port |
| 2 | 3rd scheduled DMRS port |
| 3 | 4th scheduled DMRS port Reserved |

Agreement

For a 3TX UE, support Rel-17 M-TRP PUSCH repetition where,

- Two SRS resource sets, each with up to 2 of 4-port SRS resources are configured,

Note: The configured 4 port SRS resources are used to enable 3-port SRS transmission

Asymmetric DL-sTRP/UL-mTRP

Agreement

For a UE configured with two SRS CLPC adjustment states, support Alt2 for indicating one of the SRS CLPC adjustment states to SRS:

- Alt2: When the parameter srs-PowerControlAdjustmentStates is set to 'separateClosedLoop', closedLoopIndex-r17 for SRS in the TCI state indicates one of the SRS CLPC adjustment states

  - The candidate value of i0 and i1 in closedLoopIndex-r17 for SRS refers to the first and the second CLPC adjustment state separate from PUSCH, respectively

Ericsson raised concerns on Alt2 due to potential issues with beam management.

Agreement

For indicating TPC command for those two SRS CLPC adjustment states through DCI when the UE is configured with two SRS CLPC adjustment states, support Option3:

- Option 3: enhance the legacy DCI format 2_3 of higher layer parameter srs-TPC-PDCCH-Group = typeA and typeB;

Agreement

For enhancing DCI format 2_3 for indicating TPC command for two SRS CLPC adjustment states, support Alt2:

- Alt2: Introduce one 1-bit closed-loop-indicator field for each TPC command in DCI format 2_3

  - This 1-bit closed-loop-indicator indicates the first SRS CLPC adjustment state or the second SRS CLPC adjustment state.

Note: this 1-bit indicator is present for the CC where two SRS CLPC adjustment states are configured.

Agreement

For FR1, a joint TCI state can be associated with a PL offset.

- When a joint TCI state associated with a PL offset is applied for the PUSCH/PUCCH/SRS transmission, the UE shall calculate the Tx power of the PUSCH/PUCCH/SRS based on the DL PL RS and PL offset associated with this joint TCI state.

  - Reuse the legacy uplink power control formulation by replacing legacy PL with a PL which is derived from the DL PL RS and the PL offset.

  - FFS: The UE can update UL PL in a way that new UL PL = current UL PL + an update delta indicated by the NW.

Agreement

Support applying PL offset on PDCCH-order PRACH towards a UL TRP in FR1.

- Note: The DL reference timing determination for PDCCH-order PRACH transmission to an UL TRP is still based on the DL RS defined in current RAN4 specification

- Above is subject to a separate UE capability signaling

Agreement

Consider and down-select one from the following alts for indicating a PL offset for PDCCH-order PRACH transmission at least for FR1.

- Alt1: RRC configures multiple PL offset values in PRACH-Config and PDCCH-order DCI indicates one of them through one DCI field.

- Alt2: PDCCH order DCI indicates one PL offset value

- Alt3: The PL offset associated with one of the indicated joint/UL TCI state for UL TRP in unified TCI framework is applied on the PDCCH-order PRACH transmission

- Alt4: The PDCCH order DCI indicates one TCI state associated with a PL offset and the associated PL offset is applied on the PRACH transmission.

- Alt5: RRC configures one PL offset value for PRACH and the PDCCH order DCI indicates whether this PL offset value is applied on PRACH transmission or not.

Note: Other alternatives are not precluded

Agreement

For the association between PL offset and joint/UL TCI state, consider and down-select one from the following Alts:

- Alt1a: One PL offset value is configured in a joint or UL TCI state by RRC only

- Alt1b: One PL offset value is configured in a joint or UL TCI state by RRC. A MAC CE can update the PL offset value(s) for joint or UL TCI state(s).

- Alt2a: A list of PL offset configurations is configured by RRC in BWP/CC and each PL offset configuration contains one PL offset value. One new RRC parameter is introduced in a joint or UL TCI state to indicate one of the configured PL offset configurations.

- Alt2b: A list of PL offset configurations is configured by RRC in BWP/CC and each PL offset configuration contains one PL offset value. One new RRC parameter is introduced in a joint or UL TCI state to indicate one of the configured PL offset configurations. A MAC CE can update the association between a joint or UL TCI state and PL offset configuration

- Alt3: A list of PL offset configurations is configured by RRC in BWP/CC and each PL offset configuration contains one PL offset value.  A MAC CE can activate/indicate one PL offset configuration for each activated joint or UL TCI state. In each joint or UL TCI state, the initial PL offset value is 0dB.

- Alt4: A list of PL offset values is provided in a joint or UL TCI state by RRC. Each PL offset value is applied to a corresponding measured PL range.

Other alternatives are not precluded.

Agreement

For the asymmetric DL sTRP/UL mTRP deployment scenarios, study whether and how to indicate TPC command for SRS CLPC adjustment states through DCI format 1_1 and/or 0_1 when the UE is configured two SRS CLPC adjustment states.

In RAN1#117, the following agreements were made.

UE-initiated/event-driven beam management

Agreement

On UE-initiated/event-driven beam reporting, regarding UL signaling content(s) of L1-RSRP report depending on Event-2, in a report instance, at least Option-3 is supported

- Option-3: N ≥ 1 beam(s) are reported in the report instance,

  - At least one of N reported beam(s) should satisfy the condition of Event-2

  - N is configured by gNB

    - FFS: candidate value of ‘N’.

  - FFS: RRC can enable or disable whether current beam is always reported in addition to the N beams

- FFS: Option-1/1a/1b/2.

- Above applies at least for the single CC case

Working Assumption

On beam report transmission procedure for UE-initiated/event-driven beam reporting

- For mode-A, at least support one-bit indication in the first PUCCH channel to request a resource for a second UL channel to carry beam report.

  - In such case, a periodic PUCCH resource (with PUCCH format 0/1) is configured by dedicated RRC signaling.

- For mode-B, at least support one-bit indication in the first PUCCH channel to notify a second UL channel to carry beam report.

  - In such case, a periodic PUCCH resource (with PUCCH format 0/1) is configured by dedicated RRC signaling.

- FFS: Whether/how to support multi-bit indication in the first PUCCH for mode-A and mode-B, e.g., when multi-event(s) are approved.

- FFS: details on the dedicated RRC signaling

- Above applies at least for the single CC case.

Agreement

Regarding RS measurement for the current beam for Event 2, for Option-2a, support the both schemes as follows.

- Scheme-1: RS for current beam is the QCL RS in the indicated TCI state

  - FFS: Whether/How to handle the case if only one TRS is configured in the indicated TCI state.

- Scheme-2: the RS for current beam is the SSB which is QCLed with the QCL RS in the indicated TCI state.

- Enabling one of either Scheme-1 or Scheme-2 is selected by NW.

  - FFS: The above selection is via an explicit RRC parameter or an implicit manner, e.g., if the RS(s) for new beam are CSI-RS, Scheme-1 is enabled; otherwise, Scheme-2 is enabled.

  - (Working Assumption) Enabling of either Scheme-1 or Scheme-2 should ensure the same RS type for RS measurement for current beam and new beam.

- The above QCL RS is the RS w.r.t. QCL-TypeD, if there are two QCL RSs in the indicated TCI state.

Agreement

Regarding RS measurement for the new beam for Event 2, at least Option-3a is supported

- Option-3a (explicit manner): The RS(s) for new beam(s) are explicitly configured

- FFS: Option-3b/3c

  - Option-3b: The RS(s) for new beam(s) are implicitly derived from QCL RS(s) of activated TCI state(s).

  - Option-3c: The RS(s) for new beam(s) are implicitly derived from QCL RS(s) of TCI state(s) in a configured subset of the legacy RRC-configured TCI state list

Agreement

On UE-initiated/event-driven beam reporting, regarding L1-RSRP report format Option-3 depending on Event-2, for a report instance where N ≥ 1 beam(s) are reported, the following is supported.

- RRC can enable or disable whether current beam is always reported

    - When enabled by RRC, the current beam + N beams from the measurement RSs for new beam(s) are reported

      - Note: The reported current beam is NOT counted in the N reported beams.

    - When disabled by RRC, N beams are reported.

Agreement

On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding Mode-A, the DCI format in Step-2 comprises UL-grant DCI format, and the second channel in Step-3 is at least PUSCH.

- The UL-grant DCI format at least comprises DCI format 0_1/0_2.

  - FFS: DCI format 0_3

- FFS: How to trigger the UEI beam report by the UL-grant DCI format

- FFS: the DCI format in Step-2 comprises DL-grant DCI format, and the second channel in Step-3 is PUCCH.

    - 1-bit field in the DL-grant DCI format is introduced to indicate the transmission of the UEI beam report

      - The PUCCH resource for HARQ-ACK transmission can be reused to carry both the HARQ-ACK and UEI beam report.

    - The DL-grant DCI format at least comprises DCI format 1_1/1_2.

      - FFS: DCI format 1_3

Agreement

Regarding the triggering event determination for Event 2:

- If within a time window (which is configurable), the number of Event-2 instance(s) for at least one same new beam is greater than or equal to a configurable number M, UE initiated beam report occurs.

  - Note: Event-2 instance for a new beam is determined if the L1-RSRP of the new beam becomes a threshold value better than the current beam

Above feature is subject to UE capability.

- Basic feature: Once the L1-RSRP of the new beam becomes a threshold value better than the current beam, UE initiated beam report occurs

FFS: Whether the above is captured in RAN1 or RAN2 specification.

Agreement

On UE-initiated/event-driven beam reporting, regarding trigger events, the following Event-1 and 7a/7b, are provided for down-selection or combination in RAN1#118 (possible outcome is that no new event is supported)

- Event-1: Quality of the current beam is worse than a certain threshold.

- Event-7a: Quality of at least one new beam, such as L1-RSRP, becomes a threshold value better than the RS derived from the activated TCI state with the worst quality.

- Event-7b: Quality of at least one new beam, such as L1-RSRP, becomes a threshold value better than the RS derived from the activated TCI state with the best quality.

Agreement

Regarding explicit RS configuration for new beam measurement for Event 2, down-select the following options in the RAN1#118:

- Option-1: The RS(s) for new beam(s) are explicitly configured in one RS resource set associated with an CSI reporting configuration;

  - FFS: The RS in the RS resource set can be updated by MAC-CE.

- Option-2: A list of RS(s) for new beam measurement can be configured by RRC, and a subset can be activated for new beam measurement by MAC-CE.

  - FFS: If a list size is small, MAC-CE activation is not needed

- Option-3: A list of RS resource (s) for new beam measurement can be configured by RRC, and a subset of RS resource(s) in the list can be provided for new beam measurement by indicated TCI state.

- Others are not precluded.

- FFS: Each RS for new beam measurement should be associated with a configured joint/DL TCI state which can be used as the indicated TCI state

Agreement

Regarding RS measurement for the current beam for Event 2, for Option-2a, besides for scheme-1 and scheme-2, further study the following for handling the case that only one TRS is configured in the indicated TCI state.

- Option-1: Introducing additional scheme: the RS for current beam can be a CSI-RS for beam management derived from the QCL RS in the indicated TCI state;

- Option-2: Further support TRS as measurement RS of current beam for determining L1-RSRP

- Option-3: Introducing additional scheme: The RS for current beam is explicitly configured by RRC or MAC-CE (Option-2C in RAN1 116b agreement).

- Option-4: No further enhancement (i.e., in such case, Scheme-2 is used)

- Others are not precluded.

CSI

Agreement

For the Rel-19 aperiodic standalone CJT calibration reporting, regarding the dynamic range for delay offset reporting Dn,offset, i.e. AD, at least support the following values: {0.5CP, CP}

- Decide, by RAN1#117, whether any of the following candidate values are supported: {0.75CP, 1.5CP, 14∆f, 112∆f,124∆f}

Agreement

For the Rel-19 aperiodic standalone CJT calibration reporting, regarding the dynamic range for frequency offset reporting FOn, i.e. AFO, at least support the following values: {0.1ppm, 0.2ppm}

- Decide, by RAN1#117, whether any of the following candidate values are supported: {0.025ppm, 0.05ppm, 1/(8t), 1/(16t), 1/(32t)}

Agreement

For the Rel-19 aperiodic standalone CJT calibration reporting, when ReportQuantity is ‘cjtc-P’ (DL/UL phase offset), =1 only (agreed in RAN1#116bis) implies that the measured/reported phase offsets {n,, n=0, 1, …, NTRP – 1, n≠nref} are associated with the entire configured CSI reporting band (i.e. ‘wideband’)

Agreement

For the Rel-19 aperiodic standalone CJT calibration reporting, when ReportQuantity is ‘cjtc-P’ (DL/UL phase offset), decide, by RAN1#117, whether to also support >1 (sub-band reporting) as follows:

- A sub-band size is selected from {8,16} PRBs

  - FFS: Whether the sub-band size is NW-configured via higher-layer (RRC) signalling or selected (hence reported) by the UE

- Denoting the number of sub-bands within the configured CSI reporting band as NSB-P, and the sub-bands are indexed as {0, 1, …, NSB-P –1}, decide, by RAN1#117, from the following reporting options:

  - Opt1: {(n,, n), n=0, 1, …, NTRP – 1, n≠nref}, where n,is the phase offset corresponding to sub-band 0 and the phase offset for sub-band  can be calculated as n, + n

    - Γn∈0,2πMΓ, ….,2π(MΦ-1)MΓ, where MΓ∈ {[32], [64], [128], [256]}

  - Opt2: = NSB-P, i.e. {(n,, n,, NSB-P), n=0, 1, …, NTRP – 1, n≠nref}

    - The alphabet for n, follows the previously agreed alphabet for =1, including the ‘invalid’ state

    - FFS: Whether restriction on the maximum payload size is needed

  - Note: For all the above reporting options, the UE performs measurement over the entire configured CSI reporting band

Agreement

For the Rel-19 aperiodic standalone CJT calibration reporting, when ReportQuantity is ‘cjtc-P’ (DL/UL phase offset),

- For a given phase offset reporting configuration, the UE can be configured (via higher-layer/RRC signaling) with Q associated SRS resource(s) for antenna switching

  - FFS: The supported value(s) of Q

- The UE antenna port(s) for receiving the CSI-RS configured for phase offset measurement are same as the UE antenna port(s) for transmitting the selected/configured port(s) from the associated SRS resource(s)

  - For discussion purposes only (not necessarily for specification), the UE antenna port is referred to as the ‘reference UE antenna port’

Agreement

For the Rel-19 aperiodic standalone CJT calibration reporting, regarding active resource counting and OCPU, when ReportQuantity is ‘cjtc-Dd’ (Doffset+d) or cjtc-F’ (frequency offset), fully reuse those from Rel-18 TDCP reporting

- OCPU =X.NTRP where X≥1 is defined based on UE capabilities and determined by the UE for each CJT calibration report type

Agreement

For the Rel-19 Type-II codebook refinement for 48, 64, and 128 CSI-RS ports based on the Rel-18 Type-II Doppler codebook,

- Ordering the KDOPPK CSI-RS resources ascendingly by the CSI-RS resource ID and kDOPP ={0,1,…, KDOPP  –1}, CSI-RS resources { kDOPPK, kDOPPK +1, …, (kDOPP+1)K –1} are associated with the kDOPP-th CSI-RS resource group

- FFS: If the CSI-RS resources in a resource group span two consecutive slots, m is 2.

- FFS: If the CSI-RS resources in a resource group are located in one slot, m can be configured from {1, 2}

Agreement

For the Rel-19 Type-I and Type-II codebook refinement for 48, 64, and 128 CSI-RS ports, regarding aggregation of K NZP CSI-RS resources to attain 32 < P (or PCSI-RS) ≤ 128, support only the following combinations of K and P (or PCSI-RS):

- For P (or PCSI-RS) = 48, K = 2 (each resource 24 ports) and 3 (each resource 16 ports)

- For P (or PCSI-RS) = 64, K = 2 (each resource 32 ports) and 4 (each resource 16 ports)

- For P (or PCSI-RS) = 128, K = 4 (each resource 32 ports)

Agreement

For the Rel-19 Type-I and Type-II codebook refinement for 48, 64, and 128 CSI-RS ports, regarding port mapping,

- Following legacy principle, “sequential ordering/indexing within” a group of Q indices {i0, i1, …, iQ-1} is a linearly increasing sequence such that iq < iq+1 (where q=0, 1, …, Q-2 is the port index within a CSI-RS resource, and iq or iq+1 ∈ {0, 1,…, KQ-1}) is the port index for the codebook, across the K>1 CSI-RS resources).

- After resource aggregation, P (=48, 64, or 128) ports are numbered in accordance to Table 7.4.1.5.3-1 from TS 38.211

Agreement

For the Rel-19 Type-I codebook refinement for 48, 64, and 128 CSI-RS ports, for RI=v=1, support the following:

- for each group of X1X2 SD basis vectors, a 3-bit scaling factor can be NW-configured via higher-layer (RRC) signalling, where the scaling factors are defined as scalings on the power control offset configured for the associated CSI-RS resources

  - The values of X1 and X2 for this feature are separately configured from those for CBSR

  - Separate configuration (RRC signalling) from CBSR

  - The candidate values of X1 and X2 are the same as those agreed for CBSR

- The codepoints of each of the group-specific 3-bit scaling factors are mapped to values of 1, 1/2, 1/3 ,1/4,1/6,1/8,1/12,1/16

- Note: This feature is a separate UE capability

FFS: Whether this can be extended to RI=v>1 as well as Type-II codebook refinement

Agreement

For the Rel-19 Type-I SP and Type-II codebook refinement for 48, 64, and 128 CSI-RS ports, on CBSR, the value of X1 and X2) are separately NW-configured via higher-layer (RRC) signalling from {1, 2, 4, [8, 16]}

- FFS: Dependence on each supported (X1, X2) value on the configured (N1, N2) value

Agreement

For the Rel-19 Type-I SP and Type-II codebook refinements (except based on Rel-18 Type-II Doppler) for 48, 64, and 128 CSI-RS ports, regarding CPU occupation

- For Capability 1 timeline: OCPU = ceil(P/32)

- For Capability 2 timeline: OCPU = 1

Agreement

For the Rel-19 Type-I SP codebook refinement for 48, 64, and 128 CSI-RS ports, regarding UCI omission, fully reuse the legacy rules for Rel-15 Type-I SP codebook

Agreement

For the Rel-19 Type-I SP codebook refinement for 48, 64, and 128 CSI-RS ports, regarding UCI parameters for Scheme-B RI=v=1-4:

- SD basis vector selection indicator for each layer is in Part 2 (wideband) and log2N1N2 bits per layer l=1, …, v

- Inter-pol co-phase selection indicator for each layer is in Part 2 (wideband or subband) and 2 bits (representing {+1, +j, -1, -j}) per layer l=1,…,v

Agreement

For the Rel-19 Type-I single-panel (SP) codebook refinement for 48, 64, and 128 CSI-RS ports, for Scheme-A RI=3-4 only, the legacy mapping of i1,3 to (k1,k2) for (N1=3,N2=2) from Table 5.2.2.2.1-4 of TS 38.214 is used for all of the newly supported (N1,N2) values.

- FFS: whether the i1,3 table (Table 5.2.2.2.1-4 of TS 38.214) needs to be further extended.

Agreement

On the NZP CSI-RS resource aggregation of K=2, 3 or 4 legacy NZP CSI-RS resources to attain a total of 48, 64, and 128 ports (for Rel-19 Type-I/II codebook refinement), support to configure a CSI-RS resource set with the K CSI-RS resources as the associated NZP CSI-RS for each of the SRS resource set(s) with higher layer parameter usage in SRS-ResourceSet set to 'nonCodebook',

- The previously agreed restrictions on the K resources for Rel-19 Type-I/II codebook refinement apply

- Reuse the legacy approach for triggering of the NZP-CSI-RS resources and the legacy timeline for the NZP-CSI-RS resources and SRS

Agreement

For the Rel-19 Type-I SP codebook refinement for 48, 64, and 128 CSI-RS ports with RI=5-8, support the following schemes:

- The same O1=O2 value(s) as RI=1-4 are supported

- Scheme-A (based on Scheme3 described in RAN1#116bis):

  - W1 structure:

    - The 1st SD basis vector is freely selected and subsequent 2 (RI=5-6) or 3 SD basis vectors (RI=7-8) are freely selected such that they are orthogonal in at least one dimension (horizontal or vertical).

    - The v layers are mapped to the selected SD basis vectors following legacy Rel-15 Type-I for RI=5-8.

  - W2 structure:

    - Following legacy Rel-15 Type-I RI=5-8

- Scheme-B (based on Scheme2 described in RAN1#116bis):

  - W1 structure:

    - Independent selection of different ceil(v/2) SD basis vectors for RI = v, where each SD basis vector is applied to two respective layers following legacy Rel-15 Type-I for RI=5-8, except that, if v is odd, the last SD basis vector is applied to the orphan layer.

      - FFS: mapping between the orphan layer and its selected SD basis vector and, if needed, UE reporting of the selection

      - FFS: support of 4 selected SD basis vectors for RI=5-6

    - The SD basis vectors are freely selected from a group of N1N2 orthogonal SD DFT basis vectors via combinatorial indication, as well as a layer-common (q1,q2)

  - W2 structure:

    - For the orphan layer, the inter-polarization co-phasing is selected from {1, j, -1, -j}

    - For two layers sharing a same SD basis vector, the inter-polarization co-phasing between two layers is selected from the following pairs {(1, -1), (j, -j)} to achieve inter-layer orthogonality.

- Only Scheme-A (RI=1-4+RI=5-8) and Scheme-B (RI=1-4+RI=5-8) are supported in Rel-19

Proposal 1.A.2:

For a UE configured with a total of PSRS=6 or 8 ports across ≥1 SRS resources for antenna switching intended for xT6R or xT8R, respectively, support the following fixed SRS port grouping where (with the PSRS ports indexed in an ascending order according to SRS resource ID and port number within each SRS resource):

- SRS port group 0, corresponding to CW0, comprises the first PSRS/2 out of PSRS ports; and

- SRS port group 1, corresponding to CW1, comprises the second PSRS/2 out of PSRS ports

No other spec enhancement is introduced, e.g. new CW-to-layer mapping, DL resource allocation, DCI format

Note: The above grouping assumption is to align NW and UE on the association between SRS ports and reported CQIs for the two CWs when reportQuantity = ‘cri-RI-CQI’.

Note: different SRS ports are associated with different UE antenna ports.

Note: if one single CW is scheduled, both SRS port groups can correspond to the same CW

Note: This feature is a separate UE capability and, for UEs supporting this capability, configured via RRC (FFS details on the extend of RRC configuration)

Companies are encouraged to evaluate for further discussion in RAN1#118.

Conclusion

For the Rel-19 Type-I single-panel (SP) codebook refinement for 48, 64, and 128 CSI-RS ports, there is no consensus on additionally supporting O1=O2=2

Agreement

For the Rel-19 Type-I SP codebook refinement for 48, 64, and 128 CSI-RS ports, on CBSR, the following (X1, X2) values are supported: (1,1), (2,1), (2,2), (4,1), (4,2), (4,4),

- FFS: (1,2), (1,4), (2,4)

Agreement

For the 3-bit scaling scheme for Type-I codebook, the following (X1, X2) values are supported: (2,1), (2,2), (4,1), (4,2), (4,4),

- FFS: (1,2), (1,4), (2,4), (8,1)

Conclusion

For the Rel-19 Type-I codebook refinement for 48, 64, and 128 CSI-RS ports, on the agreed 3-bit group-based scaling factor for RI=v=1, there is no consensus on supporting this feature for codebooks other than for Rel-19 Type-I SP codebook refinement

Agreement

For the Rel-19 CRI-based CSI refinement for up to 128 CSI-RS ports, for M>1, support the following:

- Resource-specific RI, i.e. RI is independently calculated and indicated for each of the selected M NZP CSI-RS resources

  - FFS: If resource-common RI indication is also supported

- 4-bit wideband CQIs are independently calculated and reported for each of the M selected NZP CSI-RS resources

- 2-bit differential SB CQIs are independently calculated and reported for each of the M selected NZP CSI-RS resource

Agreement

For the Rel-19 CRI-based CSI refinement for up to 128 CSI-RS ports, for M=2, when Rel-16 eType-II codebook is configured, FD basis selection and indication are resource-specific (per resource)

Agreement

For the Rel-19 CRI-based CSI refinement for up to 128 CSI-RS ports, for M=2, when Rel-16 eType-II codebook is configured, RRC configuration of Parameter Combination is resource-common

Agreement

For the Rel-19 CRI-based CSI refinement for up to 128 CSI-RS ports:

- Active resource counting = KS (following legacy)

Agreement

For the Rel-19 aperiodic standalone CJT calibration reporting, the UCI parameters are captured in the tables below:

- FFS: Mapping order

- FFS: Whether it is possible not to report dn

When ReportQuantity is ‘cjtc-Dd’ (Doffset+d)

| Parameter | Details/description |
| --- | --- |
| nref | Reference TRS resource set index, based on the ordering from RRC configuration: logNTRP bits |
| {Dn,offset, n=0, 1, …, NTRP – 1, n≠nref } | Delay offset for CSI-RS resource n: NTRP-1logMD bits |
| {dn, n=0, 1, …, N TRP – 1, n≠nref } | 1-bit inside/outside indicator for CSI-RS resource n: NTRP-1 bits |

When ReportQuantity is ‘cjtc-F’ (frequency offset)

| Parameter | Details/description |
| --- | --- |
| nref | Reference TRS resource set index, based on the ordering from RRC configuration: logNTRP bits |
| {FOn , n=0, 1, …, NTRP –1, n≠nref} | Frequency offset for CSI-RS resource n: NTRP-1logMF bits |

When ReportQuantity is ‘cjtc-Dd-F’ (joint Doffset+d and FO)

| Parameter | Details/description |
| --- | --- |
| nref1 | Reference TRS resource set index for Doffset+d, based on the ordering from RRC configuration: logNTRP bits |
| nref2 | Reference TRS resource set index for FO, based on the ordering from RRC configuration: logNTRP bits |
| {Dn,offset, n=0, 1, …, NTRP – 1 n≠nref1} | Delay offset for CSI-RS resource n: NTRP-1logMD bits |
| {dn, n=0, 1, …, NTRP – 1, n≠nref1 } | 1-bit inside/outside indicator for CSI-RS resource n: NTRP-1 bits |
| {FOn , n=0, 1, …, NTRP –1, n≠nref2} | Frequency offset for CSI-RS resource n: NTRP-1logMF bits |

Conclusion

For the Rel-19 aperiodic standalone CJT calibration reporting, regarding the ‘out of range’ or ‘invalid’ quantization state/hypothesis, there is no consensus on specifying a condition/event for such state.

Agreement

For the Rel-19 aperiodic standalone CJT calibration reporting, when ReportQuantity is ‘cjtc-P’ (DL/UL phase offset),

- Regarding the number of configured associated SRS resource(s) (=Q) for antenna switching xTyR, support at least Q=1 where:

  - the configured associated SRS resource is selected from all the y/x SRS resources and all the configured resource set(s)

  - FFS (by RAN1#118): whether Q>1 is also supported

  - FFS (by RAN1#118): the supported value(s) of x

- Regarding how to determine the SRS port corresponding to the ‘reference UE antenna port’, support PSRS =1 SRS port selected from all the ports from the configured Q associated SRS resource(s)

  - FFS (by RAN1#118): Whether PSRS >1 is also supported

Agreement

For the Rel-19 aperiodic standalone CJT calibration reporting, regarding timeline, fully reuse those from Rel-18 TDCP reporting

Agreement

For the Rel-19 aperiodic standalone CJT calibration reporting, regarding the applicable type(s) of the configured NTRP NZP CSI-RS resources/resource sets when ReportQuantity is ‘cjtc-Dd’ (Doffset+d) or ‘cjtc-F’ (frequency offset), all the resources across the NTRP TRS resource sets are configured with the same bandwidth

Conclusion

For the Rel-19 aperiodic standalone CJT calibration reporting, regarding the applicable type(s) of the configured NTRP NZP CSI-RS resources/resource sets when ReportQuantity is ‘cjtc-Dd’ (Doffset+d) or ‘cjtc-F’ (frequency offset), there is no consensus on:

- Supporting CSI-RS for CSI

Agreement

For the Rel-19 aperiodic standalone CJT calibration reporting, regarding the applicable type(s) of the configured NTRP NZP CSI-RS resources/resource sets when ReportQuantity is ‘cjtc-P’ (DL/UL phase offset),

- all the ‘CSI-RS for CSI’ resources within each resource set follow the legacy pre-Rel-19 rules of CSI-RS resources associated with a same resource set

- all the resources across the NTRP CSI-RS resources/resource sets are configured with the same bandwidth

Conclusion

For the Rel-19 aperiodic standalone CJT calibration reporting, regarding the applicable type(s) of the configured NTRP NZP CSI-RS resources/resource sets when ReportQuantity is ‘cjtc-P’ (DL/UL phase offset), there is no consensus on:

- Supporting multi-port CSI-RS for CSI

Conclusion

For the Rel-19 Type-II codebook refinement for 48, 64, and 128 CSI-RS ports based on the Rel-18 Type-II Doppler codebook, there is no consensus on specifying further restriction on m values based on the slot location(s) of the CSI-RS resources in a resource group

Agreement

For the Rel-19 Type-II codebook refinement for 48, 64, and 128 CSI-RS ports, on CBSR,

- N1⋅N2X1⋅X2-bit group-based CBSR

- the following (X1, X2) values are supported:

| P | (N1, N2) | (X1, X2) |
| --- | --- | --- |
| 48 | (8,3) | (1,1), (2,1), (4,1) |
|  | (6,4) | (1,1), (2,1), (2,2), |
| 64 | (16,2) | (1,1), (2,1), (2,2), (4,1), (4,2) |
|  | (8,4) | (1,1), (2,1), (2,2), (4,1), (4,2) |
| 128 | (16,4) | (1,1), (2,1), (2,2), (4,1), (4,2) |
|  | (8,8) | (1,1), (2,1), (2,2), (4,1), (4,2) |

- FFS: (1,2), (1,4), (2,4), when applicable

Conclusion

For the Rel-19 CRI-based CSI refinement for up to 128 CSI-RS ports, there is no consensus on supporting the following report quantities when Rel-15 Type-I SP codebook: ‘cri-RI-i1-CQI’, ‘cri-RI-i1’, ‘cri-RI-CQI’

R1-2405486	Moderator Summary#4 on Rel-19 CSI enhancements: Round 4	Moderator (Samsung)

Agreement

For the Rel-19 aperiodic standalone CJT calibration reporting, regarding the applicable type(s) of the configured NTRP NZP CSI-RS resources when ReportQuantity is ‘cjtc-P’ (DL/UL phase offset), one ‘CSI-RS for CSI’ resource set with NTRP resources is supported

- FFS: 1 CSI-RS resource set with J⋅NTRP single-port CSI-RS resources where J>1 is the number of CSI-RS resources per TRP

- FFS: 1 or NTRP TRS resource sets can also be used when ReportQuantity is ‘cjtc-P’ (DL/UL phase offset)

Conclusion

For the Rel-19 aperiodic standalone CJT calibration reporting, regarding the applicable type(s) of the configured NTRP NZP CSI-RS resources/resource sets when ReportQuantity is ‘cjtc-P’ (DL/UL phase offset), there is no consensus on supporting any additional time separation between RSs beyond what’s already permissible by the use of TRS resource sets

Conclusion

For the Rel-19 aperiodic standalone CJT calibration reporting, regarding the applicable type(s) of the configured NTRP NZP CSI-RS resources/resource sets when ReportQuantity is ‘cjtc-Dd’ (Doffset+d) or ‘cjtc-F’ (frequency offset), there is no consensus on supporting the following:

- Any additional time separation between RSs beyond what’s already permissible by the use of TRS resource sets

- Any restriction on the number of resources within each resource set

- CSI-RS type(s) other than TRS can be used for joint reporting of Doffset+d and FO

Agreement

For the Rel-19 aperiodic standalone CJT calibration reporting, when ReportQuantity is ‘cjtc-Dd-F’ (joint Doffset+d and FO)

- Fully reuse timeline and active resource counting from Rel-18 TDCP reporting

- OCPU = 2X.NTRP where X≥1 is defined based on UE capabilities and determined by the UE for each CJT calibration report type

Agreement

For the Rel-19 CRI-based CSI refinement for up to 128 CSI-RS ports, for A-CSI only, the NW can configure MR (<M) of KS CSI-RS resources to be selected as part of reporting the M “quadruplets”:

- (M–MR) CRIs, each with log2KS-MR bits are reported, along with the M sets of CQI/PMI/RI/(if applicable) LI

- The value of MR is NW-configured via higher-layer (RRC) signaling

- The MR selected resources are NW-configured via higher-layer (RRC) signaling

- FFS: value of MR

- This is an optional UE capability

Agreement

For the Rel-19 Type-I multi-panel (MP) codebook refinement for 48, 64, and 128 CSI-RS ports, for RI=1-4, support the following (compromise between Scheme1 and Scheme2 described in RAN1#116bis):

- W1 structure: Independent SD basis selection across all the Ng=K NZP CSI-RS resources, reusing legacy Rel-15 Type-I Pcsi-rs<16 SP SD basis selection rules with L=1 for RI=1-4

  - Ng = K = {2, 4} denotes the number of NZP CSI-RS resources associated with the Ng panels

- W2 structure:

  - Legacy Rel-15 Type-I inter-polarization co-phasing rules independently in each resource,

  - Layer-common wideband inter-resource QPSK co-phasing independently reported for resource n = 2,…,K with respect to the first resource.

Rel-19 Type-I MP does not support RI=5-8.

Reuse Rel-15 Type-I MP legacy designs for UCI omission, and CBSR.

For CSI calculation, reuse Rel-18 Type II CJT CSI-RS port ordering for UE assumption on the transmitted PDSCH symbols across antenna ports.

Agreement

For the Rel-19 CRI-based CSI refinement for up to 128 CSI-RS ports, regarding CBSR and RI restriction, down-select (by RAN1#118) one from the following alternatives:

- Alt1. KS per-resource CBSRs and KS per-resource RI restrictions

- Alt2. (legacy CRI-based) Resource-common CBSR and resource-common RI restriction

- Alt3. KS per-resource CBSRs and resource-common RI restriction

Agreement

For the Rel-19 aperiodic standalone CJT calibration reporting, when ReportQuantity is ‘cjtc-P’ (DL/UL phase offset), if  >1 (sub-band reporting) is supported, select one from option 1 and option 2, by RAN1#118, from the following:

- A sub-band size is selected from {8,16} PRBs

  - The sub-band size is NW-configured via higher-layer (RRC) signalling

- Denoting the number of sub-bands within the configured CSI reporting band as NSB-P, and the sub-bands are indexed as {0, 1, …, NSB-P –1}, decide, by RAN1#118, from the following reporting options:

  - Opt1: {(n,, n), n=0, 1, …, NTRP – 1, n≠nref}, where n,is the phase offset corresponding to sub-band 0 and the phase offset for sub-band  can be calculated as n, + n

    - Γn∈0,2πMΓ, ….,2π(MΓ-1)MΓ, where MΓ={64, 128}

  - Opt2: = NSB-P, i.e. {(n,, n,, n,NSB-P), n=0, 1, …, NTRP – 1, n≠nref}

    - The alphabet for n, follows the previously agreed alphabet for =1, including the ‘invalid’ state

    - The maximum NSB-P is [4]

  - FFS: For all the above reporting options, the UE performs measurement over the entire configured CSI reporting band

- FFS: Further restriction on CSI-RS (e.g. RE density)

Note: Companies to report whether their evaluation is based on precoded or non-precoded CSI-RS

3Tx uplink

Agreement

Update the agreement made in RAN1 #116bis as the following.

Agreement For a 3TX UE, to support 3-port SRS transmission with reusing a 4-port SRS resource, support the following for muting one of the ports of the configured 4-port SRS resource, Option 3: Always a same port is muted, e.g., i.e., the 4th port

Agreement

For codebook-based M-TRP PUSCH repetition by a 3TX UE, scheduled by DCI format 0_1/0_2,

- Reuse Rel-17 M-TRP PUSCH repetition design, where the second precoding information field only indicates TPMI index, and applies same rank as indicated by the first precoding information field.

Agreement

For codebook-based M-TRP PUSCH repetition by a 3TX UE, scheduled by DCI format 0_1/0_2,

- Introduce new tables as Table I, II, III for the second precoding information field, for maxRank=1 or 2 or 3, respectively.

  - Table I: Second precoding information for 3 antenna ports if maxRank=1

| Bit field | codebookSubset=NonCoherent |
| --- | --- |
| 0 | 1 layer: TPMI=0 |
| 1 | 1 layer: TPMI=1 |
| 2 | 1 layer: TPMI=2 |
| 3 | Reserved |

  - Table II: Second precoding information for 3 antenna ports if maxRank=2

| Bit field | codebookSubset=NonCoherent |
| --- | --- |
| 0 | 1 layer: TPMI=0 |
| 1 | 1 layer: TPMI=1 |
| 2 | 1 layer: TPMI=2 |
| 3 | 1 layer: Reserved |
| 0 | 2 layer: TPMI=0 |
| 1 | 2 layer: TPMI=1 |
| 2 | 2 layer: TPMI=2 |
| 3 | 2 layer: Reserved |

  - Table III: Second precoding information for 3 antenna ports if maxRank=3

| Bit field | codebookSubset=NonCoherent |
| --- | --- |
| 0 | 1 layer: TPMI=0 |
| 1 | 1 layer: TPMI=1 |
| 2 | 1 layer: TPMI=2 |
| 3 | 1 layer: Reserved |
| 0 | 2 layer: TPMI=0 |
| 1 | 2 layer: TPMI=1 |
| 2 | 2 layer: TPMI=2 |
| 3 | 2 layer: Reserved |
| 0 | 3 layer: TPMI=0 |
| 1~3 | 3 layer: reserved |

Agreement

For codebook-based M-TRP PUSCH repetition by a 3TX UE, for indication of PTRS-DMRS association,

- When 1 PTRS port is configured by maxNrofPorts in PTRS-UplinkConfig, reuse Rel-17 multi-TRP TDM repetition,

- When 2 PTRS ports are configured by maxNrofPorts in PTRS-UplinkConfig, and maxRank = 2 or 3,

  - A second PTRS-DMRS association field (1 bit) is used to indicate the association between PTRS ports and DMRS ports for the 2nd SRS resource set (i.e.,2nd TRP).

Agreement

For codebook-based UL transmission by a 3TX UE, subject to its capability,

- A 3TX UE may report a maximum number of 3 layers

- A 3TX UE may report a maximum number of SRS ports of up to 3

Note: SRS resource definition is not changed nor the number of SRS ports in the SRS resource.

Asymmetric DL-sTRP/UL-mTRP

Proposal 3.1:

To fulfil the asymmetric DL sTRP/UL mTRP deployment scenarios, support two TAs for single DCI based multi-TRP/panel and single TRP.

- Reuse Rel-18 specification of two TA for multi-DCI based multi-TRP/panel and remove the restriction that coresetPoolIndex needs to be configured.

Companies are encouraged to consider above for further discussion in RAN1#118

Agreement

For indicating a PL offset for PDCCH-order PRACH transmission at least for FR1, further study and down-select one from the Alt1 and Alt3 by RAN1#118 meeting:

- Alt1: RRC configures multiple PL offset values [in PRACH-Config] and PDCCH-order DCI indicates one of them through one DCI field

- Alt3: The PL offset associated with one of the indicated joint/UL TCI state for UL TRP in unified TCI framework is applied on the PDCCH-order PRACH transmission

- FFS: the details of DCI field design.

Agreement

In Rel-19, the value range of starting bit of block in DCI format 2-3 is extended from 1~31 to 1~X.

- FFS the value of X>31

FFS: Condition under which the above is applicable

Agreement

Introduce a new RRC parameter per BWP/CC to indicate that two separate SRS CLPC adjustment states are configured for SRS in a BWP/CC

Agreement

For the association between PL offset and joint/UL TCI state, support the following

- Alt1b: One PL offset value is configured in a joint or UL TCI state by RRC, where different PL offset values can be configured to different joint or UL TCI states. A MAC CE can update the PL offset value(s) for joint or UL TCI state(s).

Conclusion

There is no consensus on the following proposal:

Proposal 1.6: Support to update a UL PL for a joint/UL TCI state as follows:

- When this joint/UL TCI state is activated and it is not in the current active TCI state list, a UL PL is calculated as: UL PL = PL estimated from DL PL RS – the value of PL offset.

- When this joint/UL TCI state is activated and it is in the current active TCI state list, the UE updates the UL PL as: new UL PL = current UL PL + the updated delta indicated by the NW.

Agreement

For the asymmetric DL sTRP/UL mTRP scenarios, study and decide the value range and candidate values of PL offset value

Agreement

For the asymmetric DL sTRP/UL mTRP scenarios, study whether/how to consider PL offset in PHR calculation, including Type 1 PHR based on actual PUSCH transmission, Type 1 PHR based on reference PUSCH, Type 3 PHR based on actual SRS and Type 3 PHR based on reference SRS

Conclusion

For the asymmetric DL sTRP/UL mTRP deployment scenario, reuse the rel-17 unified TCI/ICBM and rel-18 unified TCI framework:

- When rel-17 unified TCI/ICBM is configured:

  - For FR1: one joint TCI state or {one DL TCI state + one UL TCI state} can be applied.

  - For FR2: {one DL TCI state + one UL TCI state} can be applied.

- When rel-18 unified TCI is configured:

  - For FR1: up to two joint TCI states or {one DL TCI state + up to two UL TCI state} can be applied.

  - For FR2: {one DL TCI state + up to two UL TCI states} can be applied.

#### 2.1.2	Remaining Open issues

UE-initiated/event-driven beam management:

- Remaining details on enhancement to facilitate UE-initiated/event-driven beam management for reducing overhead and/or latency, assuming the unified TCI while leveraging (as much as possible) legacy CSI measurement and reporting configuration frameworks, targeting FR2 and sTRP with intra- and inter-cell beam management

  - Remaining details on UL signaling content(s) (and procedure(s) as required) for UE-initiated/event-driven beam reporting facilitating fast beam switching

  - Remaining details on UL signaling medium/container considering the UE-initiated/event-driven nature of the UL transmission, designed primarily for the purpose of beam reporting

CSI:

- Remaining details on CSI support for up to 128 CSI-RS ports, targeting FR1

  - Remaining details on Type-I codebook refinement supporting up to a total of 128 CSI-RS ports across all resources, assuming legacy CSI-RS resources (with up to 32 CSI-RS ports per resource), based on extension of legacy codebooks

  - Remaining details on Type-II codebook refinement supporting up to a total of 128 CSI-RS ports across all resources, assuming legacy CSI-RS resources (with up to 32 CSI-RS ports per resource), based on extension of legacy codebooks, without modifying any codebook parameter other than introducing additional values for the number of ports codebook parameter(s)

  - Remaining details on extension of CRI(s)-based CSI reporting (CQI/PMI/RI calculated per CRI for ≥1 CRIs) for hybrid beamforming supporting up to a total of 128 CSI-RS ports across all resources, with up to 32 CSI-RS ports per resource, without new codebook design

- Remaining details on UE reporting enhancement for CJT deployments under non-ideal synchronization and backhaul, targeting FR1, both FDD and TDD

- Remaining details on inter-TRP time misalignment and frequency/phase offset measurement and reporting, assuming legacy CSI-RS design, with stand-alone aperiodic reporting on PUSCH

3Tx uplink:

- Remaining details on non-coherent UL codebook to facilitate 3-antenna-port codebook-based transmissions, without enhancement on UL full power transmission and without enhancement on SRS resource

Note: UL full power transmission mode 1 and 2 are not supported.

Asymmetric DL-sTRP/UL-mTRP:

- Remaining details on enhancement for asymmetric DL sTRP/UL mTRP deployment scenarios, assuming intra-band intra-DU non-co-located mTRP scenarios, without changing existing cell definition or defining a new cell (e.g. UL-only cell), assuming the Rel-17/18 unified TCI framework and fully reusing the legacy QCL/UL spatial relation rules, targeting FR1 and FR2

  - Remaining details on two closed-loop PC adjustment states for SRS, both separate from PUSCH; and pathloss offset configurations for pathloss calculation to UL TRP(s), when the pathloss RS is from DL sTRP.

## 2.2	RAN2

#### 2.2.1	Agreements

- Not yet started

#### 2.2.2	Remaining Open issues

- Not yet started

.

## 2.3	RAN3

n/a

#### 2.3.1	Agreements

#### 2.3.2	Remaining Open issues

## 2.4	RAN4

#### 2.4.1	Agreements

- Not yet started

#### 2.4.2	Remaining Open issues

- Not yet started

## 2.5	RAN5

n/a

#### 2.5.1	Agreements

#### 2.5.2	Remaining Open issues

#### 2.5.3	Remaining Open issues with cross-WG dependencies

## 2.6	RAN6

n/a

#### 2.6.1	Agreements

#### 2.6.2	Remaining Open issues

## 3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

n/a

## 3.1	SAx/CTs

#### 3.1.1	Agreements with cross-TSG impacts

#### 3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

## 4.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template