3GPP TSG RAN WG1 #117 	 R1-2405601

# Fukuoka City, Fukuoka, Japan, May 20th – 24th, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.211 | CR | 0127 | rev |  | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for hop counting in SRS for positioning with tx hopping |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Moderator (Ericsson), Huawei, HiSilicon, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos_enh2-Core |  |  |  |  |  |  | Date: |  | 2024-05-20 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The hop counter nSRSTxHopping description in 38.211 for SRS with tx hopping is not clear and points to the wrong parameter name |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarifies that the the hop counter nSRSTxHopping order should follow the one given by the parameter slotOffsetForRemainingHops in slotOffsetForRemainingHopsList |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - Incomplete description of the value range of nSRSTxHopping, so how to derive the SRS frequency hopping pattern is unclear. Lack of clarity in terms of which parameter is configured for the overlapped resources for SRS frequency hopping. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.4.1.4.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | x | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

##### 6.4.1.4.3	Mapping to physical resources

Throughout this clause, when the higher layer parameter SRShoppingNrofHops is provided for SRS-PosResource, the sounding reference signal sequence definitions applies to a given hop.

When SRS is transmitted on a given SRS resource, the sequence rpi(n,l') for each OFDM symbol l' and for each of the antenna ports of the SRS resource shall be multiplied with the amplitude scaling factor  in order to conform to the transmit power specified in [5, 38.213] and mapped in sequence starting with  to resource elements  in a slot for each of the antenna ports  according to

aKTCk'+k0pi,  l'+l0(pi)=1NapβSRSrpi(k',l')if k'=0, 1, …, Msc,bSRS-1 and l'=0,1,…,NsymbSRS-10otherwise

The length of the sounding reference signal sequence is given by

Msc,bSRS=mSRS,bNscRBKTCPF

where mSRS,b is given by a selected row of Table 6.4.1.4.3-1 with  where  is given by the field b-SRS contained in the higher-layer parameter freqHopping if configured, otherwise BSRS=0. The row of the table is selected according to the index  given by the field c-SRS contained in the higher-layer parameter freqHopping. The quantity PF ∈2, 4 is given by the higher-layer parameter FreqScalingFactor if configured, otherwise PF=1. When FreqScalingFactor is configured, the UE expects the length of the SRS sequence to be a multiple of 6.

The frequency-domain starting position k0(pi) is defined by

k0(pi)=k0(pi)+noffsetFH+noffsetRPFS+noffset2FH

where

k0pi=nshiftNscRB+kTCpi+koffsetl'+fcohnf,ns,fμ,l'' mod KTC

and
kTCpi=kTC+3KTC4 mod KTCif NapSRS=8, pi∈1003, 1007, and nSRScs,max=6kTC+KTC2 mod KTCif NapSRS=8, pi∈1002, 1006, and nSRScs,max=6kTC+KTC4 mod KTCif NapSRS=8, pi∈1001, 1005, and nSRScs,max=6kTC+KTC2 mod KTCif NapSRS=8, pi∈1001, 1003, 1005, 1007, and nSRScs,max=12kTC+KTC2 mod KTCif NapSRS=8, pi∈1001, 1003, 1005, 1007, nSRScs,max=8, and nSRScs≥nSRScs,max2kTC+KTC2 mod KTCif NapSRS=4, pi∈1001, 1003, and nSRScs,max=6 kTC+KTC2 mod KTC if NapSRS=4, pi∈1001, 1003,  nSRScs,max∈8, 12,and nSRScs≥nSRScs,max2kTCotherwise

noffsetFH=b=0BSRSmSRS,bNscRBnb

noffsetRPFS=NscRBmSRS,BSRSkF+khopmod PFPF

noffset2FH=ninithop+nSRSTxHopping mod Nhop-ninithopmSRS,0-moverlaphopNscRB

and

-	kF∈0,1,…,PF-1 is given by the higher-layer parameter StartRBIndex if configured, otherwise kF=0;

-	khop is given by Table 6.4.1.4.3-3 with

khop=nSRSb'=bhopBSRSNb' mod PF
Nbhop=1

if the higher-layer parameter EnableStartRBHopping is configured, otherwise khop=0.

-	moverlaphop∈0,1,2,4 is given by the higher-layer parameter YYY.

-	nSRSTxHopping=0,1,…,NhopsSRS-1 is the hop transmission counter in the time domain, where  nSRSTxHopping=1,2,…,NhopsSRS-1 corresponds to the order of the higher-layer parameter SlotOffsetForRemainingHops in slotOffsetForRemainingHopsList.

<--------------------------unchanged text ommited------------------------------>