3GPP TSG-RAN WG1 Meeting #116bis	R1-2403620

# Changsha, Hunan Province, China, April 15th – 19th, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.859 | CR | 0001 | Rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections on the support for Expanded and Improved NR Positioning |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Intel Corporation (Moderator), CATT, CICTCI |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_NR_pos_enh2 |  |  |  |  |  | Date: |  |  | 2024-04-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | According to the agreements in RAN1#109-e, the contents of the row of deployment layout for absolute positioning and the row of deployment layout for relative positioning/ranging in Table A.1-3 should be swapped between urban grid for V2X and highway for V2X. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Table A.1-3 of TR 38.859: Rows describing deployment layout for absolute and relative positioning/ranging are swapped between the urban grid and highway scenarios for V2X use-case. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The descriptions of the evaluation assumptions for evaluations of urban grid for V2X and highway for V2X are not correct. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

< Unchanged text omitted >

Table A.1-3: Evaluation assumptions for evaluations of sidelink positioning for V2X use-cases

| Assumptions | Value |  |
| --- | --- | --- |
| Scenarios | V2X use-cases with highway and urban grid scenarios defined in TR 37.885 [8]. - Road configuration for urban grid and highway provided in Annex A in TR 37.885 [8] is reused. |  |
|  | Urban grid for V2X | Highway for V2X |
| Carrier frequency | Uu: 4 GHz SL: 6 GHz | Uu: 2 GHz or 4GHz SL: 6 GHz |
| Deployment layout for absolute positioning | BS and UE-type RSU deployment follows the description in clause A.1.3 in TR 36.885 [9]. - Companies can provide results for additional BS/ UE-type RSU deployments, e.g., additional UE-type RSUs are added to UE-type RSU deployment in TR 36.885 [9]. | - Alt 1 as optional: BS and UE-type RSU deployment follows TR 36.885, where wrap around method of 19*3 hexagonal cells with 500m ISD in Figure A.1.3-3 of clause A.1.3 in TR 36.885 [9] is used. - Alt 2 as baseline: BSs are disabled, UE-type RSUs are uniformly located with 200m spacing on both sides of highway symmetrically. - Optional: staggered/unsymmetrical UE-type RSU distribution like NOTE: Alt 1 is assumed for evaluation of joint Uu/PC5 positioning, Alt 2 is assumed for evaluation of PC5-only positioning. |
| Deployment layout for relative positioning/ranging | - BSs are disabled (baseline), or enabled (optional) - Companies to report their assumptions - UE type RSU may be disabled or enabled (companies should report their assumption) - If enabled, UE type RSU deployment follows the description clause A.1.3 in TR 36.885 [9]. - If enabled, companies can provide additional RSU deployment, e.g., additional RSUs are added to RSU deployment in TR 36.885 [9]. | - BSs are disabled - UE type RSU may be disabled (as baseline) or enabled (as optional) - If enabled, UE-type RSUs are uniformly located with 200m spacing on both sides of highway symmetrically. - Optional: staggered/unsymmetrical UE-type RSU distribution like |
| BS Tx power | Macro BS: 49dBm |  |
| UE Tx power | Vehicle UE or UE type RSU: 23dBm |  |
| BS receiver noise figure | 5dB |  |
| UE receiver noise figure | 9 dB |  |
| UE dropping | UE dropping option A defined in clause 6.1.2 of TR 37.885 [8]: - UE dropping option A is used for the highway scenario: - Vehicle type distribution: 100% vehicle type 2. - Clustered dropping is not used. - Vehicle speed is 140 km/h in all the lanes as baseline and 70 km/h in all the lanes optionally. - UE dropping option A is used for the urban grid scenario: - Vehicle type distribution: 100% vehicle type 2. - Clustered dropping is not used. - Vehicle speed is 60 km/h in all the lanes. In the intersection, a UE goes straight, turns left, turns right with the probability of 0.5, 0.25, 0.25, respectively. |  |
| UE antenna model | Description in clause 6.1.4 in TR 37.885 [8] is reused: - Vehicle UE option 1 is the baseline (Vehicle UE antenna is modelled in Table 6.1.4-8 and 6.1.4-9 in TR 37.885 [8]) - Vehicle UE option 2 (two panels) can be optionally selected by companies. |  |
| Channel model | Description in clause 6.2 in TR 37.885 is reused. |  |

< Unchanged text omitted >