3GPP TSG RAN WG1 #117			R1-2405660

Fukuoka, Japan, May 20th – 24th, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.214 | CR | 0571 | rev | 1 | Current version: | 16.16.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on mDCI based mTRP out-of-order operation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung, Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_eMIMO-Core |  |  |  |  |  | Date: |  |  | 2024-05-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | To enable out-of-order operation for PDSCH or PUSCH, the only condition in current specification is when a UE is configured with two different coresetPoolIndexes and two scheduled PDSCHs or PUSCHs are associated to different ControlResourceSets having different values of coresetPoolIndex. A basic UE capability, multiDCI-MultiTRP-r16, is defined for a UE to support multi-DCI based multi-TRP scheme with configuration of two different coresetPoolIndexes. To support out-of-order operation for PDSCH and PUSCH, separate UE capabilities, outOfOrderOperationDL-r16 and outOfOrderOperationUL-r16, are defined which are not covered by a basic UE capability. Then, there could be a case when a UE reports a basic capability, multiDCI-MultiTRP-r16, but does not report additional optional features for out-of-order operation, outOfOrderOperationDL-r16 and/or outOfOrderOperationUL-r16. In this case, the UE can support multi-DCI based multi-TRP scheme, but cannot support out-of-order operation. Hence, we would like to add a condition, when a UE reports such optional capability supporting out-of-order operation, in the specification to enable out-of-order operation. |  |  |  |  |  |  |  |  |
| Summary of change: |  | To enable out-of-order operation, add a condition like when a UE reports such optional capability supporting out-of-order operation. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Based on current specification, a UE reporting a basic capability, multiDCI-MultiTRP-r16, but not reporting additional optional features for out-of-order operation, outOfOrderOperationDL-r16 and/or outOfOrderOperationUL-r16, shall support out-of-order operation, which is not aligned with UE capability signaling structure. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1, 6.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This is the first version of the CR. |  |  |  |  |  |  |  |  |

## 5.1	UE procedure for receiving the physical downlink shared channel

< Unchanged parts are omitted >

Except for the case when a UE is configured by higher layer parameter PDCCH-Config that contains two different values of coresetPoolIndex in ControlResourceSet and PDCCHs that schedule two PDSCHs are associated to different ControlResourceSets having different values of coresetPoolIndex and the UE reports its capability of outOfOrderOperationDL-r16, in a given scheduled cell, the UE is not expected to receive a first PDSCH and a second PDSCH, starting later than the first PDSCH, with its corresponding HARQ-ACK assigned to be transmitted on a resource ending before the start of a different resource for the HARQ-ACK assigned to be transmitted for the first PDSCH, where the two resources are in different slots for the associated HARQ-ACK transmissions, each slot is composed of Nsymslot symbols [4] or a number of symbols indicated by subslotLengthForPUCCH if provided, and the HARQ-ACK for the two PDSCHs are associated with the HARQ-ACK codebook of the same priority. Except for the case when a UE is configured by higher layer parameter PDCCH-Config that contains two different values of coresetPoolIndex in ControlResourceSet and PDCCHs that schedule two PDSCHs are associated to different ControlResourceSets having different values of coresetPoolIndex and the UE reports its capability of outOfOrderOperationDL-r16, in a given scheduled cell, the UE is not expected to receive a first PDSCH, and a second PDSCH, starting later than the first PDSCH, with its corresponding HARQ-ACK assigned to be transmitted on a resource ending before the start of a different resource for the HARQ-ACK assigned to be transmitted for the first PDSCH if the HARQ-ACK for the two PDSCHs are associated with HARQ-ACK codebooks of different priorities.

< Unchanged parts are omitted >

When PDCCHs that schedule two PDSCHs are associated to different ControlResourceSets having different values of coresetPoolIndex and the UE reports its capability of outOfOrderOperationDL-r16, the following operations are allowed:

-	For any two HARQ process IDs in a given scheduled cell, if the UE is scheduled to start receiving a first PDSCH starting in symbol j by a PDCCH associated with a value of coresetPoolIndex ending in symbol i, the UE can be scheduled to receive a PDSCH starting earlier than the end of the first PDSCH with a PDCCH associated with a different value of coresetPoolIndex that ends later than symbol i.

-	In a given scheduled cell, the UE can receive a first PDSCH in slot i, with the corresponding HARQ-ACK assigned to be transmitted in slot j, and a second PDSCH associated with a value of coresetPoolIndex different from that of the first PDSCH starting later than the first PDSCH with its corresponding HARQ-ACK assigned to be transmitted in a slot before slot j.

< Unchanged parts are omitted >

## 6.1	UE procedure for transmitting the physical uplink shared channel

< Unchanged parts are omitted >

Except for the case when a UE is configured by higher layer parameter PDCCH-Config that contains two different values of coresetPoolIndex in ControlResourceSet for the active BWP of a serving cell and PDCCHs that schedule two non-overlapping in time domain PUSCHs are associated to different ControlResourceSets having different values of coresetPoolIndex and the UE reports its capability of outOfOrderOperationUL-r16, for any two HARQ process IDs in a given scheduled cell, if the UE is scheduled to start a first PUSCH transmission starting in symbol j by a PDCCH ending in symbol i, the UE is not expected to be scheduled to transmit a PUSCH starting earlier than the end of the first PUSCH by a PDCCH that ends later than symbol i.

< Unchanged parts are omitted >

If a UE is configured by higher layer parameter PDCCH-Config that contains two different values of coresetPoolIndex in ControlResourceSet for the active BWP of a serving cell and PDCCHs that schedule two non-overlapping in time domain PUSCHs are associated to different ControlResourceSets having different values of coresetPoolIndex and the UE reports its capability of outOfOrderOperationUL-r16, for any two HARQ process IDs in a given scheduled cell, if the UE is scheduled to start a first PUSCH transmission starting in symbol j by a PDCCH associated with a value of coresetPoolIndex ending in symbol i, the UE can be scheduled to transmit a PUSCH starting earlier than the end of the first PUSCH by a PDCCH associated with a different value of coresetPoolIndex that ends later than symbol i.