3GPP TSG-RAN WG1 Meeting #117	R1-2405624

Fukuoka City, Fukuoka, Japan, May 20th-24th, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.214 | CR | 0574 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. Proposed change affects:; UICC apps; ME; X; Radio Access Network; X; Core Network. Title:; Corrections for Referencing UE Capabilities for 8TX UE. Source to WG:; Moderator (InterDigital), Ericsson, Samsung. Source to TSG:; R1. Work item code:; NR_MIMO_evo_DL_UL-Core; Date:; 2024-05-19. Category:; F; Release:; Rel-18. Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900.; Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20). Reason for change:; - According to 38.306 v18.0, UE capabilities for 8 antenna port codebooks are ‘codebook1-8TxPUSCH-r18’ (support for one of two codebooks with values n4-1 and n2-2), ‘codebook2-8TxPUSCH-r18’, ‘codebook3-8TxPUSCH-r18’, and ‘codebook4-8TxPUSCH-r18’, and there is no capability named ‘UL_8TX_Ng’.. Summary of change:; - Removing the reference to ‘UL_8TX_Ng’, and revise the text that UE should expect to be configured according its reported capabilities, which addresses both the number of port groups and the supported codebooks.. Consequences if not approved:; - Incorrect referencing to a non-existing capability.. Clauses affected:; 6.1.1.1. Y; N. Other specs; N; Other core specifications; TS/TR ... CR .... affected:; N; Test specifications; TS/TR ... CR .... (show related CRs); N; O&M Specifications; TS/TR ... CR .... Other comments:. This CR's revision history:. #### 6.1.1.1 Codebook based UL transmission -------------------------------------------Unchanged parts are omitted------------------------------------------- A UE does not expect to be configured with a value of CodebookTypeUL that does not correspond to one of the values reported in its capability for codebook based transmission with eight antenna ports. -------------------------------------------Unchanged parts are omitted-------------------------------------------. |