3GPP TSG-RAN WG1 Meeting #117	R1-2405625

Fukuoka City, Fukuoka, Japan, May 20th-24th, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 38.214 | CR | 0575 |  | rev |  | - |  |  | Current version: |  |  | 18.2.0 |  |  |  |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Proposed change affects: |  |  |  | UICC apps |  |  |  | ME | X |  | Radio Access Network | X |  | Core Network |  |  |

| Title: | Corrections for Configured Grant Operation for 8TX UE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Moderator (InterDigital), Ericsson, ASUSTek, Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MIMO_evo_DL_UL-Core |  |  |  |  |  | Date: |  |  | 2024-05-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - Configured grant operation for PUSCH supports at most 4 layers according to the conclusion from RAN1#113 below. However, configured grant operation can be controlled with both maxRank and maxRank-n8, which configure the UE for 1-4 and 5-8 layers, respectively, thereby allowing up to rank 8. This also conflicts with the statement in this section ‘A configured grant PUSCH can be transmitted with at most 4 layers’. The specification is therefore ambiguous at present, and may conflict with the agreed behavior from RAN1#113. Conclusion In Rel-18, there is no consensus to support CG transmission with dual CW PUSCH by an 8TX UE. - According to the latest agreed version of 38.331 for MIMO (in R2-2404017) a new version of the parameter maxMIMO-Layers for PUSCH is used for 5-8 layers, ‘maxMIMO-Layers-v1810’, while ‘maxMIMO-Layers’ is used for 1-4 layers. Also, the parameter ‘maxRank-n8’ is now named ‘maxRank-v1810’, but still is used for 5-8 layers, while ‘maxRank’ is used for 1-4 layers. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Delete any reference to maxRank-n8 and maxMIMO-Layers-n8 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - Inconsistency between specifications on parameter name and usage. - Incorrect descirption of configured grant operation for PUSCH |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1, 6.1.1.1, 6.1.4.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | N | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | N | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | N | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## 6.1 UE procedure for transmitting the physical uplink shared channel -------------------------------------------Unchanged parts are omitted------------------------------------------- For the PUSCH transmission corresponding to a Type 1 configured grant or a Type 2 configured grant activated by DCI format 0_0 or 0_1, the parameters applied for the transmission are provided by configuredGrantConfig except for dataScramblingIdentityPUSCH, txConfig, codebookSubset, maxRank, scaling of UCI-OnPUSCH, which are provided by pusch-Config. A configured grant PUSCH can be transmitted with at most 4 layers. For the PUSCH transmission corresponding to a Type 2 configured grant activated by DCI format 0_2, the parameters applied for the transmission are provided by configuredGrantConfig except for dataScramblingIdentityPUSCH, txConfig, codebookSubsetDCI-0-2, maxRankDCI-0-2, scaling of UCI-OnPUSCH, resourceAllocationType1GranularityDCI-0-2 provided by pusch-Config. If the UE is provided with transformPrecoder in configuredGrantConfig, the UE applies the higher layer parameter tp-pi2BPSK, if provided in pusch-Config, according to the procedure described in clause 6.1.4 for the PUSCH transmission corresponding to a configured grant. -------------------------------------------Unchanged parts are omitted------------------------------------------- #### 6.1.1.1 Codebook based UL transmission -------------------------------------------Unchanged parts are omitted------------------------------------------- When higher layer parameter ul-FullPowerTransmission is set to 'fullpowerMode2' and the higher layer parameter CodebookTypeUL is set to 'Codebook2' or 'Codebook3', and the SRS-resourceSet with usage set to 'codebook' includes one SRS resource with 8 ports, and at least one SRS resource with 2 ports or 4 ports, subject to UE capability, - when CodebookTypeUL is set to 'Codebook2', the codebookSubset associated with the 2-port SRS resource is 'nonCoherent'. - when CodebookTypeUL is set to 'Codebook2', the codebookSubset associated with the 4-port SRS resource can be configured as 'partialAndNonCoherent' or 'nonCoherent', subject to UE capability. - when CodebookTypeUL is set to 'Codebook3', the codebookSubset associated with 4 ports SRS resources is 'nonCoherent'. The maximum transmission rank may be configured by the higher layer parameter maxRank in pusch-Config for PUSCH scheduled with DCI format 0_1 or 0_3 and maxRankDCI-0-2 for PUSCH scheduled with DCI format 0_2. -------------------------------------------Unchanged parts are omitted------------------------------------------- #### 6.1.4.2 Transport block size determination For eight antenna ports PUSCH transmission, when the number of PUSCH transmission layers is greater than 4, two codewords are transmitted. If the higher layer parameter maxRank or maxMIMO-Layers in PUSCH-config is greater than 4, then one of the two transport blocks is disabled by DCI format 0_1 if IMCS = 26 and if rvid = 1 for the corresponding transport block. If both transport blocks are enabled, transport block 1 and 2 are mapped to codeword 0 and 1 respectively. If only one transport block is enabled, then the enabled transport block is always mapped to the first codeword. -------------------------------------------Unchanged parts are omitted-------------------------------------------