3GPP TSG-RAN WG1 Meeting #117	R1-2405746

# Fukuoka City, Fukuoka, Japan, May 20th-24th, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.214 | CR | 0591 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on PT-RS Coherence Conditions for 8 Tx |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Moderator (NTT DOCOMO), Ericsson, Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MIMO_evo_DL_UL-Core |  |  |  |  |  | Date: |  |  | 2024-05-27 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | For 2 and 4 port UL MIMO a capability of ‘nonCoherent’, ‘partialCoherent’, or ‘fullCoherent’ is reported using pusch-TransCoherence. According to pusch-TransCoherence, the UE supports a full/partial/noncoherent, partial/noncoherent, or noncoherent codebook subset for 2 or 4 ports. For 8 port UL MIMO, the UE reports support for one or more of ‘codebook1’, ‘codebook2’, ‘codebook3’, or ‘codebook4’, each of which corresponds to a number of antenna port groups, and where the UE maintains coherence within each group. Codebook subsets are not supported for 8 ports, and UEs are instead reconfigured with different values of CodebookTypeUL to use precoders with different numbers of mutually coherent elements. Therefore, UEs can be configured for ‘full coherent’, ‘partial coherent’, or ‘non-coherent’ operation with 2 or 4 antenna ports, but not with 8 antenna ports, where the UE instead can be configured for a codebook with its corresponding number of antenna port groups. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Full-, partial-, and non-coherent operation is associated with 2 and 4 port operation, while the amount of coherence is identified by corresponding value(s) of CodebookTypeUL for 8 antenna ports. An editorial change was made as well, adding a space in ‘ULPTRS’ in the sentence just after the first change. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | For 8 antenna port UL MIMO with PT-RS configured, the UE and the network will not know the number of PT-RS ports to use, the number of PUSCH layers in the antenna port group which are precoded coherently with the PUSCH layer/DM-RS port that PT-RS port x is associated with, and/or PUSCH to PT-RS power ratio per layer per RE. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | N | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | N | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | N | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### 6.2.3.1	UE PT-RS transmission procedure when transform precoding is not enabled

When transform precoding is not enabled and if a UE is configured with the higher layer parameter phaseTrackingRS in DMRS-UplinkConfig,

<Unchanged part omitted>

If a UE has reported the capability of supporting full-coherent UL transmission with 2 or 4 antenna ports or the capability of codebook1-8TxPUSCH-r18 with 8 antenna ports, the UE shall expect maxNrofPorts in PTRS-UplinkConfig to be configured as one if UL PT-RS is configured. If a UE has reported the capability of supporting full-coherent UL transmission and when the higher layer parameter multipanelScheme is set to 'sdmscheme', subject to UE capability, the UE can be configured with maxNrofPortsforSDM in PTRS-UplinkConfig set to n2, where at most one PT-RS port is associated with each SRS resource set with higher layer parameter usage set to 'codebook'/'nonCodebook'.

For codebook or non-codebook based UL transmission, the association between UL PT-RS port(s) and DM-RS port(s) is signalled by PTRS-DMRS association field(s) in DCI format 0_1, 0_2 and 0_3. For a PUSCH corresponding to a configured grant Type 1 transmission, the UE may assume the association between UL PT-RS port(s) and DM-RS port(s) defined by value 0 in Table 7.3.1.1.2-25, or value "00" in Table 7.3.1.1.1.2-26 or value "00" in Table 7.3.1.1.1.2-25a described in Clause 7.3.1 of [5, TS38.212].

For PUSCH scheduled by DCI format 0_0 or by activation DCI format 0_0, the UL PT-RS port is associated to DM-RS port 0.

For non-codebook based UL transmission, the actual number of UL PT-RS port(s) to transmit is determined based on SRI(s) in DCI format 0_1, 0_2 or 0_3 or higher layer parameter sri-ResourceIndicator in rrc-ConfiguredUplinkGrant. When two SRS resource sets are configured in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with higher layer parameter usage in SRS-ResourceSet set to 'noncodebook', the actual number of UL PT-RS port(s) to transmit corresponding to each SRS resource set is determined based on SRI(s) corresponding to the associated SRS resource set or higher layer parameter sri-ResourceIndicator or sri-ResourceIndicator2 corresponding to the associated SRS resource set in rrc-ConfiguredUplinkGrant. A UE is configured with the PT-RS port index for each configured SRS resource by the higher layer parameter ptrs-PortIndex configured by SRS-Config if the UE is configured with the higher layer parameter phaseTrackingRS in DMRS-UplinkConfig. If the PT-RS port index associated with different SRIs are the same, the corresponding UL DM-RS ports are associated to the one UL PT-RS port.

When the higher layer parameter multipanelScheme is set to 'sdmscheme' and two SRS resource sets are configured in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with higher layer parameter usage in SRS-ResourceSet set to 'codebook'/'nonCodebook' and higher layer parameter maxNrofPortsforSDM in PTRS-UplinkConfig set to n2, the actual number of UL PT-RS port(s) to transmit corresponding to SRS resource sets is 2.

When the higher layer parameter multipanelScheme is set to 'SFNscheme' and two SRS resource sets are configured in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with higher layer parameter usage in SRS-ResourceSet set to 'codebook'/'nonCodebook' and the higher layer parameter maxNrofPorts in PTRS-UplinkConfig is set to n2, the actual number of UL PT-RS port(s) to transmit corresponding to each SRS resource set is determined based on 1st TPMI codepoint field for 'codebook' or 1st SRI(s) codepoint field for 'nonCodebook'.

For partial-coherent and non-coherent codebook-based UL transmission with 2 or 4 antenna ports or when the higher layer parameter CodebookTypeUL is set to ‘codebook2’, ‘codebook3’, or ‘codebook4’ with 8 antenna ports, the actual number of UL PT-RS port(s) is determined based on TPMI(s) and/or number of layers which are indicated by 'Precoding information and number of layers' field(s) in DCI format 0_1, 0_2 or 0_3 or configured by higher layer parameter precodingAndNumberOfLayers:

-	if the UE is configured with the higher layer parameter maxNrofPorts in PTRS-UplinkConfig set to 'n2', the actual UL PT-RS port(s) and the associated transmission layer(s) are derived from indicated TPMI(s) as:

-	for PUSCH transmission with 2 or 4 ports, PUSCH antenna port 1000 and 1002 in indicated TPMI(s) share PT-RS port 0, and PUSCH antenna port 1001 and 1003 in indicated TPMI(s) share PT-RS port 1.

-	UL PT-RS port 0 is associated with the UL layer 'x' of layers which are transmitted with PUSCH antenna port 1000 and PUSCH antenna port 1002 in indicated TPMI(s), and UL PT-RS port 1 is associated with the UL layer 'y' of layers which are transmitted with PUSCH antenna port 1001 and PUSCH antenna port 1003 in indicated TPMI(s), where 'x' and/or 'y' are given by DCI parameter 'PTRS-DMRS association' as shown in DCI format 0_1, 0_2 and 0_3 described in Clause 7.3.1 of [5, TS38.212].

-	for PUSCH transmission with 8 ports, PUSCH antenna port 1000, 1001, 1004 and 1005 in indicated TPMI(s) share PT-RS port 0, and PUSCH antenna port 1002, 1003, 1006 and 1007 in indicated TPMI(s) share PT-RS port 1.

-	UL PT-RS port 0 is associated with the UL layer 'x' of layers which are transmitted with one or more of PUSCH antenna port 1000, 1001, 1004 and 1005 in indicated TPMI(s), and UL PT-RS port 1 is associated with the UL layer 'y' of layers which are transmitted with one or more of PUSCH antenna port 1002, 1003, 1006 and 1007 in indicated TPMI(s), where 'x' and/or 'y' are given by DCI parameter 'PTRS-DMRS association' as shown in DCI format 0_1 and DCI format 0_2 described in Clause 7.3.1 of [5, TS38.212].

If a UE is scheduled with two codewords,

-	if the UE is configured with the higher layer parameter maxNrofPorts in PTRS-UplinkConfig set to 'n1', the PT-RS port is associated with the one of DM-RS ports indicated by DCI field PTRS-DMRS association for the codeword with the higher MCS. If the MCS indices of the two codewords are the same, the PT-RS antenna port is associated with codeword 0. When a codeword is scheduled to transmit PUSCH for retransmission, the MCS for determining PT-RS association to codeword is obtained from the DCI for the same transport block in the initial transmission.

When the UE is scheduled with Qp={1,2} PT-RS port(s) in uplink and the number of scheduled layers is ,

-	If the UE is configured with higher layer parameter ptrs-Power, the PUSCH to PT-RS power ratio per layer per RE  is given by , where  is shown in the Table 6.2.3.1-3, Table 6.2.3.1-3A and Table 6.2.3.1-3B according to the higher layer parameter ptrs-Power, the PT-RS scaling factor  specified in clause 6.4.1.2.2.1 of [4, TS 38.211] is given by and also on the 'Precoding Information and Number of Layers' field in DCI.

-	The UE shall assume ptrs-Power in PTRS-UplinkConfig is set to state "00" in Table 6.2.3.1-3, Table 6.2.3.1-3A, and Table 6.2.3.1-3B  if not configured or in case of non-codebook based PUSCH.

When the higher layer parameter CodebookTypeUL is set to ‘codebook2’or ‘codebook3’ for 8 antenna  porttransmission, Lx is the number of PUSCH layers in the antenna port group which are precoded coherently with the PUSCH layer/DM-RS port that PT-RS port x is associated with, and Qp is the number of PT-RS ports scheduled to the UE.

-	When the higher layer parameter multipanelScheme is set to ‘sdmscheme’ and two SRS resource sets are configured in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with higher layer parameter usage in SRS-ResourceSet set to 'codebook'/’nonCodebook’, and codepoint "10" of SRS Resource Set indicator is indicated, for each PT-RS port is based on Table 6.2.3.1-3B, where Qp is the total number of PT-RS ports for the PUSCH.

Table 6.2.3.1-3: Factor related to PUSCH to PT-RS power ratio per layer per RE  other than 8TX PUSCH transmission and other than SDM PUSCH

| UL-PTRS-power / |  | The number of PUSCH layers ( ) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1 | 2 |  | 3 |  | 4 |  |  |
|  | All cases | Full coherent | Partial and non- coherent and non-codebook based | Full coherent | Partial and non- coherent and non-codebook based | Full coherent | Partial coherent | Non-coherent and non-codebook based |
| 00 | 0 | 3 | 3Qp-3 | 4.77 | 3Qp-3 | 6 | 3Qp | 3Qp-3 |
| 01 | 0 | 3 | 3 | 4.77 | 4.77 | 6 | 6 | 6 |
| 10 | Reserved |  |  |  |  |  |  |  |
| 11 | Reserved |  |  |  |  |  |  |  |

Table 6.2.3.1-3A: Factor related to PUSCH to PT-RS power ratio per layer per RE  for 8TX PUSCH transmission

| UL-PTRS-power / | The number of PUSCH layers (nlayerPUSCH) |  |  |
| --- | --- | --- | --- |
|  | 1-8 |  |  |
|  | CodebookTypeUL =‘codebook1’ | CodebookTypeUL =‘codebook2’ or ‘codebook3’ | CodebookTypeUL =‘codebook4’ |
| 00 | 10log10nlayerPUSCH | 10log10LxQp | 10log10Qp |
| 01 | 10log10nlayerPUSCH | 10log10nlayerPUSCH | 10log10nlayerPUSCH |
| 10 | Reserved |  |  |
| 11 | Reserved |  |  |

Table 6.2.3.1-3B: Factor related to PUSCH to PT-RS power ratio per layer per RE for SDM PUSCH

| UL-PTRS-power / | The number of PUSCH layers associated with the same SRS resource set as the PT-RS port |  |  |
| --- | --- | --- | --- |
|  | 1 | 2 |  |
|  | All cases | Full coherent | Partial and non- coherent and non-codebook based |
| 00 | 3Qp-3 | 3Qp | 3Qp-3 |
| 01 | 3Qp-3 | 3Qp | 3Qp |
| 10 | Reserved |  |  |
| 11 | Reserved |  |  |

<Unchanged part omitted>