3GPP TSG-RAN WG1 Meeting #116bis	R1-2403579

# Changsha, Hunan Province, China, April 15th – 19th, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.213 | CR | 0055 | Rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on restrictions of performing multi-channel access |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Moderator (OPPO), CATT, CICTCI, Huawei, HiSilicon, Qualcomm |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SL_enh2-Core |  |  |  |  |  | Date: |  |  | 2024-04-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In clause 4.5.6.3, the reference to define the channel frequencies of a set of channels when UE performs multi-channel access procedures is missing. In SL-U, the multiple channel access procedure is designed using NR-U uplink multiple channel access procedure as a baseline. For uplink procedure, specified in clause 4.2.1.0.4, the reference for channel frequencies refers to a restriction on frequencies between multiple carriers when UE performs uplink multi-channel access. However, for SL-U, carrier aggregation is not supported and neither restriction between carrier frequencies is needed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The whole sentence including the missing reference in clause 4.5.6.3 is removed. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE is misled to have an additional restriction on channel frequencies when UE performs multi-channel access. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.5.6.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

< Start of change request >

#### 4.5.6.3	Multi-channel access procedures for SL transmissions

The procedures described in this clause are applied for PSCCH/PSSCH/S-SSB transmission(s) and may be applied for PSFCH transmission.

A UE can access multiple channels on which SL transmissions are performed, according to the procedures described in this clause.

If a UE intends to transmit SL transmissions on a set of channels C, the following is applicable:

-	if Type 1 channel access procedure is used for SL transmissions on the set of channels C,

-	the UE may transmit on channel ci∈C using Type 2A channel access procedure as described in clause 4.5.2.1,

C

-	if Type 2A channel access procedure is performed on channel ci immediately before the UE transmission on channel cj∈C, i≠j, and

-	if the UE has accessed channel cj using Type 1 channel access procedure as described in clause 4.5.1,

-	where channel cj is selected by the UE uniformly randomly from the set of channels C before performing Type 1 channel access procedure on any channel in the set of channels C.

-	the UE may transmit on channel ci∈C using Type 1 channel access procedure as described in clause 4.5.1

-	the UE may not transmit on channel ci∈C within the bandwidth of a carrier, if the UE fails to access any of the channels, of the carrier bandwidth, on which the UE is scheduled or configured with or selects SL resources.

-	the UE may not transmit on a channel within the bandwidth of a carrier if the UE is configured without intra-cell guard band(s) on an SL bandwidth part as described in clause 7 of [8], and the UE fails to access any of the channels of the SL bandwidth part.

A CWp value is maintained independently for each channel ci∈C using the procedure described in clause 4.5.4. For determining CWp for channel ci, any PSSCH transmission that fully or partially overlaps with any channel ci∈C is used in the procedures described in clause 4.5.4.

After a UE successfully performs a multi-channel access procedure for a set of channels C, a channel occupancy is initiated for the set of channels and the UE can use the initiated channel occupancy for own subsequent transmissions (including any of S-SSB, PSFCH or PSCCH/PSSCH).

The UE shall not transmit a transmission on a channel ci∈C, for a period exceeding Tmcot,p as given in Table 4.5-1, where the value of Tmcot,p is determined using the channel access parameters used for channel cj, for accessing the channel.

< End of change request >