3GPP TSG-RAN WG1 Meeting #116bis	R1-2403580

# Changsha, Hunan Province, China, April 15th – 19th, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.213 | CR | 0056 | Rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on CAPC determination in multi-channel access procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Moderator (OPPO), Sharp, Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SL_enh2-Core |  |  |  |  |  | Date: |  |  | 2024-04-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In current specification, CAPC is determined by taking multiple SL transmissions over one or multiple consecuive slots into consideration for performing Type 1 LBT to initiate a channel occupancy on a channel. However, the CAPC determiantion in current specificaiton does not cover the case of multi-channel access procedure. As illustrated in the Figure below, if a UE does not take the future consecutive SL transmissions with higher CAPC value into consideration in determining CAPC, the UE cannot utilize the CO and would be forced to perform a new Type 1 LBT to access the channel for transmitting PSSCH (TB#2), which may cause the SL transmission to be not successfully transmitted. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add a “multi-channel access procedure” to clarify that, for multi channel access procedure, multiple SL transmisisons over one slot or mutiple consecutive slots are taken into consideration for CAPC determination. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | When a UE performs a multi-channel access procedure to initiate channel occupancy for multiple cosecutive SL transmissions within a set of channels, a consecutive SL transmission in a channel associated with higher CAPC value than the CAPC value to be used in Type 1 channel access in a channel cannot use the initiated channel occupancy. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

< Start of change request >

## 4.5	Sidelink Channel access procedures

A UE operating in sidelink resource allocation mode 1 or mode 2 and performing SL transmission(s) on channel(s) shall perform the procedures described in this clause for the UE to access the channel(s) on which the transmission(s) are performed.

In this clause, transmissions from a UE are considered as separate SL transmissions, irrespective of having a gap between transmissions or not, and XThresh for sensing is adjusted as described in clause 4.5.5 when applicable.

A UE can access a channel on which SL transmission(s) are performed according to one of Type 1 or Type 2 SL channel access procedures as described in clauses 4.5.1 and 4.5.2, respectively.

When a UE applies Type 1 channel access procedures to transmit SL transmission(s), the applicable channel access priority class (CAPC) is defined in Table 4.5-1.

When a UE applies Type 1 channel access procedures to transmit SL transmission(s) including PSSCH with user plane data and associated PSCCH, the UE determines the corresponding SL channel access priority class p  in Table 4.5-1 following the procedures described in Clause 16.9.x.2 in [9].

When a UE applies Type 1 channel access procedures to transmit SL transmission(s) including only PSFCH or only S-SSB transmission(s), the UE shall use the channel access priority class p=1 in Table 4.5-1.

A UE shall not transmit on a channel for a Channel Occupancy Time that exceeds Tslmcot,p where the channel access procedure is performed based on the channel access priority class p  associated with the UE transmissions, as given in Table 4.5-1.

When a UE applies Type 1 channel access procedure or multi-channel access procedure to initiate a channel occupancy for multiple SL transmissions over one slot or multiple consecutive slots, the highest CAPC value among the associated CAPC values with the multiple SL transmissions is used for performing the Type 1 channel access procedure or multi-channel access procedure.

If a UE fails to access the channel(s) prior to an intended SL transmission(s), Layer 1 notifies higher layers about the channel access failure and the channel(s) that the UE fails to access.

< End of change request >