3GPP TSG-RAN WG1 Meeting #117	R1-2405664

# Fukuoka City, Fukuoka, Japan, May 20th – 24th, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.213 | CR | 0057 | Rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on contention window adjustment procedures for PSCCH/PSSCH transmission |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Moderator (OPPO), Samsung, CATT, CICTCI |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SL_enh2-Core |  |  |  |  |  | Date: |  |  | 2024-05-23 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In TS 37.213, there are two methods for CW adjustment. In one method (denoted by method 1), UE adjusts the contention window size based on the HARQ feedback(s) corresponding to the PSSCH within a SL reference duration. And in method 2, UE will use the latest CW of any SL transmission in the past. However, the condition of method 1 and method 2 is not mutually exclusive in current specification. For example, PSSCH transmission with HARQ-ACK disabled can satisfy the condition of both method 1 and method 2. It will lead to an ambiguity about which method should be selected in such case. This is mainly because the condition of method 1 doesn’t restrict the HARQ feedback manner for PSSCH transmission and the condition of method 2 uses the description of SL transmission not associated with explicit HARQ-ACK feedback which also includes PSSCH transmission with HARQ-ACK disabled. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Change “at least one PSSCH” to “at least one PSSCH associated with explicit HARQ-ACK feedback(s) by the corresponding UE(s)” in the condition of method 1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The spec remains ambiguous about which method of CW adjustment should be performed for PSSCH transmission with HARQ-ACK disabled, if this CR is not approved. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.5.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

< Start of change request >

### 4.5.4	Contention window adjustment procedures for SL transmissions

If a UE transmits a SL transmission(s) including at least one PSSCH enabled with explicit HARQ-ACK feedback including ‘ACK/NACK’ using Type 1 channel access procedures associated with the channel access priority class p on a channel, the UE maintains the contention window value CWp and adjusts CWp before step 1 of the procedure described in clause 4.5.1 for the SL transmission(s) applying the following procedures:

< End of change request >