3GPP TSG- RAN WG1 Meeting #117	R1-2405537

# Fukuoka, Japan, May 20th - May 24th, 2024

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.213 | CR | 0627 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on the frequency resource of a resource pool for SL-U |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Moderator (Huawei), CATT, CICTCI, NEC, OPPO |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |
| Work item code: | NR_SL_enh2-Core |  |  |  | Date: |  | 2024-05-23 |
| Category: | F |  |  |  | Release: |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |

| Reason for change: | The number of sub-channel in a resource pool with interlace-based structure should be determined by the number of interlaces in each RB set divided by the number of interlaces per sub-channel, rather than provided by higher layer parameter sl-NumSubchannel. |  |  |  |
| --- | --- | --- | --- | --- |
| Summary of change: | In clause 16, correct the determination of the number of sub-channel in a resource pool when sl-TransmissionStructureForPSCCHandPSSCH = 'interlaceRB', which is equal to the number of interlaces in each RB set divided by the number of interlaces per sub-channel. |  |  |  |
| Consequences if not approved: | The determination of the number of sub-channel in a resource pool is incorrect. |  |  |  |
| Clauses affected: | 16 |  |  |  |
|  | Y | N |  |  |
| Other specs |  | X | Other core specifications | TS/TR ... CR ... |
| affected: |  | X | Test specifications | TS/TR ... CR ... |
| (show related CRs) |  | X | O&M Specifications | TS/TR ... CR ... |
| Other comments: |  |  |  |  |
| This CR's revision history: |  |  |  |  |

16	UE procedures for sidelink

A UE is provided by SL-BWP-Config or SL-BWP-ConfigCommon a BWP for SL transmissions (SL BWP) with numerology and resource grid determined as described in [4, TS 38.211].

For a resource pool within the SL BWP,

-	for operation without shared spectrum channel access, or for operation with shared spectrum channel access and when sl-TransmissionStructureForPSCCHandPSSCH = 'contiguousRB', the UE is provided by sl-NumSubchannel a number of sub-channels where each sub-channel includes a number of contiguous RBs provided by sl-SubchannelSize. The first RB of the first sub-channel in the SL BWP is indicated by sl-StartRB-Subchannel

-	for operation with shared spectrum channel access and when sl-TransmissionStructureForPSCCHandPSSCH = 'interlaceRB', the UE is provided a number of sub-channels in each RB set which is equal to the number of interlaces in each RB set divided by the number of interlaces per sub-channel, provided by sl-NumInterlacePerSubchannel, and the interlaces have contiguous interlace indexes

<<< UNCHANGED PARTS OMITTED >>>