3GPP TSG- RAN WG1 Meeting #116-bis	R1-2403586

# Changsha, China, April 15-April 19, 2024

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.214 | CR | 0543 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on PSSCH preparation time for multiple PSFCH occasions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Moderator (Huawei), OPPO, Qualcomm, vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SL_enh2-Core |  |  |  |  |  | Date: |  |  | 2024-4-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In SL-U, one PSSCH resource can be associated with multiple candidate PSFCH resources. Hence, the starting reference point for UE PSSCH preparation time (i.e., Tprep+δ) in mode 1 resource allocation should be updated to the last candidate PSFCH occasion corresponding to the most recent PSSCH transmission. |  |  |  |  |  |  |  |  |
| Summary of change: |  | When sl-NumPSFCH-Occasions is (pre-)configured, the last candidate PSFCH occasion is used as the starting reference point for UE preparation time in mode 1 resource allocation. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A scheduled PSSCH transmission may occur before the PSFCH reception corresponding to the most recent PSSCH transmission, if this CR is not approved. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*** Start of change request ***

8.6	UE PSSCH preparation procedure time

<omitted text>

For sidelink resource allocation mode 1, the UE does not expect that the first sidelink symbol in the sidelink allocation for a PSSCH for retransmission of a transport block and the associated PSCCH, including the DM-RS and the duplicated symbol as defined by the "Time resource assignment" field of the corresponding DCI for dynamic grant or for SL configured grant type 2, or by sl-TimeResourceCG-Type1 for configured grant type 1 starts earlier than at symbol L where L is defined as the next sidelink symbol with its CP starting Tprep+δ after the end of the last symbol of the PSFCH occasion corresponding to the most recent transmission of PSSCH for the same transport block if sl-NumPSFCH-Occasions is not (pre-)configured, or after the end of the last symbol of the last candidate PSFCH occasion corresponding to the most recent transmission of PSSCH for the same transport block if sl-NumPSFCH-Occasions is (pre-)configured, where Tprep is defined in Clause 16.5 of [6, TS 38.213] and δ=5∙10-4 s. Otherwise the UE may skip the retransmission of the PSSCH and the transmission of the corresponding PSCCH.

*** End of change request ***