3GPP TSG- RAN WG1 Meeting #116-bis	R1-2403669

# Changsha, China, April 15-April 19, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.214 | CR | 0548 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on TRIV/FRIV resource indication of SL-U |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Moderator (Huawei), CATT, CICTCI, vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SL_enh2-Core |  |  |  |  |  | Date: |  |  | 2024-04-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - In clause 8.1.2.2, the editorial error should be corrected. - In clause 8.1.5, the sub-channel number in a resource pool with interlace-based structure should be determined by the number of interlaces in each RB set divided by the number of interlace(s) per sub-channel, rather than provided by higher layer parameter sl-NumSubchannel. - The higher layer parameter transmissionStructureForPSCCHandPSSCH is not consistent with the corresponding parameter sl-TransmissionStructureForPSCCHandPSSCH in TS 38.331. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - The editorial error in clause 8.1.2.2 is corrected. - In clause 8.1.5, correct the determination of the sub-channel number in a resource pool with interlace-based structure. - Higher layer parameter transmissionStructureForPSCCHandPSSCH is corrected to sl-TransmissionStructureForPSCCHandPSSCH. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The specification is incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.2.2, 8.1.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<< UNCHANGED PARTS OMITTED >>>

8.1.2.2	Resource allocation in frequency domain

The resource allocation unit in the frequency domain is the sub-channel.

The sub-channel assignment for sidelink transmission is determined using the "Frequency resource assignment" field in the associated SCI.

The lowest sub-channel for sidelink transmission is the sub-channel on which the lowest PRB of the associated PSCCH is transmitted.

If a PSSCH scheduled by a PSCCH would overlap with resources containing the PSCCH, the resources corresponding to a union of the PSCCH that scheduled the PSSCH and associated PSCCH DM-RS are not available for the PSSCH.

When PSSCH is transmitted on multiple RB sets, the corresponding PSCCH is located on the sub-channel with the lowest index of the RB set with the lowest index within the multiple RB sets.

For operation with shared spectrum channel access for frequency range 1, if the higher layer parameter sl-TransmissionStructureForPSCCHandPSSCH is set to ‘interlaceRB’,

-	the lowest index of the RB set allocation to the initial PSSCH transmission is indicated via the field "Lowest index of the RB set allocation to the initial transmission" of the DCI format 3_0.

-	the starting RB set index of the initial PSSCH transmission of the sidelink configured grant Type 1 is indicated via the higher layer parameter sl-StartRBsetCG-Type1.

<<< UNCHANGED PARTS OMITTED >>>

8.1.5	UE procedure for determining slots and resource blocks for PSSCH transmission associated with an SCI format 1-A

The set of slots and resource blocks for PSSCH transmission is determined by the resource used for the PSCCH transmission containing the associated SCI format 1-A, and fields 'Frequency resource assignment', 'Time resource assignment' of the associated SCI format 1-A as described below.

'Time resource assignment' carries logical slot offset indication of N = 1 or 2 actual resources when sl-MaxNumPerReserve is 2, and N = 1 or 2 or 3 actual resources when sl-MaxNumPerReserve is 3, in a form of time RIV (TRIV) field which is determined as follows:

if N=1

TRIV=0

elseif N=2

TRIV=t1

else

if t2-t1-1≤15

TRIV=30t2-t1-1+t1+31

else

TRIV=3031-t2+t1+62-t1

end if

end if

where the first resource is in the slot where SCI format 1-A was received, and ti denotes i-th resource time offset in logical slots of a resource pool with respect to the first resource where for N = 2, 1≤t1≤31; and for N = 3, 1≤t1≤30, t1<t2≤31.

The starting sub-channel nsubCH,0start of the first resource is determined according to clause 8.1.2.2. The number of contiguously allocated sub-channels for each of the N resources LsubCH≥1 and the starting sub-channel indexes of resources indicated by the received SCI format 1-A, except the resource in the slot where SCI format 1-A was received, are determined from "Frequency resource assignment" which is equal to a frequency RIV (FRIV) where.

If sl-MaxNumPerReserve is 2 then

FRIV=nsubCH,1start+i=1LsubCH-1N subchannel SL+1-i

If sl-MaxNumPerReserve is 3 then

FRIV=nsubCH,1start+nsubCH,2start⋅N subchannel SL+1-LsubCH+i=1LsubCH-1N subchannel SL+1-i2

where

-	nsubCH,1start denotes the starting sub-channel index for the second resource

-	nsubCH,2start denotes the starting sub-channel index for the third resource

-	N subchannel SL is the number of sub-channels in a resource pool provided according to the higher layer parameter sl-NumSubchannel, or if the higher layer parameter sl-TransmissionStructureForPSCCHandPSSCH is set to 'interlaceRB', the number of sub-channels in each RB set which is equal to the number of interlaces in each RB set divided by the number of interlace(s) per sub-channel provided according to the higher layer parameter sl-NumInterlacePerSubchannel

If the higher layer parameter sl-TransmissionStructureForPSCCHandPSSCH is set to 'interlaceRB', the applied interlace index(s) in different RB sets are the same.

If the higher layer parameter sl-TransmissionStructureForPSCCHandPSSCH is set to 'interlaceRB', the starting RB set nRBset,0start of the first resource is determined according to the clause 8.1.2.2. The number of contiguously allocated RB sets for each of the N resources LRBset≥1 and the starting RB set indexes of resources indicated by the received SCI format 1-A, except the resource in the slot where SCI format 1-A was received, are determined from "Frequency resource assignment" which is equal to a frequency RIV (FRIV) where.

If sl-MaxNumPerReserve is 2 then

FRIVRBset=nRBset,1start+i=1LRBset-1NRBset+1-i

If sl-MaxNumPerReserve is 3 then

FRIVRBset=nRBset,1start+nRBset,2start⋅NRBset+1-LBset+i=1LRBset-1NRBset+1-i2

where

-	nRBset,1start denotes the starting RB set index for the second resource,

-	nRBset,2start denotes the starting RB set index for the third resource,

-	NRBset is the number of RB sets in a resource pool,

-	LRBset is the number of RB sets for each of the indicated resources,

-	for FRIV indication, within the resource pool, RB sets are numbered in increasing order from 0 to NRBset-1 from lowest frequency location to highest frequency location.

If the higher layer parameter sl-TransmissionStructureForPSCCHandPSSCH is set to 'interlaceRB', the resource is determined by an intersection of the interlaces corresponding to the indicated sub-channel(s) and the union of the indicated set of RB sets and intra-cell guard bands between the indicated RB sets, if any.

If TRIV indicates N < sl-MaxNumPerReserve,

-	if the higher layer parameter sl-TransmissionStructureForPSCCHandPSSCH is set to 'interlaceRB', the starting sub-channel indexes and the starting RB set indexes corresponding to sl-MaxNumPerReserve minus N last resources are not used.

-	otherwise, the starting sub-channel indexes corresponding to sl-MaxNumPerReserve minus N last resources are not used.

The number of slots in one set of the time and frequency resources for transmission opportunities of PSSCH is given by Cresel where Cresel= 10*SL_RESOURCE_RESELECTION_COUNTER [10, TS 38.321] if configured else Cresel is set to 1.

If a set of sub-channels in slot t'mSL is determined as the time and frequency resource for PSSCH transmission corresponding to the selected sidelink grant (described in [10, TS 38.321]), the same set of sub-channels in slots t'm+j×Prsvp_TX'SL are also determined for PSSCH transmissions corresponding to the same sidelink grant where j=1, 2,…, Cresel-1, Prsvp_TX, if provided, is converted from units of msec to units of logical slots, resulting in Prsvp_TX' according to clause 8.1.7, and t'0SL, t'1SL, t'2SL,… is determined by Clause 8. Here, Prsvp_TX is the resource reservation interval indicated by higher layers.

<<< UNCHANGED PARTS OMITTED >>>