3GPP TSG- RAN WG1 Meeting #116-bis	R1-2403671

# Changsha, China, April 15-April 19, 2024

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.214 | CR | 0550 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on contiguous RB based resource allocation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Moderator (Huawei), Sharp, vivo, Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SL_enh2-Core |  |  |  |  |  | Date: |  |  | 2024-04-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | For contiguous RB based resource allocation in SL-U, if the highest sub-channel of PSSCH overlaps with a single RB set and intra-cell guard band PRBs, the UE can transmit PSSCH on the PRBs belonging to the allocated sub-channel(s) except for the intra-cell guard band PRBs within the highest sub-channel. However, it cannot cover the case when the highest sub-channel of PSSCH overlaps only with intra-cell guard band PRBs, i.e. fully locates within the intra-cell guard band PRBs, as shown in the following figure (sub-channel#6). When it occurs, UE shall use RBs of the highest sub-channel for PSSCH transmission, which clearly contradicts with the principle that intra-cell guard band PRBs can be used only if both adjacent RB sets are allocated. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify that if the highest RB of the highest sub-channel of the PSSCH overlaps with intra-cell guard band PRBs, UE can transmit PSSCH on the PRBs belonging to the allocated sub-channels except for the intra-cell guard band PRBs within the highest sub-channel(s) to cover the above case. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Intra-cell guard band PRBs can be used for PSSCH transmission when only one RB set of adjacent RB sets is allocated. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This is the first version of the draft CR. |  |  |  |  |  |  |  |  |

8    Physical sidelink shared channel related procedures

<Unchanged parts are omitted>

For operation with shared spectrum channel access for frequency range 1, a sidelink resource pool can be (pre-)configured to include integer number of RB sets, and the lowest RB of the sidelink resource pool is aligned with the lowest RB of lowest RB set in the resource pool, and the highest RB of the sidelink resource pool is aligned with the highest RB of highest RB set in the resource pool. A UE can be configured with intra-cell guard bands according to the higher layer parameter intraCellGuardBandsSL-List. The configured intra-cell guard band PRBs between any two adjacent RB sets can be used only for PSSCH transmission, if and only if, the UE has successfully performed channel access procedure in both adjacent RB sets, and the UE uses both of these RB sets for PSSCH transmission. If the higher layer parameter transmissionStructureForPSCCHandPSSCH is set to ‘contiguousRB', and if more than 1 sub-channel is used for PSSCH transmission, when the highest sub-channel of PSSCH overlaps with a single RB set and intra-cell guard band PRBs, the UE can transmit PSSCH on the PRBs belonging to the allocated sub-channel(s) except for the intra-cell guard band PRBs within the highest sub-channel. If the higher layer parameter sl-TransmissionStructureForPSCCHandPSSCH is set to ‘contiguousRB', and if more than 1 sub-channel is used for PSSCH transmission, when the highest sub-channel(s) of PSSCH overlaps with intra-cell guard band PRBs only, the UE can transmit PSSCH on the PRBs belonging to the allocated sub-channel(s) except for the intra-cell guard band PRBs overlapping with the highest sub-channel(s).

<Unchanged parts are omitted>