3GPP TSG-RAN WG1 Meeting #117	R1-2405528

# Fukuoka City, Fukuoka, Japan, May 20th – 24th, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.214 | CR | 0559 | Rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on SL partial sensing for interlaced RB allocation and MCSt |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Moderator (OPPO), Samsung, CATT, CICTCI |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SL_enh2-Core |  |  |  |  |  | Date: |  |  | 2024-05-23 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The combination of MCSt and partial sensing is supported according to the current specification. The current description “any set of LsubCH contiguous sub-channels … correspond to one candidate single-slot resource” covers only a candidate single-slot resource case, but there is no corresponding description or definition for one candidate multi-slot resource (as it is intended by the specification). The support of interlaced RB resource allocation and SL partial sensing should be supported in Release 18. However, it is currently missing in the specification. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updated the description on “UE shall assume that any set of LsubCH contiguous sub-channels or LsubCH contiguous sub-channels in LRBset contiguous RB sets” as the definition for both one candidate single-slot resource and one candidate multi-slot resource in SL partial sensing. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The definition of a candidate multi-slot resource in SL partial sensing and the support of interlaced RB allocation with SL partial sensing remains unspecified in the specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

< Start of change request >

### 8.1.4	UE procedure for determining the subset of resources to be reported to higher layers in PSSCH resource selection in sidelink resource allocation mode 2

<Unchanged part omitted>

The UE shall assume that any set of LsubCH contiguous sub-channels or LsubCH contiguous sub-channels in LRBset contiguous RB sets included in the corresponding resource pool within the time interval [n+T1,n+T2] correspond to one candidate single-slot resource or the UE shall assume that any set of LsubCH contiguous sub-channels or LsubCH contiguous sub-channels in LRBset contiguous RB sets in Nslot,MCSt consecutive slots included in the corresponding resource pool within the time interval n+T1,n+T2 correspond to one candidate multi-slot resource for UE performing full sensing. The UE shall assume that any set of LsubCH contiguous sub-channels or LsubCH contiguous sub-channels in LRBset contiguous RB sets included in the corresponding resource pool in a set of Y candidate slots within the time interval n+T1,n+T2 correspond to one candidate single-slot resource or the UE shall assume that any set of LsubCH contiguous sub-channels or LsubCH contiguous sub-channels in LRBset contiguous RB sets in Nslot,MCSt consecutive slots included in the corresponding resource pool in a set of Y candidate slots within the time interval n+T1,n+T2 correspond to one candidate multi-slot resource for UE performing periodic-based partial sensing together with contiguous partial sensing and resource (re)selection triggered by periodic transmission (Prsvp_TX≠0), or in a set of Y' candidate slots within the time interval [n+T1,n+T2] correspond to one candidate single-slot resource or one candidate multi-slot resource for UE performing at least contiguous partial sensing and resource (re)selection triggered by aperiodic transmission (Prsvp_TX=0), where

< End of change request >