3GPP TSG- RAN WG1 Meeting #117	R1-2405535

# Fukuoka, Japan, May 20th - May 24th, 2024

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.214 | CR | 0560 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on guardband PRB handling |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Moderator (Huawei), vivo, Samsung, OPPO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SL_enh2-Core |  |  |  |  |  | Date: |  |  | 2024-05-23 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | For the case where the highest sub-channel(s) of a candidate resource overlaps with a single RB set and intra-cell guardband PRBs or overlap with intra-cell guardband PRBs only, it is not clear whether puncturing or rate matching is performed for the PSSCH. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify the RBs corresponding to the intra-cell guard band PRBs in the highest sub-channel(s) are not available for PSSCH if the highest sub-channel(s) of a candidate resource overlaps with a single RB set and intra-cell guardband PRBs or overlap with intra-cell guardband PRBs only. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There is confusion on the UE behavior about the PSSCH handling when the highest sub-channel(s) of a candidate resource overlaps with a single RB set and intra-cell guardband PRBs or overlap with intra-cell guardband PRBs only. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

8.1.2.2	Resource allocation in frequency domain

The resource allocation unit in the frequency domain is the sub-channel.

The sub-channel assignment for sidelink transmission is determined using the "Frequency resource assignment" field in the associated SCI.

The lowest sub-channel for sidelink transmission is the sub-channel on which the lowest PRB of the associated PSCCH is transmitted.

If a PSSCH scheduled by a PSCCH would overlap with resources containing the PSCCH, the resources corresponding to a union of the PSCCH that scheduled the PSSCH and associated PSCCH DM-RS are not available for the PSSCH.

If the highest sub-channel(s) of a PSSCH would overlap with a single RB set and the highest PRB in the highest sub-channel overlaps with intra-cell guardband PRBs, or if the highest sub-channel(s) of a PSSCH would overlap with intra-cell guardband PRBs only, the RBs corresponding to the intra-cell guard band PRBs in the highest sub-channel(s) are not available for the PSSCH.

When PSSCH is transmitted on multiple RB sets, the corresponding PSCCH is located on the sub-channel with the lowest index of the RB set with the lowest index within the multiple RB sets.

For operation with shared spectrum channel access for frequency range 1, if the higher layer parameter transmissionStructureForPSCCHandPSSCH is set to ‘interlaceRB:,

-	the lowest index of the RB set allocation to the initial PSSCH transmission is indicated via the field "Lowest index of the RB set allocation to the initial transmission" of the DCI format 3_0.

-	the starting RB set index of the initial PSSCH transmission of the sidelink configured grant Type 1 is indicated via the higher layer parameter sl-StartRBsetCG-Type1.