3GPP TSG-RAN WG1 Meeting #117	R1-2405555

# Fukuoka City, Fukuoka, Japan, May 20th – 24th, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.214 | CR | 0563 | Rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on CPE starting position determination and transmission for PSSCH/PSCCH |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Moderator (OPPO), Samsung, CATT, CICTCI |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SL_enh2-Core |  |  |  |  |  | Date: |  |  | 2024-05-23 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In clause 8.1.2.1, - The determination of CPE duration within a channel occupancy should be applied to all SL transmissions with PSSCH/PSCCH by the UE, other than the first SL transmission that initiates the channel occupancy. - The condition of transmitted or detected resource reservation for the slot of the intended PSSCH/PSCCH transmission should be based on “any one of the RB set(s)”. This is correctly captured for the initiating COT case, but not for the within COT case. - The current spec only describes how to determine the CPE duration for PSSCH/PSCCH transmissions, but the timing / OFDM symbol(s) in which the CPE is to be transmitted is unclear. - There is inconsistency of the description / terms used in the same spec paragraph to describe the intended PSCCH/PSSCH transmission for which the CPE is to be applied. This can lead to different interpretations of which PSSCH/PSCCH transmission(s) should be applied with CPE. - The index for CPE in some parts of the spec is correctly described as “the index i for 𝐶𝑖 and Δi” for SL-U operation, but not in other parts. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In clause 8.1.2.1, - It is corrected that the determination of CPE duration within a channel occupancy should be applied to all SL transmissions with PSSCH/PSCCH by the UE, other than the first SL transmission that initiates the channel occupancy. - It is clarified that the condition of transmitted or detected resource reservation for the slot of the PSSCH/PSCCH transmission should be based on “any one of the RB set(s)” for the within COT case. - It is clarified that the timing / OFDM symbol(s) in which the CPE (Text) to be applied for the intended PSSCH/PSCCH transmission is within the first one or two symbols before the first symbol of the PSSCH/PSCCH transmission. - It is clarified in the paragraphs that CPE transmission is applied for the “intended” PSCCH/PSSCH transmission to achieve a consistent description within the same spec. - The description of CPE index for SL-U operation is corrected as “the index i for 𝐶𝑖 and Δi” throughout the spec. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Specification remains to be incorrect in determining the CPE starting position for PSSCH/PSCCH transmission within a channel occupancy. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

< Start of change request >

8.1.2.1 Resource allocation in time domain

<Unchanged part omitted>

-	For operation with shared spectrum channel access in frequency range 1, for the first SL transmission with PSSCH/PSCCH by a UE to initiate a channel occupancy for a slot, if no resource reservation is transmitted or detected for the slot and any one of the RB set(s) of the intended PSCCH/PSSCH transmission, and if UE is configured with multiple CPE starting positions provided by CPEStartingPositionsPSCCH-PSSCH-InitiateCOT, the UE determines a duration of a cyclic prefix extension Text to be applied within the first one or two symbols before the first symbol of the PSSCH/PSCCH transmission according to [4, TS 38.211] where the index i for 𝐶𝑖 and Δi [4, TS 38.211] is chosen randomly from a set of values configured per priority of the PSCCH/PSSCH transmission by the higher layer parameter CPEStartingPositionsPSCCH-PSSCH-InitiateCOT. Otherwise, the UE uses a configured default cyclic prefix extension Text within the first one or two symbols before the first symbol of the PSSCH/PSCCH transmission indicated by DefaultCPEStartingPositionsPSCCH-PSSCH-InitiateCOT.

-	For operation with shared spectrum channel access in frequency range 1, for an intended SL transmission with PSSCH/PSCCH by a UE within a channel occupancy, other than the first SL transmission initiating the channel occupancy, the UE transmitting in the channel occupancy determines the duration of a cyclic prefix extension Text  to be applied within the first one or two symbols before the first symbol of the PSSCH/PSCCH transmission only according to higher layer parameter DefaultCPEStartingPositionsPSCCH-PSSCH-SharedCOT, unless the UE is configured with multiple CPE starting positions for transmitting within the channel occupancy by CPEStartingPositionsPSCCH-PSSCH-SharedCOT, in which case the UE determines the duration of a cyclic prefix extension Text to be applied within the first one or two symbols before the first symbol of the PSSCH/PSCCH transmission according to [4, TS 38.211] where the index i for 𝐶𝑖 and Δi [4, TS 38.211] is chosen randomly from a set of values configured per priority of the PSCCH/PSSCH by the higher layer parameter CPEStartingPositionsPSCCH-PSSCH-SharedCOT, if no resource reservation is transmitted or detected for the slot and any one of the RB set(s) of the intended PSCCH/PSSCH transmission, otherwise, the UE uses the configured default cyclic prefix extension Text to be applied within the first one or two symbols before the first symbol of the PSSCH/PSCCH transmission indicated by DefaultCPEStartingPositionsPSCCH-PSSCH-SharedCOT.

-	For operation with shared spectrum channel access in frequency range 1, for an intended PSSCH/PSCCH transmission by a UE that follows another SL transmission by the same UE in a channel occupancy, the UE determines the duration of a cyclic prefix extension Text to be applied within the first one or two symbols before the first symbol of the PSSCH/PSCCH transmission as follows, regardless of the duration of the cyclic prefix extension determined based on sl-CPE-StartingPositionsPSCCH-PSSCH-WithinCOT-Default or sl-CPE-StartingPositions in sl-CPE-StartingPositionsPSCCH-PSSCH-WithinCOT-List, if applicable:

-	When gap between the PSSCH/PSCCH transmission and the previous SL transmission is 1 symbol, the index i for 𝐶𝑖 and Δiis set to '1'.

-	When gap between the PSSCH/PSCCH transmission and the previous SL transmission is 2 symbols, the index i for 𝐶𝑖 and Δiis set to '3' for µ=1 and to ‘2’ for µ=2.

< End of change request >