3GPP TSG RAN WG1 Meeting #117	R1-2405709

# Fukuoka City, Japan, May 20th - 24th, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.213 | CR | 0639 | rev | - | Current version: | 17.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on SSB to CG-SDT PUSCH mapping |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Moderator(ZTE), Samsung, Ericsson, vivo, Xiaomi, New H3C |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SmallData_INACTIVE-Core |  |  |  |  |  | Date: |  |  | 2024-05-24 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In RAN1#106-e meeting, the SSB to CG PUSCH mapping method was agreed. Agreement - Each N of consecutive SSB indexes associated to one CG configuration are mapped to valid CG PUSCH resources - first, in increasing order of DMRS resource indexes, where a DMRS resource index DMRSid is determined first in an ascending order of a DMRS port index and second in an ascending order of a DMRS sequence index - second, in increasing order of CG period indexes in the association period - The mapping ratio N is explicitly signalled and the association period is implicitly derived - FFS candidate value set of mapping ratio, and whether it is configured per CG configuration or per cell - The SSB to CG PUSCH association period is the duration of multiple of CG periods depending the smallest time duration in the set determined by the CG period such that all SSBs associated with the CG configuration are mapped at least once to CG PUSCH resources. - An association pattern period includes one or more association periods and is determined so that a pattern between CG PUSCH occasions and SS/PBCH block indexes associated with the CG configuration repeats at most every 640 msec. Note: The mapping ordering and steps may be revisited if multiple CG PUSCH occasions in one CG period is supported. In RAN1#108-e meeting, it’s further clarified that non-consecutive SSB indexes are also allowed to be configured in SSB subset for mapping. Agreement Non-consecutive SSB indexes are allowed to be configured in SSB subset for SSB to CG PUSCH mapping.. Based on the above agreements, each N SSBs will be mapped to one DMRS port, where N is the mapping ratio, however, current spec does not capture the agreement completely, it’s not clear on the order of SSB indexes and how to map SSBs. For example, SSB#0, 1, 2, 3 are mapped to DMRS port#0 and 1 with mapping ratio 2, based on current spec, there might be 2 different understandings on mapping methods, i.e. - SSB#0 and 1 are mapped to DMRS port#0, then SSB#2 and 3 are mapped to DMRS port#1. - SSB#0 and 1 are mapped to DMRS port#0 and DMRS port#1 respectively, then SSB#2 and 3 are mapped to DMRS port#0 and DMRS port#1 respectively. The first understanding aligns with the agreement, but current spec may also be misinterpreted as second understanding, which would result in ambiguous implementation from BS and UE. |
| Summary of change: |  | For SSB to CG-SDT PUSCH mapping, capture the missing information from previous agreements, i.e. “Each N of NPUSCHSS/PBCH SS/PBCH block indexes in increasing order are mapped to a valid PUSCH occasion and the associated DMRS resource”, “where N is provided by sdt-SSB-PerCG-PUSCH.” |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | SSB to CG-SDT PUSCH mapping is not clear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 19.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This is the first version of the CR. |  |  |  |  |  |  |  |  |

19.1	Configured-grant based PUSCH transmission

< Unchanged text omitted >

Each N of NPUSCHSS/PBCH SS/PBCH block indexes in increasing order are mapped to a valid PUSCH occasion and the associated DMRS resource

-	first, in increasing order of DMRS resource indexes within a PUSCH occasion, where a DMRS resource index DMRSid is determined first in an ascending order of a DMRS port index and second in an ascending order of a DMRS sequence index [4, TS 38.211]

-	second, in increasing order of PUSCH configuration period indexes

where N is provided by sdt-SSB-PerCG-PUSCH.

A PUSCH occasion is valid if it does not overlap with a valid PRACH occasion as described in clause 8.1.

< Unchanged text omitted >