3GPP TSG-RAN WG1 Meeting #117	R1-2405702

Fukuoka, Japan, May 20th – 24th, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.212 | CR | 0194 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR for 38.212 on TRS occasions for idle/inactive UEs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Nokia, Mediatek, CATT, Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI18, NR_UE_pow_sav_enh-Core |  |  |  |  |  | Date: |  |  | 2024-05-24 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In RAN2#125-bis, RAN2 agreed to CRs introducing a new Rel-18 SIB for configuring TRS occasions for idle/inactive UEs, and the network either broadcasts SIB17 or the new Rel-18 SIB. This was related to the LS that RAN1 sent to RAN2 in R1-2312620. RAN2 agreed the CRs in principle: R2-2403847 Clarification on TRS in idle and inactive CATT, Ericsson CR 38.300 (Rel-18) R2-2403848 Clarification on TRS in idle and inactive Ericsson CR 38.331 (Rel-17) R2-2403849 Clarification on TRS in idle and inactive Ericsson CR 38.331 (Rel-18) 3 above are in-principle agreed In RAN2#126, the final CRs were agreed. R2-2404698 Correction on TRS for idle and inactive UEs CATT, Ericsson CR Rel-18 38.300 18.1.0 0836 2 F NR_UE_pow_sav_enh-Core R2-2403847 R2-2404988 Clarification on TRS in idle and inactive Ericsson, MediaTek, ZTE, Nokia, Huawei, HiSilicon, Apple, CATT CR Rel-17 38.331 17.8.0 4754 2 F NR_UE_pow_sav_enh-Core R2-2403848 Moved from 6.1.3.1 R2-2404989 Correction on TRS in idle and inactive Ericsson, MediaTek, ZTE, Nokia, Huawei, HiSilicon, Apple, CATT CR Rel-18 38.331 18.1.0 4755 2 F NR_UE_pow_sav_enh-Core R2-2403849 Moved from 6.1.3.1 All above agreed Undefined UE behavior for idle/inactive UEs for reception of TRS occasions configured by the new Rel-18 SIB. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Reflect the new higher layer parameters introduced for configuring TRS occasions for idle/inactive UEs in 38.212 to correct the UE behavior for idle/inactive UEs for reception of TRS occasions configured by the new Rel-18 SIB. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect specification and undefined UE behavior regarding the TRS occasion provisioning for idle/inactive UEs by the new Rel-18 SIB. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3.1.2.1, 7.3.1.3.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | Y |  | Other core specifications |  |  |  | TS38.211 (R1-2405701) TS38.213 (R1-2405703) TS38.214 (R1-2405704) |  |  |
| affected: |  |  | N | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | N | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

        - 7.3.1.2.1	Format 1_0

<Omitted Unchanged text>

The following information is transmitted by means of the DCI format 1_0 with CRC scrambled by P-RNTI:

-	Short Messages Indicator - 2 bits according to Table 7.3.1.2.1-1.

-	Short Messages - 8 bits, according to Clause 6.5 of [9, TS38.331]. If only the scheduling information for Paging, and TRS availability indication if trs-ResourceSetConfig or trs-ResourceSetConfig-r18 is configured, are carried, this bit field is reserved.

-	Frequency domain resource assignment - bits. If only the short message, and TRS availability indication if trs-ResourceSetConfig or trs-ResourceSetConfig-r18 is configured, are carried, this bit field is reserved.

-	 is the size of CORESET 0

-	Time domain resource assignment - 4 bits as defined in Clause 5.1.2.1 of [6, TS38.214]. If only the short message, and TRS availability indication if trs-ResourceSetConfig or trs-ResourceSetConfig-r18 is configured, are carried, this bit field is reserved.

-	VRB-to-PRB mapping - 1 bit according to Table 7.3.1.2.2-5. If only the short message, and TRS availability indication if trs-ResourceSetConfig or trs-ResourceSetConfig-r18 is configured, are carried, this bit field is reserved.

-	Modulation and coding scheme - 5 bits as defined in Clause 5.1.3 of [6, TS38.214], using Table 5.1.3.1-1. If only the short message, and TRS availability indication if trs-ResourceSetConfig or trs-ResourceSetConfig-r18 is configured, are carried, this bit field is reserved.

-	TB scaling - 2 bits as defined in Clause 5.1.3.2 of [6, TS38.214]. If only the short message, and TRS availability indication if trs-ResourceSetConfig or trs-ResourceSetConfig-r18 is configured, are carried, this bit field is reserved.

-	TRS availability indication - 1, 2, 3, 4, 5, or 6 bits, where the number of bits is equal to one plus the highest value of all the indBitID(s) provided by the trs-ResourceSetConfig if configured or the number of bits is equal to one plus the highest value of all the indBitID-r18(s) provided by the trs-ResourceSetConfig-r18 if configured; 0 bits otherwise.

-	Reserved bits - (8 - M) bits for operation in a cell with shared spectrum channel access in frequency range 1 or for operation in a cell in frequency range 2-2; (6 - M) bits for operation in a cell without shared spectrum channel access, where the value of M is the number of bits for the field of 'TRS availability indication' as defined above.

<Omitted Unchanged text>

        - 7.3.1.3.8	Format 2_7

DCI format 2_7 is used for notifying the paging early indication and TRS availability indication for one or more UEs.

The following information is transmitted by means of the DCI format 2_7 with CRC scrambled by PEI-RNTI:

-	Paging indication field - NPOPEINSGPO bit(s), where

-	NPOPEI is the number of paging occasions configured by higher layer parameter po-NumPerPEI as defined in Clause 10.4A in [5, TS 38.213];

-	NSGPOis the number of sub-groups of a paging occasion configured by higher layer parameter subgroupsNumPerPO.

-	Each bit in the field indicates one UE subgroup of a paging occasion.

-	TRS availability indication - 1, 2, 3, 4, 5, or 6 bits, where the number of bits is equal to one plus the highest value of all the indBitID(s) provided by the trs-ResourceSetConfig if configured or the number of bits is equal to one plus the highest value of all the indBitID-r18(s) provided by the trs-ResourceSetConfig-r18 if configured; 0 bits otherwise.

The size of DCI format 2_7 is indicated by the higher layer parameter payloadSizeDCI-2-7, according to Clause 10.4A of [5, TS 38.213]. The number of information bits in format 2_7 shall be equal to or less than the payload size of format 2_7. If the number of information bits in format 2_7 is less than the size of format 2_7, the remaining bits are reserved.

<Omitted Unchanged text>