3GPP TSG-RAN WG1 Meeting #117	R1-2405703

Fukuoka, Japan, May 20th – 24th, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.213 | CR | 0637 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR for 38.213 on TRS occasions for idle/inactive UEs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Nokia, Mediatek, CATT, Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI18, NR_UE_pow_sav_enh-Core |  |  |  |  |  | Date: |  |  | 2024-05-24 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In RAN2#125-bis, RAN2 agreed to CRs introducing a new Rel-18 SIB for configuring TRS occasions for idle/inactive UEs, and the network either broadcasts SIB17 or the new Rel-18 SIB. This was related to the LS that RAN1 sent to RAN2 in R1-2312620. RAN2 agreed the CRs in principle: R2-2403847 Clarification on TRS in idle and inactive CATT, Ericsson CR 38.300 (Rel-18) R2-2403848 Clarification on TRS in idle and inactive Ericsson CR 38.331 (Rel-17) R2-2403849 Clarification on TRS in idle and inactive Ericsson CR 38.331 (Rel-18) 3 above are in-principle agreed In RAN2#126, the final CRs were agreed. R2-2404698 Correction on TRS for idle and inactive UEs CATT, Ericsson CR Rel-18 38.300 18.1.0 0836 2 F NR_UE_pow_sav_enh-Core R2-2403847 R2-2404988 Clarification on TRS in idle and inactive Ericsson, MediaTek, ZTE, Nokia, Huawei, HiSilicon, Apple, CATT CR Rel-17 38.331 17.8.0 4754 2 F NR_UE_pow_sav_enh-Core R2-2403848 Moved from 6.1.3.1 R2-2404989 Correction on TRS in idle and inactive Ericsson, MediaTek, ZTE, Nokia, Huawei, HiSilicon, Apple, CATT CR Rel-18 38.331 18.1.0 4755 2 F NR_UE_pow_sav_enh-Core R2-2403849 Moved from 6.1.3.1 All above agreed Undefined UE behavior for idle/inactive UEs for reception of TRS occasions configured by the new Rel-18 SIB. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Reflect the new higher layer parameters introduced for configuring TRS occasions for idle/inactive UEs in 38.213 to correct the UE behavior for idle/inactive UEs for reception of TRS occasions configured by the new Rel-18 SIB. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect specification and undefined UE behavior regarding the TRS occasion provisioning for idle/inactive UEs by the new Rel-18 SIB. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.4B |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | Y |  | Other core specifications |  |  |  | TS38.211 (R1-2405701) TS38.212 (R1-2405702) TS38.214 (R1-2405704) |  |  |
| affected: |  |  | N | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | N | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## 10.4B	Indication of TRS resources

A UE in RRC_IDLE state or RRC_INACTIVE state can be provided by trs-ResourceSetConfig or by trs-ResourceSetConfig-r18 a set of TRS occasions [6, TS 38.214]. If trs-ResourceSetConfig or trs-ResourceSetConfig-r18 is provided, a DCI format 2_7, if pei-SearchSpace is provided, and a DCI format 1_0 with CRC scrambled by P-RNTI includes a TRS availability indication field [4, TS 38.212] that provides a bitmap to groups of TRS resource sets where the configuration of each TRS resource set includes an association to a bit of the bitmap. For TRS occasions provided by trs-ResourceSetConfig,  the UE can be additionally provided a multiple, by validityDuration, for a number of frames provided by defaultPagingCycle for TRS resource sets with indicated presence; if validityDuration is not provided, the multiple is equal to 2. For TRS occasions provided by trs-ResourceSetConfig-r18, the UE can be additionally provided a multiple, by validityDuration-r18, for a number of frames provided by defaultPagingCycle for TRS resource sets with indicated presence; if validityDuration-r18 is not provided, the multiple is equal to 2.

A value of '1' for a bit of the bitmap indicates presence of associated TRS resource sets for the multiple of the number of frames, starting from a SFN determined from SFN+PF_offsetmodT=0 [17, TS 38.304] that corresponds to the frame within the DRX cycle that includes the PDCCH providing the DCI format 2_7, or the DCI format 1_0 with CRC scrambled by P-RNTI, with the TRS availability indication field indicating the TRS resource sets, where T is provided by defaultPagingCycle. A value of '0' for a bit of the bitmap indicates no change to a current assumption for the availability or unavailability of associated TRS resource sets.

A UE can receive first and second PDCCHs that provide DCI format 2_7 or DCI format 1_0 with CRC scrambled by P-RNTI that indicate presence of TRS resource sets for the multiple of the number of frames, where the second PDCCH reception after the first PDCCH reception by a time that is smaller than the multiple of the number of frames.