3GPP TSG RAN Meeting #104								RP-241095

Shanghai, China, June 17-20, 2024

Agenda Item:	9.2.2

Source:	Samsung

Title:	On the evaluation of Ambient IoT

Document for:	Discussion, Decision

1	Introduction

RAN#102 approved a new RAN1-led Rel-19 SI on Ambient IoT solutions with some updates in RAN #103 in [1]. RAN 4 #110bis and RAN 4 #111 had some discussions co-existence evaluation. This contribution is to clarify the scenarios for co-existence evaluation for Ambient IoT.

2	On co-existence evaluation

The SID [1] for the RAN1-led Rel-19 SI on Ambient IoT solutions includes the following statements:

General Scope The definitions provided in TR 38.848 are taken into this SI, and the following are the exclusive general scope: …… B. Deployment Scenarios with the following characteristics, referenced to the tables in Clause 4.2.2 of TR 38.848: - Deployment scenario 1 with Topology 1 - Basestation and coexistence characteristics: Micro-cell, co-site - Deployment scenario 2 with Topology 2 and UE as intermediate node, under network control - Basestation and coexistence characteristics: Macro-cell, co-site - The location of intermediate node is indoor C. FR1 licensed spectrum in FDD. D. Spectrum deployment in-band to NR, in guard-band to LTE/NR, in standalone band(s).

In addition, in Clause 4.2.2 of TR 38.848 [2], both “Co-site” and “new site” of coexistence with existing 3GPP technologies are considered.

#### 4.2.2.1 Deployment scenario 1: Device indoors, basestation indoors With Ambient IoT device indoors and basestation indoors, this deployment scenario is characterized according to Table 4.2.2.1-1. Table 4.2.2.1-1: Characteristics of deployment scenario 1 Applicable representative use cases; Characteristics; Description (NOTE 1). Indoor inventory Indoor sensor Indoor positioning Indoor command; Environment (of device); Indoor. Basestation characteristic (if any); Micro- or pico-cell. Connectivity topology; Topology (1), (2), (3). Spectrum; Licensed FDD, licensed TDD, unlicensed. Coexistence with existing 3GPP technologies; Co-site or new site. Traffic assumption; DT and DO. Device characteristic; Device A or Device B or Device C. NOTE 1: Descriptions may not be applicable for some Devices (A, B). #### 4.2.2.2 Deployment scenario 2: Device indoors, basestation outdoors With Ambient IoT device indoors and basestation outdoors, this deployment scenario is characterized according to Table 4.2.2.2-1. Table 4.2.2.2-1: Characteristics of deployment scenario 2 Applicable representative use cases; Characteristics; Description (NOTE 1). Indoor inventory Indoor sensor Indoor positioning Indoor command; Environment (of device); Indoor. Basestation characteristic (if any); Macro- or Micro- cell BS. Connectivity topology; Topology (1), (2), (3) Note: The location of intermediate or assisting node (if any) is indoor or outdoor. Spectrum; Licensed FDD, licensed TDD, unlicensed. Coexistence with existing 3GPP technologies; Co-site or new site. Traffic assumption; DT and DO. Device characteristic; Device C may support Topology (1), (2), (3), Device A may support Topology (2), Device B may support Topology (2), (3). NOTE 1: Descriptions may not be applicable for some Devices (A, B).

However, in RAN 4 #110bis [2] and RAN 4 #111 [3], the co-existence study prioritized non-co-located case, shown as Figure 1, while the option 2-1 and option 2-2, shown as Figure 2, for co-located NR gNB and AIoT reader are still for further study.

Agreement in RAN4#110bis: Issue 2-1-1: deployment scenarios for D1T1 Option 1-1: Legacy NR gNB are outdoor macro gNB while AIoT reader/CW/devices are all indoors. Legacy NR UE is only allowed outdoors. Option 1-2: Legacy NR gNB are outdoor macro gNB while AIoT reader/CW/devices are all indoors. Legacy NR UE is indoor accessing to outdoor NR marco gNB Option 2-1: Legacy NR gNB are co-located with AIoT reader and CW. All of NR and AIoT BS/UE/Reader/Device/CW are indoors. AIoT reader /CW and Legacy gNB share same hardware Option 2-2: Legacy NR gNB are co-located with AIoT reader and CW. All of NR and AIoT BS/UE/Reader/Device/CW are indoors. AIoT reader /CW and Legacy NR gNB do not share same hardware. (less limitation on the power boosting) Agreement: - RAN4 to first evaluate co-existence for deployment scenario of option 1-1 and 1-2, and further study option 2-1 and 2-2. Issue 2-3-2: Priorities of spectrum deployment mode for co-existence evaluation Agreement: - Prioritize the following spectrum deployment mode for RAN4 co-existence evaluation - A-IoT is located within a NR transmission bandwidth configuration - A-IoT which is operating indoor shares in-band spectrum with outdoor macro BS Agreement in RAN4#111: - Consider only adjacent RB/channel co-existence evaluation for in-band deployment scenario for NR and AIOT - Encourage companies to provide the simulation results for option 1-1 and 1-2 - FFS on co-site scenario (option 2-1 and 2-2)

Based on SID, co-existence evaluation shall also include “co-site” between gNB for NR devices and Reader for A-IoT device.

Proposal: Co-existence evaluation shall include co-site scenarios of gNB for NR and Reader for A-IoT.

Figure 1 Co-existence for deployment scenario of option 1-1 and 1-2 with new site for A-IoT Reader

Figure 2 Co-existence for deployment scenario of option 2-1 and 2-2 with co-site of gNB for NR and Reader for A-IoT

3	Conclusion

This contribution discusses the SID scope and existing discussion in working group. According to SID, co-existence evaluation includes co-site scenarios, which shall be studied. Therefore, we’d like to highlight this point in RAN plenary so that WG can complete co-existence evaluation requested by SID.

Proposal: Co-existence evaluation shall include co-site scenarios of gNB for NR and Reader for A-IoT.

References

RP-240826, “Revised SID: Study on solutions for Ambient IoT (Internet of Things) in NR”, CMCC, Huawei, T-Mobile USA

TR 38.848 V18.0.0, “Study on Ambient IoT (Internet of Things) in RAN”, September 2023.

R4-2406714, “WF on Ambient IoT in NR”, CMCC, Changsha, China, 15th – 19th April, 2024

R4-2410567, “WF on co-existence evaluation for ambient IoT”, CMCC, Fukuoka, Japan, 20th May 2024 - 24th May 2024