3GPP TSG-RAN WG3 Meeting #124	R3-243063

Fukuoka, Japan, 20th – 24th May 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.413 | CR | 1136 | rev | 2 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | UL SDT for MT communication handling |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE, Qualcomm, CATT, China Telecom, Nokia, Nokia Shanghai Bell, Ericsson, Huawei, Xiaomi |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_redcap_enh-Core |  |  |  |  |  | Date: |  |  | 2024-04-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | For the UE in RRC_inactive with eDRX cycle longer than 10.24 seconds, CN based MT communication handling procedure is specified in SA2. As specified in TS 23.502, i.e., 4.8.2.2 UE Triggered Connection Resume in RRC_INACTIVE procedure. 4a. [Conditional] N2 Path switch procedure. If the accessed NG-RAN is able to retrieve the UE Context, the accessed NG-RAN node initiates N2 Path Switch procedure, i.e. steps 1 to 8 of clause 4.9.1.2.2 and including Xn data forwarding. If the Connection Resume procedure is a response to RAN paging which is triggered by 5GC due to an N2 interface procedure, NG-RAN and 5GC handle the N2 interface procedure as a collision described in clause 4.9.1.2. If Connection Inactive procedure with CN based MT communication handling (see clause 4.8.1.1a) has been performed previously then when the path switch procedure is performed downlink data or signalling delivery is triggered, if there is any. So, in case of anchor relocation, if MT communication handling is already activated, the receiving gNB shall deactivate it by sending Path switch request message to the AMF. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify that in case of anchor relocation, If MT communication handling is already activated, the receiving gNB shall deactivate it by sending Path switch request message to the AMF. Impact Analysis: Impact assessment towards the previous version of the specification (same release): This CR has isolated impact with the previous version of the specification (same release) because it clarified the UL SDT procedure combined with MT communication handling. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Overall UL SDT procedure is incomplete to support the MT Communication Handling for the UE in RRC_INACTIVE with eDRX cycle longer than 10.24 seconds. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.4.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 38.300 CR TS 38.410 CR 0050 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev0: R3-241695 Rev1: R3-242136 |  |  |  |  |  |  |  |  |

------------------------Start of the change-----------------------------

8.4.4	Path Switch Request

8.4.4.1	General

The purpose of the Path Switch Request procedure is to establish a UE associated signalling connection to the 5GC and, if applicable, to request the switch of the downlink termination point of the NG-U transport bearer towards a new termination point. The procedure uses UE-associated signalling.

8.4.4.2	Successful Operation

Figure 8.4.4.2-1: Path switch request: successful operation

The NG-RAN node initiates the procedure by sending the PATH SWITCH REQUEST message to the AMF. Upon reception of the PATH SWITCH REQUEST message the AMF shall, for each PDU session indicated in the PDU Session ID IE, transparently transfer the Path Switch Request Transfer IE to the SMF associated with the concerned PDU session.

Upon reception of the PATH SWITCH REQUEST message the AMF shall deactivate MT communication handling if already activated, as specified in TS 23.502 [10].

If the RRC Resume Cause IE is included in the PATH SWITCH REQUEST message, the AMF shall, if supported, use it as described in TS 23.502 [10] for User Plane CIoT 5GS Optimisation when the NG-RAN node is an ng-eNB.

If the RedCap Indication IE or the eRedCap Indication IE is included in the PATH SWITCH REQUEST message, the AMF shall, if supported, consider the UE respectively as a RedCap UE or an eRedCap UE that was previously served by a E-UTRA cell, and use the IE according to TS 23.501 [9].

After all necessary updates including the UP path switch have been successfully completed in the 5GC for at least one of the PDU session resources included in the PATH SWITCH REQUEST, the AMF shall send the PATH SWITCH REQUEST ACKNOWLEDGE message to the NG-RAN node and the procedure ends.

<Skip unchange part>

------------------------End of the change-----------------------------