3GPP TSG-RAN WG3 Meeting #124	R3-243805

Fukuoka, Japan, 20th-24th, May, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.455 | CR | 0149 | rev | 2 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on Measurement Reporting for BWA |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, ZTE, Xiaomi, Ericsson, Nokia, CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos_enh2-Core |  |  |  |  |  | Date: |  |  | 2024-05-20 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | According to RAN1, the gNB can indicate whether the measurements based on the processing of SRS for positioning resources across aggregated carries or explicitly indicate the aggregated resources IDs. Only the latter is supported by current spec. This feature has also be supported by LPP. For DL-TDOA and Multi-RTT, UE can report whether the measurements are based on aggregated resources: nr-RSTD-BasedOnAggregatedResources-r18 ENUMERATED {true} OPTIONAL, Then the LMF should be able to indicate a preference between reporting the Resource ID or the indication of Measurement Based On Aggregated Resources. The minimum number of maxnoaggregatedSRS-Resources should also be 2 resources… |  |  |  |  |  |  |  |  |
| 1 |  |  |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add a new indicator to TRP Measurement Result IE to indicate the measurements are performed based on SRS aggregation. Add a new bit for the Resource ID in the Measurement Characteristics Request Indicator IE Correct the minimum of the maxnoaggregatedPosSRS-Resources to 2 Impact Analysis: Impact assessment towards the previous version of the specification (same release): This CR has isolated impact with the previous version of the specification (same release) because it affects the measurement reporting for SRS bandwidth aggregation. This CR has an impact under protocol & functional point of view. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Not aligned with RAN1 parameter list. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.2.37, 9.2.81, 9.3.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | x |  | Other core specifications |  |  |  | TS/TR 38.473 CR 1425 |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev1: add the correction in the Measurement Characteristics Request Indicator IE and the minimum of the maxnoaggregatedSRS-Resources Rev2: ASN.1 correction add more co-source |  |  |  |  |  |  |  |  |

<<<<<<<<<<<<<<<<<<<< Change Begin >>>>>>>>>>>>>>>>>>>>

### 9.1.4	Messages for Measurement Information Transfer Procedures

<<<<<<<<<<<<<<<<<<<<Unchanged Text Omitted >>>>>>>>>>>>>>>>>>>>

### 9.2.37	TRP Measurement Result

This information element contains the measurement result.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Measured Result Item |  | 1 .. <maxnoPosMeas> |  |  | - |  |
| >CHOICE Measured Results Value | M |  |  |  | - |  |
| >>UL Angle of Arrival |  |  | 9.2.38 |  |  |  |
| >>UL SRS-RSRP |  |  | INTEGER (0..126) |  |  |  |
| >>UL RTOA Measurement |  |  | 9.2.39 |  |  |  |
| >>gNB Rx-Tx Time Difference |  |  | 9.2.40 |  |  |  |
| >>Z-AoA |  |  | 9.2.67 |  | YES | reject |
| >>Multiple UL-AoA |  |  | 9.2.71 |  | YES | reject |
| >>UL SRS-RSRPP |  |  | 9.2.72 |  | YES | reject |
| >>UL RSCP |  |  | 9.2.92 |  | YES | reject |
| >Time Stamp | M |  | 9.2.42 |  | - |  |
| >Measurement Quality | O |  | 9.2.43 |  | - |  |
| >Measurement Beam Information | O |  | 9.2.57 |  | - |  |
| >SRS Resource type | O |  | 9.2.73 |  | YES | ignore |
| >ARP ID | O |  | 9.2.75 |  | YES | ignore |
| >LoS/NLoS Information | O |  | 9.2.77 |  | YES | ignore |
| >Mobile TRP Location Information | O |  | 9.2.88 |  | YES | ignore |
| >Measured Frequency Hops | O |  | ENUMERATED (singleHop, multiHop, …) |  | YES | ignore |
| >Aggregated Positioning SRS Resource ID List |  | 0..1 |  | Indicates the used Positioning SRS resources across aggregated carriers. | YES | ignore |
| >>Aggregated Positioning SRS Resource ID Item |  | 2..< maxnoaggregatedPosSRS-Resources > |  |  | - |  |
| >>>Positioning SRS Resource ID | M |  | INTEGER (0..63) |  |  |  |
| >Measurement Based On Aggregated Resources | O |  | ENUMERATED(true, …) |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoPosMeas | Maximum no. of measured quantities that can be configured and reported with one positioning measurement message. Value is 16384. |
| maxnoaggregatedPosSRS-Resources | Maximum no of aggregated Positioning SRS resources per UL BWP. Value is 3. |

<<<<<<<<<<<<<<<<<<<<Unchanged Text Omitted >>>>>>>>>>>>>>>>>>>

### 9.2.81	Measurement Characteristics Request Indicator

This IE contains the measurement characteristic information requested by LMF.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Measurement characteristic request indicator | M |  | BIT STRING (SIZE(16)) | Each position in the bitmap represents a requested measurement characteristic: first bit: Measurement Beam Information Second bit: Extended Additional Path List Third bit: UL SRS-RSRPP in Additional Path Fourth Bit: Multiple UL AoA in Additional Path Fifth bit: LoS/NLoS Information Sixth bit: TRP Rx TEG association for UL-TDOA Seventh bit: TRP RxTxTEG information for DL+UL positioning. Eighth bit: SRS Resource Type Ninth bit: Multiple Measurement Instances Tenth bit: Mobile TRP location information Eleventh bit: Aggregated SRS resources IDs used for joint UL positioning measurement. Twelfth bit: Aggregated Positioning SRS resources IDs used for joint UL positioning measurement. Other bits reserved for future use. Value ‘1’ indicates ‘requested measurement characteristic’, Value ‘0’ indicates ‘not requested’. |

<<<<<<<<<<<<<<<<<<<<Unchanged Text Omitted >>>>>>>>>>>>>>>>>>>

### 9.3.5	Information Element definitions

<<<<<<<<<<<<<<<<<<<<Unchanged Text Omitted >>>>>>>>>>>>>>>>>>>>

id-PRSBWAggregationRequestIndication,

id-AggregatedPosSRSResourceID-List,

id-AggregatedPRSResourceSetList,

id-TRPPhaseQuality,

id-ValidityAreaSpecificSRSInformation,

id-TxHoppingConfiguration,

id-MeasuredFrequencyHops,

id-ReportingGranularitykminus1AdditionalPath,

id-ReportingGranularitykminus2AdditionalPath,

id-ReportingGranularitykminus3AdditionalPath,

id-ReportingGranularitykminus4AdditionalPath,

id-ReportingGranularitykminus5AdditionalPath,

id-ReportingGranularitykminus6AdditionalPath,

id-MeasBasedOnAggregatedResources

<<<<<<<<<<<<<<<<<<<<Unchanged Text Omitted >>>>>>>>>>>>>>>>>>>>

AdditionalPathListItem-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

{ ID id-MultipleULAoA	CRITICALITY ignore EXTENSION MultipleULAoA PRESENCE	optional}|

{ ID id-pathPower		CRITICALITY ignore EXTENSION UL-SRS-RSRPP PRESENCE	optional},

...

}

AggregatedPosSRSResourceID-List ::= SEQUENCE (SIZE (2..maxnoaggregatedPosSRS-Resources)) OF AggregatedPosSRSResourceID-Item

AggregatedPosSRSResourceID-Item ::= SEQUENCE {

sRSPosResource-ID	SRSPosResourceID,

iE-Extensions		ProtocolExtensionContainer { { AggregatedPosSRSResourceID-Item-ExtIEs} } OPTIONAL,

...

}

<<<<<<<<<<<<<<<<<<<<Unchanged Text Omitted >>>>>>>>>>>>>>>>>>>>

M

MeasBasedOnAggregatedResources ::= ENUMERATED { true, ... }

<<<<<<<<<<<<<<<<<<<<Unchanged Text Omitted >>>>>>>>>>>>>>>>>>>>

TrpMeasurementResult ::= SEQUENCE (SIZE (1.. maxnoPosMeas)) OF TrpMeasurementResultItem

TrpMeasurementResultItem ::= SEQUENCE {

measuredResultsValue				TrpMeasuredResultsValue,

timeStamp							TimeStamp,

measurementQuality					TrpMeasurementQuality						OPTIONAL,

measurementBeamInfo					MeasurementBeamInfo				OPTIONAL,

iE-Extensions		ProtocolExtensionContainer {{TrpMeasurementResultItem-ExtIEs}}		OPTIONAL,

...

}

TrpMeasurementResultItem-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

{ ID id-SRSResourcetype	CRITICALITY ignore EXTENSION SRSResourcetype PRESENCE optional}|

{ ID id-ARP-ID		CRITICALITY ignore EXTENSION ARP-ID 	PRESENCE optional}|

{ ID id-LoS-NLoSInformation		CRITICALITY ignore EXTENSION LoS-NLoSInformation		PRESENCE optional }|

{ ID id-Mobile-TRP-LocationInformation	CRITICALITY ignore EXTENSION Mobile-TRP-LocationInformation	PRESENCE optional }|

{ ID id-MeasuredFrequencyHops			CRITICALITY ignore EXTENSION MeasuredFrequencyHops					PRESENCE optional }|

{ ID id-AggregatedPosSRSResourceID-List	CRITICALITY ignore EXTENSION AggregatedPosSRSResourceID-List	PRESENCE optional }|

{ ID id-MeasBasedOnAggregatedResources			CRITICALITY ignore EXTENSION MeasBasedOnAggregatedResources PRESENCE optional },

...

}

<<<<<<<<<<<<<<<<<<<<Unchanged Text Omitted >>>>>>>>>>>>>>>>>>>>

9.3.7	Constant definitions

<<<<<<<<<<<<<<<<<<<<Unchanged Text Omitted >>>>>>>>>>>>>>>>>>>>

id-ValidityAreaSpecificSRSInformation							ProtocolIE-ID ::= 135

id-RequestedSRSPreconfigurationCharacteristics-List				ProtocolIE-ID ::= 136

id-SRSPreconfiguration-List										ProtocolIE-ID ::= 137

id-SRSInformation												ProtocolIE-ID ::= 138

id-TxHoppingConfiguration										ProtocolIE-ID ::= 139

id-MeasuredFrequencyHops										ProtocolIE-ID ::= 140

id-ReportingGranularitykminus1									ProtocolIE-ID ::= 141

id-ReportingGranularitykminus2									ProtocolIE-ID ::= 142

id-ReportingGranularitykminus3									ProtocolIE-ID ::= 143

id-ReportingGranularitykminus4									ProtocolIE-ID ::= 144

id-ReportingGranularitykminus5									ProtocolIE-ID ::= 145

id-ReportingGranularitykminus6									ProtocolIE-ID ::= 146

id-ReportingGranularitykminus1AdditionalPath					ProtocolIE-ID ::= 147

id-ReportingGranularitykminus2AdditionalPath					ProtocolIE-ID ::= 148

id-ReportingGranularitykminus3AdditionalPath					ProtocolIE-ID ::= 149

id-ReportingGranularitykminus4AdditionalPath					ProtocolIE-ID ::= 150

id-ReportingGranularitykminus5AdditionalPath					ProtocolIE-ID ::= 151

id-ReportingGranularitykminus6AdditionalPath					ProtocolIE-ID ::= 152

id-MeasBasedOnAggregatedResources								ProtocolIE-ID ::= xxx

<<<<<<<<<<<<<<<<<<<< Change End >>>>>>>>>>>>>>>>>>>>