3GPP TSG-RAN WG3 Meeting #124	R3-243792

# Fukuoka, Japan, 20th – 24th May, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.425 | CR | 0154 | rev | 1 | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on the UL and DL congestion information |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, CMCC, Deutsche Telekom, Nokia, Nokia Shanghai Bell, CATT, ZTE, Ericsson, Qualcomm |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_XR_enh-Core |  |  |  |  |  | Date: |  |  | 2024-05-21 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | To support ECN marking for L4S and congestion information exposure, the gNB-DU can report UL and DL congestion information to the gNB-CU via the ASSISTANCE INFORMATION DATA frame. It should be noticed that such mechanism is only applicable for NR. As specified in TS 38.425, the information elements defined in the ASSISTANCE INFORMATION DATA frame are also applicable to E-UTRA PDCP unless specified otherwise. NOTE 1: All information elements defined in Figure 5.5.2.3-1 are also applicable to E-UTRA PDCP unless specified otherwise in section 5.5.3. For better understanding, it is better to clarify that the UL and DL congestion information related IEs are not applicable for E-UTRA PDCP. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify that the UL and DL congestion information related IEs are not applicable for E-UTRA PDCP. Impact Analysis: Impact assessment towards the previous version of the specification (same release): This CR has isolated impact with the previous version of the specification (same release). This CR has an impact under functional point of view. The impact can be considered isolated because the change only affects the congestion information reporting to the gNB-CU. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The congestion information IEs may be misunderstood to be applicable for E-UTRA PDCP. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.5.3.59, 5.5.3.60, 5.5.3.61, 5.5.3.62 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev 1: R3-243792, add more co-signers. |  |  |  |  |  |  |  |  |

CHANGES START

#### 5.5.3.59	UL Congestion Information Indicator

Description: This parameter indicates the presence of UL Congestion Information. This information element is not applicable to E-UTRA PDCP.

Value range: {0= UL Congestion Information not present, 1= UL Congestion Information present}.

Field length: 1 bit.

#### 5.5.3.60	DL Congestion Information Indicator

Description: This parameter indicates the presence of DL Congestion Information. This information element is not applicable to E-UTRA PDCP.

Value range: {0= DL Congestion Information not present, 1= DL Congestion Information present}.

Field length: 1 bit.

#### 5.5.3.61	UL Congestion Information

Description: For the cases of ECN marking Request, this field indicates the percentage of UL IP packets up to two decimal points that should be ECN marked for a DRB. This information element is not applicable to E-UTRA PDCP.

For the case of Congestion Information Request, this field should be interpreted as a percentage of congestion level in UL up to two decimal points for a DRB.

As an example, value 9574 corresponds to a percentage of 95.74%.

Value range: {0..10000}.

Field length: 2 octets.

#### 5.5.3.62	DL Congestion Information

Description: For the cases of ECN marking Request, this field indicates the percentage of DL IP packets up to two decimal points that should be ECN marked for a DRB. This information element is not applicable to E-UTRA PDCP.

For the case of Congestion Information Request, this field should be interpreted as a percentage of congestion level in DL up to two decimal points for a DRB.

As an example, value 9574 corresponds to a percentage of 95.74%.

Value range: {0..10000}.

Field length: 2 octets.

CHANGES END