3GPP TSG-RAN WG3 Meeting #124	R3-243350

# Fukuoka, Japan, 20 – 24 May 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.413 | CR | 1948 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rapporteur corrections on Conditional IEs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI18 |  |  |  |  |  | Date: |  |  | 2024-05-09 |
| Category: | D |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | RAN3 agreed (as captured in RAN3 meeting minutes) for rapporteurs to update specifications with ASN.1 comments for conditional IEs. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - In the tabular, some Condition descriptions are modified for consistency. - In ASN.1, comments for conditional IEs are added or modified to align with the tabular. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistent and/or unclear specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.1.4.1, 9.1.4.8, 9.1.5.1, 9.1.5.2, 9.2.1.81, 9.2.1.101, 9.2.3.36, 9.2.3.36a, 9.3.3, 9.3.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

start of changes

#### 9.1.4.1	INITIAL CONTEXT SETUP REQUEST

This message is sent by the MME to request the setup of a UE context.

Direction: MME  eNB

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.1.1 |  | YES | reject |
| MME UE S1AP ID | M |  | 9.2.3.3 |  | YES | reject |
| eNB UE S1AP ID | M |  | 9.2.3.4 |  | YES | reject |
| UE Aggregate Maximum Bit Rate | M |  | 9.2.1.20 |  | YES | reject |
| E-RAB to Be Setup List |  | 1 |  |  | YES | reject |
| >E-RAB to Be Setup Item IEs |  | 1 .. <maxnoofE-RABs> |  |  | EACH | reject |
| >>E-RAB ID | M |  | 9.2.1.2 |  | - |  |
| >>E-RAB Level QoS Parameters | M |  | 9.2.1.15 | Includes necessary QoS parameters. | - |  |
| >>Transport Layer Address | M |  | 9.2.2.1 |  | - |  |
| >>GTP-TEID | M |  | 9.2.2.2 |  | - |  |
| >>NAS-PDU | O |  | 9.2.3.5 |  | - |  |
| >>Correlation ID | O |  | 9.2.1.80 |  | YES | ignore |
| >>SIPTO Correlation ID | O |  | Correlation ID 9.2.1.80 |  | YES | ignore |
| >>Bearer Type | O |  | 9.2.1.116 |  | YES | reject |
| >>Ethernet Type | O |  | 9.2.1.147 |  | YES | ignore |
| >>Security Indication | O |  | 9.2.1.163 |  | YES | reject |
| UE Security Capabilities | M |  | 9.2.1.40 |  | YES | reject |
| Security Key | M |  | 9.2.1.41 | The KeNB is provided after the key-generation in the MME, see TS 33.401 [15]. | YES | reject |
| Trace Activation | O |  | 9.2.1.4 |  | YES | ignore |
| Handover Restriction List | O |  | 9.2.1.22 |  | YES | ignore |
| UE Radio Capability | O |  | 9.2.1.27 |  | YES | ignore |
| Subscriber Profile ID for RAT/Frequency priority | O |  | 9.2.1.39 |  | YES | ignore |
| CS Fallback Indicator | O |  | 9.2.3.21 |  | YES | reject |
| SRVCC Operation Possible | O |  | 9.2.1.58 |  | YES | ignore |
| CSG Membership Status | O |  | 9.2.1.73 |  | YES | ignore |
| Registered LAI | O |  | 9.2.3.1 |  | YES | ignore |
| GUMMEI | O |  | 9.2.3.9 | This IE indicates the MME serving the UE. | YES | ignore |
| MME UE S1AP ID 2 | O |  | 9.2.3.3 | This IE indicates the MME UE S1AP ID assigned by the MME. | YES | ignore |
| Management Based MDT Allowed | O |  | 9.2.1.83 |  | YES | ignore |
| Management Based MDT PLMN List | O |  | MDT PLMN List 9.2.1.89 |  | YES | ignore |
| Additional CS Fallback Indicator | C-ifCSFBhighpriority |  | 9.2.3.37 |  | YES | ignore |
| Masked IMEISV | O |  | 9.2.3.38 |  | YES | ignore |
| Expected UE Behaviour | O |  | 9.2.1.96 |  | YES | ignore |
| ProSe Authorized | O |  | 9.2.1.99 |  | YES | ignore |
| UE User Plane CIoT Support Indicator | O |  | 9.2.1.113 |  | YES | ignore |
| V2X Services Authorized | O |  | 9.2.1.120 |  | YES | ignore |
| UE Sidelink Aggregate Maximum Bit Rate | O |  | 9.2.1.122 | This IE applies only if the UE is authorized for V2X services. | YES | ignore |
| Enhanced Coverage Restricted | O |  | 9.2.1.123 |  | YES | ignore |
| NR UE Security Capabilities | O |  | 9.2.1.127 |  | YES | ignore |
| CE-mode-B Restricted | O |  | 9.2.1.129 |  | YES | ignore |
| Aerial UE subscription information | O |  | 9.2.1.136 |  | YES | ignore |
| Pending Data Indication | O |  | 9.2.3.55 |  | YES | ignore |
| Subscription Based UE Differentiation Information | O |  | 9.2.1.140 |  | YES | ignore |
| Additional RRM Policy Index | O |  | 9.2.1.39a |  | YES | ignore |
| IAB Authorized | O |  | 9.2.1.146 |  | YES | ignore |
| NR V2X Services Authorized | O |  | 9.2.1.148 |  | YES | ignore |
| NR UE Sidelink Aggregate Maximum Bit Rate | O |  | 9.2.1.149 | This IE applies only if the UE is authorized for NR V2X services. | YES | ignore |
| PC5 QoS Parameters | O |  | 9.2.1.150 | This IE applies only if the UE is authorized for NR V2X services. | YES | ignore |
| UE Radio Capability ID | O |  | 9.2.1.153 |  | YES | reject |

| Range bound | Explanation |
| --- | --- |
| maxnoofE-RABs | Maximum no. of E-RAB allowed towards one UE, the maximum value is 256. |

| Condition | Explanation |
| --- | --- |
| ifCSFBhighpriority | This IE shall be present if the CS Fallback Indicator IE is set to the value “CS Fallback High Priority”. |

next change

#### 9.1.4.8	UE CONTEXT MODIFICATION REQUEST

This message is sent by the MME to provide UE Context information changes to the eNB.

Direction: MME  eNB

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.1.1 |  | YES | reject |
| MME UE S1AP ID | M |  | 9.2.3.3 |  | YES | reject |
| eNB UE S1AP ID | M |  | 9.2.3.4 |  | YES | reject |
| Security Key | O |  | 9.2.1.41 | A fresh KeNB is provided after performing a key-change on the fly procedure in the MME, see TS 33.401 [15]. | YES | reject |
| Subscriber Profile ID for RAT/Frequency priority | O |  | 9.2.1.39 |  | YES | ignore |
| UE Aggregate Maximum Bit Rate | O |  | 9.2.1.20 |  | YES | ignore |
| CS Fallback Indicator | O |  | 9.2.3.21 |  | YES | reject |
| UE Security Capabilities | O |  | 9.2.1.40 |  | YES | reject |
| CSG Membership Status | O |  | 9.2.1.73 |  | YES | ignore |
| Registered LAI | O |  | 9.2.3.1 |  | YES | ignore |
| Additional CS Fallback Indicator | C-ifCSFBhighpriority |  | 9.2.3.37 |  | YES | ignore |
| ProSe Authorized | O |  | 9.2.1.99 |  | YES | ignore |
| SRVCC Operation Possible | O |  | 9.2.1.58 |  | YES | ignore |
| SRVCC Operation Not Possible | O |  | 9.2.1.119 |  | YES | ignore |
| V2X Services Authorized | O |  | 9.2.1.120 |  | YES | ignore |
| UE Sidelink Aggregate Maximum Bit Rate | O |  | 9.2.1.122 | This IE applies only if the UE is authorized for V2X services. | YES | ignore |
| NR UE Security Capabilities | O |  | 9.2.1.127 |  | YES | ignore |
| Aerial UE subscription information | O |  | 9.2.1.136 |  | YES | ignore |
| Additional RRM Policy Index | O |  | 9.2.1.39a |  | YES | ignore |
| IAB Authorized | O |  | 9.2.1.146 |  | YES | ignore |
| NR V2X Services Authorized | O |  | 9.2.1.148 |  | YES | ignore |
| NR UE Sidelink Aggregate Maximum Bit Rate | O |  | 9.2.1.149 | This IE applies only if the UE is authorized for NR V2X services. | YES | ignore |
| PC5 QoS Parameters | O |  | 9.2.1.150 | This IE applies only if the UE is authorized for NR V2X services. | YES | ignore |
| UE Radio Capability ID | O |  | 9.2.1.153 |  | YES | reject |

| Condition | Explanation |
| --- | --- |
| ifCSFBhighpriority | This IE shall be present if the CS Fallback Indicator IE is set to the value “CS Fallback High Priority”. |

next change

#### 9.1.5.1	HANDOVER REQUIRED

This message is sent by the source eNB to the MME to request the preparation of resources at the target.

Direction: eNB  MME.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.1.1 |  | YES | reject |
| MME UE S1AP ID | M |  | 9.2.3.3 |  | YES | reject |
| eNB UE S1AP ID | M |  | 9.2.3.4 |  | YES | reject |
| Handover Type | M |  | 9.2.1.13 |  | YES | reject |
| Cause | M |  | 9.2.1.3 |  | YES | ignore |
| Target ID | M |  | 9.2.1.6 |  | YES | reject |
| Direct Forwarding Path Availability | O |  | 9.2.3.15 |  | YES | ignore |
| SRVCC HO Indication | O |  | 9.2.1.59 |  | YES | reject |
| Source to Target Transparent Container | M |  | 9.2.1.56 |  | YES | reject |
| Source to Target Transparent Container Secondary | O |  | Source to Target Transparent Container 9.2.1.56 |  | YES | reject |
| MS Classmark 2 | C- ifSRVCCtoGERAN |  | 9.2.1.64 |  | YES | reject |
| MS Classmark 3 | C- ifSRVCCtoGERAN |  | 9.2.1.65 |  | YES | ignore |
| CSG Id | O |  | 9.2.1.62 |  | YES | reject |
| Cell Access Mode | O |  | 9.2.1.74 |  | YES | reject |
| PS Service Not Available | O |  | 9.2.1.77 |  | YES | ignore |

| Condition | Explanation |
| --- | --- |
| ifSRVCCtoGERAN | This IE shall be present if the Handover Type IE is set to the value “LTEtoGERAN” and the SRVCC HO Indication IE is present. |

#### 9.1.5.2	HANDOVER COMMAND

This message is sent by the MME to inform the source eNB that resources for the handover have been prepared at the target side.

Direction: MME  eNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.1.1 |  | YES | reject |
| MME UE S1AP ID | M |  | 9.2.3.3 |  | YES | reject |
| eNB UE S1AP ID | M |  | 9.2.3.4 |  | YES | reject |
| Handover Type | M |  | 9.2.1.13 |  | YES | reject |
| NAS Security Parameters from E-UTRAN | C-iftoUTRANGERAN |  | 9.2.3.30 | The eNB shall use this IE as specified in TS 33.401 [15]. | YES | reject |
| E-RABs Subject to Forwarding List |  | 0..1 |  |  | YES | ignore |
| >E-RABs Subject to Forwarding Item IEs |  | 1 .. <maxnoofE-RABs> |  |  | EACH | ignore |
| >>E-RAB ID | M |  | 9.2.1.2 |  | - |  |
| >>DL Transport Layer Address | O |  | 9.2.2.1 |  | - |  |
| >>DL GTP-TEID | O |  | 9.2.2.2 | To deliver forwarded DL PDCP SDUs. | - |  |
| >>UL Transport Layer Address | O |  | 9.2.2.1 |  | - |  |
| >>UL GTP-TEID | O |  | 9.2.2.2 | To deliver forwarded UL PDCP SDUs. | - |  |
| E-RABs to Release List | O |  | E-RAB List 9.2.1.36 |  | YES | ignore |
| Target to Source Transparent Container | M |  | 9.2.1.57 |  | YES | reject |
| Target to Source Transparent Container Secondary | O |  | Target to Source Transparent Container 9.2.1.57 |  | YES | reject |
| Criticality Diagnostics | O |  | 9.2.1.21 |  | YES | ignore |

| Condition | Explanation |
| --- | --- |
| iftoUTRANGERAN | This IE shall be present if the Handover Type IE is set to the value “LTEtoUTRAN” or “LTEtoGERAN”. |

| Range bound | Explanation |
| --- | --- |
| maxnoofE-RABs | Maximum no. of E-RABs for one UE. Value is 256. |

#### 9.1.5.3	HANDOVER PREPARATION FAILURE

This message is sent by the MME to inform the source eNB that the Handover Preparation has failed.

Direction: MME  eNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.1.1 |  | YES | reject |
| MME UE S1AP ID | M |  | 9.2.3.3 |  | YES | ignore |
| eNB UE S1AP ID | M |  | 9.2.3.4 |  | YES | ignore |
| Cause | M |  | 9.2.1.3 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.1.21 |  | YES | ignore |

#### 9.1.5.4	HANDOVER REQUEST

This message is sent by the MME to the target eNB to request the preparation of resources.

Direction: MME  eNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.1.1 |  | YES | reject |
| MME UE S1AP ID | M |  | 9.2.3.3 |  | YES | reject |
| Handover Type | M |  | 9.2.1.13 |  | YES | reject |
| Cause | M |  | 9.2.1.3 |  | YES | ignore |
| UE Aggregate Maximum Bit Rate | M |  | 9.2.1.20 |  | YES | reject |
| E-RABs To Be Setup List |  | 1 |  |  | YES | reject |
| >E-RABs To Be Setup Item IEs |  | 1 .. <maxnoofE-RABs> |  |  | EACH | reject |
| >>E-RAB ID | M |  | 9.2.1.2 |  | - |  |
| >>Transport Layer Address | M |  | 9.2.2.1 |  | - |  |
| >>GTP-TEID | M |  | 9.2.2.2 | To deliver UL PDUs. | - |  |
| >>E-RAB Level QoS Parameters | M |  | 9.2.1.15 | Includes necessary QoS parameters. | - |  |
| >>Data Forwarding Not Possible | O |  | 9.2.1.76 |  | YES | ignore |
| >>Bearer Type | O |  | 9.2.1.116 |  | YES | reject |
| >>Ethernet Type | O |  | 9.2.1.147 |  | YES | ignore |
| >>Security Indication | O |  | 9.2.1.163 |  | YES | reject |
| Source to Target Transparent Container | M |  | 9.2.1.56 |  | YES | reject |
| UE Security Capabilities | M |  | 9.2.1.40 |  | YES | reject |
| Handover Restriction List | O |  | 9.2.1.22 |  | YES | ignore |
| Trace Activation | O |  | 9.2.1.4 |  | YES | ignore |
| Request Type | O |  | 9.2.1.34 |  | YES | ignore |
| SRVCC Operation Possible | O |  | 9.2.1.58 |  | YES | ignore |
| Security Context | M |  | 9.2.1.26 |  | YES | reject |
| NAS Security Parameters to E-UTRAN | C-iffromUTRANGERAN |  | 9.2.3.31 | The eNB shall use this IE as specified in TS 33.401 [15]. | YES | reject |
| CSG Id | O |  | 9.2.1.62 |  | YES | reject |
| CSG Membership Status | O |  | 9.2.1.73 |  | YES | ignore |
| GUMMEI | O |  | 9.2.3.9 | This IE indicates the MME serving the UE. | YES | ignore |
| MME UE S1AP ID 2 | O |  | 9.2.3.3 | This IE indicates the MME UE S1AP ID assigned by the MME. | YES | ignore |
| Management Based MDT Allowed | O |  | 9.2.1.83 |  | YES | ignore |
| Management Based MDT PLMN List | O |  | MDT PLMN List 9.2.1.89 |  | YES | ignore |
| Masked IMEISV | O |  | 9.2.3.38 |  | YES | ignore |
| Expected UE Behaviour | O |  | 9.2.1.96 |  | YES | ignore |
| ProSe Authorized | O |  | 9.2.1.99 |  | YES | ignore |
| UE User Plane CIoT Support Indicator | O |  | 9.2.1.113 |  | YES | ignore |
| V2X Services Authorized | O |  | 9.2.1.120 |  | YES | ignore |
| UE Sidelink Aggregate Maximum Bit Rate | O |  | 9.2.1.122 | This IE applies only if the UE is authorized for V2X services. | YES | ignore |
| Enhanced Coverage Restricted | O |  | 9.2.1.123 |  | YES | ignore |
| NR UE Security Capabilities | O |  | 9.2.1.127 |  | YES | ignore |
| CE-mode-B Restricted | O |  | 9.2.1.129 |  | YES | ignore |
| Aerial UE subscription information | O |  | 9.2.1.136 |  | YES | ignore |
| Pending Data Indication | O |  | 9.2.3.55 |  | YES | ignore |
| Subscription Based UE Differentiation Information | O |  | 9.2.1.140 |  | YES | ignore |
| Additional RRM Policy Index | O |  | 9.2.1.39a |  | YES | ignore |
| IAB Authorized | O |  | 9.2.1.146 |  | YES | reject |
| NR V2X Services Authorized | O |  | 9.2.1.148 |  | YES | ignore |
| NR UE Sidelink Aggregate Maximum Bit Rate | O |  | 9.2.1.149 | This IE applies only if the UE is authorized for NR V2X services. | YES | ignore |
| PC5 QoS Parameters | O |  | 9.2.1.150 | This IE applies only if the UE is authorized for NR V2X services. | YES | ignore |
| UE Radio Capability ID | O |  | 9.2.1.153 |  | YES | reject |

| Condition | Explanation |
| --- | --- |
| C-iffromUTRANGERAN | This IE shall be present if the Handover Type IE is set to the value “UTRANtoLTE” or “GERANtoLTE”. |

| Range bound | Explanation |
| --- | --- |
| maxnoofE-RABs | Maximum no. of E-RABs for one UE. Value is 256. |

next change

#### 9.2.1.81	MDT Configuration

The IE defines the MDT configuration parameters.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| MDT Activation | M |  | ENUMERATED(Immediate MDT only, Logged MDT only, Immediate MDT and Trace,…, Logged MBSFN MDT) |  | - | - |
| CHOICE Area Scope of MDT | M |  |  |  | - | - |
| >Cell based |  |  |  |  |  | - |
| >>Cell ID List for MDT |  | 1 .. <maxnoofCellIDforMDT> |  |  |  | - |
| >>>E-CGI | M |  | 9.2.1.38 |  | - | - |
| >TA based |  |  |  |  |  | - |
| >>TA List for MDT |  | 1 .. <maxnoofTAforMDT> |  |  |  | - |
| >>>TAC | M |  | 9.2.3.7 | The TAI is derived using the current serving PLMN. | - | - |
| >PLMN Wide |  |  | NULL |  | - | - |
| >TAI based |  |  |  |  | - | - |
| >>TAI List for MDT |  | 1 .. <maxnoofTAforMDT> |  |  | - | - |
| >>>TAI | M |  | 9.2.3.16 |  | - | - |
| CHOICE MDT Mode | M |  |  |  | - | - |
| >Immediate MDT |  |  |  |  |  | - |
| >>Measurements to Activate | M |  | BITSTRING (SIZE(8)) | Each position in the bitmap indicates a MDT measurement, as defined in TS 37.320 [31]. First Bit = M1, Second Bit= M2, Third Bit = M3, Fourth Bit = M4, Fifth Bit = M5, Sixth Bit = logging of M1 from event triggered measurement reports according to existing RRM configuration, Seventh Bit = M6, Eighth Bit = M7. Value “1” indicates “activate” and value “0” indicates “do not activate”. | - | - |
| >>M1 Reporting Trigger | M |  | ENUMERATED (periodic, A2event-triggered, …, A2event-triggered periodic) | This IE shall be ignored if the Measurements to Activate IE has the first bit set to “0”. | - | - |
| >>M1 Threshold Event A2 | C-ifM1A2trigger |  |  | Included in case of event-triggered or event-triggered periodic reporting for measurement M1. | - | - |
| >>>CHOICE Threshold | M |  |  |  | - | - |
| >>>>RSRP |  |  |  |  |  | - |
| >>>>>Threshold RSRP | M |  | INTEGER (0..97) | Corresponds to the thresholdRSRP in the ThresholdEUTRA IE as defined in TS 36.331 [16]. | - | - |
| >>>>RSRQ |  |  |  |  |  | - |
| >>>>>Threshold RSRQ | M |  | INTEGER (0..34) | Corresponds to the thresholdRSRQ in the ThresholdEUTRA IE as defined in TS 36.331 [16]. | - | - |
| >>M1 Periodic reporting | C-ifperiodicMDT |  |  | Included in case of periodic or event-triggered periodic reporting for measurement M1. | - | - |
| >>>Report interval | M |  | ENUMERATED (ms120, ms240, ms480, ms640, ms1024, ms2048, ms5120, ms10240, min1, min6, min12, min30, min60) | Corresponds to the ReportInterval IE as is defined in TS 36.331 [16]. | - | - |
| >>>Report amount | M |  | ENUMERATED (1, 2, 4, 8, 16, 32, 64, infinity) | Number of reports. | - | - |
| >>M3 Configuration | C-ifM3 |  | 9.2.1.86 |  | YES | ignore |
| >>M4 Configuration | C-ifM4 |  | 9.2.1.87 |  | YES | ignore |
| >>M5 Configuration | C-ifM5 |  | 9.2.1.88 |  | YES | ignore |
| >>MDT Location Information | O |  | BITSTRING(SIZE(8)) | Each position in the bitmap represents requested location information as defined in TS 37.320 [31]. First Bit = GNSS Second Bit = E-CID information. Other bits are reserved for future use and are ignored if received. Value “1” indicates “activate” and value “0” indicates “do not activate”. The eNB shall ignore the first bit unless the Measurements to Activate IE has the first bit or the sixth bit set to “1”. | YES | ignore |
| >>M6 Configuration | C-ifM6 |  | 9.2.1.101 |  | YES | ignore |
| >>M7 Configuration | C-ifM7 |  | 9.2.1.102 |  | YES | ignore |
| >>Bluetooth Measurement Configuration | O |  | 9.2.1.137 |  | YES | ignore |
| >>WLAN Measurement Configuration | O |  | 9.2.1.138 |  | YES | ignore |
| >>Sensor Measurement Configuration | O |  | 9.2.1.166 |  | YES | ignore |
| >Logged MDT |  |  |  |  |  | - |
| >>Logging interval | M |  | ENUMERATED (ms1280, ms2560, ms5120, ms10240, ms20480, ms30720, ms40960 and ms61440) | Corresponds to the LoggingInterval IE as defined in TS 36.331 [16]. | - | - |
| >>Logging duration | M |  | ENUMERATED (m10, m20, m40, m60, m90 and m120) | Corresponds to the LoggingDuration IE as defined in TS 36.331 [16]. | - | - |
| >>Bluetooth Measurement Configuration | O |  | 9.2.1.137 |  | YES | ignore |
| >>WLAN Measurement Configuration | O |  | 9.2.1.138 |  | YES | ignore |
| >>CHOICE Report Type | O |  |  |  | YES | ignore |
| >>>Periodical |  |  | NULL |  |  |  |
| >>>Event Triggered |  |  |  |  |  |  |
| >>>>Event Trigger Logged MDT Configuration | M |  | 9.2.1.165 |  |  |  |
| >>Sensor Measurement Configuration | O |  | 9.2.1.166 |  | YES | ignore |
| >Logged MBSFN MDT |  |  |  |  | YES | ignore |
| >>Logging interval | M |  | ENUMERATED (ms1280, ms2560, ms5120, ms10240, ms20480, ms30720, ms40960 and ms61440) | Corresponds to the LoggingInterval IE as defined in TS 36.331 [16]. | - | - |
| >>Logging duration | M |  | ENUMERATED (m10, m20, m40, m60, m90 and m120) | Corresponds to the LoggingDuration IE as is defined in TS 36.331 [16]. | - | - |
| >>MBSFN-ResultToLog | O |  | MBSFN-ResultToLog 9.2.1.94 |  | - | - |
| Signalling based MDT PLMN List | O |  | MDT PLMN List 9.2.1.89 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellIDforMDT | Maximum no. of Cell ID subject for MDT scope. Value is 32. |
| maxnoofTAforMDT | Maximum no. of TA subject for MDT scope. Value is 8. |

| Condition | Explanation |
| --- | --- |
| ifM1A2trigger | This IE shall be present if the Measurements to Activate IE has the first bit set to “1” and the M1 Reporting Trigger IE is set to the value “A2event-triggered” or “A2event-triggered periodic”. |
| ifperiodicMDT | This IE shall be present if the M1 Reporting Trigger IE is set to the value “periodic”, or “A2event-triggered periodic”. |
| ifM3 | This IE shall be present if the Measurements to Activate IE has the third bit set to “1”. |
| ifM4 | This IE shall be present if the Measurements to Activate IE has the fourth bit set to “1”. |
| ifM5 | This IE shall be present if the Measurements to Activate IE has the fifth bit set to “1”. |
| ifM6 | This IE shall be present if the Measurements to Activate IE has the seventh bit set to “1”. |
| ifM7 | This IE shall be present if the Measurements to Activate IE has the eighth bit set to “1”. |

next change

#### 9.2.1.101	M6 Configuration

This IE defines the parameters for M6 measurement collection.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| M6 Report Interval | M |  | ENUMERATED (ms1024, ms2048, ms5120, ms10240, …) |  | - |  |
| M6 Delay Threshold | C-ifUL |  | ENUMERATED (ms30, ms40, ms50, ms60, ms70, ms80, ms90, ms100, ms150, ms300, ms500, ms750, …) |  | - |  |
| M6 Links to log | M |  | ENUMERATED(uplink, downlink, both-uplink-and-downlink, …) |  | - |  |
| M6 Report Amount | O |  | ENUMERATED (1, 2, 4, 8, 16, 32, 64, infinity,…) |  | Yes | ignore |

| Condition | Explanation |
| --- | --- |
| ifUL | This IE shall be present if the M6 Links to log IE is set to the value “uplink” or “both-uplink-and-downlink”. |

next change

#### 9.2.3.26	SON Configuration Transfer

This IE contains the configuration information, used by e.g., SON functionality, and additionally includes the eNB identifier of the destination of this configuration information and the eNB identifier of the source of this information.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| SON Configuration Transfer |  |  |  |  |  |  |
| >Target eNB-ID | M |  |  |  |  |  |
| >>Global eNB ID | M |  | 9.2.1.37 |  |  |  |
| >>Selected TAI | M |  | TAI 9.2.3.16 |  |  |  |
| >Source eNB-ID | M |  |  |  |  |  |
| >>Global eNB ID | M |  | 9.2.1.37 |  |  |  |
| >>Selected TAI | M |  | TAI 9.2.3.16 |  |  |  |
| >SON Information | M |  | 9.2.3.27 |  |  |  |
| >X2 TNL Configuration Info | C-ifSONInformationRequest |  | 9.2.3.29 | Source eNB X2 TNL Configuration Info. | YES | ignore |
| >Synchronisation Information | C-if Activate Muting |  | 9.2.3.42 | Information on cell selected as source of synchronisation and aggressor cells. | YES | ignore |

| Condition | Explanation |
| --- | --- |
| ifSONInformationRequest | This IE shall be present if the SON Information IE contains the SON Information Request IE set to the value “X2TNL Configuration Info” |
| ifActivateMuting | This IE shall be present if the SON Information IE contains the SON Information Request IE set to the value “Activate Muting” |

#### 9.2.3.26a	EN-DC SON Configuration Transfer

This IE contains the configuration information, used by SON functionality for EN-DC for communication between a destination (target) en-gNB and a source eNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| EN-DC SON Configuration Transfer |  |  |  |  |
| >CHOICE Transfer Type | M |  |  |  |
| >>Request |  |  |  | NOTE: Used if the SON Information IE indicates a request. |
| >>>Source eNB-ID |  | 1 |  |  |
| >>>>Global eNB ID | M |  | 9.2.1.37 |  |
| >>>>Selected TAI | M |  | TAI 9.2.3.16 |  |
| >>>Target en-gNB-ID |  | 1 |  |  |
| >>>>Global en-gNB ID | M |  | 9.2.1.37a |  |
| >>>>Selected TAI | M |  | TAI 9.2.3.16 | The MME may ignore this IE if any of the following IEs are present in the EN-DC SON Configuration Transfer IE: Target eNB-ID, Associated TAI, Broadcast 5GS TAI. NOTE: The Selected TAI is a 4G TAI and should, if available, correspond to the TAI configured at the en-gNB, however, this TAI is not broadcast by the respective NR cell and may not always be available from UE reporting TAIs of overlapping E-UTRA cells. |
| >>>Target eNB-ID |  | 0..1 |  |  |
| >>>>Global eNB ID | M |  | 9.2.1.37 |  |
| >>>>Selected TAI | M |  | TAI 9.2.3.16 |  |
| >>>Associated TAI | O |  | TAI 9.2.3.16 | A 4G TAI associated with the target en-gNB as specified in TS 36.300 [15]. |
| >>>Broadcast 5GS TAI | O |  | 5GS TAI 9.3.2.52 | A Broadcast 5GS TAI of the en-gNB as specified in TS 36.300 [15]. |
| >>Reply |  |  |  | NOTE: Used if the SON Information IE indicates a reply. |
| >>>Source en-gNB-ID |  | 1 |  |  |
| >>>>Global en-gNB ID | M |  | 9.2.1.37a |  |
| >>>>Selected TAI | M |  | TAI 9.2.3.16 | NOTE: The Selected TAI contains a configured TAC of the en-gNB. |
| >>>Target eNB-ID |  | 1 |  |  |
| >>>>Global eNB ID | M |  | 9.2.1.37 |  |
| >>>>Selected TAI | M |  | TAI 9.2.3.16 |  |
| >SON Information | M |  | 9.2.3.27 |  |
| >X2 TNL Configuration Info | C-ifSONInformationRequest |  | 9.2.3.29 | Source eNB X2 TNL Configuration Info. |

| Condition | Explanation |
| --- | --- |
| ifSONInformationRequest | This IE shall be present if the SON Information IE contains the SON Information Request IE set to the value “X2 TNL Configuration Info” |

next change

### 9.3.3	PDU Definitions

-- ASN1START

-- **************************************************************

--

-- PDU definitions for S1AP.

--

-- **************************************************************

** unmodified text skipped **

-- **************************************************************

--

##### -- Handover Required

--

-- **************************************************************

HandoverRequired ::= SEQUENCE {

protocolIEs			ProtocolIE-Container		{ { HandoverRequiredIEs} },

...

}

HandoverRequiredIEs S1AP-PROTOCOL-IES ::= {

{ ID id-MME-UE-S1AP-ID							CRITICALITY reject	TYPE MME-UE-S1AP-ID						PRESENCE mandatory}|

{ ID id-eNB-UE-S1AP-ID							CRITICALITY reject	TYPE ENB-UE-S1AP-ID						PRESENCE mandatory}|

{ ID id-HandoverType							CRITICALITY reject	TYPE HandoverType						PRESENCE mandatory}|

{ ID id-Cause									CRITICALITY ignore	TYPE Cause								PRESENCE mandatory}|

{ ID id-TargetID								CRITICALITY reject	TYPE TargetID							PRESENCE mandatory}|

{ ID id-Direct-Forwarding-Path-Availability		CRITICALITY ignore	TYPE Direct-Forwarding-Path-Availability		PRESENCE optional}|

{ ID id-SRVCCHOIndication						CRITICALITY reject	TYPE SRVCCHOIndication					PRESENCE optional}|

{ ID id-Source-ToTarget-TransparentContainer	CRITICALITY reject	TYPE Source-ToTarget-TransparentContainer	PRESENCE mandatory}|

{ ID id-Source-ToTarget-TransparentContainer-Secondary	CRITICALITY reject	TYPE Source-ToTarget-TransparentContainer	PRESENCE optional}|

{ ID id-MSClassmark2							CRITICALITY reject	TYPE MSClassmark2						PRESENCE conditional}|

--	The above IE shall be present if the Handover Type IE is set to the value “LTEtoGERAN” and the SRVCC HO Indication IE is present

{ ID id-MSClassmark3							CRITICALITY ignore	TYPE MSClassmark3						PRESENCE conditional}|

--	The above IE shall be present if the Handover Type IE is set to the value “LTEtoGERAN” and the SRVCC HO Indication IE is present

{ ID id-CSG-Id									CRITICALITY reject	TYPE CSG-Id								PRESENCE optional}|

{ ID id-CellAccessMode							CRITICALITY reject	TYPE CellAccessMode						PRESENCE optional}|

{ ID id-PS-ServiceNotAvailable					CRITICALITY ignore	TYPE PS-ServiceNotAvailable			PRESENCE optional},

...

}

-- **************************************************************

--

##### -- Handover Command

--

-- **************************************************************

HandoverCommand ::= SEQUENCE {

protocolIEs			ProtocolIE-Container		{ { HandoverCommandIEs} },

...

}

HandoverCommandIEs S1AP-PROTOCOL-IES ::= {

{ ID id-MME-UE-S1AP-ID							CRITICALITY reject	TYPE MME-UE-S1AP-ID							PRESENCE mandatory}|

{ ID id-eNB-UE-S1AP-ID							CRITICALITY reject	TYPE ENB-UE-S1AP-ID							PRESENCE mandatory}|

{ ID id-HandoverType							CRITICALITY reject	TYPE HandoverType							PRESENCE mandatory}|

{ ID id-NASSecurityParametersfromE-UTRAN		CRITICALITY reject	TYPE NASSecurityParametersfromE-UTRAN			PRESENCE conditional}|

-- The above IE shall be present if Handover Type IE is set to value "LTEtoUTRAN" or "LTEtoGERAN"

{ ID id-E-RABSubjecttoDataForwardingList		CRITICALITY ignore	TYPE E-RABSubjecttoDataForwardingList			PRESENCE optional}|

{ ID id-E-RABtoReleaseListHOCmd					CRITICALITY ignore	TYPE E-RABList								PRESENCE optional}|

{ ID id-Target-ToSource-TransparentContainer	CRITICALITY reject	TYPE Target-ToSource-TransparentContainer		PRESENCE mandatory}|

{ ID id-Target-ToSource-TransparentContainer-Secondary	CRITICALITY reject	TYPE Target-ToSource-TransparentContainer	PRESENCE optional}|

{ ID id-CriticalityDiagnostics					CRITICALITY ignore	TYPE CriticalityDiagnostics				PRESENCE optional},

...

}

E-RABSubjecttoDataForwardingList ::= E-RAB-IE-ContainerList { {E-RABDataForwardingItemIEs} }

E-RABDataForwardingItemIEs S1AP-PROTOCOL-IES ::= {

{ ID id-E-RABDataForwardingItem					CRITICALITY ignore	TYPE E-RABDataForwardingItem			PRESENCE mandatory	},

...

}

E-RABDataForwardingItem ::= SEQUENCE {

e-RAB-ID							E-RAB-ID,

dL-transportLayerAddress			TransportLayerAddress 													OPTIONAL,

dL-gTP-TEID							GTP-TEID 																OPTIONAL,

uL-TransportLayerAddress			TransportLayerAddress													OPTIONAL,

uL-GTP-TEID							GTP-TEID																OPTIONAL,

iE-Extensions						ProtocolExtensionContainer { { E-RABDataForwardingItem-ExtIEs} }			OPTIONAL,

...

}

E-RABDataForwardingItem-ExtIEs S1AP-PROTOCOL-EXTENSION ::= {

...

}

-- **************************************************************

--

##### -- Handover Preparation Failure

--

-- **************************************************************

HandoverPreparationFailure ::= SEQUENCE {

protocolIEs			ProtocolIE-Container		{ { HandoverPreparationFailureIEs} },

...

}

HandoverPreparationFailureIEs S1AP-PROTOCOL-IES ::= {

{ ID id-MME-UE-S1AP-ID					CRITICALITY ignore	TYPE MME-UE-S1AP-ID					PRESENCE mandatory	}|

{ ID id-eNB-UE-S1AP-ID					CRITICALITY ignore	TYPE ENB-UE-S1AP-ID					PRESENCE mandatory	}|

{ ID id-Cause							CRITICALITY ignore	TYPE Cause							PRESENCE mandatory	}|

{ ID id-CriticalityDiagnostics			CRITICALITY ignore	TYPE CriticalityDiagnostics			PRESENCE optional	},

...

}

-- **************************************************************

--

#### -- HANDOVER RESOURCE ALLOCATION ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- Handover Request

--

-- **************************************************************

HandoverRequest ::= SEQUENCE {

protocolIEs			ProtocolIE-Container		{ {HandoverRequestIEs} },

...

}

HandoverRequestIEs S1AP-PROTOCOL-IES ::= {

{ ID id-MME-UE-S1AP-ID							CRITICALITY reject	TYPE MME-UE-S1AP-ID						PRESENCE mandatory}|

{ ID id-HandoverType							CRITICALITY reject	TYPE HandoverType						PRESENCE mandatory}|

{ ID id-Cause									CRITICALITY ignore	TYPE Cause								PRESENCE mandatory}|

{ ID id-uEaggregateMaximumBitrate				CRITICALITY reject	TYPE UEAggregateMaximumBitrate			PRESENCE mandatory}|

{ ID id-E-RABToBeSetupListHOReq					CRITICALITY reject	TYPE E-RABToBeSetupListHOReq			PRESENCE mandatory}|

{ ID id-Source-ToTarget-TransparentContainer	CRITICALITY reject	TYPE Source-ToTarget-TransparentContainer	PRESENCE mandatory}|

{ ID id-UESecurityCapabilities					CRITICALITY reject	TYPE UESecurityCapabilities			PRESENCE mandatory}|

{ ID id-HandoverRestrictionList					CRITICALITY ignore	TYPE HandoverRestrictionList			PRESENCE optional}|

{ ID id-TraceActivation							CRITICALITY ignore	TYPE TraceActivation					PRESENCE optional}|

{ ID id-RequestType								CRITICALITY ignore	TYPE RequestType						PRESENCE optional}|

{ ID id-SRVCCOperationPossible					CRITICALITY ignore	TYPE SRVCCOperationPossible			PRESENCE optional}|

{ ID id-SecurityContext							CRITICALITY reject	TYPE SecurityContext					PRESENCE mandatory}|

{ ID id-NASSecurityParameterstoE-UTRAN			CRITICALITY reject	TYPE NASSecurityParameterstoE-UTRAN		PRESENCE conditional}|

-- The above IE shall be present if the Handover Type IE is set to the value "UTRANtoLTE" or "GERANtoLTE"

{ ID id-CSG-Id									CRITICALITY reject	TYPE CSG-Id								PRESENCE optional}|

{ ID id-CSGMembershipStatus						CRITICALITY ignore	TYPE CSGMembershipStatus				PRESENCE optional}|

{ ID id-GUMMEI-ID								CRITICALITY ignore	TYPE GUMMEI								PRESENCE optional}|

{ ID id-MME-UE-S1AP-ID-2						CRITICALITY ignore	TYPE MME-UE-S1AP-ID						PRESENCE optional}|

{ ID id-ManagementBasedMDTAllowed				CRITICALITY ignore	TYPE ManagementBasedMDTAllowed			PRESENCE optional}|

{ ID id-ManagementBasedMDTPLMNList				CRITICALITY ignore	TYPE MDTPLMNList						PRESENCE optional}|

{ ID id-Masked-IMEISV							CRITICALITY ignore	TYPE Masked-IMEISV						PRESENCE optional}|

{ ID id-ExpectedUEBehaviour						CRITICALITY ignore	TYPE ExpectedUEBehaviour				PRESENCE optional}|

{ ID id-ProSeAuthorized							CRITICALITY ignore	TYPE ProSeAuthorized					PRESENCE optional}|

{ ID id-UEUserPlaneCIoTSupportIndicator			CRITICALITY ignore	TYPE UEUserPlaneCIoTSupportIndicator 		PRESENCE optional}|

{ ID id-V2XServicesAuthorized					CRITICALITY ignore	TYPE V2XServicesAuthorized			PRESENCE optional}|

{ ID id-UESidelinkAggregateMaximumBitrate		CRITICALITY ignore	TYPE UESidelinkAggregateMaximumBitrate		PRESENCE optional}|

{ ID id-EnhancedCoverageRestricted				CRITICALITY ignore	TYPE EnhancedCoverageRestricted			PRESENCE optional}|

{ ID id-NRUESecurityCapabilities				CRITICALITY ignore	TYPE NRUESecurityCapabilities			PRESENCE optional}|

{ ID id-CE-ModeBRestricted						CRITICALITY ignore	TYPE CE-ModeBRestricted					PRESENCE optional}|

{ ID id-AerialUEsubscriptionInformation			CRITICALITY ignore	TYPE AerialUEsubscriptionInformation				PRESENCE optional}|

{ ID id-PendingDataIndication					CRITICALITY ignore	TYPE PendingDataIndication			PRESENCE optional}|

{ ID id-Subscription-Based-UE-DifferentiationInfo		CRITICALITY ignore	TYPE Subscription-Based-UE-DifferentiationInfo		PRESENCE optional}|

{ ID id-AdditionalRRMPriorityIndex			CRITICALITY ignore	TYPE AdditionalRRMPriorityIndex		PRESENCE optional }|

{ ID id-IAB-Authorized		 				CRITICALITY reject	TYPE IAB-Authorized						PRESENCE optional}|

{ ID id-NRV2XServicesAuthorized						CRITICALITY ignore	TYPE NRV2XServicesAuthorized							PRESENCE optional }|

{ ID id-NRUESidelinkAggregateMaximumBitrate			CRITICALITY ignore	TYPE NRUESidelinkAggregateMaximumBitrate			PRESENCE optional }|

{ ID id-PC5QoSParameters							CRITICALITY ignore	TYPE PC5QoSParameters									PRESENCE optional }|

{ ID id-UERadioCapabilityID							CRITICALITY reject	TYPE UERadioCapabilityID								PRESENCE optional},	...

}

E-RABToBeSetupListHOReq 					::= E-RAB-IE-ContainerList { {E-RABToBeSetupItemHOReqIEs} }

E-RABToBeSetupItemHOReqIEs S1AP-PROTOCOL-IES ::= {

{ ID id-E-RABToBeSetupItemHOReq				CRITICALITY reject	TYPE E-RABToBeSetupItemHOReq			PRESENCE mandatory	},

...

}

E-RABToBeSetupItemHOReq ::= SEQUENCE {

e-RAB-ID							E-RAB-ID,

transportLayerAddress				TransportLayerAddress,

gTP-TEID							GTP-TEID,

e-RABlevelQosParameters				E-RABLevelQoSParameters,

iE-Extensions						ProtocolExtensionContainer { {E-RABToBeSetupItemHOReq-ExtIEs} }		OPTIONAL,

...

}

E-RABToBeSetupItemHOReq-ExtIEs S1AP-PROTOCOL-EXTENSION ::= {

{ ID id-Data-Forwarding-Not-Possible		CRITICALITY ignore	EXTENSION Data-Forwarding-Not-Possible	PRESENCE optional}|

{ ID id-BearerType							CRITICALITY reject	EXTENSION BearerType					PRESENCE optional}|

{ ID id-Ethernet-Type						CRITICALITY ignore	EXTENSION Ethernet-Type					PRESENCE optional}|

{ ID id-SecurityIndication				CRITICALITY reject	EXTENSION SecurityIndication			PRESENCE optional},

...

}

** unmodified text skipped **

-- **************************************************************

--

##### -- Initial Context Setup Request

--

-- **************************************************************

InitialContextSetupRequest ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { {InitialContextSetupRequestIEs} },

...

}

InitialContextSetupRequestIEs S1AP-PROTOCOL-IES ::= {

{ ID id-MME-UE-S1AP-ID					CRITICALITY reject	TYPE MME-UE-S1AP-ID						PRESENCE mandatory}|

{ ID id-eNB-UE-S1AP-ID					CRITICALITY reject	TYPE ENB-UE-S1AP-ID						PRESENCE mandatory}|

{ ID id-uEaggregateMaximumBitrate		CRITICALITY reject	TYPE UEAggregateMaximumBitrate			PRESENCE mandatory}|

{ ID id-E-RABToBeSetupListCtxtSUReq		CRITICALITY reject	TYPE E-RABToBeSetupListCtxtSUReq		PRESENCE mandatory}|

{ ID id-UESecurityCapabilities			CRITICALITY reject	TYPE UESecurityCapabilities				PRESENCE mandatory}|

{ ID id-SecurityKey						CRITICALITY reject	TYPE SecurityKey						PRESENCE mandatory}|

{ ID id-TraceActivation					CRITICALITY ignore	TYPE TraceActivation					PRESENCE optional}|

{ ID id-HandoverRestrictionList			CRITICALITY ignore	TYPE HandoverRestrictionList			PRESENCE optional}|

{ ID id-UERadioCapability				CRITICALITY ignore	TYPE UERadioCapability					PRESENCE optional}|

{ ID id-SubscriberProfileIDforRFP		CRITICALITY ignore	TYPE SubscriberProfileIDforRFP			PRESENCE optional}|

{ ID id-CSFallbackIndicator				CRITICALITY reject	TYPE CSFallbackIndicator				PRESENCE optional}|

{ ID id-SRVCCOperationPossible			CRITICALITY ignore	TYPE SRVCCOperationPossible				PRESENCE optional}|

{ ID id-CSGMembershipStatus				CRITICALITY ignore	TYPE CSGMembershipStatus				PRESENCE optional}|

{ ID id-RegisteredLAI					CRITICALITY ignore	TYPE LAI								PRESENCE optional}|

{ ID id-GUMMEI-ID						CRITICALITY ignore	TYPE GUMMEI								PRESENCE optional}|

{ ID id-MME-UE-S1AP-ID-2				CRITICALITY ignore	TYPE MME-UE-S1AP-ID						PRESENCE optional}|

{ ID id-ManagementBasedMDTAllowed		CRITICALITY ignore	TYPE ManagementBasedMDTAllowed			PRESENCE optional}|

{ ID id-ManagementBasedMDTPLMNList		CRITICALITY ignore	TYPE MDTPLMNList						PRESENCE optional}|

{ ID id-AdditionalCSFallbackIndicator	CRITICALITY ignore	TYPE AdditionalCSFallbackIndicator	PRESENCE conditional}|

-- The above IE shall be present if the CS Fallback Indicator IE is set to the value “CS Fallback High Priority”

{ ID id-Masked-IMEISV					CRITICALITY ignore	TYPE Masked-IMEISV						PRESENCE optional}|

{ ID id-ExpectedUEBehaviour				CRITICALITY ignore	TYPE ExpectedUEBehaviour				PRESENCE optional}|

{ ID id-ProSeAuthorized					CRITICALITY ignore	TYPE ProSeAuthorized					PRESENCE optional}|

{ ID id-UEUserPlaneCIoTSupportIndicator	CRITICALITY ignore	TYPE UEUserPlaneCIoTSupportIndicator	PRESENCE optional}|

{ ID id-V2XServicesAuthorized			CRITICALITY ignore	TYPE V2XServicesAuthorized				PRESENCE optional}|

{ ID id-UESidelinkAggregateMaximumBitrate		CRITICALITY ignore	TYPE UESidelinkAggregateMaximumBitrate	PRESENCE optional}|

{ ID id-EnhancedCoverageRestricted		CRITICALITY ignore	TYPE EnhancedCoverageRestricted			PRESENCE optional}|

{ ID id-NRUESecurityCapabilities				CRITICALITY ignore	TYPE NRUESecurityCapabilities			PRESENCE optional}|

{ ID id-CE-ModeBRestricted				CRITICALITY ignore	TYPE CE-ModeBRestricted			PRESENCE optional}|

{ ID id-AerialUEsubscriptionInformation	CRITICALITY ignore	TYPE AerialUEsubscriptionInformation				PRESENCE optional}|

{ ID id-PendingDataIndication			CRITICALITY ignore	TYPE PendingDataIndication				PRESENCE optional}|

{ ID id-Subscription-Based-UE-DifferentiationInfo		CRITICALITY ignore	TYPE Subscription-Based-UE-DifferentiationInfo		PRESENCE optional}|

{ ID id-AdditionalRRMPriorityIndex		CRITICALITY ignore	TYPE AdditionalRRMPriorityIndex			PRESENCE optional }|

{ ID id-IAB-Authorized					CRITICALITY ignore	TYPE IAB-Authorized						PRESENCE optional}|

{ ID id-NRV2XServicesAuthorized						CRITICALITY ignore	TYPE NRV2XServicesAuthorized							PRESENCE optional }|

{ ID id-NRUESidelinkAggregateMaximumBitrate			CRITICALITY ignore	TYPE NRUESidelinkAggregateMaximumBitrate			PRESENCE optional }|

{ ID id-PC5QoSParameters							CRITICALITY ignore	TYPE PC5QoSParameters									PRESENCE optional }|

{ ID id-UERadioCapabilityID							CRITICALITY reject	TYPE UERadioCapabilityID								PRESENCE optional},

...

}

** unmodified text skipped **

-- **************************************************************

--

##### -- UE Context Modification Request

--

-- **************************************************************

UEContextModificationRequest ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { UEContextModificationRequestIEs} },

...

}

UEContextModificationRequestIEs S1AP-PROTOCOL-IES ::= {

{ ID id-MME-UE-S1AP-ID						CRITICALITY reject	TYPE MME-UE-S1AP-ID							PRESENCE mandatory}|

{ ID id-eNB-UE-S1AP-ID						CRITICALITY reject	TYPE ENB-UE-S1AP-ID							PRESENCE mandatory}|

{ ID id-SecurityKey							CRITICALITY reject	TYPE SecurityKey							PRESENCE optional}|

{ ID id-SubscriberProfileIDforRFP			CRITICALITY ignore	TYPE SubscriberProfileIDforRFP			PRESENCE optional}|

{ ID id-uEaggregateMaximumBitrate			CRITICALITY ignore	TYPE UEAggregateMaximumBitrate			PRESENCE optional}|

{ ID id-CSFallbackIndicator					CRITICALITY reject	TYPE CSFallbackIndicator					PRESENCE optional}|

{ ID id-UESecurityCapabilities				CRITICALITY reject	TYPE UESecurityCapabilities					PRESENCE optional}|

{ ID id-CSGMembershipStatus					CRITICALITY ignore	TYPE CSGMembershipStatus					PRESENCE optional}|

{ ID id-RegisteredLAI						CRITICALITY ignore	TYPE LAI									PRESENCE optional}|

{ ID id-AdditionalCSFallbackIndicator		CRITICALITY ignore	TYPE AdditionalCSFallbackIndicator		PRESENCE conditional}|

-- The above IE shall be present if the CS Fallback Indicator IE is set to the value “CS Fallback High Priority”

{ ID id-ProSeAuthorized						CRITICALITY ignore	TYPE ProSeAuthorized						PRESENCE optional}|

{ ID id-SRVCCOperationPossible				CRITICALITY ignore	TYPE SRVCCOperationPossible					PRESENCE optional}|

{ ID id-SRVCCOperationNotPossible			CRITICALITY ignore	TYPE SRVCCOperationNotPossible			PRESENCE optional}|

{ ID id-V2XServicesAuthorized				CRITICALITY ignore	TYPE V2XServicesAuthorized					PRESENCE optional}|

{ ID id-UESidelinkAggregateMaximumBitrate	CRITICALITY ignore	TYPE UESidelinkAggregateMaximumBitrate	PRESENCE optional}|

{ ID id-NRUESecurityCapabilities			CRITICALITY ignore	TYPE NRUESecurityCapabilities				PRESENCE optional}|

{ ID id-AerialUEsubscriptionInformation		CRITICALITY ignore	TYPE AerialUEsubscriptionInformation	PRESENCE optional}|

{ ID id-AdditionalRRMPriorityIndex			CRITICALITY ignore	TYPE AdditionalRRMPriorityIndex			PRESENCE optional}|

{ ID id-IAB-Authorized						CRITICALITY ignore	TYPE IAB-Authorized							PRESENCE optional}|

{ ID id-NRV2XServicesAuthorized						CRITICALITY ignore	TYPE NRV2XServicesAuthorized							PRESENCE optional }|

{ ID id-NRUESidelinkAggregateMaximumBitrate			CRITICALITY ignore	TYPE NRUESidelinkAggregateMaximumBitrate			PRESENCE optional }|

{ ID id-PC5QoSParameters							CRITICALITY ignore	TYPE PC5QoSParameters									PRESENCE optional }|

{ ID id-UERadioCapabilityID							CRITICALITY reject	TYPE UERadioCapabilityID								PRESENCE optional},

...

}

next change

### 9.3.4	Information Element Definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

** unmodified text skipped **

EN-DCSONConfigurationTransfer ::= SEQUENCE {

transfertype				EN-DCSONTransferType,

sONInformation					SONInformation,

x2TNLConfigInfo					X2TNLConfigurationInfo 	OPTIONAL,

-- The above IE shall be present if the SON Information IE contains the SON Information Request IE and the SON Information Request IE is set to the value "X2TNL Configuration Info"

iE-Extensions			ProtocolExtensionContainer { {EN-DCSONConfigurationTransfer-ExtIEs} }			OPTIONAL,

...

}

** unmodified text skipped **

ImmediateMDT ::= SEQUENCE {

measurementsToActivate		MeasurementsToActivate,

m1reportingTrigger			M1ReportingTrigger,

m1thresholdeventA2			M1ThresholdEventA2				OPTIONAL,

-- The above IE shall be present if the Measurements to Activate IE has the first bit set to “1” and the M1 Reporting Trigger IE is set to the value “A2event-triggered” or to “A2event-triggered periodic”

m1periodicReporting			M1PeriodicReporting				OPTIONAL,

-- The above IE shall be present if the M1 Reporting Trigger IE is set to the value “periodic”, or “A2event-triggered periodic”

iE-Extensions				ProtocolExtensionContainer { { ImmediateMDT-ExtIEs} } OPTIONAL,

...

}

ImmediateMDT-ExtIEs S1AP-PROTOCOL-EXTENSION ::= {

{ ID id-M3Configuration		CRITICALITY ignore	EXTENSION M3Configuration		PRESENCE conditional}|

{ ID id-M4Configuration		CRITICALITY ignore	EXTENSION M4Configuration		PRESENCE conditional}|

{ ID id-M5Configuration		CRITICALITY ignore	EXTENSION M5Configuration		PRESENCE conditional}|

{ ID id-MDT-Location-Info	CRITICALITY ignore	EXTENSION MDT-Location-Info		PRESENCE optional}|

{ ID id-M6Configuration		CRITICALITY ignore	EXTENSION M6Configuration		PRESENCE conditional}|

{ ID id-M7Configuration		CRITICALITY ignore	EXTENSION M7Configuration		PRESENCE conditional}|

{ ID id-BluetoothMeasurementConfiguration		CRITICALITY ignore	EXTENSION BluetoothMeasurementConfiguration		PRESENCE optional}|

{ ID id-WLANMeasurementConfiguration			CRITICALITY ignore	EXTENSION WLANMeasurementConfiguration			PRESENCE optional}|

{ ID id-SensorMeasurementConfiguration			CRITICALITY ignore	EXTENSION SensorMeasurementConfiguration			PRESENCE optional},

...

}

** unmodified text skipped **

M6Configuration ::= SEQUENCE {

m6report-Interval	M6report-Interval,

m6delay-threshold	M6delay-threshold		OPTIONAL,

-- The above IE shall be present if the M6 Links to log IE is set to the value "uplink" or to "both-uplink-and-downlink"

m6-links-to-log		Links-to-log,

iE-Extensions		ProtocolExtensionContainer { { M6Configuration-ExtIEs} } OPTIONAL,

...

}

M6Configuration-ExtIEs S1AP-PROTOCOL-EXTENSION ::= {

{ ID id-M6ReportAmount		CRITICALITY ignore	EXTENSION M6ReportAmountMDT		PRESENCE optional	},

...

}

** unmodified text skipped **

SONConfigurationTransfer-ExtIEs S1AP-PROTOCOL-EXTENSION ::= {

-- Extension for Release 10 to transfer the IP addresses of the eNB initiating the ANR action --

{ID id-x2TNLConfigurationInfo		CRITICALITY ignore	EXTENSION X2TNLConfigurationInfo				PRESENCE conditional}|

-- The above IE shall be present if the SON Information IE contains the SON Information Request IE and the SON Information Request IE is set to the value "X2TNL Configuration Info"

-- Extension for Release 12 to transfer information concerning the source cell of synchronisation and the aggressor cell --

{ID id-Synchronisation-Information	CRITICALITY ignore	EXTENSION SynchronisationInformation			PRESENCE conditional},

-- The above IE shall be present if the SON Information IE contains the SON Information Request IE set to the value " Activate Muting "

...

}

end of changes