3GPP TSG-RAN WG3 Meeting #124	R3-243076

Fukuoka, Japan, 20 – 24 May, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.483 | CR | 0111 | rev | 3 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of handling GTP-U Error Indication |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Nokia, Ericsson, ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core, TEI18 |  |  |  |  |  | Date: |  |  | 2024-05-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Per C4-235602, it was agreed that when the 5G-AN receives a GTP-U Error Indication over a NG-U tunnel that is not an indirect forwarding tunnel, the 5G-AN shall initiate a PDU Session Resource Notify procedure, and indicate the release is due to receiving a GTP-U Error Indication (also see the agreed CR in R3-241117). In the CU-CP/CU-UP split architecture, it is the gNB-CU-UP that receives a GTP-U Error Indication over a NG-U tunnel. Then the gNB-CU-UP should report to the gNB-CU-CP so that the gNB-CU CP can inform the SMF. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce the User Plane Error Indicator when the gNB-CP-UP receives a GTP-U Error Indication over a NG-U tunnel. Impact Analysis: Impact assessment towards the previous version of the specification (same release): This CR has an isolated impact towards the previous version of the specification (same release) because it has an impact on the PDU Session Resource To Remove List initiated by the gNB-CU UP when the gNB-CU-UP receives a GTP-UE error indication over a NG-U tunnel. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The gNB-CU-CP cannot inform the SMF that the PDU session release is requested due to receiving a GTP-U Error Indication over NG-U. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 8.3.3.2, 9.3.3.12, 9.4.5, 9.4.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specification |  |  |  | TS 38.423 CR 1249 |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Initial version: R3-240379 Rev1: R3-241666 Make updates to introduce an explicit indicator IE, instead of cause value. Update the cover page e.g., title, other specs affected etc. Rev2: R3-242184 Update the co-source. Rev3: R3-243076 Resubmission to RAN3-124 meeting. |  |  |  |  |  |  |  |  |

Change Begins

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

[34]	3GPP TS 36.323: " Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Packet Data Convergence Protocol (PDCP) specification".

[35]	3GPP TS 29.571: "5G System; Common Data Types for Service Based Interfaces; Stage 3".

[xx]	3GPP TS 23.527: "5G System; Restoration procedures”.

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

### 8.3.3	Bearer Context Modification Required (gNB-CU-UP initiated)

#### 8.3.3.1	General

The purpose of the Bearer Context Modification Required procedure is to allow the gNB-CU-UP to modify a bearer context (e.g., due to local problems) and inform the gNB-CU-CP. The procedure uses UE-associated signalling.

#### 8.3.3.2	Successful Operation

Figure 8.3.3.2-1: Bearer Context Modification Required procedure: Successful Operation.

The gNB-CU-UP initiates the procedure by sending the BEARER CONTEXT MODIFICATION REQUIRED message to the gNB-CU-CP. The gNB-CU-CP replies with the BEARER CONTEXT MODIFICATION CONFIRM message.

If the S1 DL UP Transport Layer Information IE or the NG DL UP Transport Layer Information IE or the Redundant NG DL UP Transport Layer Information IE is contained in the BEARER CONTEXT MODIFICATION REQUIRED message, the gNB-CU-CP shall update the corresponding information.

If the gNB-CU-UP Cell Group Related Configuration IE is contained in the DRB To Modify List IE in the BEARER CONTEXT MODIFICATION REQUIRED message, the gNB-CU-CP shall try to change the cell group related configuration accordingly. If the gNB-CU-CP is not able to update the requested cell group related configuration, it shall include the Cell Group Information IE with the current cell group configuration in the DRB Modified List IE in the BEARER CONTEXT MODIFICATION CONFIRM message.

If the User Plane Error Indicator IE is included in the PDU Session Resource To Remove List IE and is set to “GTP-U Error Indication Received” in the BEARER CONTEXT MODIFICATION REQUIRED message, the gNB-CU-CP shall, if supported, consider that the PDU session resource is released due to GTP-U Error Indication received over the NG-U tunnel and inform the SMF as specified in TS 23.527 [xx].

<<<<<<<<<<<<<<<<<<<< For Information Only >>>>>>>>>>>>>>>>>>>>

#### 9.2.2.7	BEARER CONTEXT MODIFICATION REQUIRED

This message is sent by the gNB-CU-UP to inform the gNB-CU-CP that a modification of a bearer context is required (e.g., due to local problems at the gNB-CU-UP).

Direction: gNB-CU-UP  gNB-CU-CP

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU-CP UE E1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-CU-UP UE E1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| CHOICE System | M |  |  |  | YES | reject |
| >E-UTRAN |  |  |  |  |  |  |
| >>DRB To Modify List | O |  | DRB Required To Modify List E-UTRAN 9.3.3.21 |  | YES | reject |
| >>DRB To Remove List | O |  | DRB Required To Remove List E-UTRAN 9.3.3.22 |  | YES | reject |
| >NG-RAN |  |  |  |  |  |  |
| >>PDU Session Resource Required To Modify List | O |  | 9.3.3.23 |  | YES | reject |
| >>PDU Session Resource To Remove List | O |  | 9.3.3.12 |  | YES | reject |

| Range bound | Explanation |
| --- | --- |
| maxnoofDRBs | Maximum no. of DRBs for a UE. Value is 32. |
| maxnoofPDUSessionResource | Maximum no. of PDU Sessions for a UE. Value is 256. |

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

#### 9.3.3.12	PDU Session Resource To Remove List

This IE contains PDU session resource to remove related information

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| PDU Session Resource To Remove Item |  | 1..<maxnoofPDUSessionResource> |  |  | - | - |
| >PDU Session ID | M |  | 9.3.1.21 |  | - | - |
| >Cause | O |  | 9.3.1.2 |  | YES | ignore |
| >User Plane Error Indicator | O |  | ENUMERATED(GTP-U Error Indication Received, …) |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofPDUSessionResource | Maximum no. of PDU Sessions for a UE. Value is 256. |

### 9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

#### -- Information Element Definitions

--

-- **************************************************************

E1AP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) e1ap (5) version1 (1) e1ap-IEs (2) }

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

id-F1U-TNL-InfoAdded-List,

id-F1U-TNL-InfoToAddOrModify-List,

id-F1U-TNL-InfoAddedOrModified-List,

id-F1U-TNL-InfoToRelease-List,

id-BroadcastF1U-ContextReferenceE1,

id-PSIbasedDiscardTimer,

id-UserPlaneErrorIndicator,

maxnoofMBSAreaSessionIDs,

maxnoofSharedNG-UTerminations,

maxnoofMRBs,

maxnoofMBSSessionIDs,

maxnoofQoSParaSets,

maxnoofErrors,

maxnoofSliceItems,

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

PDU-Session-Resource-To-Modify-Item-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

{ID id-SNSSAI										CRITICALITY reject	EXTENSION SNSSAI												PRESENCE optional}|

{ID id-CommonNetworkInstance						CRITICALITY ignore	EXTENSION CommonNetworkInstance								PRESENCE optional	}|

{ID id-redundant-nG-UL-UP-TNL-Information			CRITICALITY ignore	EXTENSION 	UP-TNL-Information								PRESENCE optional	}|

{ID id-RedundantCommonNetworkInstance				CRITICALITY ignore	EXTENSION 	CommonNetworkInstance							PRESENCE optional	}|

{ID id-DataForwardingtoE-UTRANInformationList		CRITICALITY ignore	EXTENSION 	DataForwardingtoE-UTRANInformationList	PRESENCE optional	}|

{ID id-SecurityIndicationModify						CRITICALITY ignore	EXTENSION 	SecurityIndication								PRESENCE optional	}|

{ID id-Secondary-PDU-Session-Data-Forwarding-Information	CRITICALITY ignore	EXTENSION 	Data-Forwarding-Information		PRESENCE optional	},

...

}

PDU-Session-Resource-To-Remove-List	::= SEQUENCE (SIZE(1.. maxnoofPDUSessionResource)) OF PDU-Session-Resource-To-Remove-Item

PDU-Session-Resource-To-Remove-Item	::=	SEQUENCE {

pDU-Session-ID							PDU-Session-ID,

iE-Extensions							ProtocolExtensionContainer	{ { PDU-Session-Resource-To-Remove-Item-ExtIEs } }	OPTIONAL,

...

}

PDU-Session-Resource-To-Remove-Item-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

{ID id-Cause		CRITICALITY ignore	EXTENSION Cause		PRESENCE optional}|

{ID id-UserPlaneErrorIndicator		CRITICALITY ignore	EXTENSION UserPlaneErrorIndicator		PRESENCE optional},

...

}

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

UplinkOnlyROHC ::= SEQUENCE {

maxCID							INTEGER (0..16383, ...),

rOHC-Profiles					INTEGER (0..511, ...),

continueROHC					ENUMERATED {true, ...}	OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { UplinkOnlyROHC-ExtIEs } } 		OPTIONAL

}

UplinkOnlyROHC-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

URIaddress ::= VisibleString

UserPlaneErrorIndicator ::= ENUMERATED {

gTP-U-error-indication-received,

...

}

UEInactivityInformation ::= INTEGER (1..7200, ...)

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

### 9.4.7	Constant Definitions

-- ASN1START

-- **************************************************************

--

#### -- Constant definitions

--

-- **************************************************************

E1AP-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) e1ap (5) version1 (1) e1ap-Constants (4) }

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

id-F1U-TNL-InfoToAddOrModify-List								ProtocolIE-ID ::= 210

id-F1U-TNL-InfoAddedOrModified-List								ProtocolIE-ID ::= 211

id-F1U-TNL-InfoToRelease-List									ProtocolIE-ID ::= 212

id-BroadcastF1U-ContextReferenceE1								ProtocolIE-ID ::= 213

id-PSIbasedDiscardTimer											ProtocolIE-ID ::= 214

id-UserPlaneErrorIndicator										ProtocolIE-ID ::= aaa

Change Ends