3GPP TSG-RAN WG3 Meeting #124	R3-243037

# Fukuoka, Japan, 20 – 24 May, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.473 | CR | 1377 | rev | 1 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of Time Reference Information |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Nokia, ZTE, Ericsson, Deutsche Telekom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_IIOT-Core |  |  |  |  |  | Date: |  |  | 2024-05-10 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In the Reference Time Information Report message, the Time Reference Information IE is included to indicate the time reference information, wherein it can be observed that the presence for the Uncertainty IE and the Time Information Type IE are optional in the Tabular, but are mandatory in the ASN.1. As specified in TS 38.331 below, both the uncertainty and the timeInfoType IEs are optional. ReferenceTimeInfo-r16 ::= SEQUENCE { time-r16 ReferenceTime-r16, uncertainty-r16 INTEGER (0..32767) OPTIONAL, -- Need S timeInfoType-r16 ENUMERATED {localClock} OPTIONAL, -- Need S referenceSFN-r16 INTEGER (0..1023) OPTIONAL -- Cond RefTime } Note that the presence of the Reference SFN IE in the Time Reference Information IE is correct since the gNB-CU should always include the referenceSFN and send to UE in the RRC DLInformationTransfer message. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Update the presence of the uncertainty and the timeInformationType to be optional in ASN.1. - Update the semantic descriptions for the Reference SFN, Uncertainty and Time Information Type IEs. Impact Analysis: Impact assessment towards the previous version of the specification (same release): This CR has isolated impact with the previous version of the specification (same release) because it impacts the Time Reference Information when the Reference Time Information Report procedure is enabled. This CR is NBC. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect delivery of the Uncertainty and Time Information Type IEs to the UE when these two IEs are not needed. Misalignment between the ASN.1 and the Tabular. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.3.1.148, 9.3.1.149, 9.4.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Initial version: R3-241689 Rev1: R3-243037 Resubmission to RAN3-124 meeting. |  |  |  |  |  |  |  |  |

CHANGES START

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

#### 9.3.1.148	Time Reference Information

This IE contains the time reference information.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Reference Time | M |  | 9.3.1.149 |  |
| Reference SFN | M |  | INTEGER (0..1023) | Corresponds to the referenceSFN contained in the ReferenceTimeInfo IE, as defined in TS 38.331 [8] |
| Uncertainty | O |  | INTEGER (0..32767, …) | Corresponds to the uncertainty contained in the ReferenceTimeInfo IE, as defined in TS 38.331 [8]. |
| Time Information Type | O |  | ENUMERATED (localClock) | Corresponds to the timeInfoType contained in the ReferenceTimeInfo IE, as defined in TS 38.331 [8]. |

#### 9.3.1.149	Reference Time

This IE provides the accurate Reference Time information.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Reference Time | M |  | OCTET STRING | Includes the ReferenceTime IE contained in the ReferenceTimeInfo IE as defined in TS 38.331 [8]. |

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

### 9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

F1AP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) f1ap (3) version1 (1) f1ap-IEs (2) }

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

TimeReferenceInformation ::= SEQUENCE {

referenceTime					ReferenceTime,

referenceSFN					ReferenceSFN,

uncertainty						Uncertainty				OPTIONAL,

timeInformationType				TimeInformationType		OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {TimeReferenceInformation-ExtIEs} }	OPTIONAL

}

TimeReferenceInformation-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

TimeInformationType ::= ENUMERATED {localClock}

TimeStamp ::= SEQUENCE {

systemFrameNumber		SystemFrameNumber,

slotIndex				TimeStampSlotIndex,

measurementTime			RelativeTime1900	OPTIONAL,

iE-Extension			ProtocolExtensionContainer { { TimeStamp-ExtIEs} }	OPTIONAL

}

TimeStamp-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

CHANGES END