3GPP TSG RAN Meeting #104                                                  RP-241132

Shanghai, China, June 17-20, 2024

Agenda Item:	9.1.3.5

Source:	Spreadtrum Communications

Title: 	Considerations on enhancements of Network energy savings for NR

Document for:	Discussion and decision

# Introduction

The paper discusses some sub-topics for R19 eNES.

# On-demand SIB1 for idle/inactive UEs

In RAN1#117, evaluation results are collected and some observations are drawn.

According to evaluation results and observations, the energy saving gain for NES cell is promising in most cases (e.g., SIB1 periodicity is 20ms or 40ms, SSB and SIB1 PDSCH are TDMed), and the energy saving loss for cell A is trivial in most case (e.g., on-demand SIB1 rate is larger than 30%).

According to WID, whether on-demand SIB1 for idle/inactive UEs enters normative work needs to be confirmed in September RAN plenary.

Therefore, we have the following observation:

Observation 1: According to RAN1 evaluations, the NES gain of on-demand SIB1 for idle/inactive UEs is justified.

Although according to R19 eNES WID [1], whether on-demand SIB1 for idle/inactive UEs enters normative work will be checked in RAN#105, this checkpoint can be done in advance at RAN#104 as RAN1 has finished the evaluation. The earlier to enter into normative work will be benefit to have a robust R19 eNES specification. In addition, solving this issue in RAN#104 can save more time for RAN#105 to discuss other issues.

- Study procedures and signaling method(s) to support on-demand SIB1 for UEs in idle/inactive mode, including: [RAN1/2/3] - Checkpoint for normative work in RAN#105

Proposal 1: Confirm to specify procedures and signaling method(s) to support on-demand SIB1 for UEs in idle/inactive mode, and do the relative revision to R19 eNES WID.

# SSB adaptation in time domain

SSB adaptation in time domain was widely discussed in RAN1, but it was controversial that whether SSB periodicity is larger than 20ms is feasible for UEs. If it is not feasible that SSB periodicity is larger than 20ms, SSB adaptation in time domain is meaningless for PCell or cell (re-)selected by idle UEs.

Observation x: If it is not feasible that SSB periodicity is larger than 20ms, SSB adaptation in time domain is meaningless for PCell or cell (re-)selected by idle UEs.

## Impact to legacy UEs

As shown in Appendix-2, we think impact to legacy UEs is trivial.

## Evaluation results in SI

Since RAN1 had agreement that baseline is adaptation of SSB periodicity, we check the following evaluation results in SI shown in Appendix-1. We observe that most cases with promising NES gain have assumptions of zero (empty) or low load. In these cases, we focused on SSB adaptation in a PCell or a cell (re-)selected by idle UEs due to zero or low load. It is not reasonable to assume SCell usage in zero or low load.

Observation 2: In SI evaluations, we focused on SSB adaptation in a PCell or a cell (re-)selected by idle UEs due to zero or low load assumption in most cases with promising NES gain.

## About “greenfield” in SID

In R18 NES SID [2], it mentioned that some techniques can be deployed in “greenfield”, which does not need consider backward compatibility in some cases.

Note 1: legacy UEs should be able to continue accessing a network implementing Rel-18 network energy savings techniques, with the possible exception of techniques developed specifically for greenfield deployments.

From perspective of commercialization, R19 eNES-capable UEs will be in market for long time, since 5G/6G may coexist for long time, like 4G/5G.

Observation 3: In R18 NES SID, some techniques can be deployed in “greenfield”, which does not need consider backward compatibility in some cases.

## Summary

According to above observations, for R19 eNES WID, we suggest that we should assume some cells only allow R19 eNES-capable UEs to camp on or access, and the according note can be added in WID.

Proposal 2: For R19 eNES WID, a note can be added:  For SSB adaptation in time domain, it does not exclude that some cells only allow R19 eNES-capable UEs to camp on or access.

# Conclusion

We have the following proposals.

Proposal 1: Confirm to specify procedures and signaling method(s) to support on-demand SIB1 for UEs in idle/inactive mode, and do the relative revision to R19 eNES WID.

Proposal 2: For R19 eNES WID, a note can be added:  For SSB adaptation in time domain, it does not exclude that some cells only allow R19 eNES-capable UEs to camp on or access.

# Reference

- RP-240170, “Revised WID for Enhancements of network energy savings for NR”, Ericsson, RAN#103, Mar.  18th – 21st, 2024.

- RP-221443, “Revised SI: Study on network energy savings for NR”, Huawei, RAN#96, June  6th – 9th, 2023.

# Appendix-1: Evaluations results for adaptation of SSB periodicity

Evaluation results for technique A-1-3 are shown as follow. It can be observed that most cases with promising NES gain have zero (empty) or low load.

Table 6.1.1.2-3: BS energy savings by adapting SSB/SIB1 periodicities

| Company | ES scheme | BS Category | Load scenario | ES gain (%) | UPT/Access delay/latency/UE power consumption, etc. (loss w.r.t. baseline) | Baseline configuration/assumption/Other notable setting |
| --- | --- | --- | --- | --- | --- | --- |
| CMCC [23] | SSB periodicity 20ms, SIB repetition period 40ms. | cat.2 | Zero | 13.7% |  | Baseline: normal SSB/SIB1 transmission, with 20ms repetition period for both. |
|  | SSB and SIB1 repetition period 40ms. |  |  | 17.6% |  |  |
|  | SSB periodicity 20ms, SIB repetition period 40ms. | cat.1 | Zero | 25.7% |  |  |
|  | SSB and SIB1 repetition period 40ms. |  |  | 28.7% |  |  |
| vivo [10] [20] | Period adaptation of common signals and channels (ES scheme: 160ms SSB and SIB1, 160ms RACH listening) | Cat1 | Zero | 78.8% | UE power consumption: 0% | Baseline scheme: 20ms SSB and SIB1, 20ms RACH listening |
|  |  | Cat2 |  | 16.6% | UE power consumption: 0% |  |
| NOKIA/NSB [12] | SSB/SIB1/RO monitoring period= 160ms | Cat 2 | Zero, Low, Light, Medium | 48.4%, 44.3%, 43.7%, 39.9% | UPT: 0 Mbps, 83 Mbps, 70 Mbps, 55 Mbps | SSB/SIB1/random-access occasion (RO) monitoring periodicity @ 20ms UEs are initially in RRC_idle state |
|  | SSB/SIB1/RO monitoring period= 640ms |  | Zero, Low, Light, Medium | 53.6%, 49.0%, 48.8%, 46.1% | UPT: 0 Mbps, 29 Mbps, 27 Mbps, 25 Mbps |  |
|  | SSB/SIB1/RO monitoring period= 1280ms |  | Zero, Low, Light, Medium | 83.6%, 51.3%, 51.7%, 50.6% | UPT: 0 Mbps, 11.2 Mbps, 11 Mbps, 10.5 Mbps |  |
| Spreadtrum [13] | Prolonging the periodicity of SSB/SIB1/paging: 1) SSB burst periodicity is 160ms, and SIB1 repetition periodicity is 160ms. 2) PF periodicity at gNB side is 160ms (T=1280ms, N=8). 3) gNB can enter light sleep for Cat 1, but can only enter micro sleep for Cat 2. | Cat 1 | Zero | Set 1- Set 3: 23.8%, 19.6%, 16.3% |  | 1) SSB burst periodicity is 20ms, and SIB1 repetition periodicity is 20ms. 2) PF periodicity at gNB side is 20ms (T=1280ms, N=64). 3) gNB can enter light sleep for Cat 1, but can only enter micro sleep for Cat 2. |
|  |  | Cat 2 |  | Set 1- Set 3: 9.3%, 8.3%, 9.4% |  |  |
|  | Transmission window of SSB/SIB1/paging: 1) SSB burst periodicity is 20ms, and SIB1 repetition periodicity is 20ms. 2) PF periodicity at gNB side is 20ms (T=1280ms, N=64). 3) gNB can enter light sleep for Cat 1, but can only enter micro sleep for Cat 2. | Cat 1 |  | Set 1- Set 3: 23.8%, 19.6%, 16.3% |  | 1) The transmission window periodicity is 1280ms, and the transmission window duration is 160ms. 2) SSB burst periodicity is 20ms within the transmission window, and SIB1 repetition periodicity is 20ms within the transmission window. 3) PF periodicity at gNB side is 160ms (T=1280ms, N=8) within the transmission window. 4) gNB can enter light sleep for Cat 1, and can enter both light sleep and micro sleep for Cat 2 (at the tail of the transmission window). |
|  |  | Cat 2 |  | Set 1- Set 3: 51.5%, 47.3%, 20.6% |  |  |
| Intel [22] | Increasing the common channel/signal periodicity | Cat 1 | Low | 40.1% | UPT: 819.66 Mbps Avg EE* (baseline): 5.10 Avg EE (ES scheme): 9.17 | Baseline: SSB/PRACH: 20 msec periodicity; SIB periodicity 40ms ES scheme: SSB/SIB1/PRACH: 160 msec periodicity. EE* is defined as cell throughput (in Mbps) / average power consumption (in relative power), and averaged from all BS. |
|  |  |  |  | 45.0% | UPT: 819.66 Mbps Avg EE (baseline): 5.10 Avg. EE (ES scheme): 10.60 | Baseline: SSB/PRACH: 20 msec periodicity; SIB periodicity 40ms ES scheme: SSB/SIB1/PRACH: 640 msec periodicity |
|  |  |  | Light | 14.6% | UPT: 611.45Mbps Avg EE (baseline): 2.66 Avg. EE (ES scheme): 3.31 | Baseline: SSB/PRACH: 20 msec periodicity; SIB periodicity 40ms ES scheme: SSB/SIB1/PRACH: 160 msec periodicity |
|  |  |  |  | 16.8% | UPT: 611.45Mbps Avg EE (baseline): 2.66 Avg. EE (ES scheme): 3.46 | Baseline: SSB/PRACH: 20 msec periodicity; SIB periodicity 40ms ES scheme: SSB/SIB1/PRACH: 640 msec periodicity |
|  |  |  | Medium | 6.2% | UPT: 457.92Mbps Avg EE (baseline): 1.50 Avg. EE (ES scheme): 1.63 | Baseline: SSB/PRACH: 20 msec periodicity; SIB periodicity 40ms ES scheme: SSB/SIB1/PRACH: 160 msec periodicity |
|  |  |  |  | 7.1% | UPT: 457.92Mbps Avg EE (baseline): 1.50 Avg. EE (ES scheme): 1.65 | Baseline: SSB/PRACH: 20 msec periodicity; SIB periodicity 40ms ES scheme: SSB/SIB1/PRACH: 640 msec periodicity |
|  |  | Cat2 | Low | 8.2% | UPT: 819.66Mbps Avg EE (baseline): 35.82 Avg. EE (ES scheme): 39.23 | Baseline: SSB/PRACH: 20 msec periodicity; SIB periodicity 40ms ES scheme: SSB/SIB1/PRACH: 160 msec periodicity |
|  |  |  |  | 10.9% | UPT: 819.66Mbps Avg EE (baseline): 35.82 Avg. EE (ES scheme): 40.09 | Baseline: SSB/PRACH: 20 msec periodicity; SIB periodicity 40ms ES scheme: SSB/SIB1/PRACH: 1280 msec periodicity |
|  |  |  | Light | 5.1% | UPT: 611.45Mbps Avg EE (baseline): 20.75 Avg. EE (ES scheme): 22.00 | Baseline: SSB/PRACH: 20 msec periodicity; SIB periodicity 40ms ES scheme: SSB/SIB1/PRACH: 160 msec periodicity |
|  |  |  |  | 5.8% | UPT: 611.45Mbps Avg EE (baseline): 20.75 Avg. EE (ES scheme): 22.19 | Baseline: SSB/PRACH: 20 msec periodicity; SIB periodicity 40ms ES scheme: SSB/SIB1/PRACH: 1280 msec periodicity |
|  |  |  | Medium | 3.0% | UPT: 457.92Mbps Avg EE (baseline): 12.44 Avg. EE (ES scheme): 12.89 | Baseline: SSB/PRACH: 20 msec periodicity; SIB periodicity 40ms ES scheme: SSB/SIB1/PRACH: 160 msec periodicity |
|  |  |  |  | 3.4% | UPT: 457.92Mbps Avg EE (baseline): 12.44 Avg. EE (ES scheme): 12.96 | Baseline: SSB/PRACH: 20 msec periodicity; SIB periodicity 40ms. ES scheme: SSB/SIB1/PRACH: 1280 msec periodicity |
| CATT [25] | Adaptation of common signals and channels | Cat 1 | Zero load | 10.2%, 72.7%, 84.8% |  | Baseline: 20ms SSB; ES scheme: SSB: 40ms, 80ms, 160ms for each load |
|  |  |  | Low load | 3.4%, 18.8%, 19.7% |  | Baseline: SLS; (DRX-cycle, on duration timer, inactivity timer) = (160ms, 8ms, 100ms); SSB periodicity 20ms; CSI-RS/TRS 10ms; ES scheme: SSB: 40ms, 80ms, 160ms for each load |
|  |  |  | Light load | 1.9%, 5.2%, 5.6% |  | Baseline: SLS; (DRX-cycle, on duration timer, inactivity timer) = (160ms, 8ms, 100ms); SSB periodicity 20ms; CSI-RS/TRS 10ms; ES scheme: SSB: 40ms, 80ms, 160ms for each load |
|  |  |  | Medium load | 1.3%, 2.2%, 2.6% |  | Baseline: SLS; (DRX-cycle, on duration timer, inactivity timer) = (160ms, 8ms, 100ms); SSB periodicity 20ms; CSI-RS/TRS 10ms; ES scheme: SSB: 40ms, 80ms, 160ms for each load |
| Fujitsu [11] | SSB/SIB1 period= 40ms | Cat2 | Zero, low, light, medium | 17.9%, 13.7%, 11.1%, 8.6% |  | Baseline scheme: 20 ms SSB/SIB1 period |
|  | SSB/SIB1 period= 80ms |  | Zero, low, light, medium | 26.8%, 20.6%, 16.7%, 12.8% |  |  |
|  | SSB/SIB1 period= 160ms |  | Zero, low, light, medium | 31.4%, 24.1%, 19.4%, 15.0% |  |  |
|  | SSB/SIB1 period= 40ms |  | Zero, low, light, medium | 18.3%, 12.6%, 9.4%, 6.9% |  |  |
|  | SSB/SIB1 period= 80ms |  | Zero, low, light, medium | 27.4%, 18.8%, 14.1%, 10.4% |  |  |
|  | SSB/SIB1 period= 160ms |  | Zero, low, light, medium | 32.0%, 22.0%, 16.5%, 12.1% |  |  |
| Ericsson [18] | 40ms SSB+SIB1 | Cat1 | Zero | 0.9% |  | Baseline scheme: 20ms SSB + 160ms SIB1 ES: one SSB. Energy calculation: per symbol energy consumption is modeled. |
|  | 80ms SSB+SIB1 |  |  | 48.5% |  |  |
|  | 160ms SSB+SIB1 |  |  | 72.6% |  |  |
|  | 40ms SSB+SIB1 |  |  | -6.2% |  | Baseline scheme: 20ms SSB + 160ms SIB1 ES: Four SSBs. Energy calculation: per symbol energy consumption is modeled. |
|  | 80ms SSB+SIB1 |  |  | 43.8% |  |  |
|  | 160ms SSB+SIB1 |  |  | 70.5% |  |  |
| Qualcomm [17] | Adaptation of Common Signals and Channels | Category 1 | No Load | 13.9% | Access delay/latency: additional 20 ms; UE power consumption increment: 99% | Note: "SSB period of 40 ms" without any network traffic either in DL or UL. Therefore, there are no statistics for UPT, latency, etc.. |

# Appendix-2: Impact to legacy UEs with SSB adaptation

## Appendix-2-1: Whether legacy UEs can support SSB periodicity larger than 20ms

Some issues were raised in RAN1, e.g.

- Issue-1: Legacy UEs cannot perform initial cell selection (i.e. cell search) with SSB periodicity larger than 20ms, since 20ms is default SSB periodicity in TS 38.213.

- Issue-2: Legacy UEs cannot camp on or be served by PCell with SSB periodicity larger than 20ms.

For Issue-1, 20ms is just default SSB periodicity, which can be regarded as typical periodicity in real deployment. However, it does not mean UE cannot perform cell search with SSB periodicity larger than 20ms. According to specs, UE should follow spec to support all possible SSB periodicities. Of course, the latency may be larger.

Observation 4: Legacy UEs can perform cell search with SSB periodicity larger than 20ms, but the latency may be larger.

For Issue-2, 20ms is default SSB periodicity, but it does not mean that UE cannot camp on a cell or be served by a PCell with SSB periodicity larger than 20ms. Of course, the mobility and the performance may be impacted.

Observation 5: Legacy UEs can camp on a cell or be served by a PCell with SSB periodicity larger than 20ms, but the mobility and performance may be impacted.

## Appendix-2-1: Trivial negative impact to legacy UEs

In R19 eNES WID [1], it is mentioned that

Note: there shall be no negative impact to legacy UEs, unless significant benefits are shown

According to above observations, for PCell or standalone cell, legacy UEs may be impacted by SSB periodicity larger than 20ms. However, in SI evaluation, empty or low load was widely assumed in companies’ evaluations. In empty of low load case, network have some compensation for the connected UEs, e.g. low MCS, so the negative impact to legacy UEs may be trivial.

Observation 6: In empty of low load case, the negative impact to legacy UEs may be trivial.