| 3GPP TR 38.897 V1.0.0 (2024-06) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; High power for NR TDD intra-band carrier aggregation in frequency range FR1; (Release 18) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2023, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	4

1	Scope	6

2	References	6

3	Definitions of terms, symbols and abbreviations	6

3.1	Terms	6

3.2	Symbols	6

3.3	Abbreviations	6

4	Background	6

5	High power for NR TDD intra-band carrier aggregation in frequency range FR1	7

5.1	CA_n77(2A) with UL CA_n77(2A)	8

5.1.1	Configurations	8

5.1.2	UE maximum output power	8

5.1.3	UE additional maximum output power reduction	8

5.2	CA_n78(2A) with UL n78	9

5.2.1	Configurations	9

5.2.2	UE maximum output power	9

5.2.3	UE additional maximum output power reduction	9

Annex A (informative): Change history	10

Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

1	Scope

The present document is a technical report for release 18 basket WI High power UE (power class 1.5 or 2) for intra-band Carrier Aggregation combinations of a single NR FR1 TDD band.

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP RP-222647: “WID on High power UE (power class 1.5 or 2) for intra-band Carrier Aggregation combinations of a single NR FR1 TDD band”.

3	Definitions of terms, symbols and abbreviations

3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

3.2	Symbols

For the purposes of the present document, the following symbols apply:

3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

<ABBREVIATION>	<Expansion>

TDD				Time Division Duplex

UE				User Equipment

4	Background

At 3GPP RAN#97-e meeting, a basket Work Item on “High power UE (power class 1.5 or 2) for intra-band Carrier Aggregation combinations of a single NR FR1 TDD band” [2] was approved for Rel-18. The objectives of the core part are as follows:

This WI will address the following intra-band Carrier Aggregation cases of a single NR FR1 TDD band:

- with PC2 for UL intra-band CA configuration

- with PC2 for single uplink carrier

- with PC1.5 for single uplink carrier

- with the following objectives:

1) Specify the band-combination specific RF requirements including

a) UE maximum output power and Tx power tolerance

b) A-MPR requirements if needed

2) Define requirements for PC1.5 according to following conditions

a) Include intra-band DL CA band combinations with PC1.5 on the single carrier uplink in supporting of 2x26dBm PA configuration.

Note 0: Other additional requirements due to the high power on UL may be added via WID revisions, if necessary.

Note 1: The generic requirements of high power UEs for the band combinations considered in this WI are covered in REL-17 NR_RF_FR1_enh.

Note 2: RAN4 will not apply its block agreement process to CRs of this basket WI.

The NR intra-band CA band combination configurations of frequency range FR1 that are considered in this WI are defined in the Table 1 below:

Table 1: NR Intra band CA configuration

| NR CA configuration |  |  | Uplink CA configuration or single uplink carrier |  | Power class |  | contact name, company |  | Contact email |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n77(2A) |  |  | CA_n77(2A) |  | PC2 |  | Ron Borsato AT&T |  | ronald.borsato@att.com |  |
| CA_n78(2A) |  |  | n78A |  | PC2 |  | Ryu Kitagawa, NTT DOCOMO, INC. Per Lindell Ericsson |  | ryuu.kitagawa.pn@nttdocomo.com per.lindell@ericsson.com |  |
| CA_n78(2A) |  |  | CA_n78(2A) |  | PC2 |  | Per Lindell Ericsson |  | per.lindell@ericsson.com |  |
| CA_n77(3A) |  |  | n77A |  | PC2 |  | Yasuki Suzuki KDDI Per Lindell Ericsson |  | ui-suzuki@kddi.com per.lindell@ericsson.com |  |
| CA_n77C |  |  | CA_n77C |  | PC2 |  | Zheng Zhao, Verizon |  | Zheng.zhao@verizonwireless.com |  |
| CA_n41(A-C) |  |  | CA_n41C |  | PC2 |  | Bill Shvodian T-Mobile USA |  | bill.shvodian@t-mobile.com |  |
| CA_n77(3A) |  |  | CA_n77(2A) |  | PC2 |  | Takehiko Narita SoftBank Per Lindell Ericsson |  | takehiko.narita@g.softbank.co.jp per.lindell@ericsson.com |  |
| CA_n79C |  |  | CA_n79C |  | PC2 |  | Ziwei Chen CMCC |  | chenziwei@chinamobile.com |  |
| CA_n41B |  |  | CA_n41B |  | PC2 |  | Yasuki Suzuki KDDI |  | ui-suzuki@kddi.com |  |
| CA_n40B |  |  | n40A |  | PC2 |  | Wilfredo Trocel nbn co Limited |  | wilfredotrocel@nbnco.com.au |  |
| CA_n40B |  |  | CA_n40B |  | PC2 |  | Wilfredo Trocel nbn co Limited |  | wilfredotrocel@nbnco.com.au |  |
| CA_n40(2A) |  |  | n40A |  | PC2 |  | Wilfredo Trocel nbn co Limited |  | wilfredotrocel@nbnco.com.au |  |
| CA_n40(2A) |  |  | CA_n40(2A) |  | PC2 |  | Wilfredo Trocel nbn co Limited |  | wilfredotrocel@nbnco.com.au |  |
| CA_n77C |  |  | n77A |  | PC1.5 |  | Zheng Zhao, Verizon |  | Zheng.zhao@verizonwireless.com |  |
| CA_n78(2A) |  |  | n78A |  | PC1.5 |  | Ryu Kitagawa, NTT DOCOMO, INC. Per Lindell Ericsson |  | ryuu.kitagawa.pn@nttdocomo.com per.lindell@ericsson.com |  |
| CA_n77(3A) |  |  | n77A |  | PC1.5 |  | Yasuki Suzuki KDDI Per Lindell Ericsson |  | ui-suzuki@kddi.com per.lindell@ericsson.com |  |
| CA_n78C |  |  | n78A |  | PC1.5 |  | Charley Cao Telstra |  | chariey.cao@team.telstra.com |  |
| CA_n41B |  |  | n41A |  | PC1.5 |  | Yasuki Suzuki KDDI |  | ui-suzuki@kddi.com |  |
| CA_n40B |  |  | n40A |  | PC1.5 |  | Wilfredo Trocel nbn co Limited |  | wilfredotrocel@nbnco.com.au |  |
| CA_n40(2A) |  |  | n40A |  | PC1.5 |  | Wilfredo Trocel nbn co Limited |  | wilfredotrocel@nbnco.com.au |  |

The present document is a technical report for this basket Work Item.

5	High power for NR TDD intra-band carrier aggregation in frequency range FR1

5.1	CA_n77(2A) with UL CA_n77(2A)

5.1.1	Configurations

Table 5.1.1-1: NR CA configurations and bandwidth combination sets defined for intra-band non-contiguous CA

| NR CA Configuration | Uplink CA Configurations or single uplink carrier3 | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Maximum Aggregated bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n77(2A) | CA_n77(2A)1 | 20, 40, 80, 100 | 20, 40, 80, 100 |  |  | 200 | 0 |
|  |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  | 200 | 1 |
|  |  | See n77 channel bandwidths in Table 5.3.5-1 of TS38.101-1 for each carrier |  |  |  | 200 | 4 and 5 |
| NOTE 1: Power Class 2 is allowed for this uplink combination or single uplink carrier in this downlink/uplink combination. NOTE 2: Power Class 1.5 is allowed for this uplink combination or single uplink carrier in this downlink/uplink combination. NOTE 3: Only single uplink carriers with power class other than PC3 are listed. |  |  |  |  |  |  |  |

5.1.2	UE maximum output power

For intra-band non-contiguous carrier aggregation with two uplink carriers the maximum output power is specified in Table 5.1.2-1.

Table 5.1.2-1: UE Power Class for intraband non-contiguous CA

| NR CA Configuration | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 4 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n77(2A) |  |  | 26 | +2/-3 | 23 | +2/-3 |  |  |
| NOTE 1: An uplink CA configuration in which the band has NOTE 3 in Table 6.2.1-1 is allowed to reduce the lower tolerance limit by 1.5 dB when the transmission bandwidths of the band are confined within FUL_low and FUL_low + 4 MHz or FUL_high - 4 MHz and FUL_high. NOTE 2: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 3: For intra-band non-contiguous carrier aggregation the maximum power requirement shall apply to the total transmitted power over all component carriers (per UE). |  |  |  |  |  |  |  |  |

5.1.3	UE additional maximum output power reduction

There is no A-MPR issue for this CA configuration.

5.2	CA_n78(2A) with UL n78

5.2.1	Configurations

Table 5.2.1-1: NR CA configurations and bandwidth combination sets defined for intra-band non-contiguous CA

| NR CA Configuration | Uplink CA Configurations or single uplink carrier3 | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Maximum Aggregated bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n78(2A) | n781,2 | 10, 20, 40, 50, 60, 80, 90, 100 | 10, 20, 40, 50, 60, 80, 90, 100 |  |  | 200 | 0 |
|  |  | 10, 20, 25, 30, 40, 50, 60, 80, 90, 100 | 10, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  |  | 200 | 1 |
|  |  | 10, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 10, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  | 200 | 2 |
| NOTE 1: Power Class 2 is allowed for this uplink combination or single uplink carrier in this downlink/uplink combination NOTE 2: Power Class 1.5 is allowed for this uplink combination or single uplink carrier in this downlink/uplink combination NOTE 3: Only single uplink carriers with power class other than PC3 are listed. |  |  |  |  |  |  |  |

5.2.2	UE maximum output power

For intra-band non-contiguous carrier aggregation with one uplink carrier on the PCC, the requirements shown below apply for power class 3 and other power classes if indicated in clause 5.5A.2 of TS38.101-1.

Table 5.2.2-1: UE Power Class

| NR band | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 4 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n78 |  |  | 293 | +2/-3 | 26 | +2/-3 | 23 | +2/-3 |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance NOTE 2: Power class 3 is default power class unless otherwise stated NOTE 3: Achieved via dual Tx |  |  |  |  |  |  |  |  |

5.2.3	UE additional maximum output power reduction

There is no A-MPR issue for this CA configuration.

Annex A (informative): Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2022-11 | RAN4 #105 | R4-2220577 |  |  |  | TR skeleton | 0.0.1 |
| 2023-02 | RAN4 #106 | R4-2302146 |  |  |  | The following TPs are approved: R4-2302142 TP on TR 38.897 for DL CA_n77(2A) with UL PC2 CA_n77(2A) R4-2302143 TP on TR 38.897 for DL CA_n78(2A) with UL PC2 and PC1.5 n78 | 0.1.0 |
| 2023-04 | RAN4 #106bis | R4-2305500 |  |  |  | Remove the DL CA configuration with UL PC1.5 CA from TR | 0.2.0 |