| 3GPP TR 38.743 V1.0.0 (2024-06) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group RAN; Evolved Universal Terrestrial Radio Access (E-UTRA) and NR; Study on enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN (Release 19) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet https://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2024, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	4

1	Scope	5

2	References	5

3	Definitions of terms, symbols and abbreviations	5

3.1	Terms	5

3.2	Symbols	5

3.3	Abbreviations	5

4	Use cases and Solutions	6

4.1	AI/ML based Network Slicing	6

4.1.1	Use case description	6

4.1.2	Solutions and standard impacts	6

4.2	AI/ML based Coverage and Capacity Optimization	6

4.2.1	Use case description	6

4.2.2	Solutions and standard impacts	6

5	Rel-18 Leftovers and solutions	6

5.1	Mobility optimization for NR-DC	6

5.2	Split architecture support for Rel-18 use cases	6

5.3	Energy saving enhancements	6

5.4	Continuous MDT collection targeting the same UE across RRC states	6

5.5	Multiple-hop UE trajectory across gNBs	6

6	Conclusion	7

Annex <A> (informative): Change history	8

Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall	indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should	indicates a recommendation to do something

should not	indicates a recommendation not to do something

may	indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can	indicates that something is possible

cannot	indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will	indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not	indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

1	Scope

The present document provides the description and investigation of new AI/ML based use cases, i.e., Network Slicing and Coverage and Capacity Optimization, and its corresponding solutions, and initial analysis of Rel-18 leftovers.

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[x]	3GPP TS 38.300: " NR; NR and NG-RAN Overall Description".

3	Definitions of terms, symbols and abbreviations

3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

example: text used to clarify abstract rules by applying them literally.

3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

<ABBREVIATION>	<Expansion>

4	Use cases and Solutions

4.1	AI/ML based Network Slicing

4.1.1	Use case description

Editor Note: Capture the description of use case

Support of network slicing in NG-RAN is defined in TS38.300 [x].

The NG-RAN plays a key role in taking mobility, load balancing and Radio Resources Management decisions for the purpose of meeting target requirements derived from the SLA of each supported network slice.

AI/ML function can analyze metrics related to network and UE level performance related to perform optimal resource management and mobility decisions for network slicing to meet the requirements.

4.1.2	Solutions and standard impacts

Editor Note: Capture the solutions for the use case, including potential standard impacts on existing Nodes, functions, and interfaces

#### 4.1.2.1	Locations for AI/ML Model Training and AI/ML Model Inference

The following solutions can be considered for supporting AI/ML-based network slicing:

-	AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB.

-	AI/ML Model Training and AI/ML Model Inference are both located in the gNB.

In case of CU-DU split architecture, the following solutions are possible:

-	AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB-CU.

-	AI/ML Model Training and Model Inference are both located in the gNB-CU.

#### 4.1.2.4	Input data of AI/ML based Network Slicing:

To predict the optimized network slicing decisions, a gNB may need the following information as input data for AI/ML-based network slicing:

From local node:

-	Measured/Predicted radio resource status per slice

-	Measured/Predicted slice available capacity

-	Legacy predicted UE trajectory

From neighbouring gNBs:

-	Measured/Predicted radio resource status per slice

-	Measured/Predicted slice available capacity

From the UE:

-	UE measurement report (e.g., UE RSRP, RSRQ, SINR measurement, etc), including cell level and beam level UE measurements

#### 4.1.2.5	Output data of AI/ML based Network Slicing:

AI/ML-based network slicing model in a gNB can generate the following information as output:

- Predicted radio resource status per slice

- Predicted slice available capacity

- Resource management decisions for resources within RRM policies (used by gNB internally)

- Slice aware mobility decisions (used by gNB internally)

#### 4.1.2.6	Feedback of AI/ML based Network Slicing:

To optimize the performance of AI/ML-based network slicing model, the following feedback can be considered to be collected from gNBs:

- Measured Radio resource status per slice

- Measured Slice available capacity

- Legacy UE performance feedback for those UEs handed over from the source gNB

- Finer granularity UE performance feedback for those UEs handed over from the source gNB. FFS on the exact level of finer granularity.

#### 4.1.2.7	Potential standard impacts:

Following standard impacts is listed for subsequent Rel-19 normative work compared with what was specified during Rel-18.

Xn interface:

- Enhanced existing procedure to collect predicted information between gNBs:

  - Predicted radio resource status per slice

  - Predicted slice available capacity

4.2	AI/ML based Coverage and Capacity Optimization

4.2.1	Use case description

Editor Note: Capture the description of use case

The objective of NR Coverage and Capacity Optimization (CCO) function is to detect and resolve or mitigate CCO issues. An NG-RAN node may autonomously adjust within and switch among coverage configurations. When a change is executed, a NG-RAN node may notify its neighbour NG-RAN nodes with the list of cells and SSBs with modified coverage included.

In the legacy CCO solution, a reactive approach is used: when the gNB (gNB-CU in case of CU-DU split architecture) detects a CCO issue which negatively impacts network and UE performance after it has already occurred, the gNB (gNB-DU in case of CU-DU split architecture) attempts to resolve or mitigate it.

With an AI/ML based CCO, a more proactive approach is used to prevent (or limiting at an early stage) the rise of a CCO issue with the consequent degradation of network (and UE) performance.

4.2.2	Solutions and standard impacts

Editor Note: Capture the solutions for the use case, including potential standard impacts on existing Nodes, functions, and interfaces

4.2.2.1 Locations for AI/ML Model Training and AI/ML Model Inference

The following solutions can be considered for supporting AI/ML-based CCO:

- AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB.

- AI/ML Model Training and AI/ML Model Inference are both located in the gNB.

In case of CU-DU split architecture, the following solutions are possible:

- AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB-CU.

- AI/ML Model Training and Model Inference are both located in the gNB-CU.

5	Rel-18 Leftovers and solutions

Editor Note: Such topics are listed here for further selection/down selection for normative work.

5.1	Mobility optimization for NR-DC

Editor Note: Capture the description and its potential standard impacts.

5.1.1 Use case description

Mobility Optimization for NR-DC is studied by assuming inference at the MN only. The main use case is limited to Dual Connectivity only (e.g., no conditional Dual Connectivity procedures are in scope).

5.2	Split architecture support for Rel-18 use cases

Editor Note: Capture the description and its potential standard impacts.

5.2.1 Use case description

The split architecture should be enhanced to support the Rel-18 use cases, e.g, Load Balancing, Energy Saving, and Mobility Optimization.

In case of CU-DU architecture, the following solutions are possible:

- AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB-CU(-CP);

- AI/ML Model Training and Model Inference are both located in the gNB-CU(-CP).

5.2.2 Potential Standard impacts

The following standard impacts are listed for subsequent Rel-19 normative work compared with what was specified during Rel-18.

E1 interface:

- Measured UE performance from gNB-CU-UP to gNB-CU-CP.

F1 interface:

-	Measured Energy Cost from gNB-DU to gNB-CU.

5.3	Energy saving enhancements

Editor Note: Capture the description and its potential standard impacts.

5.4	Continuous MDT collection targeting the same UE across RRC states

Editor Note: Capture the description and its potential standard impacts.

5.4.1	Use Case Description

The problem of continuous data collection for management-based MDT can be described as follows: a UE in the NG-RAN can be configured with management-based Logged MDT when in RRC_Idle and RRC_Inactive states and with management-based Immediate MDT when in RRC_Connected state. Differently from signalling-based MDT, in management-based MDT, a UE is not uniquely identified in the MDT activation. Therefore, when a UE transits to RRC_Connected state from RRC_Idle/RRC_Inactive (during which Logged MDT data have been collected) or when a UE is handed over between gNBs, the network does not have standardized means to select again the same UE for continuous MDT for subsequent MDT data collection.

The Data Collection continuity in this scenario can be split into two tasks as below:

- Problem A (measurement continuity): how to ensure that the same UE collecting MDT measurements during the same RRC state and across different RRC states.

- Problem B (trace correlation): how to ensure that the TCE which eventually receives the MDT reports can associate the received logged and immediate MDT measurements to a continuous data collection period from the same UE.

5.5	Multiple-hop UE trajectory across gNBs

Editor Note: Capture the description and its potential standard impacts.

5.5.1 Use case description

In Rel-18, the cell-based UE trajectory prediction is limited to the first-hop target NG-RAN node. Multi-hop predicted UE trajectory across gNBs consists of a list of cells belonging to one or more gNBs where the UE is expected to connect and these cells are listed in chronological order.

6	Conclusion

Annex <A> (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |