3GPP TSG RAN Meeting #104	 RP-241172

Shanghai, China, June 17-20, 2024

Agenda item:		9.3.1.2	Evolution of NR duplex operation: Sub-band full duplex (SBFD) [RAN1 WI: NR_duplex_evo]

Source:	Nokia

Title:	Discussion on SBFD WID updates

Document for:	Discussion

# 1	Introduction

In RAN#102 the work item on Evolution of NR duplex operation was approved. The current work item description (RP-240789) has the following objectives, which are subject to the progress in RAN1 working group with a checkpoint in RAN#104:

- For subband non-overlapping full duplex (SBFD) operation at gNB side within a TDD carrier: - Specify semi-static indication of time location of SBFD subbands to UEs in RRC_CONNECTED mode [RAN1, RAN2] - Indication of time location of SBFD subbands in SIB is not precluded - Specify semi-static indication of frequency domain location of SBFD subbands to UEs in RRC_CONNECTED mode [RAN1, RAN2] - Indication of frequency domain location of SBFD subbands in SIB is not precluded - Specify SBFD operation to support random access in SBFD symbols by UEs in RRC CONNECTED mode [RAN1, RAN2] - Study and specify, if justified, SBFD operation to UE in RRC_IDLE/INACTIVE mode for random access [RAN1, RAN2] - RAN#104 to check whether to proceed normative work - Specify UE transmission, reception and measurement behavior and procedures in SBFD symbols and/or non-SBFD symbols for SBFD aware UE [RAN1, RAN2] - Transmission and reception behaviours on SBFD subbands configured in DL and/or flexible symbol indicated by TDD-UL-DL-ConfigCommon - UL transmissions within UL subband only - DL receptions within DL subband(s) only, except for CLI measurement by the UE outside of the DL subbands Note: When flexible symbols are used, it is not expected that any legacy Uplink symbol is converted to Downlink/SBFD symbols - Enhancement on resource allocation in frequency domain in SBFD symbols, including - resource allocation in frequency domain for PDSCH/CSI-RS across two DL subbands in SBFD symbols - handling of unaligned boundaries between SBFD subband(s) and RBG, CSI reporting subband, CSI-RS resource, PRG - Enhancements on physical channels/signals and procedure across SBFD symbols and non-SBFD symbols in different slots, where each transmission/reception within a slot has either all SBFD or all non-SBFD symbols, including - resource allocation in frequency domain for transmission or reception in SBFD symbols and non-SBFD symbols with different available frequency resource in different slots - CSI report of which associated CSI-RS instances occur in both SBFD symbols and non-SBFD symbols in different slots - Configurations for SRS, PUCCH and PUSCH on SBFD symbols and non-SBFD symbols, e.g., resources, frequency hopping parameters, UL power control parameters and/or beam/spatial relation - Collision handling between DL reception in DL subband(s) and UL transmission in UL subband in a SBFD symbol - Followings are assumed based on TR 38.858 - SBFD at the gNB side - Half duplex operation at the UE side - FR1 and FR2-1 - SBFD operation Option 4, i.e., both time and frequency locations of subbands for SBFD operation are known to SBFD aware UEs - Coexistence between non-SBFD aware UEs (including legacy UEs) and SBFD aware UEs in the cell operating SBFD at gNB side - SBFD scheme within a single configured DL and UL BWP pair with aligned center frequencies - One UL subband for SBFD operation in an SBFD symbol (excluding legacy UL symbol/slot) within a TDD carrier - Mechanisms for SBFD operation shall also consider the adjacent channel coexistence between two operators - Specify enhancements for CLI handling [RAN1, RAN2, RAN3]: - Support gNB-to-gNB CLI handling scheme(s) (the detailed schemes are to be down-selected from those in TR38.858 by RAN1#117) - Support UE-to-UE CLI handling scheme(s) (the detailed schemes are to be down-selected from those in TR38.858 by RAN1#117) - Note: Without dedicated optimization for dynamic/flexible TDD. - Specify BS RF requirements for SBFD operation at gNB [RAN4] - Specify applicable RRM core requirements for CLI handling mechanisms [RAN4] - Specify other RRM core requirements for SBFD operation, if identified [RAN4] - Note: RAN3 will not specify enhancements to network signalling to support inter-operator coordination for CLI handling

In this contribution we provide our proposed way forward on the above objectives based on the reached progress in the RAN working groups.

# 2	Discussion

## 2.1	SBFD operation to UE in RRC_IDLE/INACTIVE mode for random access

According to the work item description, RAN#104 needs to determine whether the work on SBFD operation to UE in RRC_IDLE/INACTIVE mode for random access should proceed to normative work or not. RAN1 has discussed this topic during three meetings where the following was finally agreed in RAN1#117 in May 2024:

Agreement Support random access in SBFD symbols for UEs in RRC_IDLE/INACTIVE mode.

Given this agreement, we propose that the corresponding work item objective is rephrased to clarify that SBFD operation in RRC_IDLE/INACTIVE mode for random access is to be specified in Release 19.

Observation 1: RAN1 has agreed in RAN1#117 to support random access in SBFD symbols for UEs in RRC_IDLE/INACTIVE mode.

Proposal 1: Update the Rel-19 Evolution of NR duplex operation objective to clarify that SBFD operation in RRC_IDLE/INACTIVE mode for random access is to be specified in the Release 19 NR_duplex_evo work item.

On the more technical side, RAN1 has discussed the topics of random access for both RRC_IDLE/INACTIVE and RRC_CONNECTED in the same agenda item. In case contention based random access (CBRA) is used, not much difference has been identified for the CBRA operation when triggered by a UE in RRC_IDLE, INACTIVE or CONNECTED mode. For this reason, to simplify the specifications, we think RAN1 and RAN2 shall strive to have a unified solution for RRC_IDLE mode and RRC_Connected mode.

Observation 2: Based on the technical analyses conducted in RAN1, not much difference has been identified for the contention-based random access (CBRA) operation when triggered by a UE in RRC_IDLE, INACTIVE or CONNECTED mode.

Proposal 2: RAN1 and RAN2 to strive for a unified solution for RRC_IDLE mode and RRC_ CONNECTED mode.

The WID may be updated as follows:

- For subband non-overlapping full duplex (SBFD) operation at gNB side within a TDD carrier: - <omitted text> - Study and sSpecify, if justified, SBFD operation to UE in RRC_IDLE/INACTIVE mode for random access [RAN1, RAN2] - RAN1 and RAN2 to strive for a unified solution for RRC_IDLE mode and RRC_ CONNECTED mode RAN#104 to check whether to proceed normative work - <omitted text>

## 2.2	Enhancements for CLI handling

The WID currently states that the detailed CLI handling schemes are to be down-selected by RAN1#117 in May. In this regard, RAN1 has reached the following agreements:

Agreement For enhancements for CLI handling, the following are recommended from RAN1's perspective: - Information exchange of semi-static cell-specific SBFD time and frequency location configuration [RAN3] - Information exchange of measurement resource configuration, i.e., SSB and/or periodic NZP CSI-RS [RAN3] - Information exchange of strongest DL beam information [RAN3] - Information exchange of CLI-mitigation request [RAN3] - Note: No normative behavior is implied for the gNB receiving the CLI-mitigation request. No need to further send a stop message for CLI mitigation. Detailed signaling can be discussed in RAN3. - UL resource muting for PUSCH, including [RAN1, RAN2, RAN4] - Indication/Determination of UL resource muting for PUSCH based on semi-static configuration, assuming comb-2 for both DFT-S-OFDM and CP-OFDM in each allocated PRB and up to 2 symbols in time domain - Note: No new DCI field / MAC CE is introduced - PUSCH resource mapping, i.e., rate-matching around the muted REs - UCI resource determination in symbols with muted REs - Note: No impact on data and control multiplexing as defined in section 6.2.7, TS 38.212. - Note: UL resource muting does not apply for Msg A PUSCH and Msg 3 PUSCH. - Note: UL resource muting applies only for UEs in RRC_CONNECTED mode. - Note: UE assumes that the UL resource muting pattern does not overlap UL DMRS or PT-RS in the same symbol. - Note: Power boosting is assumed for REs in the symbol with UL resource muting. PUSCH transmit power does not change across symbols. - Note: No changes on TBS determination for PUSCH. - Note: Check impact on transmit signal quality/MPR requirement, if any, with RAN4 - Note: This feature is subject to UE capability - L1 based UE-to-UE CLI measurement and reporting based on existing CSI framework, including [RAN1, RAN2, RAN4] - Measurement resources - Periodic, semi-persistent, or aperiodic measurement resource (set), i.e., SRS-RSRP resource or CLI-RSSI resource - Note: The measurement resources for SRS-RSRP are based on the existing legacy RS patterns. No specification impact for SRS configuration information exchange between gNBs. - Configuration/Determination of ‘typeD’ QCL assumptions for the CLI measurement resource - Measurement reporting - At least aperiodic reporting - Note: Periodic and semi-persistent reporting can be considered - New report quantities: e.g. L1-SRS-RSRP, L1-CLI-RSSI and/or measurement resource indices - Note: At least wideband reporting is supported - UCI bits generation - Priority rules for multiple CSI reporting - Note: The existing CSI processing unit, CPU occupation rule and timeline for L1 beam reporting are reused for L1 UE-to-UE CLI measurement and reporting as starting point. - CLI measurement accuracy requirement [RAN4] - Note: The above information exchanges are subject to RAN3’s assessment of feasibility and specification impact.

Based on these agreements, we think the CLI-related work item objectives could be modified to reflect the latest RAN1 progress:

Observation 2: RAN1 has made down-selection in RAN1 #117 for the detailed schemes for gNB-to-gNB CLI handling and UE-to-UE CLI handling.

Proposal 3: Update the Rel-19 Evolution of NR duplex operation objective to clarify the work scope for CLI handling schemes to be specified in the Release 19 NR_duplex_evo work item. Proposed update of WID is as follows.

- Specify enhancements for CLI handling [RAN1, RAN2, RAN3]: - Support gNB-to-gNB CLI handling scheme(s) (the detailed schemes are to be down-selected from those in TR38.858 by RAN1#117) - Specify inter-gNB signaling for information exchange of [RAN3] - semi-static cell-specific SBFD time and frequency location configuration, and - CLI measurement resource configuration, i.e., SSB and/or periodic NZP CSI-RS - Strongest DL beam information and/or CLI-mitigation request - Note: No normative behavior is implied for the gNB receiving the CLI-mitigation request. No need to further send a stop message for CLI mitigation. Detailed signaling can be discussed in RAN3. - Specify UL resource muting for PUSCH including [RAN1, RAN2, RAN4] - Indication/Determination of UL resource muting for PUSCH based on semi-static configuration, assuming comb-2 for both DFT-S-OFDM and CP-OFDM in each allocated PRB and up to 2 symbols in time domain without introducing new DCI field or MAC-CE - PUSCH resource mapping, i.e., rate-matching around the muted REs - UCI resource determination in symbols with muted REs - Note: No impact on data and control multiplexing as defined in section 6.2.7, TS 38.212. - Note: UL resource muting does not apply for Msg A PUSCH and Msg 3 PUSCH. - Note: UL resource muting applies only for UEs in RRC_CONNECTED mode. - Note: UE assumes that the UL resource muting pattern does not overlap UL DMRS or PT-RS in the same symbol. - Note: Power boosting is assumed for REs in the symbol with UL resource muting. PUSCH transmit power does not change across symbols. - Note: No changes on TBS determination for PUSCH. - Note: Check impact on transmit signal quality/MPR requirement, if any, [RAN4] - Support UE-to-UE CLI handling scheme(s) (the detailed schemes are to be down-selected from those in TR38.858 by RAN1#117) - Specify L1 based UE-to-UE CLI measurement and reporting based on existing CSI framework, including [RAN1, RAN2, RAN4] - Measurement resources - Periodic, semi-persistent, or aperiodic measurement resource (set), i.e., SRS-RSRP resource or CLI-RSSI resource - Note: The measurement resources for SRS-RSRP are based on the existing legacy RS patterns. No specification impact for SRS configuration information exchange between gNBs. - Configuration/Determination of ‘typeD’ QCL assumptions for the CLI measurement resource - Measurement reporting - At least aperiodic reporting - Note: Periodic and semi-persistent reporting can be considered - New report quantities: e.g. L1-SRS-RSRP, L1-CLI-RSSI and/or measurement resource indices - UCI bits generation - Priority rules for multiple CSI reporting - Note: The existing CSI processing unit, CPU occupation rule and timeline for L1 beam reporting are reused for L1 UE-to-UE CLI measurement and reporting as starting point. - Note: Without dedicated optimization for dynamic/flexible TDD.

# 3	Conclusions and proposed WI modifications

In this contribution we have discussed the scope and proposed changes of the Rel-19 NR_duplex_evo work item based on RAN1 progress.

For SBFD operation for random access, the following proposals, observations and proposed WID modifications have been made:

Observation 1: RAN1 has agreed in RAN1#117 to support random access in SBFD symbols for UEs in RRC_IDLE/INACTIVE mode.

Proposal 1: Update the Rel-19 Evolution of NR duplex operation objective to clarify that SBFD operation in RRC_IDLE/INACTIVE mode for random access is to be specified in the Release 19 NR_duplex_evo work item.

Observation 2: Based on the technical analyses conducted in RAN1, not much difference has been identified for the contention-based random access (CBRA) operation when triggered by a UE in RRC_IDLE, INACTIVE or CONNECTED mode.

Proposal 2: RAN1 and RAN2 to strive for a unified solution for RRC_IDLE mode and RRC_ CONNECTED mode.

- For subband non-overlapping full duplex (SBFD) operation at gNB side within a TDD carrier: - <omitted text> - Study and sSpecify, if justified, SBFD operation to UE in RRC_IDLE/INACTIVE mode for random access [RAN1, RAN2] - RAN#104 to check whether to proceed normative work - RAN1 and RAN2 to strive for a unified solution for RRC_IDLE mode and RRC_ CONNECTED mode - <omitted text>

For CLI handling, the following proposals, observations and proposed WID modifications have been made:

Observation 2: RAN1 has made down-selection in RAN1 #117 for the detailed schemes for gNB-to-gNB CLI handling and UE-to-UE CLI handling.

Proposal 3: Update the Rel-19 Evolution of NR duplex operation objective to clarify the work scope for CLI handling schemes to be specified in the Release 19 NR_duplex_evo work item. Proposed update of WID is as follows.

- Specify enhancements for CLI handling [RAN1, RAN2, RAN3]: - Support gNB-to-gNB CLI handling scheme(s) (the detailed schemes are to be down-selected from those in TR38.858 by RAN1#117) - Specify inter-gNB signaling for information exchange of [RAN3] - semi-static cell-specific SBFD time and frequency location configuration, and - CLI measurement resource configuration, i.e., SSB and/or periodic NZP CSI-RS - Strongest DL beam information and/or CLI-mitigation request - Note: No normative behavior is implied for the gNB receiving the CLI-mitigation request. No need to further send a stop message for CLI mitigation. Detailed signaling can be discussed in RAN3. - Specify UL resource muting for PUSCH including [RAN1, RAN2, RAN4] - Indication/Determination of UL resource muting for PUSCH based on semi-static configuration, assuming comb-2 for both DFT-S-OFDM and CP-OFDM in each allocated PRB and up to 2 symbols in time domain without introducing new DCI field or MAC-CE - PUSCH resource mapping, i.e., rate-matching around the muted REs - UCI resource determination in symbols with muted REs - Note: No impact on data and control multiplexing as defined in section 6.2.7, TS 38.212. - Note: UL resource muting does not apply for Msg A PUSCH and Msg 3 PUSCH. - Note: UL resource muting applies only for UEs in RRC_CONNECTED mode. - Note: UE assumes that the UL resource muting pattern does not overlap UL DMRS or PT-RS in the same symbol. - Note: Power boosting is assumed for REs in the symbol with UL resource muting. PUSCH transmit power does not change across symbols. - Note: No changes on TBS determination for PUSCH. - Note: Check impact on transmit signal quality/MPR requirement, if any, [RAN4] - Support UE-to-UE CLI handling scheme(s) (the detailed schemes are to be down-selected from those in TR38.858 by RAN1#117) - Specify L1 based UE-to-UE CLI measurement and reporting based on existing CSI framework, including [RAN1, RAN2, RAN4] - Measurement resources - Periodic, semi-persistent, or aperiodic measurement resource (set), i.e., SRS-RSRP resource or CLI-RSSI resource - Note: The measurement resources for SRS-RSRP are based on the existing legacy RS patterns. No specification impact for SRS configuration information exchange between gNBs. - Configuration/Determination of ‘typeD’ QCL assumptions for the CLI measurement resource - Measurement reporting - At least aperiodic reporting - Note: Periodic and semi-persistent reporting can be considered - New report quantities: e.g. L1-SRS-RSRP, L1-CLI-RSSI and/or measurement resource indices - UCI bits generation - Priority rules for multiple CSI reporting - Note: The existing CSI processing unit, CPU occupation rule and timeline for L1 beam reporting are reused for L1 UE-to-UE CLI measurement and reporting as starting point. - Note: Without dedicated optimization for dynamic/flexible TDD.

# References

- RP-240789: WID revision: Evolution of NR duplex operation: Sub-band full duplex (SBFD). March 2024.