3GPP TSG RAN meeting #104										RP-241174

Shanghai, China, June 17-20, 2024

## Status Report to TSG

Agenda item:			9.4.1.1

| WI / SI Name | Enhancement of NR Dynamic spectrum sharing (DSS) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: No | Performance part: Yes |  | Testing part: No |
| Acronym | NR_DSS_enh-Perf |  |  |  |  |
| Unique ID | 940093 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-232957 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: mm/yyyy | Core part: 12/2023 | Performance part: 06/2024 | Testing part: mm/yyyy |  |
| Overall Completion level | Study Item: xx % | Core part: 100% | Performance Part: 100% | Testing part: xx% |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN1 |
| --- | --- | --- |
| Rapporteur | Name | Ravikiran Nory |
|  | Company | Ericsson |
|  | Email | ravikiran.nory@ericsson.com |

## 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

## 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

## 2.1	RAN1

Rapporteur Note: Core part completed in RAN#102 in Dec 2023.

Rapporteur Note: The main work for RAN1 is completed in Aug 2022. But the corresponding catB CRs will only be brought for approval towards end of WI in Q3, 2023.

#### 2.1.1	Agreements

##### NR PDCCH reception in symbols with LTE CRS REs

###### RAN1#109-e (May 2022)

Agreement

To evaluate the following options:

  - Option-1-1: No NR-PDCCH-DMRS is transmitted for only the REs overlapping with LTE-CRS of the OFDM symbol, NR-PDCCH is punctured on REs colliding with LTE-CRS, NR-PDCCH must span at least 2 consecutive symbols with at least 1 symbol not overlapping with LTE-CRS

  - Option-1-2: No NR-PDCCH-DMRS is transmitted in any RE of the OFDM symbol, NR-PDCCH is transmitted on REs not colliding with LTE-CRS including the original DMRS, NR-PDCCH is punctured on REs colliding with LTE-CRS, NR-PDCCH must span at least 2 consecutive symbols with at least 1 symbol not overlapping with LTE-CRS

  - Option-2: NR-PDCCH or NR-PDCCH-DMRS is transmitted on REs not colliding with LTE-CRS, NR-PDCCH and NR-PDCCH-DMRS may or may not be punctured on REs colliding with LTE-CRS

    - No puncture is baseline (UE side)

Observation

For evaluations consider the following options:

Option 1-1:

- PDCCH and PDCCH DMRS mapping to REs: Legacy

- PDCCH REs overlapping with LTE CRS: Receiver punctures

- PDCCH DMRS REs overlapping with LTE CRS: All DMRS REs on overlapping symbol Not used for CE, or legacy pattern is assumed

- gNB transmits: Irrelevant what the gNB transmits on REs overlapping with the LTE CRS REs as indicated in the CRS RM pattern.

- Channel estimator: operate on clean symbol DMRS only, Legacy

Option 1-2:

- PDCCH and PDCCH DMRS mapping to REs: New PDCCH rate-matching

  - No PDCCH DMRS on the symbol overlapping with LTE CRS

- PDCCH REs overlapping with LTE CRS: Receiver punctures

- PDCCH DMRS REs overlapping with LTE CRS: Not expected

- Channel estimator (UE assumption): Operate on clean symbol DMRS only

- gNB transmits: Irrelevant what the gNB transmits on REs overlapping with the LTE CRS REs as indicated in the CRS RM pattern

Option 2:

- PDCCH and PDCCH DMRS mapping to REs: Legacy

- PDCCH REs overlapping with LTE CRS: Baseline: Process as legacy

- PDCCH DMRS REs overlapping with LTE CRS: Aware or unaware

- Channel estimator: Baseline: Process as legacy (Receiver does not puncture DMRS), Optional: Advanced receiver (Use the DMRS other than legacy behavior)

- gNB transmits:

  - Baseline: May puncture the PDCCH/PDCCH DMRS, or may superposition the two.

  - Optional: may puncture LTE CRS of Port#2&3.

  - Impact to LTE UEs should be considered if superposition is used.

Agreement

For evaluations consider the following list of scenarios:

Scenario#1A: 1 symbol CORESET, overlapped with CRS – Option 2 only

Scenario#2: 2 symbols CORESET, including 1 overlapping symbol and 1 clean symbol – Option 1-1/1-2/2

Scenario#3: 3 symbols CORESET, including 1 overlapping symbol and 2 clean symbols – Option 1-1/1-2/2

Agreement

LLS simulations assumptions, [] are optional:

| Parameters | Values |
| --- | --- |
| Carrier frequency | 2 GHz |
| SCS | 15 kHz |
| Bandwidth | 20 MHz [5, 10 MHz], LTE bandwidth equal or smaller than NR |
| Channel model | TDL-C 300, [TDL-A 300] |
| Correlation | Low |
| Number of BS antennas | 4 Tx, (M,N,P,Mg,Ng;Mp,Np)= (1,2,2,1,1;1,1), [2 Tx, (M,N,P,Mg,Ng;Mp,Np)= (1,1,2,1,1;1,1).] |
| Number of UE antennas | 2 Rx (M,N,P,Mg,Ng;Mp,Np)= (1,1,2,1,1;1,1) |
| DCI payload (excluding CRC) | 60 bits [50bits] |
| Interleaving | Non-Interleaved, [Interleaved] |
| Precoding | Precoder cycling per REG bundle |
| REG bundle size | 6 PRBs |
| CRS | single 4 port CRS pattern, [additional 4 port CRS pattern] |
| Channel estimation | practical – companies to report details |
| UE speed | 30 kmph [3kmph, 120 kmph, 350 kmph] |
| Power ratio of LTE-CRS RE/NR PDCCH RE, Power ratio of LTE-CRS RE/NR PDCCH-DMRS RE | Companies to report (if applicable) |

Agreement

SLS simulations assumptions, [] are optional:

| Parameters | Values |
| --- | --- |
| Carrier frequency | 2.1 GHz |
| SCS | 15 kHz |
| Simulation bandwidth | 20 MHz [5, 10 MHz]，same as that in LLS |
| BS antenna height | 25 m |
| UE height | 1.5m |
| TRP transmit power | 49 dBm 20 MHz |
| Scenario | Urban Macro (500m ISD), [Rma (1732m ISD)] |
| Device deployment | 80% indoor, 20% outdoor (Uma) [50% indoor,50% in-car (Rma)] |
| UE speeds | Indoor users: 3km/h |
|  | Outdoor users (in-car): 30 km/h |
| BS noise figure | 5 dB |
| BS antenna element gain | 8 dBi |
| UE noise figure | 9 dB |
| Thermal noise level | -174 dBm/Hz |
| Traffic geometry | Full Buffer |
| Macro sites | 19 |
| Downtilt | 102° or according to Scenario |
| Minimum BS to UE distance | 35m |
| KPI | Companies to report (e.g., total PDCCH capacity, PDCCH coverage/outage, Potential degradation of LTE, whether and how to achieve coexistence with legacy UEs) |
| Others | Companies to report (e.g., fraction of LTE UEs, fraction of Rel-18 DSS NR UEs, etc.). Companies to report considered baseline(s). Baseline(s) aim(s) to be comparable to the evaluated option(s) |

###### RAN1#110 (August 2022)

Agreement

If the feature “Reception of NR PDCCH candidates that overlap with LTE CRS REs “is supported, it is RAN1 understanding that the feature is supported by UE performing channel estimation with a regular legacy DMRS pattern in frequency dimension, i.e., no change to UE assumption on PDCCH DMRS RE positions/pattern in a symbol that are used for the purpose of channel estimation.

Agreement

Reception of NR PDCCH candidates that overlap with LTE CRS REs is supported by Rel18 UEs

PDCCH candidates and PDCCH-DMRS RE mapping are based on that of R15 from UE side.

Note: depends on UE capability

Following options can be used for PDCCH-DMRS channel estimation

- legacy CE assumption

  - RAN1 consider support this, if no RAN4 performance requirements are defined

- CE on clean symbol(s) only (this channel estimation option does not apply for 1 symbol CORESET)

Note: Restriction on the symbols and/or LTE CRS patterns applicable for above agreements can be considered during UE capability session.

Conclusion

RAN1 understands above agreements applies to 15kHz SCS only.

###### RAN1#110bis-e (October 2022)

Rapporteur Note: No new agreements.

###### RAN1#111 (November 2022)

Rapporteur Note: No new agreements.

###### RAN1#112 (February 2023)

Rapporteur Note: No new agreements.

###### RAN1#112b-e (April 2023)

Agreement

The draft CR in R1-2304277 for Rel-18 TS38.211 eDSS is endorsed in principle.

Agreement

The draft CR in R1-2304195 for Rel-18 TS38.213 eDSS is endorsed in principle.

Agreement

Regarding component 1 of FG 52-1,

  - Component 1 of FG 52-1 is revised to “Reception of NR PDCCH candidates in REs that overlap with LTE CRS when UE is provided with LTE CRS RM pattern(s) by configuration of one CRS rate matching pattern or multiple non-overlapping CRS rate matching patterns”.

  - FFS whether to separate the capability to support reception of NR PDCCH candidates in REs that overlap with LTE CRS when UE is provided with LTE CRS RM patterns by configuration of multiple non-overlapping CRS rate matching patterns

Agreement:

Introduce following FGs

| 52. NR_DSS_enh | 52-1 | Reception of NR PDCCH candidates overlapping with LTE CRS REs | Reception of NR PDCCH candidates that overlap with LTE CRS REs within a NR carrier using 15 kHz SCS 1) Reception of NR PDCCH candidates in REs that overlap with LTE CRS when UE is provided with LTE CRS RM pattern by configuration of one CRS rate matching pattern [via lte-CRS-ToMatchAround] [2) PDCCH-DMRS channel estimation Value 1: PDCCH-DMRS channel estimation based on legacy CE assumption Value 2: for CORESET duration greater than 1 symbol, PDCCH-DMRS channel estimation based on PDCCH-DMRS REs in symbols not overlapping with LTE CRS Other values FFS] [3) Reception of NR PDCCH candidates that overlap with LTE CRS REs only on the 2nd symbol of an NR slot] [4) NR PDCCH that overlaps with LTE CRS REs is in [Type-1 CSS with dedicated RRC configuration, Type-3 CSS, and/or] USS that are monitored within the first 3 OFDM symbols of a slot] | 5-28 | Yes | N/A | UE is not required to support reception of NR PDCCH candidates overlapping with LTE CRS REs when it is provided with LTE CRS RM pattern by higher layers | [Per Band] | [No] | [No] | [N/A] | [Component 2 candidate value set: {Value 1, Value2, other values FFS}] [Component 3 candidate value set: {Value 1, other values FFS}] [Note: From UE perspective, PDCCH candidates and PDCCH-DMRS RE mapping are based on that of R15] [Note: For component 2, RAN1 consider support legacy CE only if no RAN4 performance requirements are defined for this option] Note: it is RAN1 understanding that the feature is supported by UE performing channel estimation with a regular legacy DMRS pattern in frequency dimension, i.e., no change to UE assumption on PDCCH DMRS RE positions/pattern in a symbol that are used for the purpose of channel estimation. | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 52. NR_DSS_enh | 52-1a | Reception of NR PDCCH candidates overlapping with LTE CRS REs with multiple non-overlapping CRS rate matching patterns | 1) Reception of NR PDCCH candidates in REs that overlap with LTE CRS when UE is provided with LTE CRS RM patterns by configuration of multiple non-overlapping CRS rate matching patterns [via lte-CRS-PatternList1-r16] | 52-1, 14-1 | Yes | N/A |  | [Per Band] | [No] | [No] | [N/A] | Note: it is RAN1 understanding that the feature is supported by UE performing channel estimation with a regular legacy DMRS pattern in frequency dimension, i.e., no change to UE assumption on PDCCH DMRS RE positions/pattern in a symbol that are used for the purpose of channel estimation. | Optional with capability signaling |
| 52. NR_DSS_enh | 52-1b | NR PDCCH reception that overlaps with LTE CRS within a single span of 3 consecutive OFDM symbols that is within the first 4 OFDM symbols in a slot | 1) NR PDCCH that overlaps with LTE CRS REs is in [Type-1 CSS with dedicated RRC configuration, Type-3 CSS, and/or] USS that are monitored within a single span of 3 consecutive OFDM symbols that is within the first 4 OFDM symbols in a slot | 52-1, 22-12 | Yes | N/A |  | [Per Band] | [No] | [No] | [N/A] |  | Optional with capability signaling |
| 52. NR_DSS_enh | 52-2 | Two LTE-CRS overlapping rate matching patterns within a part of NR carrier using 15 kHz overlapping with a LTE carrier [(regardless of support or configuration of multi-TRP)] | 1) Support of two LTE-CRS overlapping rate matching patterns configured by lte-CRS-PatternList3-r18 and lte-CRS-PatternList4-r18 within a part of NR carrier using 15 kHz overlapping with a LTE carrier [(regardless of support or configuration of multi-TRP)] 2) Maximum number of LTE-CRS rate matching patterns in total within a NR carrier using 15 kHz SCS: {2,3,4,5,6} - FFS: total number reported by UE in FG 14-1 and FG 52-2 shall not exceed 6 - FFS: Note: If a UE supports FG52-2 and FG14-1, the maximum numbers reported for FG 52-2 should be equal to or larger than the numbers reported for FG 14-1 3) Maximum number of LTE-CRS non-overlapping rate matching patterns within a NR carrier using 15 kHz SCS: {1,2,3} - FFS: total number reported by UE in FG 14-1 and FG 52-2 shall not exceed 3 - FFS: Note: If a UE supports FG52-2 and FG14-1, the maximum numbers reported for FG 52-2 should be equal to or larger than the numbers reported for FG 14-1 | [5-28] | Yes | N/A | [UE support for two overlapping LTE-CRS RM patterns within a part of NR carrier using 15 kHz overlapping with a LTE carrier when UE is not supporting or configured with multi-TRP is not enabled] | [Per Band] | [No] | [No] | [N/A] | FFS: Note: If a UE supports FG52-2 and FG14-1, the maximum numbers reported for FG 52-2 should be equal to or larger than the numbers reported for FG 14-1 | Optional with capability singaling |
| 52. NR_DSS_enh | 52-2a | Two LTE-CRS overlapping rate matching patterns with two different values of coresetPoolIndex within a part of NR carrier using 15 kHz overlapping with a LTE carrier | Support of two LTE-CRS overlapping rate matching patterns configured by lte-CRS-PatternList3-r18 and lte-CRS-PatternList4-r18 with two different values of coresetPoolIndex within a part of NR carrier using 15 kHz overlapping with a LTE carrier FFS additional clarification of relation between Rel-16 capability | 52-2, 16-2a | Yes | N/A |  | [Per Band] | [No] | [No] | [N/A] |  | Optional with capability singaling |
| 52. NR_DSS_enh | 52-3 | FFS: Support reception of NR PDCCH candidates overlapping with LTE CRS REs when two LTE-CRS overlapping rate matching patterns are configured |  | 52-1, at least one of 52-2 or 14-1a | Yes | N/A |  | [Per Band] | No | No | N/A |  |  |

###### RAN1#113 (May 2023)

Agreement:

- Component 1 in FG 52-1 is confirmed as: Reception of NR PDCCH candidates in REs that overlap with LTE CRS when UE is provided with LTE CRS RM pattern by configuration of one CRS rate matching pattern [via lte-CRS-ToMatchAround]

Agreement:

- Component 1 in FG 52-1a is confirmed as: Reception of NR PDCCH candidates in REs that overlap with LTE CRS when UE is provided with LTE CRS RM patterns by configuration of one or multiple non-overlapping CRS rate matching patterns [via lte-CRS-PatternList1-r16 [or lte-CRS-PatternList3-r18]

Agreement

- Regarding component 2 on CE in FG 52-1,

  - Update the component 2 as “Reception of a NR PDCCH candidate in REs that overlap with LTE CRS: candidate value set {a) when at least one symbol of the NR PDCCH candidate is not overlapped with LTE CRS, b) when some or all of symbols of NR PDCCH candidate overlap with LTE CRS}”

  - Note is confirmed as: For component 2, RAN1 considers support value b) in component 2 only if RAN4 performance requirements for value b) are not defined

Agreement

- Component 3 in FG 52-1 is updated as: Reception of NR PDCCH candidates that overlap with LTE CRS REs only on the X-th 2nd symbols of an NR slot. Candidate values for X: {only 2nd symbol, 1st and 2nd symbols}

Agreement

- Component 4 in FG 52-1 is confirmed as: NR PDCCH that overlaps with LTE CRS REs is in [Type-1 CSS with dedicated RRC configuration, Type-3 CSS, and/or USS] that are monitored within the first 3 OFDM symbols of a slot

- FG 52-1b is confirmed as follows

| 52. NR_DSS_enh | 52-1b | NR PDCCH reception that overlaps with LTE CRS within a single span of 3 consecutive OFDM symbols that is within the first 4 OFDM symbols in a slot | 1) NR PDCCH that overlaps with LTE CRS REs is in [Type-1 CSS with dedicated RRC configuration, Type-3 CSS, and/or USS] that are monitored within a single span of 3 consecutive OFDM symbols that is within the first 4 OFDM symbols in a slot | 52-1, 22-12 | Yes | N/A |  | [Per Band] | [No] | [No] | [N/A] |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Agreement

- Reporting type of FG 52-1/52-1a is per band for FR1 only

Conclusion

There is no consensus in RAN1 whether to introduce FG 52-3 for Support reception of NR PDCCH candidates overlapping with LTE CRS REs when two LTE-CRS overlapping rate matching patterns are configured

###### RAN1#114 (Aug 2023)

Agreement

- For component 4 of FG52-1 and component 1 of FG52-1b, NR PDCCH that overlaps with LTE CRS REs is in Type-1 CSS with dedicated RRC configuration, Type-3 CSS, and/or USS.

Agreement

- Prerequisite FGs of FG 52-1a is updated as “52-1, at least one of {14-1, 52-2}”

- Component 1 is updated as: Reception of NR PDCCH candidates in REs that overlap with LTE CRS when UE is provided with LTE CRS RM patterns by configuration of one or multiple non-overlapping CRS rate matching patterns via lte-CRS-PatternList1-r16 if the UE supports FG 14-1 or lte-CRS-PatternList3-r18 if the UE support FG 52-2

Agreement

- For FG52-1b, the reporting type is per band.

Agreement

- For FG52-1, the note “Note: From UE perspective, PDCCH candidates and PDCCH-DMRS RE mapping are based on that of R15” is removed.

Rapporteur Note: Agreed RAN1 CRs listed below.

| TD# | Title | Source to WG | Rel | TS | Vsn | Work Item | CR | Rev | Cat |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| R1-2308694 | Introduction of dynamic spectrum sharing (DSS) enhancements | Samsung | Rel-18 | 38.213 | 17.6.0 | NR_DSS_enh-Core | 0500 |  | B |
| R1-2308727 | Introduction of dynamic spectrum sharing enhancements | Ericsson | Rel-18 | 38.211 | 17.5.0 | NR_DSS_enh-Core | 0112 |  | B |

###### RAN1#114bis (October 2023)

Rapporteur Note: Below are agreements from maintenance discussions (including UE capability updates) for this WI.

Agreement

- Adopt the TP to Section 7.3.2.2, TS38.211.

------------------------------- start TP to 38.211 -----------------------

7.3.2.2	Control-resource set (CORESET)

<unchanged parts omitted>

-	if the higher-layer parameter precoderGranularity equals allContiguousRBs,

-	the UE may assume the same precoding being used across the all resource-element groups within the set of contiguous resource blocks in the CORESET;

-	the UE may assume that no resource elements in the CORESET overlap with an SSB;

-	if the UE is not provided with the higher-layer parameter pdcchCandidateReception-WithCRSOverlap, the UE may assume that no resource elements in the CORESET overlap with  LTE cell-specific reference signals as indicated by the higher-layer parameter lte-CRS-ToMatchAround, lte-CRS-PatternList1, or lte-CRS-PatternList2, or lte-CRS-PatternList3.

<unchanged parts omitted>

-------------------------------- end TP -----------------------------------

Agreement

- Adopt the TP to Section10.1, TS38.213.

------------------------------- start TP to 38.213 -----------------------

10.1	 UE procedure for determining physical downlink control channel assignment

< Unchanged parts are omitted >

When precoderGranularity = allContiguousRBs, a UE does not expect

-	to be configured a set of resource blocks of a CORESET that includes more than four sub-sets of resource blocks that are not contiguous in frequency

-	any RE of a CORESET to overlap with any RE determined from

-	lte-CRS-ToMatchAround or LTE-CRS-PatternList if the UE is not provided pdcchCandidateReception-WithCRSOverlap and the UE does not indicate an associated capability corresponding to the configuration of lte-CRS-ToMatchAround or of LTE-CRS-PatternList

-	a SS/PBCH block.

< Unchanged parts are omitted >

-------------------------------- end TP -----------------------------------

Agreement

- Update the component 2 of FG52-1

2) Reception of a NR PDCCH candidate in REs that overlap with LTE CRS: candidate value set { a) when at least one symbol of the NR PDCCH candidate and the DMRS for demodulation of the NR PDCCH candidate is not overlapped with LTE CRS, b) when some or all of symbols of NR PDCCH candidate overlap with LTE CRS}

For the editors:

The above endorsed text proposals to TS 38.211 and 38.213, respectively, are also collected in R1-2310596. Please consider them in the next specification revision.

Agreement

- The “Need of FR1/FR2 differentiation” fields for FGs 52-1/52-1a/52-1b are updated to “N/AFR1 only”

###### RAN1#115 (November 2023)

Rapporteur Note: Below are agreements from maintenance discussions (including UE capability updates) for this WI.

Agreement

The TP for issue 3 in section 2.3 of R1-2312367 is endorsed for Section 7.3.2.2 of TS 38.211.

Conclusion

- There is no consensus in RAN1 to introduce following FGs in Rel-18

  - Puncturing the PDCCH REs colliding with LTE CRS in the PDCCH decoding process	{Supported}

  - Usage of PDCCH DMRS REs on the PDCCH symbols overlapping with the CRS symbols	{DMRS REs not overlapping with LTE CRS REs, none of the DMRS REs}

##### UE support for two overlapping CRS rate matching patterns

###### RAN1#109-e (May 2022)

Working Assumption

Introduce two new RRC parameters (i.e., list3 and list4). gNB cannot configure legacy list1/2 and new list3/4 simultaneously. If new list3/4 are configured, UE applies list3/4 regardless of whether CORESETPoolIndex= 1 is configured for any of DL BWPs of the serving cell.

Working Assumption

  - Introduce two new RRC parameters lte-CRS-PatternList3-r18 and lte-CRS-PatternList4-r18 in ServingCellConfig around which the UE shall do rate matching for PDSCH scheduled by PDCCH with CRC scrambled by C-RNTI, MCS-C-RNTI, CS-RNTI, or PDSCHs with SPS.

    - The network does not configure lte-CRS-PatternList3-r18 and any of lte-CRS-ToMatchAround, lte-CRS-PatternList1-r16 and lte-CRS-PatternList2-r16 simultaneously. Lte-CRS-PatternList4-r18 is configured if lte-CRS-PatternList3-r18 is configured in ServingCellConfig.

    - For case when UE is not configured with two CORESET pools two different values of coresetPoolIndex in ControlResourceSet, and configured with lte-CRS-PatternList3-r18 and lte-CRS-PatternList4-r18, both lte-CRS-PatternList3-r18 and lte-CRS-PatternList4-r18 are applied

    - For case when UE is configured with two CORESET pools two different values of coresetPoolIndex in ControlResourceSet, and configured with lte-CRS-PatternList3-r18 and lte-CRS-PatternList4-r18

      - If UE is configured with crs-RateMatch-PerCoresetPoolIndex, lte-CRS-PatternList3-r18 is applied if the PDSCH is associated with coresetPoolIndex set to ‘0’, and lte-CRS-PatternList4-r18 is applied if the PDSCH is associated with coresetPoolIndex set to ‘1’

      - Otherwise, both lte-CRS-PatternList3-r18 and lte-CRS-PatternList4-r18 are applied.

    - The legacy configuration rule in TS 38.331 is applied in Rel-18 DSS, i.e.,

      - “The first LTE CRS pattern in lte-CRS-PatternList4 shall be fully overlapping in frequency with the first LTE CRS pattern in lte-CRS-PatternList3, The second LTE CRS pattern in this list shall be fully overlapping in frequency with the second LTE CRS pattern in lte-CRS-PatternList31, and so on.”

Agreement

- Introduce new UE capability(ies) for support of two overlapping LTE CRS patterns in Rel-18 DSS if the UE is NOT configured by higher layer parameter PDCCH-Config with two different values of coresetPoolIndex in ControlResourceSet.

  - NW can configure two LTE-CRS overlapping rate matching patterns without coresetPoolIndex only if the UE indicates support of the new capability(ies).

- Clarify that the Rel-16 UE capability overlapRateMatchingEUTRA-CRS-r16 is subject to support of multiDCI-Multi-TRP-r16.

- Maximum number of LTE-CRS rate matching patterns for PDSCH supported by a UE (i.e., maxNumberPatterns-r16 and maxNumberNon-OverlapPatterns-r16) is kept unchanged.

###### RAN1#110 (August 2022)

Agreement

  - Introduce two new RRC parameters lte-CRS-PatternList3-r18 and lte-CRS-PatternList4-r18 in ServingCellConfig around which the UE shall do rate matching for PDSCH scheduled by PDCCH with CRC scrambled by C-RNTI, MCS-C-RNTI, CS-RNTI, or PDSCHs with SPS.

    - The network does not configure lte-CRS-PatternList3-r18 and any of lte-CRS-ToMatchAround, lte-CRS-PatternList1-r16 and lte-CRS-PatternList2-r16 simultaneously. Lte-CRS-PatternList4-r18 is configured only if lte-CRS-PatternList3-r18 is configured in ServingCellConfig.

    - For case when UE is not configured with two different values of coresetPoolIndex in ControlResourceSet, and configured with lte-CRS-PatternList3-r18 and lte-CRS-PatternList4-r18, both lte-CRS-PatternList3-r18 and lte-CRS-PatternList4-r18 are applied

    - For case when UE is configured with two different values of coresetPoolIndex in ControlResourceSet, and configured with lte-CRS-PatternList3-r18 and lte-CRS-PatternList4-r18

      - If UE is configured with crs-RateMatch-PerCoresetPoolIndex, lte-CRS-PatternList3-r18 is applied if the PDSCH is associated with coresetPoolIndex set to ‘0’, and lte-CRS-PatternList4-r18 is applied if the PDSCH is associated with coresetPoolIndex set to ‘1’

      - Otherwise, both lte-CRS-PatternList3-r18 and lte-CRS-PatternList4-r18 are applied.

    - The legacy configuration rule in TS 38.331 is applied in Rel-18 DSS, i.e.,

      - “The first LTE CRS pattern in lte-CRS-PatternList4 shall be fully overlapping in frequency with the first LTE CRS pattern in lte-CRS-PatternList3, The second LTE CRS pattern in this list shall be fully overlapping in frequency with the second LTE CRS pattern in lte-CRS-PatternList3, and so on.”

The working Assumption made in RAN1#109e does not need to be confirmed.

Working Assumption(RAN1#109e)

  - Introduce two new RRC parameters lte-CRS-PatternList3-r18 and lte-CRS-PatternList4-r18 in ServingCellConfig around which the UE shall do rate matching for PDSCH scheduled by PDCCH with CRC scrambled by C-RNTI, MCS-C-RNTI, CS-RNTI, or PDSCHs with SPS.

    - The network does not configure lte-CRS-PatternList3-r18 and any of lte-CRS-ToMatchAround, lte-CRS-PatternList1-r16 and lte-CRS-PatternList2-r16 simultaneously. Lte-CRS-PatternList4-r18 is configured if lte-CRS-PatternList3-r18 is configured in ServingCellConfig.

    - For case when UE is not configured with two different values of coresetPoolIndex in ControlResourceSet, and configured with lte-CRS-PatternList3-r18 and lte-CRS-PatternList4-r18, both lte-CRS-PatternList3-r18 and lte-CRS-PatternList4-r18 are applied

    - For case when UE is configured with two different values of coresetPoolIndex in ControlResourceSet, and configured with lte-CRS-PatternList3-r18 and lte-CRS-PatternList4-r18

      - If UE is configured with crs-RateMatch-PerCoresetPoolIndex, lte-CRS-PatternList3-r18 is applied if the PDSCH is associated with coresetPoolIndex set to ‘0’, and lte-CRS-PatternList4-r18 is applied if the PDSCH is associated with coresetPoolIndex set to ‘1’

      - Otherwise, both lte-CRS-PatternList3-r18 and lte-CRS-PatternList4-r18 are applied.

    - The legacy configuration rule in TS 38.331 is applied in Rel-18 DSS, i.e.,

      - “The first LTE CRS pattern in lte-CRS-PatternList4 shall be fully overlapping in frequency with the first LTE CRS pattern in lte-CRS-PatternList3, The second LTE CRS pattern in this list shall be fully overlapping in frequency with the second LTE CRS pattern in lte-CRS-PatternList3, and so on.”

Agreement

lte-CRS-PatternList3-r18 and lte-CRS-PatternList4-r18 are applicable to 15 kHz SCS PDSCH only.

Agreement

The following TP for sub-clause 5.1.4.2 of TS 38.214 h20 is endorsed in principle.

## 5.1.4.2 PDSCH resource mapping with RE level granularity <omitted text> A UE may be configured with any of the following higher layer parameters: - REs indicated by the 'RateMatchPatternLTE-CRS' in lte-CRS-ToMatchAround in ServingCellConfig or ServingCellConfigCommon configuring cell-specific RS, in 15 kHz subcarrier spacing applicable only to 15 kHz subcarrier spacing PDSCH, of one LTE carrier in a serving cell are declared as not available for PDSCH. - REs indicated by 'RateMatchPatternLTE-CRS' in lte-CRS-PatternList1-r16 or lte-CRS-PatternList3-r18 in ServingCellConfig configuring cell-specific RS, in 15 kHz subcarrier spacing applicable only to 15 kHz subcarrier spacing PDSCH, of one LTE carrier in a serving cell are declared as not available for PDSCH. - For the UE for broadcast reception, REs indicated by 'RateMatchPatternLTE-CRS' in PDSCH-Config-MCCH or PDSCH-Config-MCCH configuring cell-specific RS, in 15 kHz subcarrier spacing applicable only to 15 kHz subcarrier spacing PDSCH, of one LTE carrier in a serving cell are declared as not available for PDSCH. - Each RateMatchPatternLTE-CRS configuration contains v-Shift consisting of LTE-CRS-vshift(s), nrofCRS-Ports consisting of LTE-CRS antenna ports 1, 2 or 4 ports, carrierFreqDL representing the offset in units of 15 kHz subcarriers from (reference) point A to the LTE carrier centre subcarrier location, carrierBandwidthDL representing the LTE carrier bandwidth, and may also configure mbsfn-SubframeConfigList representing MBSFN subframe configuration. A UE determines the CRS position within the slot according to Clause 6.10.1.2 in [15, TS 36.211], where slot corresponds to LTE subframe. - If the UE is configured by higher layer parameter PDCCH-Config with two different values of coresetPoolIndex in ControlResourceSet and is also configured by the higher layer parameter lte-CRS-PatternList1-r16 and lte-CRS-PatternList2-r16 in ServingCellConfig, the following REs are declared as not available for PDSCH: - if the UE is configured with crs-RateMatch-PerCoresetPoolIndex, REs indicated by the CRS pattern(s) in lte-CRS-PatternList1-r16 if the PDSCH is associated with coresetPoolIndex set to ‘0’, or the CRS pattern(s) in lte-CRS-PatternList2-r16 if PDSCH is associated with coresetPoolIndex set to ‘1’; - otherwise, REs indicated by lte-CRS-PatternList1-r16 and lte-CRS-PatternList2-r16, in ServingCellConfig. - If the UE is not configured by higher layer parameter PDCCH-Config with two different values of coresetPoolIndex in ControlResourceSet, and if the UE is configured by higher layer parameter lte-CRS-PatternList3-r18 and lte-CRS-PatternList4-r18 in ServingCellConfig, REs indicated by lte-CRS-PatternList3-r18 and lte-CRS-PatternList4-r18 are declared as not available for PDSCH. - If the UE is configured by higher layer parameter PDCCH-Config with two different values of coresetPoolIndex in ControlResourceSet and is also configured by the higher layer parameter lte-CRS-PatternList3-r18 and lte-CRS-PatternList4-r18 in ServingCellConfig, the following REs are declared as not available for PDSCH: - if the UE is configured with crs-RateMatch-PerCoresetPoolIndex, REs indicated by the CRS pattern(s) in lte-CRS-PatternList3-r18 if the PDSCH is associated with coresetPoolIndex set to ‘0’, or the CRS pattern(s) in lte-CRS-PatternList4-r18 if PDSCH is associated with coresetPoolIndex set to ‘1’; - otherwise, REs indicated by lte-CRS-PatternList3-r18 and lte-CRS-PatternList4-r18, in ServingCellConfig. <omitted text>

Agreement

- Agree the following RRC parameters for support of two overlapping LTE-CRS patterns.

- Send an LS to RAN2 about the agreed RRC parameters together with all RAN1 agreements in AI 9.9.2.

| WI code | Sub-feature group | Parameter name in the spec | New or existing? | Description | Value range | Per (UE, cell, TRP, …) | UE-specific or Cell-specific | Specification |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR_DSS_enh | Support for two overlapping CRS rate matching patterns | lte-CRS-PatternList3 | New | A list of LTE CRS patterns around which the UE shall do rate matching for PDSCH. The LTE CRS patterns in this list shall be non-overlapping in frequency. This list can be configured if [Rel-18 DSS capability(ies)] specified in TS 38.306 [26] is indicated. The network does not configure this field and lte-CRS-ToMatchAround, or this field and lte-CRS-PatternList1, or this field and lte-CRS-PatternList2 simultaneously. | LTE-CRS-PatternList-r16 | ServingCellConfig | UE-specific | 38.331 |
| NR_DSS_enh | Support for two overlapping CRS rate matching patterns | lte-CRS-PatternList4 | New | A list of LTE CRS patterns around which the UE shall do rate matching for PDSCH. The LTE CRS patterns in this list shall be non-overlapping in frequency. This list can be configured if [Rel-18 DSS capability(ies)] specified in TS 38.306 [26] is indicated. The first LTE CRS pattern in this list shall be fully overlapping in frequency with the first LTE CRS pattern in lte-CRS-PatternList3. The second LTE CRS pattern in this list shall be fully overlapping in frequency with the second LTE CRS pattern in lte-CRS-PatternList3, and so on. Network configures this field only if the field lte-CRS-ToMatchAround is not configured and the field lte-CRS-PatternList3 is configured. | LTE-CRS-PatternList-r16 | ServingCellConfig | UE-specific | 38.331 |

Agreement

The draft LS R1-2208193 is endorsed in principle by replacing

- Clarify that the Rel-16 UE capability overlapRateMatchingEUTRA-CRS-r16 is subject to support of multiDCI-Multi-TRP-r16.

With

- Clarify that the Rel-16 UE capability overlapRateMatchingEUTRA-CRS-r16 is subject to support of multiDCI-Multi-TRP-r16 in Rel-18 ASN.1.

Agreement

Final LS R1-2208194 is endorsed.

###### RAN1#110bis-e (October 2022)

Rapporteur Note: No new agreements.

###### RAN1#111 (November 2022)

Rapporteur Note: No new agreements.

###### RAN1#112 (February 2023)

Rapporteur Note: No new agreements.

###### RAN1#112b-e (April 2023)

Agreement

The draft CR in R1-2304277 for Rel-18 TS38.211 eDSS is endorsed in principle.

Agreement

The draft CR in R1-2304195 for Rel-18 TS38.213 eDSS is endorsed in principle.

Agreement

The draft CR in R1-2304202 for Rel-18 TS38.214 eDSS is endorsed in principle.

Agreement:

Introduce following FGs

| 52. NR_DSS_enh | 52-1 | Reception of NR PDCCH candidates overlapping with LTE CRS REs | Reception of NR PDCCH candidates that overlap with LTE CRS REs within a NR carrier using 15 kHz SCS 1) Reception of NR PDCCH candidates in REs that overlap with LTE CRS when UE is provided with LTE CRS RM pattern by configuration of one CRS rate matching pattern [via lte-CRS-ToMatchAround] [2) PDCCH-DMRS channel estimation Value 1: PDCCH-DMRS channel estimation based on legacy CE assumption Value 2: for CORESET duration greater than 1 symbol, PDCCH-DMRS channel estimation based on PDCCH-DMRS REs in symbols not overlapping with LTE CRS Other values FFS] [3) Reception of NR PDCCH candidates that overlap with LTE CRS REs only on the 2nd symbol of an NR slot] [4) NR PDCCH that overlaps with LTE CRS REs is in [Type-1 CSS with dedicated RRC configuration, Type-3 CSS, and/or] USS that are monitored within the first 3 OFDM symbols of a slot] | 5-28 | Yes | N/A | UE is not required to support reception of NR PDCCH candidates overlapping with LTE CRS REs when it is provided with LTE CRS RM pattern by higher layers | [Per Band] | [No] | [No] | [N/A] | [Component 2 candidate value set: {Value 1, Value2, other values FFS}] [Component 3 candidate value set: {Value 1, other values FFS}] [Note: From UE perspective, PDCCH candidates and PDCCH-DMRS RE mapping are based on that of R15] [Note: For component 2, RAN1 consider support legacy CE only if no RAN4 performance requirements are defined for this option] Note: it is RAN1 understanding that the feature is supported by UE performing channel estimation with a regular legacy DMRS pattern in frequency dimension, i.e., no change to UE assumption on PDCCH DMRS RE positions/pattern in a symbol that are used for the purpose of channel estimation. | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 52. NR_DSS_enh | 52-1a | Reception of NR PDCCH candidates overlapping with LTE CRS REs with multiple non-overlapping CRS rate matching patterns | 1) Reception of NR PDCCH candidates in REs that overlap with LTE CRS when UE is provided with LTE CRS RM patterns by configuration of multiple non-overlapping CRS rate matching patterns [via lte-CRS-PatternList1-r16] | 52-1, 14-1 | Yes | N/A |  | [Per Band] | [No] | [No] | [N/A] | Note: it is RAN1 understanding that the feature is supported by UE performing channel estimation with a regular legacy DMRS pattern in frequency dimension, i.e., no change to UE assumption on PDCCH DMRS RE positions/pattern in a symbol that are used for the purpose of channel estimation. | Optional with capability signaling |
| 52. NR_DSS_enh | 52-1b | NR PDCCH reception that overlaps with LTE CRS within a single span of 3 consecutive OFDM symbols that is within the first 4 OFDM symbols in a slot | 1) NR PDCCH that overlaps with LTE CRS REs is in [Type-1 CSS with dedicated RRC configuration, Type-3 CSS, and/or] USS that are monitored within a single span of 3 consecutive OFDM symbols that is within the first 4 OFDM symbols in a slot | 52-1, 22-12 | Yes | N/A |  | [Per Band] | [No] | [No] | [N/A] |  | Optional with capability signaling |
| 52. NR_DSS_enh | 52-2 | Two LTE-CRS overlapping rate matching patterns within a part of NR carrier using 15 kHz overlapping with a LTE carrier [(regardless of support or configuration of multi-TRP)] | 1) Support of two LTE-CRS overlapping rate matching patterns configured by lte-CRS-PatternList3-r18 and lte-CRS-PatternList4-r18 within a part of NR carrier using 15 kHz overlapping with a LTE carrier [(regardless of support or configuration of multi-TRP)] 2) Maximum number of LTE-CRS rate matching patterns in total within a NR carrier using 15 kHz SCS: {2,3,4,5,6} - FFS: total number reported by UE in FG 14-1 and FG 52-2 shall not exceed 6 - FFS: Note: If a UE supports FG52-2 and FG14-1, the maximum numbers reported for FG 52-2 should be equal to or larger than the numbers reported for FG 14-1 3) Maximum number of LTE-CRS non-overlapping rate matching patterns within a NR carrier using 15 kHz SCS: {1,2,3} - FFS: total number reported by UE in FG 14-1 and FG 52-2 shall not exceed 3 - FFS: Note: If a UE supports FG52-2 and FG14-1, the maximum numbers reported for FG 52-2 should be equal to or larger than the numbers reported for FG 14-1 | [5-28] | Yes | N/A | [UE support for two overlapping LTE-CRS RM patterns within a part of NR carrier using 15 kHz overlapping with a LTE carrier when UE is not supporting or configured with multi-TRP is not enabled] | [Per Band] | [No] | [No] | [N/A] | FFS: Note: If a UE supports FG52-2 and FG14-1, the maximum numbers reported for FG 52-2 should be equal to or larger than the numbers reported for FG 14-1 | Optional with capability singaling |
| 52. NR_DSS_enh | 52-2a | Two LTE-CRS overlapping rate matching patterns with two different values of coresetPoolIndex within a part of NR carrier using 15 kHz overlapping with a LTE carrier | Support of two LTE-CRS overlapping rate matching patterns configured by lte-CRS-PatternList3-r18 and lte-CRS-PatternList4-r18 with two different values of coresetPoolIndex within a part of NR carrier using 15 kHz overlapping with a LTE carrier FFS additional clarification of relation between Rel-16 capability | 52-2, 16-2a | Yes | N/A |  | [Per Band] | [No] | [No] | [N/A] |  | Optional with capability singaling |
| 52. NR_DSS_enh | 52-3 | FFS: Support reception of NR PDCCH candidates overlapping with LTE CRS REs when two LTE-CRS overlapping rate matching patterns are configured |  | 52-1, at least one of 52-2 or 14-1a | Yes | N/A |  | [Per Band] | No | No | N/A |  |  |

###### RAN1#113 (May 2023)

Agreement

- FG 52-2 is confirmed as: Two LTE-CRS overlapping rate matching patterns within a part of NR carrier using 15 kHz overlapping with a LTE carrier [(regardless of support or configuration of multi-TRP)]

- Component 1 in FG 52-2 is confirmed as: Support of two LTE-CRS overlapping rate matching patterns configured by lte-CRS-PatternList3-r18 and lte-CRS-PatternList4-r18 within a part of NR carrier using 15 kHz overlapping with a LTE carrier [(regardless of support or configuration of multi-TRP)] for the case when crs-RateMatchPerCoresetPoolIndex is not configured

Agreement

- “FFS: total number reported by UE in FG 14-1 and FG 52-2 shall not exceed 6” is removed from component 2 in FG 52-2

- “FFS: Note: If a UE supports FG52-2 and FG14-1, the maximum numbers reported for FG 52-2 should be equal to or larger than the numbers reported for FG 14-1” is removed from component 2 in FG 52-2

- “FFS: total number reported by UE in FG 14-1 and FG 52-2 shall not exceed 3” is removed from component 3 in FG 52-2

- “FFS: Note: If a UE supports FG52-2 and FG14-1, the maximum numbers reported for FG 52-2 should be equal to or larger than the numbers reported for FG 14-1” is removed from component 3 in FG 52-2

- Add a note in FG 52-2: If a UE supports FG52-2 and FG14-1, FG14-1 is reported for list1/2 and FG52-2 is reported for list3/4

Agreement

- Prerequisite FG for FG 52-2 is confirmed as FG 5-28

Agreement

- The consequence if UE does not support FG52-2 is “UE does not support two LTE-CRS overlapping rate matching patterns configured by lte-CRS-PatternList3-r18 and lte-CRS-PatternList4-r18 within a part of NR carrier using 15 kHz overlapping with a LTE carrier”

Agreement

- “FFS additional clarification of relation between Rel-16 capability” is removed from FG 52-2a

- Component 1 in FG 52-2a is confirmed as: Support of two LTE-CRS overlapping rate matching patterns configured by lte-CRS-PatternList3-r18 and lte-CRS-PatternList4-r18 with two different values of coresetPoolIndex within a part of NR carrier using 15 kHz overlapping with a LTE carrier for the case when crs-RateMatchPerCoresetPoolIndex is configured

Agreement

- Reporting type of FG52-2/2a is per band for FR1 only

###### RAN1#114 (Aug 2023)

Rapporteur Note: Agreed RAN1 CRs listed below.

| TD# | Title | Source to WG | Rel | TS | Vsn | Work Item | CR | Rev | Cat |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| R1-2308724 | Introduction of dynamic spectrum sharing enhancements | Nokia | Rel-18 | 38.214 | 17.6.0 | NR_DSS_enh-Core | 0443 |  | B |
| R1-2308727 | Introduction of dynamic spectrum sharing enhancements | Ericsson | Rel-18 | 38.211 | 17.5.0 | NR_DSS_enh-Core | 0112 |  | B |

###### RAN1#114bis (October 2023)

Rapporteur Note: Below are agreements from maintenance discussions (including UE capability updates) for this WI.

Agreement

- The components 2 and 3 of FG52-2 are updated as below.

  - 2) Maximum number of LTE-CRS rate matching patterns in total within a NR carrier using 15 kHz SCS with candidate value set: {2,3,4,5,6}

  - 3) Maximum number of LTE-CRS non-overlapping rate matching patterns within a NR carrier using 15 kHz SCS with candidate value set: {1,2,3}

###### RAN1#115 (November 2023)

Rapporteur Note: No new agreements.

#### 2.1.2	Remaining Open issues

- None

## 2.2	RAN2

Rapporteur Note: Core part completed in RAN#102 in Dec 2023.

#### 2.2.1	Agreements

###### RAN2#119bis-e (October 2022)

Endorse the Rel-18 TS 38.331 CR, the modification is the same as the TS 38.331 TP in R2-2210297.

Endorse the Rel-18 TS 38.306 CR, the modification is same as in R2-2210586, no need to update Rel-16/17 specs with the assumption that the same condition is already applicable to Rel-16/17 UEs based on RAN1 spec.

RAN2 waits for RAN1 regarding the RRC configuration and UE capability for PDCCH on CRS

R2-2210993	Running 38.331 CR for R18 DSS		Ericsson, ZTE Cporporation	DraftCR	Rel-18	38.331	NR_DSS_enh

Running CR is endorsed

R2-2210994	Running 38.306 CR for R18 DSS		Ericsson, ZTE Cporporation	DraftCR	Rel-18	38.306	NR_DSS_enh

Running CR is endorsed

###### RAN2#120 (November 2022)

R2-2211910	Running TS38.306 CR for R18 DSS	ZTE Corporation, Ericsson	draftCR	Rel-18	38.306	17.2.0	B	NR_DSS_enh

R2-2212386	Running 38.331 CR for R18 DSS	Ericsson, ZTE Corporation	CR	Rel-18	38.331	17.2.0	3697	-	B	NR_DSS_enh-Core

Both CRs above are endorsed (as running CRs)

###### RAN2#121 (February 2023)

Rapporteur Note: No new agreements.

###### RAN2#121bis-e (April 2023)

Rapporteur Note: No new agreements.

###### RAN2#122 (May 2023)

Rapporteur Note: No new agreements.

###### RAN2#123 (Aug 2023)

Rapporteur Note: No new agreements.

###### RAN2#123bis (October 2023)

R2-2310959	Running 38.331 CR for R18 DSS	Ericsson, ZTE Corporation	CR	Rel-18	38.331	17.6.0	3888	1	B	NR_DSS_enh-Core	R2-2301405

=>	The CR is agreed in principle

R2-2310673	Running 38.306 CR for R18 DSS	ZTE Corporation, Ericsson	draftCR	Rel-18	38.306	17.6.0	B	NR_DSS_enh

=>	The CR is agreed in principle

###### RAN2#124 (November 2023)

R2-2312770	Introduction of R18 DSS in 38.306	ZTE Corporation, Ericsson	CR	Rel-18	38.306	17.6.0		0993	-	B	NR_DSS_enh

=>	The CR is endorsed and will be merged with mega CR

R2-2312995	Introduction of R18 DSS	Ericsson, ZTE Corporation	CR	Rel-18	38.331	17.6.0		4360	3	B	NR_DSS_enh-Core	R2-2312993

=>	The CR is agreed

R2-2312144	Running UE capability CR on 38.306  for Rel-18 R1 R4 feature lists	Intel Corporation	draftCR	Rel-18	38.306	17.6.0	NR_MIMO_evo_DL_UL, NR_netcon_repeater, NR_DSS_enh, NR_MC_enh, NR_FR1_lessthan_5MHz_BW, NR_BWP_wor, NR_redcap_enh, NR_XR_enh, TEI18

=>	The CR is endorsed

R2-2312145	Running UE capability CR on 38.331  for Rel-18 R1 R4 feature lists	Intel Corporation	draftCR	Rel-18	38.331	17.6.0	NR_MIMO_evo_DL_UL, NR_netcon_repeater, NR_DSS_enh, NR_MC_enh, NR_FR1_lessthan_5MHz_BW, NR_BWP_wor, NR_redcap_enh, NR_XR_enh, TEI18

=>	The CR is endorsed

#### 2.2.2	Remaining Open issues

- None

## 2.3	RAN3

#### 2.3.1	Agreements

#### 2.3.2	Remaining Open issues

## 2.4	RAN4

RAN4 work is scheduled to start in Q4, 2023 according to agreed time plan in [2],[3].

#### 2.4.1	Agreements

##### NR PDCCH reception in symbols with LTE CRS REs

###### RAN4#108bis (October 2023)

Whether to define PDCCH demodulation requirements for Rel-18 eDSS feature

Agreement:

- Define PDCCH demodulation requirements for eDSS

Evaluation scenario

Agreement:

- 2-symbol CORESET

- Basic assumptions

  - Rate matching pattern

    - Option 1: Single non-overlapping CRS rate matching patterns for PDCCH demodulation.

    - Other options not precluded

  - 4 CRS ports for LTE

  - Symbol #0: LTE PDCCH/PCFICH

  - Symbol #1: NR PDCCH overlapping with LTE CRS

  - Symbol #2: NR PDCCH

gNB assumption for PDCCH symbols overlapped with LTE CRS

Agreement:

- PDCCH data and DMRS REs overlapped with LTE CRS are punctured.

UE receiver assumption (e.g., channel estimation)

Agreement:

- PDCCH channel estimation is assumed to use only the clean PDCCH symbol.

###### RAN4#109 (November 2023)

CRS rate matching pattern assumption

Agreement:

- Single LTE CRS rate matching pattern

UE receiver assumption

Agreement:

- Leave the receiver design to implementation for evaluation purposes.

- Further check submitted results. If there is a large variation in results, then discuss receiver assumptions.

Common parameters

Agreement:

- DCI format: 1_0

- CCE to REG mapping: Non-interleaved

- REG bundle size: 6 PRB

- Precoder: Random precoder with precoder cycling per REG bundle

- LTE CRS: 4 Ports

- TX assumption for overlapping REs: Puncture PDCCH and DMRS REs overlapping with LTE CRS

- LTE PDCCH, PDSCH:  No transmission

- Slots with LTE PBCH/PSS/SSS: No PDCCH transmission

- Channel estimation: PDCCH CE only on clean symbols

Payload bits

Agreement:

- 39 bits

Channel bandwidth

Agreement:

- For FDD, consider 10MHz/15kHz

Aggregation level

Agreement:

- 4, 8 (for evaluation purpose)

Channel model

Agreement:

- TDLA30-10 and TDLC300-100

- Discuss whether to down-select in the next meeting

Power ratio of LTE-CRS RE/NR PDCCH-DMRS RE

Agreement:

- 0dB

###### RAN4#110 (February 2024)

Antenna configuration

Agreement:

- 2x4, 4x2 for both FDD and TDD

Channel bandwidth

Agreement:

- For TDD, consider 10MHz/15kHz

Aggregation level

Agreement:

- 4, 8

Channel model

Agreement:

- TDLA30-10 and TDLC300-100

Payload

Agreement:

- Remain 39 bits

PDCCH test cases for requirement

Agreement:

| Test number | Duplex | Bandwidth (MHz) | CORESET RB | CORESET duration | Aggregation level | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) |
| 1 | FDD | 10 | 48 | 2 | 4 | TDLA30-10 | 2x4 Low | 1 | TBD |
| 2 | FDD | 10 | 48 | 2 | 8 | TDLC300-100 | 4x2 Low | 1 | TBD |
| 3 | TDD | 10 | 48 | 2 | 4 | TDLA30-10 | 2x4 Low | 1 | TBD |
| 4 | TDD | 10 | 48 | 2 | 8 | TDLC300-100 | 4x2 Low | 1 | TBD |

How to align the results

Way Forward:

- Companies are encouraged to double check their submitted results and update in the next meeting if necessary.

###### RAN4#110bis (April 2024)

Whether to change the current test case setup

Agreement:

  - Agreed test cases for requirement:

| Test number | Duplex | Bandwidth (MHz) | CORESET RB | CORESET duration | Aggregation level | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) |
| 1 | FDD | 10 | 48 | 2 | 4 | TDLA30-10 | 2X4 Low | 1 | TBD |
| 2 | FDD | 10 | 48 | 2 | 8 | TDLC300-100 | 4X2 Low | 1 | TBD |
| 3 | TDD | 10 | 48 | 2 | 4 | TDLA30-10 | 4x4 Low | 1 | TBD |
| 4 | TDD | 10 | 48 | 2 | 8 | TDLC300-100 | 2x2 Low | 1 | TBD |

Payload

Agreement:

  - Remain 39bits for all test cases unless any issue identified.

Following draft CRs are endorsed:

R4-2406014	Draft CR to 38.101-4: PDCCH requirement for DSS enhancement (FDD 2Rx)	Type: draftCR	38.101-4 v18.3.0	CR-  rev  Cat: B (Rel-18) 	Source: MediaTek inc.

R4-2406015	DraftCR for PDCCH requirements for eDSS - TDD with 2RX		Type: draftCR	38.101-4 v18.3.0	CR-  rev  Cat: B (Rel-18)	Source: Apple

R4-2406016	draft CR Reference measurement channel for DSS enhancement demodulation requirement FDD and TDD	Type: draftCR	38.101-4 v18.3.0	  CR-  rev  Cat: B (Rel-18)	Source: Huawei,HiSilicon

R4-2406017	Draft CR on introduction of PDCCH requirement for DSS enhancement -TDD 4Rx		Type: draftCR	38.101-4 v18.3.0	  CR-  rev  Cat: B (Rel-18)	Source: ZTE Corporation

R4-2406018	Drat CR on the Introduction of PDCCH requirement for DSS enhancement -FDD 4Rx	Type: draftCR	38.101-4 v18.3.0	  CR-  rev  Cat: B (Rel-18)	Source: Nokia

R4-2405485	Draft BigCR to 38.101-4 on Introducing PDCCH demodulation requirements for DSS enhancement	Type: draftCR	38.101-4 v18.3.0	  CR-  rev  Cat: B (Rel-18)	Source: Ericsson

###### RAN4#111 (May 2024)

Following draft CRs are endorsed:

R4-2409873	DraftCR for PDCCH requirements for eDSS - TDD with 2RX	Type: draftCR	38.101-4 v18.3.0	  CR-  rev  Cat: B (Rel-18)	Source: Apple

R4-2409992	Drat CR on the Introduction of PDCCH requirement for DSS enhancement -FDD 4Rx	Type: draftCR	38.101-4 v18.3.0	  CR-  rev  Cat: B (Rel-18)	Source: Nokia

R4-2409990	[NR_DSS_enh-Perf] Draft CR to 38.101-4 Applicability rule for NR PDCCH demodulation requirements under DSS scenario		Type: draftCR	38.101-4 v18.3.0	  CR-  rev  Cat:  (Rel-18)	Source: Qualcomm India Pvt Ltd

R4-2409991  Draft CR to 38.101-4: PDCCH requirement for DSS enhancement (FDD 2Rx)	Source: MediaTek

R4-2409849  Draft CR on introduction of PDCCH requirement for DSS enhancement -TDD 4Rx	Source: ZTE Corporation,Sanechips

Following big CR is agreed by post meeting email approval:

R4-2408753	Big CR to 38.101-4 Introduction of PDCCH requirement for DSS enhancement	Type: CR	38.101-4 v18.3.0	  CR-0536  rev  Cat: B (Rel-18)	Source: Ericsson

##### UE support for two overlapping CRS rate matching patterns

###### RAN4#108bis (October 2023)

Whether to define the PDSCH demodulation requirements with two overlapping CRS rate matching patterns

Agreement:

- Not define PDSCH demodulation requirements.

###### RAN4#109 (November 2023)

Rapporteur Note: No new agreements.

###### RAN4#110 (February 2024)

Rapporteur Note: No new agreements.

###### RAN4#110bis (April 2024)

Rapporteur Note: No new agreements.

###### RAN4#111 (May 2024)

Rapporteur Note: No new agreements.

#### 2.4.2	Remaining Open issues

- None

## 2.5	RAN5

#### 2.5.1	Agreements

#### 2.5.2	Remaining Open issues

#### 2.5.3	Remaining Open issues with cross-WG dependencies

## 2.6	RAN6

#### 2.6.1	Agreements

#### 2.6.2	Remaining Open issues

## 3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

## 3.1	SAx/CTs

#### 3.1.1	Agreements with cross-TSG impacts

#### 3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

## 4.	References

#### General

[1] RP-221622 – “Revised WID on Enhancement of NR Dynamic spectrum sharing (DSS)”, Ericsson, RAN#96, Budapest, Jun 2022.

[2] RP-213469 – “Summary for RAN Rel-18 Package”, RAN Chair (Qualcomm), RAN1 Chair (Samsung), RAN2 Chair (MediaTek), RAN3 Chair (ZTE), RAN4 Chair (Huawei), RAN#94e, Dec 2021.

[3] RP-213697 – “Overview of time unit allocation for all RAN REL-18 projects”,	RAN1 Chair (Samsung), RAN2 Chair (MediaTek), RAN3 Chair (ZTE), RAN4 Chair (Huawei), RAN#94e, Dec 2021.

#### RAN1#109-e (May 2022)

45 contributions (for details, please see A.I. 9.9, 9.9.1, 9.9.2 and 9.9.3 in RAN1#109-e Tdoc list - link)

#### RAN1#110 (August 2022)

40 contributions (for details, please see A.I. 9.9, 9.9.1, 9.9.2 in RAN1#110 Tdoc list - link)

#### RAN1#110bis-e (October 2022)

R1-2210358	Moderator summary for Rel. 18 NR DSS	Moderator (Intel Corporation)

R1-2208491	Discussion on remaining issues for Rel-18 DSS	ZTE

R1-2208578	Maintenance for Rel-18 DSS	Spreadtrum Communications

R1-2208675	Remaining issues on Rel-18 NR Dynamic spectrum sharing	vivo

R1-2208859	Discussion on remaining issues in Rel-18 eDSS	OPPO

R1-2209275	Discussion on NR PDCCH reception in symbols with LTE CRS REs	xiaomi

R1-2209814	Remaining issues on NR PDCCH reception in symbols with LTE CRS REs	Huawei, HiSilicon

R1-2209929	Remaining issues on NR PDCCH reception in symbols with LTE CRS REs	NTT DOCOMO, INC.

R1-2210017	Maintenance for Rel-18 DSS	Qualcomm Incorporated

R1-2210193	[eDSS] RRC configuration and UE capability for PDCCH on CRS	Nokia, Nokia Shanghai Bell

#### RAN2#119bis-e (October 2022)

R2-2210992	Report of [AT119bis-e][016][NR18] DSS enhancement (ZTE)	ZTE

R2-2210993	Running 38.331 CR for R18 DSS		Ericsson, ZTE Cporporation	DraftCR	Rel-18	38.331	NR_DSS_enh

R2-2210994	Running 38.306 CR for R18 DSS		Ericsson, ZTE Cporporation	DraftCR	Rel-18	38.306	NR_DSS_enh

R2-2209314	LS to RAN2 on two overlapping LTE-CRS patterns in Rel-18 DSS (R1-2208194; contact: ZTE)	RAN1	LS in	Rel-18	NR_DSS_enh	To:RAN2

R2-2210636	Work plan for Rel18 WI on Enhancement of NR Dynamic spectrum sharing (DSS)	Ericsson	discussion

R2-2210133	RRC configuration and UE capability for PDCCH on CRS	Nokia, Nokia Shanghai Bell	discussion	Rel-18	NR_DSS_enh

R2-2210297	Discussion on two overlapping LTE-CRS patterns in Rel-18 DSS	ZTE Corporation, Sanechips, Ericsson	discussion	Rel-18	NR_DSS_enh-Core

#### RAN1#111 (November 2022)

R1-2211037	Remaining issues on Rel-18 NR Dynamic spectrum sharing	vivo

R1-2211286	[eDSS] RRC configuration and UE capability for PDCCH on CRS	Nokia, Nokia Shanghai Bell

R1-2211487	Discussion on remaining issues in Rel-18 eDSS	OPPO

R1-2211652	Discussion on remaining issues for Rel-18 DSS	ZTE

R1-2212013	Remaining issues on NR PDCCH reception in symbols with LTE CRS REs	NTT DOCOMO, INC.

R1-2212454	Remaining issues on NR PDCCH reception in symbols with LTE CRS REs	Huawei, HiSilicon

#### RAN2#120 (November 2022)

R2-2211105	LS to RAN2 on two overlapping LTE-CRS patterns in Rel-18 DSS (R1-2208194; contact: ZTE)	RAN1	LS in	Rel-18	NR_DSS_enh	To:RAN2

R2-2211910	Running TS38.306 CR for R18 DSS	ZTE Corporation, Ericsson	draftCR	Rel-18	38.306	17.2.0	B	NR_DSS_enh

R2-2212386	Running 38.331 CR for R18 DSS	Ericsson, ZTE Corporation	CR	Rel-18	38.331	17.2.0	3697	-	B	NR_DSS_enh-Core

#### RAN1#112 (February 2023)

R1-2300485	Remaining issues on Rel-18 NR Dynamic spectrum sharing	vivo

R1-2301060	[eDSS] RRC configuration and UE capability for PDCCH on CRS	Nokia, Nokia Shanghai Bell

R1-2301524	Remaining issues on NR PDCCH reception in symbols with LTE CRS REs	NTT DOCOMO, INC.

R1-2301572	Discussion on remaining issues in Rel-18 eDSS	OPPO

R1-2301711	Remaining issues on NR PDCCH reception in symbols with LTE CRS REs	Huawei, HiSilicon

R1-2301744	Discussion on remaining issues for Rel-18 DSS	ZTE

#### RAN2#121 (February 2023)

R2-2301321	Running 38.306 CR for R18 DSS	ZTE Corporation, Ericsson	draftCR	Rel-18	38.306	17.3.0	B	NR_DSS_enh-Core

R2-2301405	Running 38.331 CR for R18 DSS	Ericsson, ZTE Corporation	CR	Rel-18	38.331	17.3.0	3888	-	B	NR_DSS_enh-Core

#### RAN1#112b-e (April 2023)

R1-2302516	Discussion on UE features for eDSS	vivo

R1-2302578	Discussion on UE features for Rel-18 eDSS	OPPO

R1-2302626	Discussion on UE features for eDSS	Spreadtrum Communications

R1-2302764	Discussion on UE feature for eDSS	ZTE

R1-2302896	Initial views on UE features for eDSS	Nokia, Nokia Shanghai Bell

R1-2303160	Discussion on UE features for eDSS	Samsung

R1-2303342	UE feature discussion for Rel-18 eDSS	MediaTek Inc.

R1-2303513	On Rel-18 eDSS UE features	Apple

R1-2303622	UE features for eDSS	Qualcomm Incorporated

R1-2303737	Discussion on UE features for eDSS	NTT DOCOMO, INC.

R1-2303763	Initial UE feature list for Rel18 DSS Enhancements WI	WI Rapporteur(Ericsson)

R1-2303764	UE features for Rel18 DSS Enhancements	Ericsson

R1-2303864	UE features for eDSS	Huawei, HiSilicon

R1-2304214	Summary on UE features for eDSS		Moderator (NTT DOCOMO, INC.)

R1-2304225	Session Notes for 9.17.14	Ad-Hoc Chair (NTT DOCOMO, INC.)

R1-2302518	Discussion on Rel-18 Multi-carrier enhancements, eDSS, NCR and BWP without restriction	vivo

R1-2302579	Discussion on remaining issues in Rel-18 eDSS and MC-enh	OPPO

R1-2302740	Introduction of dynamic spectrum sharing enhancements	Ericsson LM

R1-2302884	On MC-enhancements, BWP without restrictions and eDSS	Nokia, Nokia Shanghai Bell

R1-2303162	Introduction of dynamic spectrum sharing (DSS) enhancements		Samsung

R1-2303166	Discussion on remaining issues for eDSS and MC	Spreadtrum Communications

R1-2303404	Draft CRs and higher layer signalling for others	ZTE

R1-2303624	Views on MC-Enh, BWP without restriction, and eDSS		Qualcomm Incorporated

R1-2303740	Discussion on remaining issues on MC enhancements, eDSS and BWP without restriction	NTT DOCOMO, INC.

R1-2303765	Initial higher layer parameter list for Rel18 DSS Enhancements WI	WI Rapporteur(Ericsson)

R1-2303766	Discussion on MCE, eDSS and NCR for Rel18	Ericsson

R1-2303858	Remaining issues for eDSS, NCR, multicarrier enhancements	Huawei, HiSilicon

R1-2304003	Introduction of dynamic spectrum sharing enhancements	Nokia

R1-2304191	Summary of email discussions [112bis-e-R18-38.213-NR_eDSS]	Moderator (Samsung)

R1-2304195	Introduction of dynamic spectrum sharing (DSS) enhancements	Samsung

R1-2304202	Introduction of dynamic spectrum sharing enhancements	Nokia

R1-2304207	Summary of email discussion on eDSS enhancements [112bis-e-R18-38.214-eDSS]	Moderator (Nokia)

R1-2304220	Summary of Email discussion on Rel-18 RRC parameters for LS to RAN2 and RAN3	Moderator (Ericsson)

R1-2304221	Collection of updated higher layers parameter list for Rel-18  NR	Moderator (Ericsson)

R1-2304238	Consolidated higher layers parameter list for Rel-18 NR	Moderator (Ericsson)

R1-2304277	Introduction of dynamic spectrum sharing enhancements	Ericsson

#### RAN1#113 (May 2023)

R1-2304515	Discussion on UE features for eDSS	vivo

R1-2304585	Discussion on UE features for eDSS	Spreadtrum Communications

R1-2304605	Discussion on UE feature for eDSS	ZTE

R1-2304672	UE features for eDSS	Huawei, HiSilicon

R1-2304889	Discussion on UE features for eDSS	xiaomi

R1-2305282	On Rel-18 eDSS UE features	Apple

R1-2305376	UE features for eDSS	Qualcomm Incorporated

R1-2305472	Discussion on UE features for eDSS enhancement	OPPO

R1-2305547	UE features for eDSS	Samsung

R1-2305579	UE features for Rel18 DSS Enhancements	Ericsson

R1-2305626	Discussion on UE features for eDSS	NTT DOCOMO, INC.

R1-2305668	UE feature discussion for Rel-18 eDSS	MediaTek Inc.

R1-2305727	On UE features for eDSS	Nokia, Nokia Shanghai Bell

R1-2306187	Session Notes for 9.16.13	Ad-Hoc Chair (NTT DOCOMO)

R1-2306193	Summary on UE features for eDSS	Moderator (NTT DOCOMO, INC.)

R1-2304517	Discussion on Rel-18 Multi-carrier enhancements, eDSS and NCR	vivo

R1-2304586	Discussion on remaining issues in RRC signaling for Rel-18 multi-carrier enhancement and eDSS	Spreadtrum Communications

R1-2305475	Discussion on remaining issues in Rel-18 eDSS	OPPO

R1-2305627	Discussion on higher layer parameters and remaining issues on Rel-18 WIs	NTT DOCOMO, INC.

R1-2305716	Higher layer signaling for NR Rel-18	Ericsson

R1-2306113	Summary of higher layer parameters for R18 DSS Enhancements	Moderator (Ericsson)

R1-2306266	Discussion summary on higher layer parameter list for Rel-18 DSS Enhancements WI	WI Rapporteur (Ericsson)

#### RAN1#114 (Aug 2023)

R1-2308515	Session Notes for 9.16.13	Ad-Hoc Chair (NTT DOCOMO)

R1-2306484	UE features for eDSS	Huawei, HiSilicon

R1-2306595	On UE features for eDSS	Nokia, Nokia Shanghai Bell

R1-2306679	Discussion on UE features for eDSS	Spreadtrum Communications

R1-2306787	Discussion on UE features for eDSS	vivo

R1-2306991	Discussion on UE feature for eDSS	ZTE

R1-2307320	Views On UE features for Rel-18 NR eDSS	Apple

R1-2307365	Discussion on UE features for eDSS	xiaomi

R1-2307507	Discussion on UE features for eDSS	NTT DOCOMO, INC.

R1-2307580	Discussion on UE features for eDSS enhancement	OPPO

R1-2307721	UE features for eDSS	Samsung

R1-2307970	UE features for eDSS	Qualcomm Incorporated

R1-2307994	UE features for Rel18 DSS Enhancements	Ericsson

R1-2306485	Discussions on draft CRs and higher layer signalling for other Rel-18 items	Huawei, HiSilicon

R1-2307510	Discussion on higher layer parameters and remaining issues on Rel-18 Wis	NTT DOCOMO, INC.

#### RAN1#114bis (October 2023)

R1-2308855	On UE features for eDSS	Nokia, Nokia Shanghai Bell

R1-2308921	UE features for eDSS	Huawei, HiSilicon

R1-2308922	Maintenance of NR PDCCH reception in symbols with LTE CRS REs	Huawei, HiSilicon

R1-2309135	Maintenance of NR PDCCH reception in symbols with LTE CRS	ZTE

R1-2309140	Discussion on UE feature for eDSS	ZTE

R1-2309622	Remaining issues for PDCCH reception in overlapping with LTE CRS	OPPO

R1-2309629	Discussion on UE features for eDSS	OPPO

R1-2310009	Remaining Issues on NR PDCCH Reception in Symbols with LTE CRS REs	Langbo

R1-2310045	Maintenance on NR PDCCH reception in symbols with LTE CRS REs	NTT DOCOMO, INC.

R1-2310064	Discussion on UE features for eDSS	NTT DOCOMO, INC.

R1-2310096	Maintenance for reception of NR PDCCH candidates overlapping with LTE CRS REs	Ericsson

R1-2310099	UE features for Rel18 DSS Enhancements		Ericsson

R1-2310155	NR PDCCH reception in symbols with LTE CRS REs	Qualcomm Incorporated

R1-2310177	UE features for eDSS	Qualcomm Incorporated

R1-2310361	Moderator Summary#1 for Rel. 18 eDSS	Moderator (Intel Corporation)

R1-2310549	Session notes for 8.10 (Maintenance on enhancement of NR Dynamic Spectrum Sharing)	Ad-Hoc Chair (CMCC)

R1-2310596	Endorsed text proposals for TS38.211 and TS38.213 for Rel. 18 eDSS	Moderator (Intel Corporation)

R1-2310621	Summary on UE features for eDSS	Moderator (NTT DOCOMO, INC.)

R1-2310629	Session Notes for 8.16.10	Ad-Hoc Chair (NTT DOCOMO)

#### RAN2#123bis (October 2023)

R2-2310953	RAN2 updates for DSS	Ericsson, ZTE Corporation	discussion

R2-2310959	Running 38.331 CR for R18 DSS		Ericsson, ZTE Corporation	CR	Rel-18

R2-2310673	Running 38.306 CR for R18 DSS		ZTE Corporation, Ericsson	draftCR	Rel-18

#### RAN4#108bis (October 2023)

R4-2315849	Work plan for Enhancement of NR dynamic spectrum sharing WI	Ericsson

R4-2316639	Enhancement of NR Dynamic Spectrum Sharing: General and Work Plan	Nokia, Nokia Shanghai Bell

R4-2315347	Discussion on the demodulation performance requirements for enhanced NR dynamic spectrum sharing	Qualcomm Inc

R4-2315488	UE demodulation performance requirements for NR dynamic spectrum sharing	Apple

R4-2315711	Discussion on NR DSS demodulation requirements	ZTE Corporation

R4-2315850	On the UE demodulation requirements for enhancement of NR Dynamic Spectrum Sharing (DSS)	Ericsson

R4-2315989	Discussions on Rel-18 DSS demodulation and CSI requirements	Huawei, HiSilicon

R4-2316470	Discussion on requirements for DSS enhancements	MediaTek inc.

R4-2316640	Enhancement of NR Dynamic Spectrum Sharing: UE Demodulation Performance Requirements	Nokia, Nokia Shanghai Bell

R4-2317959	Topic summary for [108bis][328] NR_DSS_enh	Moderator (Ericsson)

R4-2316964	Offline meeting minutes on NR_DSS_enh		Ericsson

R4-2317000	WF on NR_DSS_enh	Ericsson

#### RAN1#115 (November 2023)

R1-2310898	On UE features for eDSS	Nokia, Nokia Shanghai Bell

R1-2311023	Maintenance of NR PDCCH reception in symbols with LTE CRS	ZTE

R1-2311190	Discussion on UE features for eDSS	Spreadtrum Communications

R1-2311274	Remaining issues for PDCCH reception in overlapping with LTE CRS	OPPO

R1-2311450	Monitoring of PDCCH candidates overlapping with LTE CRS	Nokia, Nokia Shanghai Bell

R1-2311696	Maintenance of NR PDCCH reception in symbols with LTE CRS	Apple

R1-2312153	UE features for eDSS	Huawei, HiSilicon

R1-2312219	Maintenance of NR PDCCH reception in symbols with LTE CRS REs	Huawei, HiSilicon

R1-2312367	Moderator Summary#1 for Rel. 18 eDSS	Moderator (Intel Corporation)

R1-2312509	Session notes for 8.10 (Maintenance on enhancement of NR Dynamic Spectrum Sharing)	Ad-Hoc Chair (Huawei)

R1-2312580	Session Notes for 8.16.10	Ad-Hoc Chair (NTT DOCOMO)

R1-2312588	Summary on UE features for eDSS	Moderator (NTT DOCOMO)

#### RAN2#124 (November 2023)

R2-2312770	Introduction of R18 DSS in 38.306	ZTE Corporation, Ericsson	CR	Rel-18

R2-2312995	Introduction of R18 DSS	Ericsson, ZTE Corporation	CR	Rel-18	38.331

R2-2312144	Running UE capability CR on 38.306  for Rel-18 R1 R4 feature lists	Intel Corporation

R2-2312145	Running UE capability CR on 38.331  for Rel-18 R1 R4 feature lists	Intel Corporation

#### RAN4#109 (November 2023)

R4-2318588	UE demodulation performance requirements for NR dynamic spectrum sharing	Apple

R4-2318664	Discussion on PDCCH requirements for DSS enhancements	MediaTek inc.	Noted

R4-2318729	Discussion on the demodulation performance requirements for enhanced NR dynamic spectrum sharing	Qualcomm India Pvt Ltd

R4-2319222	On UE demodulation requirement for enhancement of Dynamic Spectrum Sharing	Ericsson

R4-2319545	Discussion on NR DSS demodulation requirements	ZTE Corporation

R4-2320200	Discussions on performance requirements for Rel-18 DSS	Huawei, HiSilicon

R4-2320585	Enhancement of NR Dynamic Spectrum Sharing: UE Demodulation Performance Requirements	Nokia, Nokia Shanghai Bell

R4-2318223	Topic summary for [109][331] NR_DSS_enh	Moderator (Ericsson)

R4-2321139	WF on [109][331] NR_DSS_enh	Ericsson

#### RAN4#110 (February 2024)

R4-2400468	UE demodulation performance requirements for NR dynamic spectrum sharing	Apple

R4-2400529	Discussion on PDCCH requirements for DSS enhancement	MediaTek inc.

R4-2400530	Simulation results  on PDCCH requirements for DSS enhancement	MediaTek inc.

R4-2400734	[NR_DSS_enh-Perf]Discussion on the demodulation performance requirements for DSS enhancements	Qualcomm India Pvt Ltd

R4-2401550	On PDCCH demodulation requirements for DSS enhancement	Ericsson

R4-2401551	Simulation resutls for DSS enhancement	Ericsson

R4-2401552	Simulation result collection for DSS enhancement demodulation requirement	Ericsson

R4-2401680	Discussion on performance requirements for Rel-18 eDSS	Huawei,HiSilicon

R4-2401681	Simulation results for eDSS performance	Huawei,HiSilicon

R4-2402051	Discussion on enhancement of NR dynamic spectrum sharing demodulation requirements	ZTE Corporation

R4-2402052	Simulation results for eDSS demodulation requirements	ZTE Corporation

R4-2402720	Enhancement of NR dynamic spectrum sharing: UE Demodulation Performance Requirements	Nokia, Nokia Shanghai Bell

R4-2402721	On PDCCH of Enhancement of NR Dynamic Spectrum Sharing: Simulation Results	Nokia, Nokia Shanghai Bell

R4-2402668	Topic summary for [110][328] NR_DSS_enh	Moderator (Ericsson)

R4-2402873	WF on [110][328] NR_DSS_enh	Ericsson

#### RAN4#110bis (April 2024)

R4-2404146	Simulation results  on PDCCH requirements for DSS enhancement	MediaTek inc.

R4-2404150	Draft CR to 38.101-4: PDCCH requirement for DSS enhancement (FDD 2Rx)	MediaTek inc.

R4-2404307	UE demodulation performance requirements for NR dynamic spectrum sharing	Apple

R4-2404308	DraftCR for PDCCH requirements for eDSS - TDD with 2RX	Apple

R4-2405164	Discussions on performance requirements for Rel-18 DSS	Huawei,HiSilicon

R4-2405165	draft CR Reference measurement channel for DSS enhancement demodulation requirement FDD and TDD	Huawei,HiSilicon

R4-2405210	Simulation results for eDSS demodulation requirements	ZTE Corporation

R4-2405215	Draft CR on introduction of PDCCH requirement for DSS enhancement -TDD 4Rx	ZTE Corporation

R4-2405478	On PDCCH demodulation requirements for DSS enhancement	Ericsson

R4-2405479	Simulation results for Rel-18 DSS enhancement demodualtion	Ericsson

R4-2405480	Simulation results collection for DSS enhancement demodulation requirement	Ericsson

R4-2405485	Draft BigCR to 38.101-4 on Introducing PDCCH demodulation requirements for DSS enhancement	Ericsson

R4-2405791	Enhancement of NR dynamic spectrum sharing: UE Demodulation Performance Requirements	Nokia

R4-2405792	On PDCCH of Enhancement of NR Dynamic Spectrum Sharing: Simulation Results	Nokia

R4-2405793	Drat CR on the Introduction of PDCCH requirement for DSS enhancement -FDD 4Rx	Nokia

R4-2405936	[NR_DSS_enh-Perf] Simulation results for NR DSS enhancements	Qualcomm India Pvt Ltd

#### RAN4#111 (May 2024)

R4-2407016	Simulation results on PDCCH requirements for DSS enhancement	MediaTek inc.

R4-2407262	UE demodulation performance requirements for NR dynamic spectrum sharing	Apple

R4-2407263	DraftCR for PDCCH requirements for eDSS - TDD with 2RX	Apple

R4-2408060	Enhancement of NR dynamic spectrum sharing: UE Demodulation Performance Requirements	Nokia

R4-2408061	On PDCCH of Enhancement of NR Dynamic Spectrum Sharing: Simulation Results	Nokia

R4-2408062	Drat CR on the Introduction of PDCCH requirement for DSS enhancement -FDD 4Rx	Nokia

R4-2408638	[NR_DSS_enh-Perf] Draft CR to 38.101-4 Applicability rule for NR PDCCH demodulation requirements under DSS scenario	Qualcomm India Pvt Ltd

R4-2408677	[NR_DSS_enh-Perf] Simulation results for NR PDCCH performance requirements under DSS enhancements	Qualcomm India Pvt Ltd

R4-2408750	On PDCCH demodulation requirements for DSS enhancement	Ericsson

R4-2408751	Simulation results collection for DSS enhancement demodulation requirement	Ericsson

R4-2408752	Simulation results for Rel-18 DSS enhancement demodualtion	Ericsson

R4-2408753	Big CR to 38.101-4 Introduction of PDCCH requirement for DSS enhancement	Ericsson

R4-2409013	Discussion on performance requirements for Rel-18 DSS	Huawei,HiSilicon

R4-2409088	Simulation results for eDSS demodulation requirements	ZTE Corporation, Sanechips

30.05.2024		minor adaptations for RAN #104

16.02.2024		minor adaptations for RAN #103

10.11.2023		minor adaptations for RAN #102

02.08.2023		minor adaptations for RAN #101

26.04.2023		minor adaptations for RAN #100

01.02.2023		minor adaptations for RAN #99

27.10.2022		minor adaptations for RAN #98e

01.08.2022		minor adaptations for RAN #97e

21.05.2022		minor adaptations for RAN #96

10.01.2022		minor adaptations for RAN #95e

04.10.2021		minor adaptations for RAN #94e

08.08.2021		minor adaptations for RAN #93e

17.05.2021		minor adaptations for RAN #92e

28.01.2021		minor adaptations for RAN #91e

09.11.2020		minor adaptations for RAN #90e

31.08.2020		minor adaptations for RAN #89e

20.04.2020		minor adaptations for RAN #88e

18.02.2020		minor adaptations for RAN #87e

14.11.2019		minor adaptations for RAN #86

18.08.2019		minor adaptations for RAN #85

12.05.2019		minor adaptations for RAN #84

27.02.2019		minor adaptations for RAN #83

21.11.2018		completion levels with colours added (for RAN #82)

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects (for RAN #81)

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template