3GPP TSG RAN Meeting #104	RP-241188

Shanghai, China, June 17-20, 2024

# Source:	Verizon

# Title:	Objectives for SCS

# Document for:	Discussion/Decision

Agenda Item:	9.1.5

# Introduction

FCC has proposed a new framework [1] for Supplemental Coverage from Space (SCS) for satellite operators to collaborate with existing mobile spectrums from terrestrial operators to offer ubiquitous connectivity directly to consumer handsets, in which the SCS is in a secondary basis.

For 3GPP defined 4G and 5G networks, this SCS framework is a crucial component, in which satellite networks work to seamlessly provide coverage which cannot be achieved on the existing networks’ own. The goal of this SCS communication is to bring life-saving connectivity to remote areas, interim emergency (or 911) calls, and test routing requirements to ensure help is available even without a terrestrial connection.

In this contribution we present some objectives for potential 3GPP work.

# SCS framework

The SCS framework from FCC can accelerate innovative SCS operations with the need to retain service quality of terrestrial networks, protect spectrum usage rights, and minimize the risk of harmful interference to the terrestrial services, both domestically and internationally. This SCS framework represents a step to encourage a development while minimizing risks to the terrestrial service.

  - Objective of SCS operation

One of the goals of this SCS framework is to enable consumers in areas not covered by terrestrial networks to stay connected using their existing devices via satellite-based communications. In fact, the FCC rules have provided the related SCS spectrum availability, spectrum licensing framework, and technical and operational requirements.

The existing LTE and NR devices developed by the 3GPP for mobile communication are the essential requirement in designing the SCS operation. The SCS feature will be played on these UE devices by using the terrestrial spectrums to the satellite communication in the providing service to the end users, in which the UE device will send uplink data (or voice frame) toward the satellite, the downlink data are received by the UE device.

Objective 1: The SCS requirements shall target on the 3GPP LTE and NR UE devices for service of geographically independent area (GIA).

The satellite will be the central controller in the SCS operation; that is, it is in charge of the spectrum allocation and management. The described satellite-assisted systems can be seen as an extension to the conventional terrestrial operations. Thus, it is necessary to use the same LTE and NR frame structures as the terrestrial operations.

Objective 2: The same frame structure defined by 3GPP for LTE and NR operations shall be used in SCS capable devices.

  - Spectrum allocation for SCS operation

For initial development, FCC permits frequency allocated only to authorize SCS bi-directional, secondary mobile-satellite service (MSS) operations from the terrestrial spectrums listed below following a licensing framework.

Considering the initial SCS development is targeting on the existing 3GPP UE devices over GIA areas, it is necessary to reuse the existing 3GPP band definitions for the 700MHz, the 850MHz, and the PCS spectrums for the SCS device development. The existing terrestrial bands will be an effective way to optimize the growing service, service development, and straightforward component in necessity to communication channels for the SCS operation. 3GPP also should study the regulatory spectrums allocated for SCS, and are still not in its band definition yet.

Objective 3: Initial SCS capability shall be targeted on the existing 3GPP band definitions for the 700MHz, the 850MHz, and the PCS spectrums. The existing bands shall be used in SCS operation via satellite-based communications.

Objective 4: 3GPP should study these allocated spectrums which are allocated as the SCS bands, but are not defined by 3GPP yet.

  - Interference free for terrestrial operation

A key component of the entry criteria is that the MSS (secondary mobile-satellite service) operation must be secondary to terrestrial operations for all SCS bands. This means that SCS operations shall not cause harmful interference to nor claim protection from harmful interference from present terrestrial primary operations.

This indicates that bidirectional MSS operation (satellite-to-device and device-to-satellite) shall apply to the entire spectrum range of each of the SCS band. As a satellite communication, the existing part 25 rules in US regulation will be applied in order to review SCS applications and ensure satellite operators which are technically and legally qualified. Any harmful interference that might arise from satellite partners’ deployment within the licensees’ markets must be resolved before the spectrum can be used for SCS.

Therefore, it is necessary to identify any new SCS RF requirements, verify potential harmful interference upon SCS deployment, mitigate and ensure the terrestrial service being fully protected based on the secondary market rules in development of SCS to operation in the defined “GIA”.

Objective 5: In development, it shall ensure the terrestrial spectrums to be fully protected from any possible harmful interferences from SCS, in case new RF parameters are introduced.

  - GIA co-channel interference

The SCS framework is designed for expanding satellite coverage to GIA areas. Although there may be no adjacent terrestrial or no broadband coverage, the co-channel interference between would be still a challenge given at play when introducing satellite communications over the terrestrial spectrums in a GIA area for the SCS operation. So far, there are two leasing methods defined by FCC for getting the terrestrial licenses to a satellite operator for SCS operation.

Either method, the boundaries of a GIA area for the SCS operation must meet the safeguards with the technical parameters to avoid co-channel interference with one another’s operations overlay on top of a terrestrial mobile operator’s network or licenses’ right from harmful interference. In case the SCS system’s parameters could fall within the relevant parameters published in the ITU, international coordination would be considered as part of the SCS development based on regulator requirement.

Objective 6: 3GPP needs to identify possible parameters to avoid GIA co-channel interference with other operations overlay on top of a terrestrial mobile.

Objective 7: 3GPP may need to identify the relevant parameters published in the ITU if the SCS system’s technical parameters can fall in.

  - SCS roaming

FCC has lowered the roaming requirement for the SCS operation at this time because of additional complexity and additional technical considerations that could affect the development of SCS collaborations.

For SCS 911 calls and texts, it is mainly to be placed outdoors with the user device having a view of the GPS satellites in the sky. Given that the user devices typically have GPS receivers, it should be able to determine its location, and the SCS should be able to provide the needed assistance information. Thus, there is no further study needed for roaming in the SCS development (e.g., the cell section/re-section, handover) in between SCS operation and terrestrial networks.

Objective 8: The RF RRM requirement for the roaming is not required at this time. The GPS receiver will be used in devices for necessary location assistance information in the SCS operation.

  - Additional GIA technical requirements

The SCS operation needs to coordinate the parameters necessary to avoid co-channel interference with one another’s operations. FCC has also provided various technical requirements for the SCS operation.

    - Boundary limits for co-channel

At the boundaries of a GIA, the FCC requirement for the signal level from SCS, at and beyond the terrestrial operator’s licensed area, is the same as those defined for terrestrial operation in each respective band. FCC maintains the existing market area boundary limits established in parts 22, 24, and 27 for SCS operation. The regulated limits, which should be aggregate signals from all beams, are as below.

Objective 9: At the GIA boundaries, the in-band field signal strength cannot exceed the following limits by a satellite providing SCS service

- 40 dBµV/m for the 600 MHz, 700 MHz, and 800 MHz bands

- 47 dBµV/m for the AWS-H Block and PCS band.

Objective 10: A validation method shall be defined for verification of the area boundary limits.

    - OOBE limit

A regulated OOBE limit is defined for the SCS operation to protect adjacent band operations from the risk of harmful interference.

This is a uniform OOBE limit for the operation across all of the SCS bands, is expressed as a terrestrial power flux-density (PFD) limit, and is in line with the requirement placed on terrestrial services.

At 1.5 meters above ground, the aggregate out-of-band PFD limit is -120 dBW/m2/MHz for the 600MHz, 700MHz, 800MHz, and PCS 1990-1995 MHz bands. This OOBE limit shall be ensured, measured, and enforced on the ground to the SCS operation to confirm the adjacent band devices are protected.

Objective 11: 3GPP requirement mast meet the aggregate out-of-band PFD limit, i.e., -120 dBW/m2/MHz, for the 600MHz, 700MHz, 800MHz, and PCS 1990-1995 MHz bands at 1.5 meters above ground level.

# Core objective

For enabling SCS capability on the existing UE devices, 3GPP needs to identify if there are any open issues and if there is any new requirement needed in collaboration with the terrestrial mobile service to ensure interference free to the incumbent terrestrial mobile services.

The main objective of SCS operation is to take into account the regulated SCS framework and allow for the existing (LTE and NR) terrestrial UE transmitters and receivers characteristics to enable the SCS capability based on the existing 3GPP frame structures.

# Conclusion

Recognizing that SCS has the potential to bring life-saving connectivity to remote areas and ensure emergency communications are supported, this new service can be taken as a major step towards enabling satellite operators and terrestrial wireless providers to collaborate on offering ubiquitous connectivity across.

If a 3GPP action is needed, it should aim to expand access to communications services, drive technological innovation, and promote efficient spectrum use. For this, we have following objectives to the possible identified works,

Objective 1: The SCS requirements shall target on the 3GPP LTE and NR UE devices for service of geographically independent area (GIA).

Objective 2: The same frame structure defined by 3GPP for LTE and NR operations shall be used in SCS capable devices.

Objective 3: Initial SCS capability shall be targeted on the existing 3GPP band definitions for the 700MHz, the 850MHz, and the PCS spectrums. The existing bands shall be used in SCS operation via satellite-based communications.

Objective 4: 3GPP should study these allocated spectrums which are allocated as the SCS bands, but are not defined by 3GPP yet.

Objective 5: In development, it shall ensure the terrestrial spectrums to be fully protected from any possible harmful interferences from SCS, in case new RF parameters are introduced.

Objective 6: 3GPP needs to identify possible parameters to avoid GIA co-channel interference with other operations overlay on top of a terrestrial mobile.

Objective 7: 3GPP may need to identify the relevant parameters published in the ITU if the SCS system’s technical parameters can fall in.

Objective 8: The RF RRM requirement for the roaming is not required at this time. The GPS receiver will be used in devices for necessary location assistance information in the SCS operation.

Objective 9: At the GIA boundaries, the in-band field signal strength cannot exceed the following limits by a satellite providing SCS service

- 40 dBµV/m for the 600 MHz, 700 MHz, and 800 MHz bands

- 47 dBµV/m for the AWS-H Block and PCS band.

Objective 10: A validation method shall be defined for verification of the area boundary limits.

Objective 11: 3GPP requirement mast meet the aggregate out-of-band PFD limit, i.e., -120 dBW/m2/MHz, for the 600MHz, 700MHz, 800MHz, and PCS 1990-1995 MHz bands at 1.5 meters above ground level.

# References

[1]	FCC 24-28A1, FCC-24-28A1.pdf, FCC