3GPP TSG RAN Meeting #104	RP-24xxxx

Shanghai, China, June 17 - 20, 2024

# Source:	ZTE, Samsung

# Title:	New WID: Rel-19 NR intra-band contiguous CA with UL MIMO

# Document for:	Approval

Agenda Item:	9.1.5

# 3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title: Rel-19 NR intra-band contiguous CA with UL MIMO

Acronym:

Unique identifier: {A number to be provided by MCC at the plenary}

NOTE:	For new WIs/SIs leave the Unique identifier empty and make a proposal for an Acronym.

For a revised WI/SI: Take Unique identifier and acronym as shown in 3GPP workplan.

If this is a RAN WID including Core and Perf. part, then Title, Acronym and Unique identifier refer to the feature WI.

Please tick (X) the applicable box(es) in the table below:

Either:

| This WID includes a Core part | X |
| --- | --- |
| This WID includes a Performance part |  |

# Potential target Release: Rel-19

Note that this field above indicates the proposed Release at the time of submission of the WID to TSG approval. It can later be changed without a need to revise the WID. The updated target Release is indicated in the Work Plan. NOTE: In case of contradiction with the target dates of clause 5, clause 5 determines the target release.

# 1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  | X |  |  |  |
| No | X |  | X | X | X |
| Don't know |  |  |  |  |  |

# 2	Classification of the Work Item and linked work items

2.1	Primary classification

This work item is a …

|  | Feature |
| --- | --- |
| X | Building Block |
|  | Work Task |
|  | Study Item |

NOTE:	Normally, Core/Perf./Testing parts in RAN WIDs are Building Blocks. Only if they are under an SA or CT umbrella, they are defined as work tasks. If you are in doubt, please contact MCC.

2.2	Parent Work Item

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |

NOTE:	RAN agreed some time ago, that it describes the feature WI + Core/Perf. part WI or Testing part WI in one 	WID. Therefore the table above should just include the feature WI data (In case the feature covers Core and 	Perf. part, please list under Working Group the leading WG of the Core part).

2.3	Other related Work Items and dependencies

NOTE:	Also related or dependent WIs/SIs in other TSGs should be indicated.

| Other related Work Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
| 960095 | NR_bands_UL_MIMO_R18 | {optional free text} |
| 970087 | HPUE_NR_FR1_TDD_intra_CA_R18 | {optional free text} |
| 890162 | NR_RF_FR1_enh-Core | {optional free text} |

# 3	Justification

There are a lot of NR FR1 bands supporting intra-band contiguous CA operation, meanwhile UL MIMO feature can also be supported in the same band. In current TS38.101 specification, there are two bands specified with PC2/3 NR intra-band CA configuration with UL MIMO defined, i.e. CA_n41C, CA_n78C. The general RF requirements have already been defined.

In the R19 WID NR_ENDC_RF_Ph4, there are four NR intra-band CA configuration with UL MIMO selected as example band combinations, i.e. CA_n41C, CA_n78C, CA_n77C and CA_n79C. It can be seen that the corresponding lower power class (i.e. PC2/PC3) configurations are still missing in some of them (i.e. CA_n77C and CA_n79C). The absence of a lower power class configuration would cause problems in duty cycle based SAR solution to high power class configuration. The agreement in RAN4 in R4-2406583 is to have the Rel-19 basket WIs to cover the necessary requirements for the lower order power class cases (PC2 and PC3) of same PC1.5 band combination.

Moreover, considering the demands from operators, and more NR intra-band UL contiguous CA with UL MIMO will be requested by operators, using Basket WID approach is an efficient and routine way.

Therefore, a basket WI should be created to include the corresponding lower order power class cases (PC2 and PC3) for NR intra-band contiguous CA with UL MIMO in Rel-19 for the benefit of the WID management.

The preconditions to propose NR intra-band contiguous CA with UL MIMO in Rel-19 are as follows.

- NR Intra band CA shall be completed and specified in advance.

- NR band supporting UL MIMO shall be completed and specified in advance.

# 4	Objective

4.1	Objective of SI or Core part WI or Testing part WI

4.1.1	Objective

- Specify the UE maximum output power requirement for PC3/PC2 NR intra-band contiguous CA with UL MIMO.

- Specify the band specific A-MPR requirements for the proposed bands, if needed

- NOTE 1: This basket WI is not subject to block approval procedure

PC3/PC2 NR intra-band contiguous CA in this WI are introduced in a REL-independent way starting from release 15 according to Table 5.2.1-1 of TS 38.307. However no changes to TS 38.307 are needed.

4.1.2	Way of working

The new request adding NR intra-band contiguous CA with UL MIMO should be submitted on RAN4 reflector before tdoc submission deadline to the next RAN4 meeting (1 week before the meeting).

The basket WI will then be updated with the new requests (section 4.1.3, Table 4.1.3-1) and submitted to next RAN4 meeting for endorsement, before submission to RAN meeting for approval.

When the work is completed, all draft CRs related to one request will be submitted in the same RAN4 meeting to check consistency. If they are endorsed, the basket WI Rapporteur will merge all draft CRs from all requests in big CRs (one per TS specification).

After the RAN4 meeting preceding a RAN meeting, those big CRs will be sent on RAN4 reflector for email approval (1 week) and, if agreed, they will be submitted to following RAN meeting.

4.1.3	Requests overview

Table 4.1.3-1: Requests tracking for NR intra-band contiguous CA with UL MIMO

| NR CA configuration | Contact name, company | Contact email | Other supporting companies | note | status |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Note: minimum 3 |  |  |
| CA_n79C | Wubin, Zhou ZTE | zhou.wubin@zte.com.cn | CMCC, Samsung, Sanechips | Both PC3 and PC2 | New |

4.2	Objective of Performance part WI

4.3	RAN time budget request (not applicable to RAN5 WIs/SIs)

NOTE:	For all new RAN related WIs/SIs which are not led by RAN WG5 the WI/SI rapporteur has to fill out the attached Excel table to request time budgets for corresponding RAN WG meetings.
The Excel table has to be filled out for all affected RAN WGs and up to the target date of the WI/SI.
One time unit (TU) corresponds to ~ 2 hours in the meeting.
If no TU is needed, then leave the field empty otherwise enter a number >0 in the field.

For revisions of already approved WI/SI descriptions: Please remove the Excel table from the WID/SID's zip file. The time budgets are already recorded. If you want to modify them, then this has to be done via the status report and not via a revised WID/SID.

If this WID is covering Core and Performance part, then please fill out one line for each part in the attached Excel table.

additional comments to the time budget request in the attached Excel table:

# 5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Remarks |

NOTE:	If this is a RAN WI including Core and Perf. part, then all new Core part specs have to be listed first and then all new Perf. part specs. Indicate "Core part" or "Perf. part" under Remarks for each spec.
By default a new specs can only be new for one of both parts.

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| 38.101-1 | Add NR intra-band contiguous CA with UL MIMO related RF requirements for NR FR1 bands | RAN #xxx | Core part |

NOTE:	If this is a RAN WI including Core and Perf. part, then all new Core part specs have to be listed first and then all new Perf. part specs. Indicate "Core part" or "Perf. part" under Remarks for each spec.
If an existing spec is affected by both (Core part and Perf. part), then it has to be listed twice with appropriate approval dates.

6	Work item Rapporteur(s)

Wubin Zhou, ZTE, Zhou.wubin@zte.com.cn

Yuanyuan Zhang, Samsung, tina55.zhang@samsung.com

# 7	Work item leadership

RAN WG4

# 8	Aspects that involve other WGs

None

NOTE:	For RAN WIs: Section 8 applies only to WGs outside of TSG RAN because RAN WG aspects have to be covered in section 4.

# 9	Supporting Individual Members

{At least 4 supporting Individual Members are needed. There is an expectation that these companies will provide resources to progress the work. Note that having 4 supporting companies is a necessary but not sufficient condition: the usual TSG approval process by consensus is needed for the WID approval.}

Supporting IM name