3GPP TSG RAN Meeting #104									RP-241267

Shanghai, China, June 17-20, 2024

## Status Report to TSG

Agenda item:			9.4.4.6

| WI / SI Name | Even Further RRM enhancement for NR and MR-DC |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: Yes |  | Testing part: No |
| Acronym | NR_RRM_enh3 |  |  |  |  |
| Unique ID | 950078 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-221696 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: | Core part: 12/2023 | Performance part: 06/2024 | Testing part: |  |
| Overall Completion level | Study Item: | Core part: Overall:100% RAN4: 100% | Performance Part: 100% | Testing part: |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN4 |
| --- | --- | --- |
| Rapporteur | Name | Jie Cui |
|  | Company | Apple |
|  | Email | jie_cui@apple.com |
| Rapporteur | Name | Roy Hu |
|  | Company | OPPO |
|  | Email | hurongyi@oppo.com |

## 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

## 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

## 2.1	RAN1

#### 2.1.1	Agreements

#### 2.1.2	Remaining Open issues

## 2.2	RAN2

#### 2.2.1	Agreements

#### 2.2.2	Remaining Open issues

## 2.3	RAN3

#### 2.3.1	Agreements

#### 2.3.2	Remaining Open issues

## 2.4	RAN4

#### 2.4.1	Agreements

RAN4#110bis meeting

The following tdocs have been approved/endorsed in RAN4#110bis meeting for performance part:

| Tdoc number | Title | Source |
| --- | --- | --- |
| R4-2405719 | DraftCR HO with PSCell for FR1-FR1 to FR1-FR1 | Qualcomm Incorporated |
| R4-2405965 | draftCR on TC for SCell activation delay with shorter measurement interval enhancement | MediaTek inc. |
| R4-2406325 | Draft CR to TS 38.133 for NR-DC FR1-FR1 scg activation and deactivation test case | Ericsson |
| R4-2406326 | Draft CR on performance requirements for FR1-FR1 NR-DC | OPPO |
| R4-2406331 | (NR_RRM_enh3-Core) draft CR on R18 RRM enhancement core maintenance | Apple |
| R4-2406333 | draftCR on TCs for FR2 unknown SCell activation with FG31-2 and FG31-3 | Nokia, Nokia Shanghai Bell |
| R4-2406334 | [NR_RRM_enh3-Perf] Draft CR on TC for Multiple SCell activation delay with FR1 unknown SCell with L3 report | ZTE Corporation |
| R4-2406335 | DraftCR on TC2 for FR2 PUCCH SCell activation FG31-1 | Huawei, HiSilicon |
| R4-2406336 | draft CR on test case for R18 FR2 SCell activation delay reduction | vivo |
| R4-2406337 | DraftCR FR2 SCell activation with L3 reporting during activaiton | Qualcomm Incorporated |
| R4-2406339 | Way Forward for [110bis][207] NR_RRM_enh3_part1 | Apple |
| R4-2406426 | FR1-FR1 SCG activation | Nokia |
| R4-2406507 | Draft Big CR to TS 38.133 on core requirement maintenance for Even Further RRM enhancement for NR and MR-DC | Apple, OPPO |
| R4-2406508 | Draft Big CR to TS 38.133 on performance requirements for Even Further RRM enhancement for NR and MR-DC | Apple, OPPO |

RRM performance part for FR2 SCell activation delay reduction

Issue 2-1: How to configure M to have valid L3 report for FG31-1 TC

Agreement:

- Calculate the M values based on UE capability in the corresponding sections.

Agreement:

DCI transmission timing: cover two case:

- Case 1: n+3ms+THARQ+M-k2

  - FFS: UE pass condition is either UE to L1 or L3 report.

- Case 2: n+[7]ms+ THARQ

- where k2=1 with type A mapping and startSymbolAndLength (SLIV) = 42 (L=4, and S=0).

DL scheduling for data starts from n+3ms.

Issue 2-2: whether to verify requirement without L3 reporting for FG31-1 TC

Agreement:

  - For all FG31-1 TCs, only verify the activation delay when L3 report is triggered.

    - FFS UE reporting L1 is also a pass condition.

Issue 2-3: whether to verify requirement for L1 report before and after TCI configuration in FG31-1 TC

- Option 1 (CMCC):

  - for the test cases on L3 report based enhancement, it is proposed to consider both the case with L1 measurement and the case without L1 measurement.

- Option 2 (CTC, Apple):

  - When TCI configuration are based on L1-RSRP reporting, legacy requirements and existing test cases can be applied. It’s proposed only to define test case for TCI configuration based on the latest valid L3-RSRP reporting, i.e., the latest valid L3-RSRP reporting is before TCI configuration.

- Agreement:

  - Up to the conclusion of “FFS” part in issue 2-2

Issue 2-4: FG31-1 TC for CSSF2

- Proposal (HW):

  - To verify CSSF = 2 in TC#2, and clarify the applicability rule that for UE supporting two PUCCH group, UE is required to pass TC#2.

- Recommended WF

  - TBD

Issue 2-5: DRX configuration for FG31-3 TC

Agreement:

Pcell is always scheduled during ON duration when DRX is configured.

Interruption is not tested when DRX is configured.

Issue 2-6: Testing timeline in FG31-1/2/3 TC

- Option 1 (HW):

  - Define Time periods for TC for FG 31-1 as Table I of R4-240998.

- Option 2 (Ericsson):

  - RAN4 to agree on the following procedure for L3-reporting based SCell activation and SCell deactivation

    - Cell power keeps remaining same to avoid event-triggered reporting

    - Before start of T1, MO is configured. Before end of T1, and after sufficiently large time to perform measurement on the neighbour cell, SCell is added. T1 ends with SCell being added.

    - T2: start from receiving MAC-CE for SCell activation to L3 reporting

    - T3: start from L3 reporting to CSI report

    - T4: start from receiving SCell deactivation command.

- Option 3 (MTK):

  - The timeline design for R18 SCell activation TCs:

    - For FG31-2 and FG31-3 TCs, the timeline is designed same as legacy TCs’ timeline for SCell activation using three successive time periods (T1, T2 and T3).

    - For FG31-1 TCs, the timeline is designed using four successive time periods (T1, T2, T3 and T4) to include the time period for L3 report.

- Recommended WF

  - Discuss in the CR.

Issue 2-7: SSB configuration in FG31-1 TC for FR1 (TC5/TC7)

- Option 1 (vivo):

  - For SSB configuration of the to-be-activated SCell, use SSB.3 for 15kHz SCS and SSB.4 for 30kHz SCS, so that 2 SSBs are available for FR1 SCell activation test cases.

- Recommended WF

  - Discuss in the CR.

RAN4#111 meeting

The following tdocs have been approved/endorsed in RAN4#110bis meeting for performance part:

| Tdoc number | Title | Source |
| --- | --- | --- |
| R4-2408528 | Draft CR to TS 38.133 for test case of SCG activation in FR1-FR1 NR-DC | Ericsson |
| R4-2410251 | draft CR on R18 FR2 SCell activation delay reduction | vivo |
| R4-2410252 | [NR_RRM_enh3-Core] Draft CR on SCell activation enhancement of R18 RRM enhancement core maintenance | ZTE Corporation, Sanechips |
| R4-2410253 | DraftCR on maintenance for R18 eFeRRM SCell activation | Huawei, HiSilicon |
| R4-2410254 | correction on TCs for FR2 unknown SCell activation with FG31-1 | Nokia, Nokia Shanghai Bell |
| R4-2410255 | draft CR on test case for R18 FR2 SCell activation delay reduction | vivo |
| R4-2410257 | DraftCR on TC maintenance for R18 eFeRRM SCell activation | Huawei, HiSilicon |
| R4-2410424 | [NR_RRM_enh3-Perf] Draft CR on TC for Multiple SCell activation delay with FR1 unknown SCell with L3 report | ZTE Corporation, Sanechips |
| R4-2410259 | Way Forward for [111][205] NR_RRM_enh3_part1 | Apple |
| R4-2410343 | WF on improvement of SCG activation requirements for FR1-FR1 NR-DC | OPPO |

RRM performance part for FR2 SCell activation delay reduction

Issue 2-1: whether to verify requirement with L1 report for FG31-1 TC

Agreement:

- TE sends TCI activation CMD after L3 report

  - If DCI is scheduled within the time margin, UE is required to respond with the L3 report as long as there is a valid measurement result regardless whether there is L1 reporting before n+3ms+THARQ+M.

Issue 2-2: DCI transmission timing for case 2 in FG31-1 TC

Agreement: DCI transmission for case 2 is n+7ms+ THARQ.

Issue 2-3: FG31-1 TC for CSSF2

Agreement: Clarify the applicability rule that for UE supporting two PUCCH group, UE is required to pass TC#2 in the agreed TC list (R4-2403466).

#### 2.4.2	Remaining Open issues

## 2.5	RAN5

#### 2.5.1	Agreements

#### 2.5.2	Remaining Open issues

#### 2.5.3	Remaining Open issues with cross-WG dependencies

## 2.6	RAN6

#### 2.6.1	Agreements

#### 2.6.2	Remaining Open issues

## 3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

## 3.1	SAx/CTs

#### 3.1.1	Agreements with cross-TSG impacts

#### 3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

## 4.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

RAN4 #110bis meeting

| TDoc | Title | Source |
| --- | --- | --- |
| R4-2404356 | (NR_RRM_enh3-Core) draft CR on R18 RRM enhancement core maintenance | Apple |
| R4-2404357 | On FR2 SCell activation performance part | Apple |
| R4-2404370 | Maintenance on FR2 SCell activation delay reduction | Nokia, Nokia Shanghai Bell |
| R4-2404371 | 38.133 CR on multilple SCell activation with L3 reporting | Nokia, Nokia Shanghai Bell |
| R4-2404372 | Performance requirement for FR2 SCell activation delay reduction | Nokia, Nokia Shanghai Bell |
| R4-2404373 | draftCR on TCs for FR2 unknown SCell activation with FG31-2 and FG31-3 | Nokia, Nokia Shanghai Bell |
| R4-2404695 | Discussion on FR1-FR1 SCG activation | Nokia |
| R4-2404696 | FR1-FR1 SCG activation | Nokia |
| R4-2404717 | Discussion on performance requirements for FR2 SCell activation delay reduction | CMCC |
| R4-2404778 | [NR_RRM_enh3-Perf] Draft CR on TC for Multiple SCell activation delay with FR1 unknown SCell with L3 report | ZTE Corporation |
| R4-2404818 | Topic summary for [110bis][207] NR_RRM_enh3_part1 | Moderator (Apple) |
| R4-2404819 | Topic summary for [110bis][208] NR_RRM_enh3_part2 | Moderator (OPPO) |
| R4-2404845 | Draft CR to TS 38.133 for NR-DC FR1-FR1 scg activation and deactivation test case | Ericsson |
| R4-2404850 | Discussion on RRM performance requirements for FR2 SCell activation delay reduction | China Telecom |
| R4-2404914 | Draft CR on performance requirements for FR1-FR1 NR-DC | OPPO |
| R4-2404996 | Discussion on enhancement for FR2 SCell activation delay reduction | Huawei, HiSilicon |
| R4-2404997 | DraftCR on maintenance for R18 eFeRRM | Huawei, HiSilicon |
| R4-2404998 | Discussion on performance requirements for FR2 SCell activation delay reduction | Huawei, HiSilicon |
| R4-2404999 | DraftCR on TC2 for FR2 PUCCH SCell activation FG31-1 | Huawei, HiSilicon |
| R4-2405356 | draft CR on test case for R18 FR2 SCell activation delay reduction | vivo |
| R4-2405718 | DraftCR FR2 SCell activation with L3 reporting during activaiton | Qualcomm Incorporated |
| R4-2405719 | DraftCR HO with PSCell for FR1-FR1 to FR1-FR1 | Qualcomm Incorporated |
| R4-2405772 | RRM core requirements maintenance for FR2 SCell activation delay reduction | Ericsson |
| R4-2405773 | RRM performance requirements for FR2 SCell activation delay reduction | Ericsson |
| R4-2405965 | draftCR on TC for SCell activation delay with shorter measurement interval enhancement | MediaTek inc. |
| R4-2405966 | Discussion on the performance requirements for FR2 SCell activation enhancement | MediaTek inc. |
| R4-2406324 | Discussion on FR1-FR1 SCG activation | Nokia |
| R4-2406325 | Draft CR to TS 38.133 for NR-DC FR1-FR1 scg activation and deactivation test case | Ericsson |
| R4-2406326 | Draft CR on performance requirements for FR1-FR1 NR-DC | OPPO |
| R4-2406327 | DraftCR HO with PSCell for FR1-FR1 to FR1-FR1 | Qualcomm Incorporated |
| R4-2406331 | (NR_RRM_enh3-Core) draft CR on R18 RRM enhancement core maintenance | Apple |
| R4-2406332 | DraftCR on maintenance for R18 eFeRRM | Huawei, HiSilicon |
| R4-2406333 | draftCR on TCs for FR2 unknown SCell activation with FG31-2 and FG31-3 | Nokia, Nokia Shanghai Bell |
| R4-2406334 | [NR_RRM_enh3-Perf] Draft CR on TC for Multiple SCell activation delay with FR1 unknown SCell with L3 report | ZTE Corporation |
| R4-2406335 | DraftCR on TC2 for FR2 PUCCH SCell activation FG31-1 | Huawei, HiSilicon |
| R4-2406336 | draft CR on test case for R18 FR2 SCell activation delay reduction | vivo |
| R4-2406337 | DraftCR FR2 SCell activation with L3 reporting during activaiton | Qualcomm Incorporated |
| R4-2406338 | draftCR on TC for SCell activation delay with shorter measurement interval enhancement | MediaTek inc. |
| R4-2406339 | Way Forward for [110bis][207] NR_RRM_enh3_part1 | Apple |
| R4-2406426 | FR1-FR1 SCG activation | Nokia |
| R4-2406507 | Draft Big CR to TS 38.133 on core requirement maintenance for Even Further RRM enhancement for NR and MR-DC | Apple, OPPO |
| R4-2406508 | Draft Big CR to TS 38.133 on performance requirements for Even Further RRM enhancement for NR and MR-DC | Apple, OPPO |

RAN4 #111 meeting

| TDoc | Title | Source |
| --- | --- | --- |
| R4-2407300 | On RRM core requirements maintenance for Scell activation enhancement | Apple |
| R4-2407301 | Big CR to TS 38.133 on core requirement maintenance for Even Further RRM enhancemen for NR and MR-DC | Apple |
| R4-2407302 | Big CR to TS 38.133 on performance requirements for Even Further RRM enhancement for NR and MR-DC | Apple |
| R4-2407358 | On FG31-1 test cases for Scell activation enhancement | Apple |
| R4-2407736 | Maintenance on FR2 SCell activation delay reduction | Nokia, Nokia Shanghai Bell |
| R4-2407737 | 38.133 CR on multilple SCell activation with L3 reporting | Nokia, Nokia Shanghai Bell |
| R4-2407738 | Performance requirement for FR2 SCell activation delay reduction | Nokia, Nokia Shanghai Bell |
| R4-2407739 | correction on TCs for FR2 unknown SCell activation with FG31-1 | Nokia, Nokia Shanghai Bell |
| R4-2407765 | Discussion on maintenance of R18 FR2 SCell activation delay requirements | vivo |
| R4-2407766 | draft CR on R18 FR2 SCell activation delay reduction | vivo |
| R4-2407767 | Discussion on test cases of FR2 SCell activation delay reduction | vivo |
| R4-2407768 | draft CR on test case for R18 FR2 SCell activation delay reduction | vivo |
| R4-2408002 | Topic summary for [111][205] NR_RRM_enh3_part1 | Moderator (Apple) |
| R4-2408003 | Topic summary for [111][206] NR_RRM_enh3_part2 | Moderator (OPPO) |
| R4-2408161 | Discussion on FR1-FR1 SCG activation | Nokia |
| R4-2408162 | FR1-FR1 SCG activation | Nokia |
| R4-2408243 | Discussion on the core maintenance of SCell activation delay reduction | ZTE Corporation, Sanechips |
| R4-2408261 | [NR_RRM_enh3-Core] Draft CR on L3 reporting relevant R18 RRM enhancement core maintenance | ZTE Corporation, Sanechips |
| R4-2408262 | [NR_RRM_enh3-Core] Draft CR on SCell activation enhancement of R18 RRM enhancement core maintenance | ZTE Corporation, Sanechips |
| R4-2408263 | [NR_RRM_enh3-Perf] Draft CR on TC for Multiple SCell activation delay with FR1 unknown SCell with L3 report | ZTE Corporation, Sanechips |
| R4-2408308 | Draft CR on maintenance for R18 SCell activation enhancement | China Telecom |
| R4-2408309 | Discussion on RRM core requirements maintenance for FR2 SCell activation delay reduction | China Telecom |
| R4-2408310 | Discussion on RRM performance requirements for FR2 SCell activation delay reduction | China Telecom |
| R4-2408429 | eFeRRM-Core remaining issues for L3 report based SCell activation | Qualcomm Incorporated |
| R4-2408430 | eFeRRM-Perf remaining issues for L3 report based SCell activation | Qualcomm Incorporated |
| R4-2408528 | Draft CR to TS 38.133 for test case of SCG activation in FR1-FR1 NR-DC | Ericsson |
| R4-2408563 | Discussion on Core requirements maintenance for R18 eFeRRM SCell activation | Huawei, HiSilicon |
| R4-2408564 | DraftCR on maintenance for R18 eFeRRM SCell activation | Huawei, HiSilicon |
| R4-2408565 | Discussion on performance requirements for FR2 SCell activation delay reduction | Huawei, HiSilicon |
| R4-2408566 | DraftCR on TC maintenance for R18 eFeRRM SCell activation | Huawei, HiSilicon |
| R4-2409707 | Core maintenance: L3 reporting in multiple SCell activation | Ericsson |
| R4-2409708 | Performance maintenance for Scell activation | Ericsson |
| R4-2409709 | draft CR to TS 38.133 on SCell activation enhancement test cases | Ericsson |
| R4-2410251 | draft CR on R18 FR2 SCell activation delay reduction | vivo |
| R4-2410252 | [NR_RRM_enh3-Core] Draft CR on SCell activation enhancement of R18 RRM enhancement core maintenance | ZTE Corporation, Sanechips |
| R4-2410253 | DraftCR on maintenance for R18 eFeRRM SCell activation | Huawei, HiSilicon |
| R4-2410254 | correction on TCs for FR2 unknown SCell activation with FG31-1 | Nokia, Nokia Shanghai Bell |
| R4-2410255 | draft CR on test case for R18 FR2 SCell activation delay reduction | vivo |
| R4-2410256 | [NR_RRM_enh3-Perf] Draft CR on TC for Multiple SCell activation delay with FR1 unknown SCell with L3 report | ZTE Corporation, Sanechips |
| R4-2410257 | DraftCR on TC maintenance for R18 eFeRRM SCell activation | Huawei, HiSilicon |
| R4-2410258 | draft CR to TS 38.133 on SCell activation enhancement test cases | Ericsson |
| R4-2410259 | Way Forward for [111][205] NR_RRM_enh3_part1 | Apple |
| R4-2410424 | [NR_RRM_enh3-Perf] Draft CR on TC for Multiple SCell activation delay with FR1 unknown SCell with L3 report | ZTE Corporation, Sanechips |

01.08.2022		minor adaptations for RAN #97e

21.05.2022		minor adaptations for RAN #96

10.01.2022		minor adaptations for RAN #95e

04.10.2021		minor adaptations for RAN #94e

08.08.2021		minor adaptations for RAN #93e

17.05.2021		minor adaptations for RAN #92e

28.01.2021		minor adaptations for RAN #91e

09.11.2020		minor adaptations for RAN #90e

31.08.2020		minor adaptations for RAN #89e

20.04.2020		minor adaptations for RAN #88e

18.02.2020		minor adaptations for RAN #87e

14.11.2019		minor adaptations for RAN #86

18.08.2019		minor adaptations for RAN #85

12.05.2019		minor adaptations for RAN #84

27.02.2019		minor adaptations for RAN #83

21.11.2018		completion levels with colours added (for RAN #82)

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects (for RAN #81)

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template