3GPP TSG RAN meeting #104										RP- 241272

Shanghai, China, June 17-20, 2024

Status Report to TSG

Agenda item:			9.4.4.5

| WI / SI Name | Enhanced NR support for high speed train scenario in frequency range 2 (FR2) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: Yes |  | Testing part: No |
| Acronym | NR_HST_FR2_enh |  |  |  |  |
| Unique ID | 950079 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-222272 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: n/a | Core part: 09/2023 | Performance part: 06/2024 (extended for 1Q from original completion data 03/2024) | Testing part: n/a |  |
| Overall Completion level | Study Item: n/a | Core part: 100% | Performance Part: 100% | Testing part: n/a |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN4 |
| --- | --- | --- |
| Rapporteur | Name | WANG, He (Jackson) |
|  | Company | Samsung |
|  | Email | h0809.wang@samsung.com |

1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | Yes/No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

2.4	RAN4

2.4.1	Agreements

RAN4 #110-bis (Changsha, China, 15th – 19th April, 2024)

1. RRM requirement

Draft Big CR (R4-2406408) to TS 38.133 on Rel-18 HST FR2 RRM performance requirements is endorsed with the following endorsed dCRs contained:

- R4-2406406, Test case requirements for cell reselection to FR2 inter-frequency NR carrier for FR2 HST

- R4-2406407, Draft CR for 38.133 on test case for FR2 HST intra-band CA without SSB time index detection when DRX is not used

- R4-2406409, Draft CR to 38.133 on test case for SSB based L1-RSRP for FR2 PC6 UE with multi-Rx

- R4-2406410, Draft CR to 38.133 on TCI State Switch for HST FR2 Enhanced Test Cases

- R4-2406411, Draft CR to 38.133 on Enhanced Intra-Frequency Measurements Test Case for HST FR2 Enhanced

WF on FR2 HST RRM performance requirements is approved in R4-2406412, with details provided as follows:

Way forwards on NR HST FR2 performance part are provided as:

Topic #1: RRM Performance Requirements for Rel-18 FR2 HST

Table 1 The planned RRM performance test cases list (for information)

| TC index | Corresponding Core Requirement | Necessity of New Test Case | Company for CR volunteer |
| --- | --- | --- | --- |
| TC1 | Requirements for Cell Re-selection | New test case needed. (New test case for A.7.1.1.X Cell reselection to FR2 inter-frequency NR case for Rel-18 FR2 HST intra-band CA) | Intel |
| TC2 | Requirements for SCell Activation Delay for Deactivated SCell | No new test case needed. |  |
| TC3-TC6 | Requirements for Inter-frequency measurement with measurement gaps | No new test case needed. |  |
|  |  | No new test case needed. |  |
|  |  | New test case needed. (New test case for A.7.6.2.X3 SA event triggered reporting tests for Rel-18 FR2 HST intra-band CA with SSB time index detection when DRX is not used (Pcell in FR2)) | Huawei |
|  |  | New test case needed. (New test case for A.7.6.2.X4 SA event triggered reporting tests for Rel-18 FR2 HST intra-band CA without SSB time index detection when DRX is not used (Pcell in FR2)) | Ericsson |
| TC7 | Requirements for Inter-frequency measurement without measurement gaps | No new test case needed. |  |
| TC8 | Requirements for One shot large UL timing adjustment for FR2 Power Class 6 UE | New test case(s) needed. | Nokia |
|  | Requirements for MAC-CE based TCI state switch delay in HST FR2 scenarios |  |  |
| TC9 | Requirements for SSB based radio link monitoring | No new test case needed. |  |
| TC10 | Requirements for SSB based beam failure detection | No new test case needed. |  |
| TC11 | Requirements for SSB based L1-RSRP | New test case needed. (New test case for A.7.6.3.X SSB based L1-RSRP measurement when DRX is used for FR2-1 PC6 UEs supporting SimultaneousReceptionFR2HST-r18) | Samsung |
| TC12 | Requirements for Intra-frequency measurements with/without measurement gaps | New test case needed. (New test case for A.7.6.1.X SA event triggered reporting test without gap under non-DRX for UE supporting [measurementEnhancementCAInterFreqFR2-r18]) | Nokia |

Sub-topic 1-1 Test case design on SSB based L1-RSRP in Rel-18 FR2 HST

Issue 1-1-1 AoA setup for multi-Rx chain DL reception in Rel-18 FR2 HST

Agreement:

- Necessity: It is necessary to define a new 2AoA setup for multi-Rx chain DL reception in Rel-18 FR2 HST.

  - The new 2AoA setup for FR2 HST RRM test cases is: 1 AoA in Rx beam peak direction, 1 in non Rx beam peak

  - The new 2AoA setup is applicable to support the SSB based L1-RSRP measurement test on FR2-1 PC6 UEs supporting SimultaneousReceptionFR2HST-r18

- How to design: Define the new 2AoA setup in new sub-Clause (s) in A.3.15.4 based on the RF requirements defined in Section 7.3K.6 TS 38.101-2.

Issue 1-1-2 Test case SSB configuration

Agreement:

- RAN4 to use two SSBs in SSB based L1-RSRP measurement test on FR2-1 PC6 UEs supporting SimultaneousReceptionFR2HST-r18

Sub-topic 1-2 Test case design on UL timing adjustment/TCI state switching in Rel-18 FR2 HST

Issue 1-2-1 Whether to split the testing of the UL timing adjustment and TCI state switching delay

Agreement from Wednesday RRM session:

- Introduce a new TC with 0 indication from MAC CE. Further discuss offline whether the corresponding Rel-17 legacy test can be skipped or kept.

- Rel-17 HST UL timing TC (A.7.5.8.3) is applicable to PC6 devices supporting enhanced MAC CE

Issue 1-2-2 How to design the MAC-CE indication based TCI state switch delay TC

Way Forward: Address directly at the CR stage.

Issue 1-2-3 Whether to define a new TC to verify the TCI state switching delay requirement in Multi-Rx scenario

Agreement from Wednesday RRM session:

For FR2 HST, it is RAN4 common understanding that the TCI state switching delay requirements defined in Rel-17 are applicable for the Rel-18 scenario with simultaneous reception.

Not introduce a new Rel-18 test case on TCI state switching delay for the PC6 UE receiving two simultaneous DL signals associated with two different QCL TypeD RSs.

Sub-topic 1-3 Missing testing parameters

Agreement:

- RAN4 to define Minimum SSB_RP values in the Table B.2.3-2: Conditions for inter-frequency measurements in FR2 for PC6 UEs

  - The Minimum SSB_RP values shall be defined based on the identified values agreed in the RAN5 reply LS discussion

Sub-topic 1-4 Test cases for inter-frequency intra-band CA measurements

Way forward:

- Update draft CR to use two RF channels (two cells) for SA inter-frequency test cases for HST FR2 PC6 UEs in Rel-18.

2. Demodulation requirement

WF (R4-2406004) on FR2 HST demodulation requirements is approved, with the following agreements and way forwards contained

Way forwards on FR2 HST demodulation requirements are provided as:

Sub-topic 1-1: PDSCH requirement with multi-Rx reception and CA requirements

Issue 1-1-1: PDSCH requirement for multi-Rx chain reception

Agreement:

- Updated the SNR requirement based on latest simulation summary of R4-2405860

  - Additional margin for MCS 19 is 0.8dB

Issue 1-1-2:   PDSCH requirement for CA

Agreement:

- Updated the SNR requirement based on latest simulation summary of R4-2405860

  - Additional margin for MCS 17 is 0.8dB

Sub-topic 2-1: Feature lists

Issue 2-1-1:  UE feature lists for FR2 PDSCH requirements with CA

Agreement:

- Not to introduce new UE feature components on enhanced demodulation requirements for CA HST FR2

Issue 2-1-2:  UE feature lists for FR2 PDSCH requirements with multi-Rx chain reception

Agreement:

- Not to introduce new UE feature components on enhanced demodulation requirements for FR2-1 PC6 UEs with simultaneous DL signals reception associated with two different QCL TypeD RSs.

The following CRs were endorsed

| Tdoc number | Title | Source | Status |
| --- | --- | --- | --- |
| R4-2406003 | Draft CR on PDSCH requirement with multi-Rx reception for FR2 HST (TS38.101-4, Rel-18) | Huawei | Endorsed |
| R4-2406005 | Draft CR on PDSCH requirements with CA | Nokia | Endorsed |
| R4- 2405985 | Big CR for TS 38.101-4 on Rel-18 FR2 HST demodulation requirements | Samsung | Email approval |

RAN4 #111 (Fukuoka City, Fukuoka, Japan, 20th – 24th May, 2024)

1. RRM requirement

Reply LS to RAN2 (R4-2410285) on RRM intra-frequency CA measurement enhancements for NR FR2 HST is approved

CR (R4-2410298) to TS 38.133 on HST FR2 Enhanced RRM Core Maintenance is agreed.

Big CR (R4-2410378) to TS 38.133 on Rel-18 HST FR2 RRM performance requirements is agreed with the following endorsed dCRs contained:

- R4-2406406, Test case requirements for cell reselection to FR2 inter-frequency NR carrier for FR2 HST

- R4-2410286, Test case on SA event triggered reporting for Rel-18 FR2 HST inter-frequency measurement with SSB time index detection when DRX is not used (Pcell in FR2)

- R4-2410287, Draft CR to 38.133 on UL Timing and TCI State Switch Test Case for HST FR2 Enhanced

- R4-2410288, Draft CR to 38.133 on Minimum SSB_RP Values for PC6 UEs

- R4-2410289, Draft CR to 38.133 on Enhanced Intra-Frequency Measurements Test Case for HST FR2 Enhanced

- R4-2410408, Draft CR for 38.133 on test case for FR2 HST intra-band CA without SSB time index detection when DRX is not used

- R4-2410291, Draft CR to 38.133 on test case for SSB based L1-RSRP for FR2 PC6 UE with multi-Rx

- R4-2410292, Draft CR to 38.133 on new 2AoA setup for multi-Rx chain DL reception in Rel-18 FR2 HST

2. Demodulation requirement

The following CRs were endorsed

| Tdoc number | Title | Source | Status |
| --- | --- | --- | --- |
| R4-2409840 | Big CR for TS 38.101-4 on Rel-18 FR2 HST demodulation requirements | Samsung | Email approval |
| R4-2409839 | Draft CR on PDSCH requirement with multi-Rx reception for FR2 HST (TS38.101-4, Rel-18) | Huawei | Endorsed |
| R4-2408063 | Draft CR on PDSCH requirements with CA | Nokia | Endorsed |

2.4.2	Remaining Open issues

Core Part:

- No remaining work expected.

Performance Part:

- No remaining work expected.

4.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

RAN4#110-bis contributions:

- R4-2406408   Draft Big CR to TS 38.133 on Rel-18 HST FR2 RRM performance requirements    Samsung, Nokia, Nokia Shanghai Bell

- R4-2406406   Test case requirements for cell reselection to FR2 inter-frequency NR carrier for FR2 HST    Intel Corporation

- R4-2406407   Draft CR for 38.133 on test case for FR2 HST intra-band CA without SSB time index detection when DRX is not used    Ericsson

- R4-2405691   Discussion on performance requirement for FR2 HST    Samsung

- R4-2406409   Draft CR to 38.133 on test case for SSB based L1-RSRP for FR2 PC6 UE with multi-Rx    Samsung

- R4-2405730   On HST FR2 Enhanced RRM Performance Requirements  Nokia

- R4-2406410   Draft CR to 38.133 on TCI State Switch for HST FR2 Enhanced Test Cases  Nokia

- R4-2406411   Draft CR to 38.133 on Enhanced Intra-Frequency Measurements Test Case for HST FR2 Enhanced  Nokia

- R4-2404823   Topic summary for [110bis][212] NR_HST_FR2_enh    Moderator (Samsung)

- R4-2406412   WF on FR2 HST RRM performance requirements    Samsung

- R4-2405860      Simulation results summary for Rel-18 FR2 HST demodulation requirements                           Samsung

- R4-2405985      Big CR for TS 38.101-4 on Rel-18 FR2 HST demodulation requirements                                  Samsung

- R4-2405089      UE demodulation requirements for FR2 HST with CA                                                                Ericsson

- R4-2405090      UE demodulation requirements for FR2 HST multi-Rx reception                                               Ericsson

- R4-2405127      Simulation results on UE multi-Rx demodulation requirements for HST FR2                             Huawei

- R4-2406003      Discussion on UE multi-Rx demodulation requirements for HST FR2                                        Huawei

- R4-2405788      Simulation Results on HST FR2 Enhanced with Multi-Rx Chain DL Reception                          Nokia

- R4-2405840      Topic summary for [110bis][321] NR_HST_FR2_enh_Demod                                            Moderator (Samsung)

- R4-2406004      WF on FR2 HST demodulation requirements	                                                                        Samsung

- R4-2406005      Draft CR on PDSCH requirements with CA                                                                                  Nokia

RAN4#111 contributions:

- R4-2408592   Reply LS on RRM enhancements for NR FR2 HST           Huawei, HiSilicon

- R4-2408609   Reply LS to RAN2 on RRM enhancements for NR FR2 HST          Ericsson

- R4-2408645   On LS Reply on RRM enhancements for NR FR2 HST to RAN2         Nokia

- R4-2408646   Draft LS Reply on RRM enhancements for NR FR2 HST to RAN2         Nokia

- R4-2408646   Draft LS Reply on RRM enhancements for NR FR2 HST to RAN2         Nokia

- R4-2410298   CR to 38.133 on HST FR2 Enhanced RRM Core Maintenance         Nokia

- R4-2408887   Discussion on intra-band CA intra-frequency configuration for Rel-18 FR2 HST   Samsung

- R4-2410285   Reply LS to RAN2 on RRM intra-frequency CA measurement enhancements for NR FR2 HST    Samsung

- R4-2410286  Test case on SA event triggered reporting for Rel-18 FR2 HST inter-frequency measurement with SSB time index detection when DRX is not used (Pcell in FR2)            Huawei, HiSilicon

- R4-2410287   Draft CR to 38.133 on UL Timing and TCI State Switch Test Case for HST FR2 Enhanced         Nokia

- R4-2410288   Draft CR to 38.133 on Minimum SSB_RP Values for PC6 UEs         Nokia

- R4-2410289    Draft CR to 38.133 on Enhanced Intra-Frequency Measurements Test Case for HST FR2 Enhanced         Nokia

- R4-2410408   Draft CR for 38.133 on test case for FR2 HST intra-band CA without SSB time index detection when DRX is not used         Ericsson

- R4-2410291   Draft CR to 38.133 on test case for SSB based L1-RSRP for FR2 PC6 UE with multi-Rx   Samsung

- R4-2410292   Draft CR to 38.133 on new 2AoA setup for multi-Rx chain DL reception in Rel-18 FR2 HST    Samsung, Nokia, Qualcomm, Ericsson

- R4-2410378   Big CR to TS 38.133 on Rel-18 HST FR2 RRM performance requirements   Samsung

- R4-2408066      On Finalizing the Big CR of HST FR2 Enhanced: Channel Modeling                                        Nokia

- R4-2409840      Big CR for TS 38.101-4 on Rel-18 FR2 HST demodulation requirements                           Samsung

- R4-2408063      Draft CR on PDSCH requirements with CA                                                                               Nokia

- R4-2408064      On Finalizing the Big CR of HST FR2 Enhanced: PDSCH with Carrier Aggregation                Nokia

- R4-2408065      On Finalizing the Big CR of HST FR2 Enhanced: PDSCH with Multi-Rx                                  Nokia

- R4-2409839      Draft CR on PDSCH requirement with multi-Rx reception for FR2 HST (TS38.101-4, Rel-18)   Huawei

- R4-2410115      Topic summary for [111][321] NR_HST_FR2_enh_Demod                                            Moderator (Samsung)

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template