# 3GPP TSG-RAN WG4 Meeting # 111                                                              R4-2408378

# Fukuoka, Japan, May. 20th – 24th, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-3 | CR | 1228 | rev |  | Current version: | 18.5.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (NR_newRAT-Core) Correct the NOTE for harmonic mixing MSD valid test point |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation, Sanechips |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core, DC_R16_1BLTE_1BNR_2DL2UL-Core, DC_R17_1BLTE_1BNR_2DL2UL-Core, DC_R18_1BLTE_1BNR_2DL2UL-Core, ENDC_UE_PC2_R17_NR_TDD-Core, HPUE_FR1_DC_LTE_NR_R18- Core, NR_6GHz_unlic_full |  |  |  |  |  |  | Date: |  | 2024-04-29 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In the agreed CR R4-2401255, we correct the NOTE for harmonic mixing MSD valid test point for inter-band NR CA, and the corrections have already been implemented in TS38.101 v18.5.0. For inter-band ENDC, similar corrections should be done. We copied the reasons in CR R4-2401255 as below which can also be applied to inter-band ENDC. Similar with UL/DL frequency condition of harmonic MSD, for harmonic mixing MSD test point, except the nth aggressor UL frequency hit the mth victim victim DL frequency, the valid aggressor UL carrier frequency must be restricted in the range of BWchannel/2 higher than the lower edge and lower than the upper edge of aggressor UL band, respectively. Out of this range, it is invalid MSD test point. Also, the NOTEs in Table 7.3B.2.3.1-5 are missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. Add the condition/equation of FUL_lowHB+BWCℎannelHB/2≤fULHB≤FUL_ℎigℎHB−BWCℎannelHB/2 to restrict the aggressor UL carrier frequency in the range of BWchannel/2 higher than the lower edge and lower than the upper edge of aggressor UL band, respectively. 2. Add the NOTEs in Table 7.3B.2.3.1-5. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - There may exist invalid test points for harmonic mixing MSD requirements. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3B.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  | X |  | Test specifications |  |  | TS38.521-3 |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

| This CR's revision history: |  |
| --- | --- |

# << Start of change >>

Table 7.3B.2.3.1-3: Reference sensitivity QPSK PREFSENS (EN-DC with n46)

| E-UTRA or NR Band / Channel bandwidth of the affected DL band / MSD |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 80 MHz (dB) | 90 MHz (dB) | 100 MHz (dB) |
| 2 | n461 |  |  |  | N/A |  | N/A |  | N/A | N/A |  |  |
| n46 | 22,3 | 28 | 28 | 28 | 28 |  |  |  |  |  |  |  |
| n46 | 482,4 | 22.6 | 19.5 | 17.8 | 16.6 |  |  |  |  |  |  |  |
| 66 | n46 |  |  |  | N/A |  | N/A |  | N/A | N/A |  |  |
| NOTE 1: These requirements apply when there is at least one individual RE within the downlink (victim) transmission bandwidth which falls into the reference sensitivity exclusion region as specified in Table 6.x.1.7-2 and Table 6.x.1.7-3. NOTE 2: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (higher) band and when the frequency range of relative higher band’s uplink channel bandwidth or uplink 1st adjacent channel bandwidth is fully or partially overlapped with the downlink transmission bandwidth of a victim (lower) band. NOTE 3: The requirements for a victim (lower) band apply for UL EARFCN of the aggressor (higher) band (superscript HB) such that in MHz and FUL_lowHB+BWCℎannelHB/2≤fULHB≤FUL_ℎigℎHB−BWCℎannelHB/2 with the DL carrier frequency in the lower band and fULHB the channel bandwidth configured in the higher band, both in MHz. Note 4: The requirements should be verified for UL NR-ARFCN of the aggressor (high) band (superscript HB) such that fULHB=15∗fDLLB0.1 in MHz and FUL_lowHB+BWCℎannelHB/2≤fULHB≤FUL_ℎigℎHB−BWCℎannelHB/2 with with carrier frequency in the victim (lower) band in MHz and the channel bandwidth configured in the higher band. |  |  |  |  |  |  |  |  |  |  |  |  |

Table 7.3B.2.3.1-4: Void

Table 7.3B.2.3.1-5: Uplink configuration for reference sensitivity exceptions due to receiver harmonic mixing for EN-DC paring with n46

| E-UTRA or NR Band / SCS / Channel bandwidth of the affected DL band / UL RB allocation of the agressor band |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | SCS of UL band (kHz) | 5 MHz (LCRB) | 10 MHz (LCRB) | 15 MHz (LCRB) | 20 MHz (LCRB) | 25 MHz (LCRB) | 40 MHz (LCRB) | 50 MHz (LCRB) | 60 MHz (LCRB) | 80 MHz (LCRB) | 90 MHz (LCRB) | 100 MHz (LCRB) |
| n46 | 2 | 15 | 25 | 50 | 75 | 100 |  |  |  |  |  |  |  |
| n46 | 48 | 15 | 12 | 25 | 36 | 50 |  |  |  |  |  |  |  |

7.3B.2.3.2	Reference sensitivity exceptions due to receiver harmonic mixing for EN-DC in NR FR1

Sensitivity degradation is allowed for different combinations of UL configurations and DL channel bandwidths if a band is impacted by receiver harmonic mixing due to another band part of the same EN-DC configuration. Reference sensitivity exceptions for the victim band (low) and uplink/downlink configurations due to UL harmonic from a PC3 aggressor UL band (high) for either single band uplink or PC3 or PC2 EN-DC are specified in Table 7.3B.2.3.2-1.

Reference sensitivity exceptions and uplink/downlink configurations due to harmonic mixing from a PC2 aggressor UL for PC2 EN-DC are specified in Table 7.3B.2.3.2-1a.

For these exceptions, only the listed test points in Table 7.3B.2.3.2-1 and Table 7.3B.2.3.2-1a need to be tested.

Table 7.3B.2.3.2-1: Reference sensitivity exceptions (MSD) due to receiver harmonic mixing for EN-DC in NR FR1

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| 1 | n71 | 5 | 15 | 25 (RBstart=0) | 5 | 26.8 | NOTE 4 | UL1/DL3 |
| 1 | n71 | 20 | 15 | 100 (RBstart=0) | 20 | 15.6 | NOTE 4 | UL1/DL3 |
| 1 | n105 | 5 | 15 | 25 (RBstart=0) | 5 | 26.8 | NOTE 4 | UL1/DL3 |
| 1 | n105 | 20 | 15 | 100 (RBstart=0) | 20 | 15.6 | NOTE 4 | UL1/DL3 |
| 2 | n71 | 5 | 15 | 25 (RBstart=0) | 5 | 26.8 | NOTE 4 | UL1/DL3 |
| 2 | n71 | 20 | 15 | 50 (RBstart=0) | 20 | 15.6 | NOTE 4 | UL1/DL3 |
| n2 | 71 | 5 | 15 | 25 (RBstart=0) | 5 | 26.8 | NOTE 4 | UL1/DL3 |
| n2 | 71 | 20 | 15 | 50 (RBstart=0) | 20 | 15.6 | NOTE 4 | UL1/DL3 |
| 7 | n2612 | 5 | 15 | 25 (RBstart=0) | 5 | [2.0] | NOTE 4 | UL1/DL3 |
| 7 | n105 | 5 | 15 | 25 (RBstart=0) | 5 | 5.7 |  |  |
| 7 | n105 | 20 | 15 | 100 (RBstart=0) | 20 | 2.7 |  |  |
| n25 | 71 | 5 | 15 | 25 (RBstart=0) | 5 | 26.8 | NOTE 4 | UL1/DL3 |
| n25 | 71 | 20 | 15 | 50 (RBstart=0) | 20 | 15.6 | NOTE 4 | UL1/DL3 |
| n38 | 59 | NA | NA | NA | NA | NA | NOTE 4 | UL1/DL3 |
| n40 | 20 | 5 | 15 | 25 (RBstart=0) | 5 | 27.8 | NOTE 4 | UL1/DL3 |
| n40 | 20 | 20 | 15 | 100 (RBstart=0) | 20 | 20.3 | NOTE 4 | UL1/DL3 |
| n40 | 28 | 5 | 15 | 25 (RBstart=0) | 5 | 37.8 | NOTE 4 | UL1/DL3 |
| n40 | 28 | 20 | 15 | 100 (RBstart=0) | 20 | 30.3 | NOTE 4 | UL1/DL3 |
| n41 | 5 | 10 | 15 | 25 (RBstart=0) | 5 | 24.3 | NOTE 4 | UL1/DL3 |
| n41 | 5 | 10 | 15 | 50 (RBstart=0) | 10 | 24.3 | NOTE 4 | UL1/DL3 |
| n41 | 18 | 10 | 15 | 25 (RBstart=0) | 5 | 24.3 | NOTE 4 | UL1/DL3 |
| n41 | 18 | 15 | 15 | 75 (RBstart=0) | 15 | 22.5 | NOTE 4 | UL1/DL3 |
| n41 | 26 | 10 | 15 | 25 (RBstart=0) | 5 | 24.3 | NOTE 4 | UL1/DL3 |
| n41 | 26 | 15 | 15 | 75 (RBstart=0) | 15 | 22.5 | NOTE 4 | UL1/DL3 |
| n46 | 2 | 5 | 15 | 25 (RBstart=0) | 5 | 28 | NOTE 4 | UL1/DL3 |
| n46 | 48 | 5 | 15 | 12 (RBstart=0) | 5 | 22.6 | NOTE 2 | UL2/DL3 |
| n46 | 48 | 20 | 15 | 100 (RBstart=0) | 20 | 16.6 | NOTE 2 | UL2/DL3 |
| 48 | n12 | 5 | 15 | 25 (RBstart=0) | 5 | 31 | NOTE 2 | UL1/DL5 |
| 48 | n12 | 10 | 15 | 50 (RBstart=0) | 10 | 28 | NOTE 2 | UL1/DL5 |
| n77 | 2 | 10 | 15 | 25 (RBstart=0) | 5 | 6.1 |  |  |
| n77 | 2 | 20 | 15 | 100 (RBstart=0) | 20 | 3.7 |  |  |
| n77 | 3 | 10 | 15 | 25 (RBstart=0) | 5 | 5.7 |  |  |
| n77 | 3 | 20 | 15 | 100 (RBstart=0) | 20 | 2.7 |  |  |
| n77 | 7 | 10 | 15 | 12 (RBstart=0) | 5 | 10.4 | NOTE 8 | UL2/DL3 |
| n77 | 7 | 10 | 15 | 50 (RBstart=0) | 20 | 10.4 | NOTE 8 | UL2/DL3 |
| n77 | 12 | 10 | 15 | 25 (RBstart=0) | 5 | 31 | NOTE 2 | UL1/DL5 |
| n77 | 12 | 10 | 15 | 50 (RBstart=0) | 10 | 28 | NOTE 2 | UL1/DL5 |
| n77 | 13 | 10 | 15 | 25 (RBstart=0) | 5 | 31 | NOTE 2 | UL1/DL5 |
| n77 | 13 | 10 | 15 | 50 (RBstart=0) | 10 | 28 | NOTE 2 | UL1/DL5 |
| n77 | 14 | 10 | 15 | 25 (RBstart=0) | 5 | 31 | NOTE 2 | UL1/DL5 |
| n77 | 14 | 10 | 15 | 50 (RBstart=0) | 10 | 28 | NOTE 2 | UL1/DL5 |
| n77 | 19 | 10 | 15 | 25 (RBstart=0) | 5 | 7.2 |  |  |
| n77 | 19 | 15 | 15 | 75 (RBstart=0) | 15 | 3.8 |  |  |
| n77 | 25 | 10 | 15 | 25 (RBstart=0) | 5 | 6.1 |  |  |
| n77 | 25 | 20 | 15 | 100 (RBstart=0) | 20 | 3.7 |  |  |
| n77 | 28 | 10 | 15 | 25 (RBstart=0) | 5 | 28 | NOTE 2 | UL1/DL5 |
| n77 | 28 | 20 | 15 | 100 (RBstart=0) | 20 | 22 | NOTE 2 | UL1/DL5 |
| n77 | 29 | 10 | 15 | 25 (RBstart=0) | 5 | 31 | NOTE 2 | UL1/DL5 |
| n77 | 29 | 10 | 15 | 50 (RBstart=0) | 10 | 28 | NOTE 2 | UL1/DL5 |
| n77 | 41 | 10 | 15 | 12 (RBstart=0) | 5 | 10.4 | NOTE 8 | UL2/DL3 |
| n77 | 41 | 10 | 15 | 50 (RBstart=0) | 20 | 10.4 | NOTE 8 | UL2/DL3 |
| n78 | 3 | 10 | 15 | 25 (RBstart=0) | 5 | 5.7 |  |  |
| n78 | 3 | 20 | 15 | 100 (RBstart=0) | 20 | 2.7 |  |  |
| n78 | 8 | 10 | 15 | 25 (RBstart=0) | 5 | 5.7 |  |  |
| n78 | 8 | 20 | 15 | 100 (RBstart=0) | 20 | 2.7 |  |  |
| n78 | 12 | 10 | 15 | 25 (RBstart=0) | 5 | 31 | NOTE 2 | UL1/DL5 |
| n78 | 12 | 10 | 15 | 50 (RBstart=0) | 10 | 28 | NOTE 2 | UL1/DL5 |
| n78 | 13 | 10 | 15 | 25 (RBstart=0) | 5 | 31 | NOTE 2 | UL1/DL5 |
| n78 | 13 | 10 | 15 | 50 (RBstart=0) | 10 | 28 | NOTE 2 | UL1/DL5 |
| n78 | 19 | 10 | 15 | 25 (RBstart=0) | 5 | 7.2 |  |  |
| n78 | 19 | 15 | 15 | 75 (RBstart=0) | 15 | 3.8 |  |  |
| n78 | 40 | 10 | 15 | 12 (RBstart=0) | 5 | 10.4 | NOTE 8 | UL2/DL3 |
| n78 | 40 | 20 | 15 | 50 (RBstart=0) | 20 | 10.4 | NOTE 8 | UL2/DL3 |
| n78 | 41 | 10 | 15 | 12 (RBstart=0) | 5 | 10.4 | NOTE 8 | UL2/DL3 |
| n78 | 41 | 20 | 15 | 50 (RBstart=0) | 20 | 10.4 | NOTE 8 | UL2/DL3 |
| n79 | 8 | 10 | 15 | 25 (RBstart=0) | 5 | 25 | NOTE 2 | UL1/DL5 |
| n79 | 8 | 20 | 15 | 100 (RBstart=0) | 20 | 22 | NOTE 2 | UL1/DL5 |
| n79 | 11 | 10 | 15 | 25 (RBstart=0) | 5 | 39.3 | NOTE 4 | UL1/DL3 |
| n79 | 11 | 15 | 15 | 75 (RBstart=0) | 15 | 34.5 | NOTE 4 | UL1/DL3 |
| n79 | 19 | 10 | 15 | 25 (RBstart=0) | 5 | 29.5 | NOTE 2 | UL1/DL5 |
| n79 | 19 | 15 | 15 | 75 (RBstart=0) | 15 | 24.7 | NOTE 2 | UL1/DL5 |
| n79 | 21 | 10 | 15 | 25 (RBstart=0) | 5 | 39.3 | NOTE 4 | UL1/DL3 |
| n79 | 21 | 15 | 15 | 75 (RBstart=0) | 15 | 34.5 | NOTE 4 | UL1/DL3 |
| n79 | 26 | 10 | 15 | 25 (RBstart=0) | 5 | 27 | NOTE 2 | UL1/DL5 |
| n79 | 26 | 15 | 15 | 75 (RBstart=0) | 15 | 22.2 | NOTE 2 | UL1/DL5 |
| NOTE 1: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (higher) band for which the mixing product due to harmonic of victim (lower) band LO with leakage of aggressor (higher) band is within the downlink transmission bandwidth of a victim (lower) band. NOTE 2: The requirements should be verified for DL EARFCN of the victim (lower) band (superscript LB) such that and FUL_lowHB+BWCℎannelHB/2≤fULHB≤FUL_ℎigℎHB−BWCℎannelHB/2 with the DL carrier frequency in the lower band and fULHB the UL carrier frequency and BWCℎannelHB the channel bandwidth configured in the higher band, both in MHz. NOTE 3: Void. NOTE 4: The requirements should be verified for DL EARFCN or NR ARFCN of the victim (lower) band (superscript LB) such that and FUL_lowHB+BWCℎannelHB/2≤fULHB≤FUL_ℎigℎHB−BWCℎannelHB/2 with the DL carrier frequency in the lower band and fULHB the UL carrier frequency and BWCℎannelHB the channel bandwidth configured in the higher band, both in MHz. NOTE 5: Void NOTE 6: Void NOTE 7: Void NOTE 8: The requirements should be verified for DL EARFCN of the victim (lower) band (superscript LB) such that fDLLB=fULHB/0.150.1 and BWCℎannelHB the channel bandwidth configured with the DL carrier frequency in the lower band and fULHB the UL carrier frequency and BWCℎannelHB the channel bandwidth configured in the higher band, both in MHz. NOTE 9: No requirements apply for the case that there is at least one individual RE within the uplink transmission bandwidth of the relative higher band and when the frequency range of relative higher band’s uplink channel bandwidth or uplink 1st adjacent channel bandwidth is fully or partially overlapped with the 3 times of the frequency range of the relative lower band’s downlink channel bandwidth. The reference sensitivity is only verified when this is not the case. NOTE 10: MSD test point can be chosen according to supported BW and lowest SCS supported by the UE. NOTE 11: The MSD test points cannot be verified for the band combination in US due to the Band n77 frequency range restriction. NOTE 12: The requirements should be verified for the lowest NR ARFCN of the affected DL (lower) band and for the highest NR ARFCN of the UL (higher) band |  |  |  |  |  |  |  |  |

Table 7.3B.2.3.2-1a: Reference sensitivity exceptions (MSD) due to receiver harmonic mixing for PC2 EN-DC in NR FR1

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n41 | 18 | 5 | 15 | 25 (RBstart=0) | 5 | 27.3 | NOTE 4 | UL1/DL3 |
| n77 | 2 | 10 | 15 | 25 (RBstart=0) | 5 | 9.1 |  |  |
| n77 | 2 | 20 | 15 | 100 (RBstart=0) | 20 | 6.7 |  |  |
| n77 | 3 | 10 | 15 | 25 (RBstart=0) | 5 | 8.1 |  |  |
| n77 | 3 | 20 | 15 | 100 (RBstart=0) | 20 | 5.7 |  |  |
| n77 | 12 | 10 | 15 | 25 (RBstart=0) | 5 | 34 | NOTE 1 | UL1/DL5 |
| n77 | 12 | 10 | 15 | 50 (RBstart=0) | 10 | 31 | NOTE 1 | UL1/DL5 |
| n77 | 13 | 10 | 15 | 25 (RBstart=0) | 5 | 34 | NOTE 1 | UL1/DL5 |
| n77 | 13 | 10 | 15 | 50 (RBstart=0) | 10 | 31 | NOTE 1 | UL1/DL5 |
| n77 | 14 | 10 | 15 | 25 (RBstart=0) | 5 | 34 | NOTE 1 | UL1/DL5 |
| n77 | 14 | 10 | 15 | 50 (RBstart=0) | 10 | 31 | NOTE 1 | UL1/DL5 |
| n77 | 19 | 10 | 15 | 25 (RBstart=0) | 5 | 9.8 |  |  |
| n77 | 19 | 15 | 15 | 75 (RBstart=0) | 15 | 5.8 |  |  |
| n77 | 28 | 10 | 15 | 25 (RBstart=0) | 5 | 32 | NOTE 1 | UL1/DL5 |
| n77 | 28 | 20 | 15 | 100 (RBstart=0) | 20 | 25 | NOTE 1 | UL1/DL5 |
| n772 | 29 | 10 | 15 | 25 (RBstart=0) | 5 | 34 | NOTE 1 | UL1/DL5 |
| n772 | 29 | 10 | 15 | 50 (RBstart=0) | 10 | 31 | NOTE 1 | UL1/DL5 |
| n77 | 41 | 10 | 15 | 12 (RBstart=0) | 5 | 19.4 | NOTE 4 | UL2/DL3 |
| n77 | 41 | 20 | 15 | 50 (RBstart=0) | 20 | 19.4 | NOTE 4 | UL2/DL3 |
| n78 | 8 | 10 | 15 | 25 (RBstart=0) | 5 | 8.1 | NOTE 5 | UL1/DL4 |
| n78 | 8 | 20 | 15 | 20 (RBstart=0) | 20 | 4.3 | NOTE 5 | UL1/DL4 |
| n78 | 5 | 10 | 15 | 25 (RBstart=0) | 5 | 8.1 | NOTE 5 | UL1/DL4 |
| n78 | 13 | 10 | 15 | 25 (RBstart=0) | 5 | 34 | NOTE 1 | UL1/DL5 |
| n78 | 19 | 10 | 15 | 25 (RBstart=0) | 5 | 9.8 |  |  |
| n78 | 19 | 15 | 15 | 75 (RBstart=0) | 15 | 5.8 |  |  |
| n78 | 28 | 10 | 15 | 25 (RBstart=0) | 5 | 31 | NOTE 1 | UL1/DL5 |
| n78 | 28 | 20 | 15 | 100 (RBstart=0) | 20 | 25 | NOTE 1 | UL1/DL5 |
| n78 | 41 | 10 | 15 | 12 (RBstart=0) | 5 | 19.4 | NOTE 4 | UL2/DL3 |
| n78 | 41 | 20 | 15 | 50 (RBstart=0) | 20 | 19.4 | NOTE 4 | UL2/DL3 |
| n78 | 40 | 10 | 15 | 50 (RBstart=0) | 5 | 16.2 | NOTE 4 | UL2/DL3 |
| n79 | 19 | 10 | 15 | 25 (RBstart=0) | 5 | 32.5 | NOTE 1 | UL1/DL5 |
| n79 | 19 | 15 | 15 | 75 (RBstart=0) | 15 | 27.7 | NOTE 1 | UL1/DL5 |
| n79 | 21 | 10 | 15 | 25 (RBstart=0) | 5 | 42.3 | NOTE 3 | UL1/DL3 |
| n79 | 21 | 15 | 15 | 75 (RBstart=0) | 15 | 37.5 | NOTE 3 | UL1/DL3 |
| NOTE 1: The requirements should be verified for DL EARFCN of the victim (lower) band (superscript LB) such that and FUL_lowHB+BWCℎannelHB/2≤fULHB≤FUL_ℎigℎHB−BWCℎannelHB/2 with the DL carrier frequency in the lower band and fULHB the UL carrier frequency and BWCℎannelHB the channel bandwidth configured in the higher band, both in MHz. NOTE 2: For a UE which supports this band combination only when the Band n77 frequency range restriction defined in NOTE 12 of Table 5.2-1 from TS 38.101-1 applies, the MSD test point(s) cannot be verified for the band combination and the test point(s) can be skipped. NOTE 3: The requirements should be verified for DL EARFCN or NR ARFCN of the victim (lower) band (superscript LB) such that and FUL_lowHB+BWCℎannelHB/2≤fULHB≤FUL_ℎigℎHB−BWCℎannelHB/2 with the DL carrier frequency in the lower band and fULHB the UL carrier frequency and BWCℎannelHB the channel bandwidth in the higher band, both in MHz. NOTE 4: The requirements should be verified for DL EARFCN of the victim (lower) band (superscript LB) such that fDLLB=fULHB/0.150.1 and FUL_lowHB+BWCℎannelHB/2≤fULHB≤FUL_ℎigℎHB−BWCℎannelHB/2 with the DL carrier frequency in the lower band and fULHB the UL carrier frequency and BWCℎannelHB the channel bandwidth in the higher band, both in MHz. NOTE 5: The requirements should be verified for UL NR-ARFCN of the aggressor (higher) band (superscript HB) such that in MHz and FUL_lowHB+BWCℎannelHB/2≤fULHB≤FUL_ℎigℎHB−BWCℎannelHB/2 with the carrier frequency in the victim (lower) band and the channel bandwidth configured in the higher band |  |  |  |  |  |  |  |  |

Table 7.3B.2.3.2-2: Void

# << End change >>