3GPP TSG-RAN4 Meeting #111	R4-2408474

# Fukuoka City, Fukuoka, Japan, 20th May 2024 - 24th May 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.141 | CR | 1085 | rev | - | Current version: | 17.13.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (TEI17) CR to 37.141 on Removal of Additional spurious emission limits for GSM (BC2) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17 |  |  |  |  |  | Date: |  |  | 2024-05-13 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | An issue with the MSR BS spurious emission limits was identified in a review of the European harmonised standard for MSR BS. It was pointed out that having two sets of limits with different measurement bandwidths is confusing and that also the second set of limits were not aligned with international recommendations (ERC Rec 74-01). The unwanted emission limits for GSM were developed within the CEPT GSM project over the years 1986-1989 in the group CEPT/CCH/GSM/WP2. The first set of spurious emission limits proposed for GSM can be found in an ad hoc report from June 1988 (WP2 Doc 4.88 Attachment 1, accessible with 3GU login). The limits were intended for the new Recommendation GSM 05.05, which corresponds to present 3GPP TS 45.005. It can be seen that the limits in the present TS 45.005 and the 1988 proposed values for GSM 05.05 bear a striking resemblance, both in terms of limits and measurement bandwidths. A subset of those values is present in the MSR BS specifications as “Additional minimum requirement for BC2 (Category B)” and apply for a BS operating in Band Category 2 when GSM/EDGE is configured. This was considered an important aspect by many GSM proponents when the MSR BS specifications were developed back in 2009, since it would mean that GSM operation with new MSR BS would be restricted by the same limits as legacy GSM BTS with single carrier transceivers. The situation is very different today when most GSM is deployed with MSR BS and GSM BTS with SC transceivers may not have been produced since more than 10 years. It should also be noted that the 35+ year old GSM spurious limits are not aligned with the limits in SM.329-12 or ERC Recommendation 74-01. The Category B limits in those recommendations were in fact develop 10 years after the development of GSM. The major difference is the measurement bandwidths, where the GSM limits are expressed for several non-standard bandwidths at certain offsets. The conclusion is that the GSM additional limits for BC2 are not anymore relevant, since they use non-standard measurement bandwidths based on work that predates the present international recommendations for unwanted emissions and are thus not aligned. Removing them will make the MSR BS specification better aligned with the international recommendation in SM.329-12 and ultimately with the Radio Regulations. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The “Additional minimum requirement for BC2 (Category B)” for spurious emissions is removed. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The MSR BS emission limits would not be aligned with international recommendations such as SM.329-12. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.6.1.5, 6.6.1.5.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 37.104 |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### 6.6.1.5	Test requirements

The measurement result in step 2 of 6.6.1.4.2 shall not exceed the maximum level specified in Table 6.6.1.5.1-1 to Table 6.6.1.5.6-1 if applicable for the BS under test.

The test requirements of either clause 6.6.1.5.1 (Category A limits) or clause 6.6.1.5.2 (Category B limits) shall apply.

##### 6.6.1.5.1	Spurious emissions (Category A)

The power of any spurious emission shall not exceed the limits in Table 6.6.1.5.1-1

Table 6.6.1.5.1-1: BS Spurious emission limits, Category A

| Frequency range | Maximum level | Measurement Bandwidth | Note |
| --- | --- | --- | --- |
| 9kHz - 150kHz |  | 1 kHz | Note 1 |
| 150kHz - 30MHz |  | 10 kHz | Note 1 |
| 30MHz - 1GHz | -13 dBm | 100 kHz | Note 1 |
| 1GHz - 12.75 GHz |  | 1 MHz | Note 2 |
| 12.75 GHz – 5th harmonic of the upper frequency edge of the DL operating band in GHz |  | 1 MHz | Note 2, Note 3 |
| NOTE 1: Bandwidth as in ITU-R SM.329 [13], s4.1 NOTE 2: Bandwidth as in ITU-R SM.329 [13], s4.1. Upper frequency as in ITU-R SM.329 [13], s2.5 table 1 NOTE 3: This spurious frequency range applies only for operating bands for which the 5th harmonic of the upper frequency edge of the DL operating band is reaching beyond 12.75 GHz. |  |  |  |

##### 6.6.1.5.2	Spurious emissions (Category B)

The power of any spurious emission shall not exceed the limits in Table 6.6.1.5.2-1

Table 6.6.1.5.2-1: BS Spurious emissions limits, Category B

| Frequency range | Maximum Level | Measurement Bandwidth | Note |
| --- | --- | --- | --- |
| 9 kHz 150 kHz | -36 dBm | 1 kHz | Note 1 |
| 150 kHz 30 MHz | -36 dBm | 10 kHz | Note 1 |
| 30 MHz 1 GHz | -36 dBm | 100 kHz | Note 1 |
| 1 GHz 12.75 GHz | -30 dBm | 1 MHz | Note 2 |
| 12.75 GHz 5th harmonic of the upper frequency edge of the DL operating band in GHz | -30 dBm | 1 MHz | Note 2, Note 3 |
| NOTE 1: Bandwidth as in ITU-R SM.329 [13], s4.1 NOTE 2: Bandwidth as in ITU-R SM.329 [13], s4.1. Upper frequency as in ITU-R SM.329 [13], s2.5 table 1 NOTE 3: This spurious frequency range applies only for operating bands for which the 5th harmonic of the upper frequency edge of the DL operating band is reaching beyond 12.75 GHz. |  |  |  |

##### 6.6.1.5.3	Void