3GPP TSG-WG4 Meeting #111	R4-2407645

# Fukuoka, Japan, May 20th – 24th, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR | 2234 | rev | - | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR 38.101-1 correction CR for Rx requirements with TxD indication |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17 |  |  |  |  |  | Date: |  |  | 2024-05-07 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | txDiversity2Tx-r18 was introduced in Rel-18, which can also be used to indicate a single band (LS R4-2321983). The Rx requirements with Tx UL configuration with TxD indication should be extended to capability of txDiversity2Tx-r18. As requested by RAN4, the capability should be supported from Rel-17 (LS R4-2403857). |  |  |  |  |  |  |  |  |
| Summary of change: |  | Extend TxD capability to include indication of txDiversity2Tx-r18 for PC2 UL band which may cause REFSENS degradation. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Ambiguity exists in the specificication. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3.2, 7.3A.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-1 |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change>

7.3.2	Reference sensitivity power level

The throughput shall be ≥ 95 % of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2.2, A3.2 and A.3.3 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1) with parameters specified in Table 7.3.2-1a, Table 7.3.2-1b, Table 7.3.2-1c, Table 7.3.2-1d and Table 7.3.2-2.

Table 7.3.2-1a: Two antenna port reference sensitivity QPSK PREFSENS for FDD bands

| Operating band / SCS / Channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 35 MHz (dBm) | 40 MHz (dBm) | 45 MHz (dBm) | 50 MHz (dBm) |
| n1 | 15 |  | -100.0 | -96.8 | -95.0 | -93.8 | -92.7 | -91.9 |  | -90.6 | -90.1 | -89.6 |
|  | 30 |  |  | -97.1 | -95.1 | -94.0 | -92.8 | -92.0 |  | -90.7 | -90.2 | -89.7 |
|  | 60 |  |  | -97.5 | -95.4 | -94.2 | -93.0 | -92.1 |  | -90.9 | -90.3 | -89.7 |
| n2 | 15 |  | -98 | -94.8 | -93 | -91.8 | -90.7 | -84.1 | -83.6 | -81.5 |  |  |
|  | 30 |  |  | -95.1 | -93.1 | -92 | -90.8 | -84.2 | -83.7 | -81.6 |  |  |
|  | 60 |  |  | -95.5 | -93.4 | -92.2 | -90.9 | -84.3 | -83.8 | -81.7 |  |  |
| n3 | 15 |  | -97.0 | -93.8 | -92.0 | -90.8 | -89.7 | -88.9 | -86.2 | -82.3 | -81.3 | -79.7 |
|  | 30 |  |  | -94.1 | -92.1 | -91.0 | -89.8 | -89.0 | -86.3 | -82.4 | -81.4 | -79.8 |
|  | 60 |  |  | -94.5 | -92.4 | -91.2 | -90.0 | -89.1 | -86.4 | -82.6 | -81.5 | -79.9 |
| n5 | 15 |  | -98.0 | -94.8 | -93.0 | -86.8 | -84.8 |  |  |  |  |  |
|  | 30 |  |  | -95.1 | -93.1 | -88.6 | -84.9 |  |  |  |  |  |
| n71 | 15 |  | -98.0 | -94.8 | -93.0 | -91.8 | -90.7 | -89.9 | -89.2 | -88.6 |  | -81.5 |
|  | 30 |  |  | -95.1 | -93.1 | -92.0 | -90.8 | -90.0 | -89.3 | -88.7 |  | -81.5 |
|  | 60 |  |  | -95.5 | -93.4 | -92.2 | -91.0 | -90.1 | -89.4 | -88.9 |  | -81.5 |
| n8 | 15 |  | -97.0 | -93.8 | -91.4 | -85.8 | -83.6 | -81.3 | -78.4 |  |  |  |
|  | 30 |  |  | -94.1 | -91.7 | -87.2 | -84.7 | -81.4 | -78.5 |  |  |  |
| n12 | 15 |  | -97.0 | -93.8 | -84.0 |  |  |  |  |  |  |  |
|  | 30 |  |  | -94.1 | -84.1 |  |  |  |  |  |  |  |
| n13 | 15 |  | -97.0 | -93.8 |  |  |  |  |  |  |  |  |
|  | 30 |  |  | -94.1 |  |  |  |  |  |  |  |  |
| n14 | 15 |  | -97.0 | -93.8 |  |  |  |  |  |  |  |  |
|  | 30 |  |  | -94.1 |  |  |  |  |  |  |  |  |
| n18 | 15 |  | -100.0 | -96.8 | -95.0 |  |  |  |  |  |  |  |
|  | 30 |  |  | -97.1 | -95.1 |  |  |  |  |  |  |  |
| n20 | 15 |  | -97.0 | -93.8 | -91.0 | -89.8 |  |  |  |  |  |  |
|  | 30 |  |  | -94.1 | -91.1 | -90.0 |  |  |  |  |  |  |
| n24 | 15 |  | -100.0 | -96.8 |  |  |  |  |  |  |  |  |
|  | 30 |  |  | -97.1 |  |  |  |  |  |  |  |  |
|  | 60 |  |  | -97.5 |  |  |  |  |  |  |  |  |
| n25 | 15 |  | -96.5 | -93.3 | -91.5 | -90.3 | -89.3 | -82.2 | -81.7 | -79.5 | -77.6 |  |
|  | 30 |  |  | -93.6 | -91.6 | -90.5 | -89.4 | -82.3 | -81.8 | -79.6 | -77.7 |  |
|  | 60 |  |  | -94.0 | -91.9 | -90.7 | -89.6 | -82.4 | -81.9 | -79.7 | -77.8 |  |
| n26 | 15 | -99.7 | -97.56 | -94.56 | -92.76 | -87.6 | -84.5 | -81.7 |  |  |  |  |
|  | 30 |  |  | -94.86 | -92.76 | -87.7 | -84.6 | -81.8 |  |  |  |  |
| n28 | 15 | -100.2 | -98.5 | -95.5 | -93.5 | -90.8 | -84.2 | -78.5 |  |  |  |  |
|  | 30 |  |  | -95.6 | -93.6 | -91.0 | -84.2 | -78.6 |  |  |  |  |
| n30 | 15 |  | -99.0 | -95.8 |  |  |  |  |  |  |  |  |
|  | 30 |  |  | -96.1 |  |  |  |  |  |  |  |  |
| n31 | 15 | -95.7 | -93.5 |  |  |  |  |  |  |  |  |  |
| n65 | 15 |  | -99.5 | -96.3 | -94.5 | -93.3 |  |  |  |  |  | -89.2 |
|  | 30 |  |  | -96.6 | -94.6 | -93.5 |  |  |  |  |  | -89.3 |
|  | 60 |  |  | -97.0 | -94.9 | -93.7 |  |  |  |  |  | -89.4 |
| n66 | 15 |  | -99.5 | -96.3 | -94.5 | -93.3 | -92.2 | -91.4 | -90.7 | -90.1 | -89.6 |  |
|  | 30 |  |  | -96.6 | -94.6 | -93.5 | -92.3 | -91.5 | -90.8 | -90.2 | -89.7 |  |
|  | 60 |  |  | -97.0 | -94.9 | -93.7 | -92.5 | -91.6 | -90.9 | -90.4 | -89.8 |  |
|  | 15 |  | -100.0 | -96.8 | -95.0 | -93.8 | -92.7 |  |  |  |  |  |
| n70 | 30 |  |  | -97.1 | -95.1 | -94.0 | -92.8 |  |  |  |  |  |
|  | 60 |  |  | -97.5 | -95.4 | -94.2 | -93.0 |  |  |  |  |  |
| n71 | 15 |  | -97.2 | -94.0 | -91.6 | -86.0 | -84.19 -74.810 | -82.59 -67.110 | -80.79 -64.010 |  |  |  |
|  | 30 |  |  | -94.3 | -91.9 | -87.4 | -84.29 -74.910 | -82.69 -67.210 | -80.89 -64.110 |  |  |  |
| n72 | 15 | -95.7 | -93.5 |  |  |  |  |  |  |  |  |  |
| n74 | 15 |  | -99.53 | -96.33 | -94.53 | -89.33 |  |  |  |  |  |  |
|  | 30 |  |  | -96.63 | -94.63 | -89.53 |  |  |  |  |  |  |
|  | 60 |  |  | -97.03 | -94.93 | -89.63 |  |  |  |  |  |  |
| n85 | 15 | -99.2 | -97.0 | -93.8 | -84.0 |  |  |  |  |  |  |  |
|  | 30 |  |  | -94.1 | -84.1 |  |  |  |  |  |  |  |
| n100 | 15 | -102.2 | -100 |  |  |  |  |  |  |  |  |  |
| n105 | 15 |  | -97.28 | -94.0 | -91.6 | -86.9 | -85.1 | -83.8 | -82.5 |  |  |  |
|  | 30 |  |  | -94.3 | -91.9 | -87.9 | -85.5 | -84.3 | -82.6 |  |  |  |
| n106 | 15 | -99.2 |  |  |  |  |  |  |  |  |  |  |
| NOTE 1: Four Rx antenna ports shall be the baseline for this operating band except for two Rx vehicular UE and two Rx antenna port XR UEs indicating UE capability [2Rx XR]. Four Rx antenna ports for RedCap UE is not supported for this operating band. NOTE 2: The transmitter shall be set to PUMAX as defined in clause 6.2.4 NOTE 3: The requirement is modified by -0.5 dB when the assigned NR channel bandwidth is confined within 1475.9 - 1510.9 MHz. NOTE 4: Void NOTE 5: Void NOTE 6: Values are modified by -0.5dB when carrier channel BW is between 865MHz and 894MHz. NOTE 7: Void. NOTE 8: DL channels overlapping the 612-617MHz range have 0.5dB added to the REFSENS NOTE 9: Applies to UEs that support a maximum uplink BW of 20 MHz in this band. NOTE 10: Applies to UEs that support optional symmetric UL/DL for this BW. |  |  |  |  |  |  |  |  |  |  |  |  |

Table 7.3.2-1b: Two antenna port reference sensitivity QPSK PREFSENS for TDD, SDL and FDD with variable duplex operation bands

| Operating band / SCS / Channel bandwidth / REFSENS |  |  |  |  |
| --- | --- | --- | --- | --- |
| Operating band | SCS kHz | Channel bandwidth (MHz) | REFSENS (dBm)8 | Duplex Mode |
| n297 | 15 | 5,10 | -97 + 10log10(NRB/25) | SDL |
|  | 30 | 10 | -94.1 + 10log10(NRB/24) |  |
| n34 | 15 | 5, 10, 15 | -100 + 10log10(NRB/25) | TDD |
|  | 30 | 10, 15 | -97.1 + 10log10(NRB/24) |  |
|  | 60 | 10, 15 | -97.5 + 10log10(NRB/11) |  |
| n381 | 15 | 5, 10, 15, 20, 25, 30, 40 | -100 + 10log10(NRB/25) | TDD |
|  | 30 | 10, 15, 20, 25, 30, 40 | -97.1 + 10log10(NRB/24) |  |
|  | 60 | 10, 15, 20, 25, 30, 40 | -97.5 + 10log10(NRB/11) |  |
| n39 | 15 | 5, 10, 15, 20, 25, 30, 35, 40 | -100 + 10log10(NRB/25) | TDD |
|  | 30 | 10, 15, 20, 25, 30, 35, 40 | -97.1 + 10log10(NRB/24) |  |
|  | 60 | 10, 15, 20, 25, 30, 35, 40 | -97.5 + 10log10(NRB/11) |  |
| n40 | 15 | 5, 10, 15, 20, 25, 30, 40, 50 | -100 + 10log10(NRB/25) | TDD |
|  | 30 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | -97.1 + 10log10(NRB/24) |  |
|  | 60 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | -97.5 + 10log10(NRB/11) |  |
| n411, n901 | 15 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 | -94.8 + 10log10(NRB/52) | TDD |
|  | 30 | 10, 15, 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 | -95.1 + 10log10(NRB/24) |  |
|  | 60 | 10, 15, 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 | -95.5 + 10log10(NRB/11) |  |
| n481 | 15 | 5, 10, 15, 20, 30, 40, 505 | -99 + 10log10(NRB/25) | TDD |
|  | 30 | 10, 15, 20, 30, 40, 505, 605, 705, 805, 905, 1005 | -96.1 + 10log10(NRB/24) |  |
|  | 60 | 10, 15, 20, 30, 40, 505, 605, 705, 805, 905, 1005 | -96.5 + 10log10(NRB/11) |  |
| n50 | 15 | 5, 10, 15, 20, 30, 40, 50 | -100 + 10log10(NRB/25) | TDD |
|  | 30 | 10, 15, 20, 30, 40, 50, 60, 80 | -97.1 + 10log10(NRB/24) |  |
|  | 60 | 10, 15, 20, 30, 40, 50, 60, 80 | -97.5 + 10log10(NRB/11) |  |
| n51 | 15 | 5 | -100 | TDD |
| n53 | 15 | 5, 10 | -100 + 10log10(NRB/25) | TDD |
|  | 30 | 10 | -97.1 |  |
|  | 60 | 10 | -97.5 |  |
| n54 | 15 | 5 | -100 | TDD |
| n677 | 15 | 5, 10, 15, 20 | -100 + 10log10(NRB/25) | SDL |
|  | 30 | 10, 15, 20 | -97.1 + 10log10(NRB/24) |  |
| n757 | 15 | 5,10,15,20,25,30,40,50 | -100 + 10log10(NRB/25) | SDL |
|  | 30 | 10,15,20,25,30,40,50 | -97.1 + 10log10(NRB/24) |  |
|  | 60 | 10,15,20,25,30,40,50 | -97.5 + 10log10(NRB/11) |  |
| n767 | 15 | 5 | -95.3 + 10log10(NRB/52) | SDL |
| n771,4 | 15 | 10, 15, 20, 25, 30, 40, 50 | -95.3 + 10log10(NRB/52) | TDD |
|  | 30 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | -95.6 + 10log10(NRB/24) |  |
|  | 60 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | -96.0 + 10log10(NRB/11) |  |
| n781 | 15 | 10, 15, 20, 25, 30, 40, 50 | -95.8 + 10log10(NRB/52) | TDD |
|  | 30 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | -96.1 + 10log10(NRB/24) |  |
|  | 60 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | -96.5 + 10log10(NRB/11) |  |
| n791 | 15 | 10, 20, 30, 40, 50 | -95.8 + 10log10(NRB/52) | TDD |
|  | 30 | 10, 20, 30, 40, 50, 60, 70, 80, 90, 100 | -96.1 + 10log10(NRB/24) |  |
|  | 60 | 10, 20, 30, 40, 50, 60, 70, 80, 90, 100 | -96.5 + 10log10(NRB/11) |  |
| n91 | 15 | 5 | -100 | FDD |
| n92 | 15 | 5,10,15,20 | -100 + 10log10(NRB/25) | FDD |
|  | 30 | 10,15,20 | -97.1 + 10log10(NRB/24) |  |
| n93 | 15 | 5 | -100 | FDD |
| n94 | 15 | 5,10,15,20 | -100 + 10log10(NRB/25) | FDD |
|  | 30 | 10,15,20 | -97.1 + 10log10(NRB/24) |  |
| n101 | 15 | 5, 10 | -100 + 10log10(NRB/25) | TDD |
|  | 30 | 10 | -97.1 + 10log10(NRB/24) |  |
| n1041,10 | 15 | 20, 30, 40, 50 | -90.7 + 10log10(NRB/106) | TDD |
|  | 30 | 20, 30, 40, 50, 60, 70, 80, 90, 100 | -90.8 + 10log10(NRB/51) |  |
|  | 60 | 20, 30, 40, 50, 60, 70, 80, 90, 100 | -91.1 + 10log10(NRB/24) |  |
| n10911 | 15 | 5,10,15,20,25,30,40,50 | -100 + 10log10(NRB/25) | FDD |
|  | 30 | 10,15,20,25,30,40,50 | -97.1 + 10log10(NRB/24) |  |
| NOTE 1: Four Rx antenna ports shall be the baseline for this operating band except for two Rx vehicular UE and two Rx antenna port XR UEs indicating UE capability [2Rx XR]. Four Rx antenna ports for RedCap UE is not supported for this operating band. NOTE 2: The transmitter shall be set to PUMAX as defined in clause 6.2.4. NOTE 3: Void NOTE 4: The requirement is modified by -0.5 dB when the assigned UE channel bandwidth is confined within 3300 - 3800 MHz. NOTE 5: For these bandwidths, the minimum requirements are restricted to operation when carrier is configured as a downlink carrier part of CA configuration. NOTE 6: Void NOTE 7: For SDL bands, the reference sensitivity requirements shall be verified by inter-band CA combinations with SDL band, which are supported by UE. NOTE 8: The REFSENS value is rounded to the nearest number down to one decimal point. “NRB” in REFSENS formula is the maximum transmission bandwidth configuration as defined in Table 5.3.2-1. NOTE 9: Void. NOTE 10: A UE may implement two RX antenna ports for band n104 when conditions are met. The exact conditions are FFS. NOTE 11: Applies for DL channels for which channels edges are > 15 MHz away from 2xFUL at 15 kHz SCS and > 30 MHz away from 2xFUL at 30 kHz SCS. In case of UL second harmonic direct hit, the value is modified to -71.9 dBm for all channel bandwidths. |  |  |  |  |

For power class 2 UEs, certain degradation of the reference sensitivity in Table 7.3.2-1a is allowed. The maximum amount of degradation is specified in Table 7.3.2-1c, and in Table 7.3.2-1d for a UE that indicates txDiversity-r16 or txDiversity2Tx-r18 [15].

Table 7.3.2-1c Reference Sensitivity Degradation from PC3 to PC2 for FDD bands for UE not supporting Tx Diversity

| Operating Band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 35 MHz (dB) | 40 MHz (dB) | 45 MHz (dB) | 50 MHz (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n1 | 0 | 0 | 0 | 0 | 0 | 0 | - | 0 | 0 | 0 |
| n2 | 0.8 | 0.9 | 1.1 | 1.2 | 1.3 | 2.7 | 2.8 | 3.5 |  |  |
| n3 | 0.5 | 0.5 | 0.5 | 0.5 | 0.6 | 0.8 | 1.1 | 1.5 | 2.3 | 2.8 |
| n8 | 0.5 | 0.7 | 0.8 | 2.3 | 2.8 | 3.2 | 3.1 |  |  |  |
| n14 | 0.6 | 0.8 |  |  |  |  |  |  |  |  |
| n25 | 0.8 | 0.8 | 0.9 | 1.1 | 1.3 | 2.7 | 2.8 | 3.5 | 3.7 |  |
| n66 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |  |
| n71 | 0.5 | 0.9 | 0.9 | 2.2 | 2.42 2.53 | 2.52 2.43 | 2.92 3.13 |  |  |  |
| n70 | 0 | 0 | 0 | 0 | 0 |  |  |  |  |  |
| NOTE 1: The transmitter shall be set to PUMAX as defined in clause 6.2.4 NOTE 2: Applies to UEs that support a maximum uplink BW of 20 MHz in this band. NOTE 3: Applies to UEs that support optional symmetric UL/DL for this BW. |  |  |  |  |  |  |  |  |  |  |

<Unchanged part is omitted>

<Next Change>

7.3A.4	Reference sensitivity exceptions due to UL harmonic interference for CA

<Unchanged part is omitted>

For a PC2 aggressor NR UL band for NR DL CA FR1, the maximum amount of degradation is specified in Table 7.3A.4-2a, and Table 7.3A.4-2b for a UE that indicates txDiversity-r16 or txDiversity2Tx-r18 [15].

Table 7.3A.4-2a: Reference sensitivity exceptions and uplink/downlink configurations due to UL harmonic from a PC2 aggressor NR UL band for NR DL CA FR1 for UE not supporting Tx Diversity

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n3 | n78 | 5 | 15 | 25 (RBstart=0) | 10 | 27.1 | NOTE 2 | UL2/DL1 direct-hit |
| n3 | n78 | 10 | 15 | 50 (RBstart=0) | 100 | 16.6 | NOTE 2 | UL2/DL1 direct-hit |
| NOTE 1: Void NOTE 2: The requirements should be verified for UL NR-ARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. |  |  |  |  |  |  |  |  |

Table 7.3A.4-2b: Reference sensitivity exceptions and uplink/downlink configurations due to UL harmonic from a PC2 aggressor NR UL band for NR DL CA FR1 for UE supporting Tx Diversity

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n3 | n78 | 5 | 15 | 25 (RBstart=0) | 10 | 32.3 | NOTE 2 | UL2/DL1 direct-hit |
| NOTE 1: Void NOTE 2: The requirements should be verified for UL NR-ARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band.. |  |  |  |  |  |  |  |  |

<Unchanged part is omitted>

<End of Change>